HDINFO Copyright 1993 - James Thorpe

OVERVIEW:

HDINFO - provides information about IDE hard disk

drives.

It identifies the number of Cylinders, Heads, and

Sectors/Cylinder for any IDE hard disk.  The

information is read from the hard drive directly,

and therefore it does not matter what the current

CMOS parameters are.  If the current CMOS settings

do not give 100% utilisation of drive space, then

optimal CMOS parameters are offered which will.

Ideally, you should run this program when first

installing a hard disk to find the optimal

parameters for it BEFORE the hard drive is

partitioned and formatted.  Choose BIOS hard disk

drive type 47, enter the appropriate figures, then

partition and format the drive.

PLEASE NOTE: If you are already using the hard drive

do NOT change the CMOS setting if they are not

optimal for the disk.  You will be unable to access

the drive properly if the CMOS parameters are

changed from those with which the drive was

partitioned and formatted.

SOURCE CODE:

The full source code for the program has been

distributed with the EXE file. There are several

things to note:

1.	Borland C++ is necessary to compile the code in

HDWIN.CPP, which uses Borland's Turbo Vision

user interface.  The code in HDINFO.CPP (where

the real work is done) is not compiler specific

and should compile with little, if any,

modification using most C++ compilers.

2.	I run the Borland compiler from my D: drive.

If you use C: you will need to change the

directories settings for the Project file

before compiling.

3.	I have included all the source code to

demonstrate Turbo Vision programming and direct

hardware access in C/C++, in other words, for

educational purposes.  You are free to utilise

my code in your own programs PROVIDED that you

do not make any profit from it.  If you wish to

use my code in a commercial application, then

let me know, and we can come to some

arrangement - or better yet, offer me a job.

DISCLAIMER:

I dislike having to do this but ...

Every reasonable effort has been made to ensure that

this program will function without damage to the

hard disk or its contents.  It is supplied free of

charge, as a useful public utility, and is run at

risk of the end-user.  The author makes no

guarantees and will accept no liability for damage

or loss caused by the use of this program.

CONTACT:

Should you have any questions regarding this

program, or just want a general chat, I can be

contacted in the following ways.

Compuserve: 	ID# 100035,101

Mail:		James Thorpe

4/4 Bradly Ave

Kirribilli

Sydney

Australia	2061

Phone:		61 2 957-4450

Hope you find HDINFO useful.

James Thorpe.