Note to SIO2PC users:

KEYER.OBJ is a program for ham radio which took up about 2 years

of my programming time before I started SIO2PC. I'm throwing it in

just as a curiosity, to fill in disk space. If you have friends who

are ham operators, they may be interested in it. If not, you may

want to erase KEYER2.DOC (this file) and KEYER.DOC (short form of

instructions for KEYER.OBJ) from the disk and KEYER.OBJ from the

ATARI.ATR disk image. If you want to play with it, plug a joystick

into stick #2, run KEYER.OBJ and send some Morse code.

Instructions: The WA5BDU Keyer

OPTION: Enter command mode.

SELECT: Keyboard to MEMO/TRANSMIT window toggle.

C O M M A N D S

[OPTION] plus ...

HS: Help Screen

HP: Help text dump to printer

SP: Speed

RT: Ratio, doT

RH: Ratio, dasH

SW: Spacing, Word (in 1/2 dot length units)

SC: Spacing, Char (in 1/2 dot length units)

CS: Console Speaker toggle (400/800 only)

VO: Video Off/On toggle

KD: Key Down (tune)

OA: On Air/Off Air toggle

IK: Inverse Keying (Invert output logic)

TO: Sidetone frequency

OM: Open message (Create or Edit; ESC to exit)

BE: Buffer Enable (Start/Stop sending)

TAB key: Same as above

BC: Buffer Clear (SHIFT-CLEAR is the same thing)

MA: Thru MZ: Send message A-Z immediately.

Inverse A-Z: Put message A-Z in buffer to send

PL: Parameter Load (Set up keyer the way you like it)

PS: Save current parameters to file

NOTE: File AUTOPARA.KEY will auto-load

TL: Textfile Load (Load message file)

TS: Save messages to file

NOTE: File AUTOTEXT.KEY will auto-load.

Messages can call each other (embed inverse A-Z in message)

A message can call itself.  Hit any key to stop the infinite

loop.

Commands may be embedded within messages, "*" signals start

of command.

INTERFACING: Stick 2: Pin 1: Output, Pin 2: Command button

(optional), Pin 3: Dash paddle contact, Pin 4: Dot contact,

Pin 8: Common

Special codes from paddle: .-... (AS) enters COMMAND mode

(same as OPTION), .----- (EOM) exits message edit mode (same

as ESC key).

WA5BDU, 300 S. VANCOUVER, RUSSELLVILLE, AR 72801