SPECTRUM TO ST FILE CONVERTER

All those Speccy games you've got lying about are useless if you can't get them

off the damn tapes, aren't they? Well plug your tape recorder into the parallel

port of your ST, and you can load, complete with stripey screen, any normal

Basic, Code or Data file you want! It can't (yet) handle turbo loads or

headerless files, but we're working on it...

The connection between the tape player and the ST is as follows:

First you need a 3.5mm jack plug - you can cut the end off a Speccy EAR lead,

that'll do nicely. Now solder the wire leading to the middle of the plug to pin

8 of a 25-pin D plug. Solder the wire leading to the outside of the plug to pin

20 and you're all set. Now run SPECTOST.PRG and connect the tape recorder using

the magic widget you've just put together, and play your tape!

The core code is a heavily modified version of a SCREEN$ converter I got from

ST User a long long time ago, but now it just ports any file, not just screens,

complete with a Speccy Emulator compatible header.

Have fun...