Return-Path: ralphey@zikzak.apana.org.au

Received: from ralphey@zikzak.apana.org.au by smug.student.adelaide.edu.au (5.65/AndrewR-940303) id AA03609; Sun, 2 Oct 94 13:56:10 +0930

Received: (from ralphey@localhost) by zikzak.apana.org.au (8.6.9/8.6.9) id EAA28274 for hilander@smug.student.adelaide.edu.au; Sun, 2 Oct 1994 04:28:33 GMT

From: Russell Alphey <ralphey@zikzak.apana.org.au>

Message-Id: <199410020428.EAA28274@zikzak.apana.org.au>

Subject: 64net man

To: hilander

Date: Sun, 2 Oct 1994 14:28:32 +1000 (EST)

X-Mailer: ELM [version 2.4 PL23]

Mime-Version: 1.0

Content-Type: text/plain; charset=US-ASCII

Content-Transfer-Encoding: 7bit

Content-Length: 56908

Hi Paul,

Here's the 1.68 version of the manual, still woefully incomplete, but

getting there.  Any comments greatly appreciated, as always.

R.

------cut here--------

*** This is the 941001 version of the manual, and refers to v1.68 of 64NET.

Questions / notes to Paul requiring answer/clarification:

Byte 002F in the .N64 file hasn't been defined/allocated???  (Only been

waiting 3 months for an answer on this :)

Devices A: - Z: default to 64NET partitions 1 - 26...do they also default

to partition names "A" - "Z"?  IIf not, can they?

64NET Version 1.68

(C) Copyright Paul Gardner-Stephen 1992-1994

Manual Version 1.68

(C) Copyright Paul Gardner-Stephen and Russell Alphey 1994

Note that this manual, like the software it describes, is still

evolving, and therefore may not accurately describe the version of

the 64NET software you are using/testing.

Whilst the manual author is trying to keep up to date, and welcomes

any corrections or suggestions, he can only say that if you get

into difficulties due to this manual...

TOUGH!

Note also that this manual is going to be thoroughly revised and

reordered soon...

Copyright Notice:

(C) Copyright Paul Gardner-Stephen 1992-1994

The 64NET program software, cable, and manual are copyrighted,

and all rights are reserved by Paul Gardner-Stephen.  This distribution

of 64NET is intended for use on one fileserving IBM compatible computer,

and either one (1), or a maximum of four (4) Commodore 64 compatible

computers at any one time, subject to licence conditions.  The original

purchaser may make one backup copy of the software for the IBM compatible,

and may copy the 64NET wedge file for the Commodore 64 onto as many of

the same original purchasers disks or data-cassettes, with a maximum of

four (4) iterations of the wedge running at any one time, again subject

to licence conditions.  Lawful users of this product are hereby

licenced only to LOAD and EXECUTE the programs included in the 64NET

distribution, and to READ, EDIT, & WRITE the configuration files of the

same, where explicitly mentioned in this manual.  Duplication, copying,

selling, or otherwise distributing this product is a violation of the

law, and a breach of copyright, and the software license of this product.

It is also an offense to duplicate the 64NET key file provided with

registered copies.  Further, you may not modify 64NET to bypass the

checking for a valid key file, nor may you use key files other than

those provided by Paul Gardner-Stephen, or his authorised nominees.

This manual is also copyrighted, and all rights reserved.  It may not be

photocopied, reproduced, translated or reduced to any electronic medium

or machine readable form without prior consent, in writing, from Paul

Gardner-Stephen or his authorised nominees.

IBM is a registered trademark of International Business Machines Inc.

CBM, Commodore 64, 1541, 1571, and 1581 are registered trademarks of

Commodore Business Machines Inc.

JiffyDOS is a registered trademark of Creative Micro Designs.

Warranty:

This distribution is provided as is without warranty of any kind,

either expressed or implied, including, but limited to, the implied

warranties of merchantability and fitness for a particular use, except

for the use of occupying disk space on the supplied disks.

You, the buyer, assume all risks for quality and performance of this

product, and for any damage, including loss of data, which may occur as

a direct or indirect consequence of using this distribution of 64NET.

In no event will Paul Gardner-Stephen or his authorised nominees

be responsible for any damages, including loss of profits or other

incidental or consequential damages arising out of the use of, or

inability to use, 64NET, even if Paul Gardner-Stephen or an authorised

dealer of 64NET has been advised of the possibility of such damages, or

for any claim by any other party.

In some regions the exclusion of implied warranties is forbidden by

law, in which case the above exclusion may not apply to you.  Should

any part of this warranty be shown invalid or unenforceable, then the

validity and enforceability of the remainder of this warranty shall not

be affected thereby.

Table of Contents:

Copyright notice

Warranty

1. Introduction

2. Installation of 64NET

- Adding registration details

3. Starting 64NET

4. Features of 64NET

5. 64NET Functions

Basic

Advanced

6. 64NET Utilities

C64

PC

7. User Customisation of 64NET

8. 64NET Limitations

9. 64NET Trouble-shooting Hints & Tips

10. Programming with 64NET.

Appendices

A. Compatibility List

B. 64NET Support

C. The Future of 64NET

D. Constructing the 64NET cable.

E. Differences between the Public Domain and Commercial releases

of 64NET.

F. 64NET File Format.

G. 64NET Error and Status Messages.

H. 64NET Reserved Filenames and their Functions.

I. Files provided on the 64NET system disk.

J. Installing 64NET permanently in your Commodore 64.

K. Glossary.

L. 64NET Registration Form.

M. Recognised PC filetypes.

N. Multi-lingual operation.

1. Introduction:

The Commodore 64 is, without question, the finest home computer

ever released.  Some of the features which made it the leader include high

quality audio output, ease of use, the ability to be able to be

plugged into a standard television set, and a plethora of arcade

standard games available. Compared to other computers with

similar capabilities the 64 was (and still is!) also great value for money.

However, the major problem with it, which was often pointed out by

its detractors, was the slow speed of its mass storage

devices such as the datasette and the 1541 disk drive.  In more recent

times such devices as the 1571 and 1581 disk drives from Commodore, and

various high capacity floppy and hard drives from third party suppliers

have improved this situation markedly, although not cheaply.

Commodore 64 owners have long looked at the cheap storage available

to PC owners and bemoaned their inability

to take advantage of this, or their inability to use the storage on

their home PC for access by their 64.

NOT ANY MORE!

Paul Gardner-Stephen is proud to announce 64NET(c), the cheap,

complete system which allows Commodore 64 owners to take advantage of

the cheap mass storage available on PCs.  With the price and

availability of low-end PCs, 64NET is the cheapest mass storage for the

Commodore 64 available today.  64NET even supports up to 4 Commodore 64s

from the one PC simultaneously (if so licenced)!

64NET requires no special skills from its users, nor any

programming.  In fact, if you purchased 64NET with its interface cable

you can be accessing your PC disks from your 64 immediately after loading

and running the 64NET program, ie. a matter of minutes!

With 64NET you can access up to 65535 logical drives on your

PC server from your Commodore 64 using standard Commodore commands.  There

is also no limit to the size of the PC drives accessable to the

Commodore 64, subject to DOS limitations,

due to the way 64NET interfaces the Commodore 64 to the PC.

64NET also supports the command structures of other disk systems

popular on the Commodore 64, eg. partition and subdirectory access, and

also allows the user to chose the device number which is the 64NET disk

drive.

Whilst 64NET was developed to allow Commodore 64 owners access to

the hard drives of their PCs, it is equally happy accessing floppy

disk drives, CD-ROM drives, and network drives.  In fact, 64NET will

cheerfully access any device that presents itself as having a DOS file

system!

2. Installation of 64NET:

64NET is designed to be as simple as possible to install, and

setup.  Whilst none of our testers had any difficulty performing the

installation, we advise you to read the installation instructions

carefully, and to seek advice where you feel uncertain as to anything

contained herein.  As pointed out earlier, we take no

responsibility for any damage caused due to a faulty installation, even

where such damage is caused by errors or omissions in these instructions.

Pre-install Checklist:

Before attempting to install 64NET, make sure the 64NET distribution

kit you received from your supplier contains all of the following

items.

1. A double sided disk, with the Commodore 64 64NET wedge programs

on one side,

and the IBM PC 64NET server software on the other.

2. This manual.

3. Either the patented 64NET cable, if you purchased it as part of your

kit, or a detailed cable construction sheet if you did not.  Note that

the construction details are also duplicated in appendix D of this manual.

If you did not purchase the 64NET cable then you will have to construct one

before you can use 64NET.

Hardware Installation:

1) Turn off both the Commodore 64 and IBM compatible.  Failure to do

this before plugging in the 64NET cable could result in the damage

of your computers due to shorting.  Again we stress that no

responsibility will be accepted due to your failure to follow this

instruction.

2) Connect your 64NET cable's DB25 connector to one of the parallel

printer ports of the IBM compatible computer.  It does not matter which

parallel port you use if you have more than one, as 64NET will

figure out which is being used!

3) Connect the other end of the same cable to the user port of the

Commodore 64 (or compatible).  Be sure to connect it the correct

way up as the user port connector unfortunately is symmetric,

therefore making it possible to insert the plug upside down.  The

simplest way of inserting the cable correctly is to make sure that

the plug goes in with the cabled pins to the bottom, assuming that this

component of the plug is visible.  If the plug is protected by a shell,

then insert it with the labelled side up (this should mean that the

cable points away from the expansion (cartridge) port, if the shells

are the same as those obtained by the 64NET test team).

4) Repeat steps 2 and 3 for each Commodore 64 computer you are connecting

to your 64NET server computer.

5) Turn all computers back on.

Congratulations!  You have successfully installed the hardware

component of 64NET.

Software Installation:

Before starting the software installatio of 64NET, make sure that you

have made a backup of your 64NET software disk(s).  Instructions on how

to do this are not the province of this manual.

1) Create a subdirectory on the hard disk of your PC, from the root

directory

with the name 64NET.  This can be accomplished with the following

commmands in DOS (assuming installation to the C: drive):

C:

MD \64NET

2) Copy the contents of the IBM compatible distribution diskette into

this subdirectory.  This can be accomplished with the following

commands in DOS (assuming installation from the A: drive to the C:

drive):

C:

CD \64NET

COPY A:*.*

This completes the installation of the server component of 64NET.  See

Appendix I for a complete list of files which should now be found in

the 64NET directory on the hard disk.

3) Copy the file named "64NET WEDGE" from the Commodore 64 distribution

diskette to your utility disk, or to a blank diskette.  This will

become your 64NET work (startup) disk.

4) Repeat 3) for each Commodore 64 you will concurrently be using with

64NET, subject to the licence you obtained when you purchased 64NET.

5) Place your original distribution diskettes into a nice, safe, cool,

dry place.

6) Congratulations!  You have completed the 64NET software installation.

You are now ready to progress to using 64NET!

Adding registration details.

If you have purchased a registered copy of 64NET initially, then you

need not read this section any further, as the normal installation

process will automatically detect this.

If, however, you have decided to register 64NET after having used the

public domain version, rest assured.  Upgrading your version is a

simple task, and does not overwrite your current configuration.

You will receive a file from your distributor called 64NET.KEY - place

this in your 64NET directory in place of any file of that name which

may be there.

That's all there is to it!

Note that this key file is registered to you, and you alone.  Do not

copy or otherwise distribute it.  Each key file is uniquely identifiable

so people using unauthrosied copies will be prosecuted, as will those

providing the key file.

3. Starting 64NET:

1) Move to your 64NET directory on the IBM compatible, eg CD \64NET

2) Type 64NET and press the ENTER key.

Note that the PC software MUST be run before the 64NET C64 wedge

software to

enable the server to synchronise with the clients.  Failure to do this

will not result in any damage to either system, but it will cause

64NET to fail to work, resulting in undoubted disappointment to the

user.

This is not true of the 64NET C64 kernel replacement software, which

synchronises with the server on each access of the server.

You should see a screen similar to that in figure 1, below.

.....Insert screen shot here.....

Figure 1, Typical 64NET server initial screen.

Note that the rapidly "flashing" number on the screen is normal, and

shows that 64NET is polling each of the possible parallel ports to

see if a 64NET command has been received.  This occurs whether or

not your system contains multiple parallel ports or not.

The messages at the bottom of the screen are explained later, eg.

F1, F2.

3) At this point you have 2 choices as to the type of installation used

on your C64.  The choice you make generally depends on whether or not

you wish to use software or hardware found to be incompatible with

64NET.  Examples of this are listed in Appendix A.

Normally it is expected that you will want to use the 64NET kernel

replacement software which copies the kernel ROM to the underlying

RAM, then patches that RAM with the 64NET access routines.  The

method of doing this is described in part A below.  If you

have software or hardware which expects to have free access to that

RAM for its own purposes, then the 64NET kernel software will be

corrupted by this, and undoubtedly fail!  A possible hardware

solution to this is described in Appendix J.

If you chose, or are forced, not to use the 64NET kernel replacement

software, then you will need to install the 64NET wedge software, as

shown in part B, below.  Note that you have several choices as to the

location in memory that the wedge is located, to provide maximum

flexibility, and hence compatibility, with other software.

Common to either choice of C64 64NET installation is the

default device number of the 64NET server.  64NET currently installs

its server as device 7.  This is done to avoid any possible device

number conflicts with hardware already attached to your system.

If for any reason you wish to alter the device number the 64NET

server responds to, then all you need do is alter the value stored

in location 167 ($A7) to the device number you wish.  This means

that 64NET will allow your PC server disk drives to be accessed as

anything from device 0 right through to device 255!  Of course, not all

software wil accept such outlandish numbers, and certain numbers

will cause problems to the operating system.  That's where the

flexibility of 64NET shines, in allowing you to choose device

numbers.

Part A. Installing the 64NET kernel replacement software.

LOAD and RUN the "64NET KERNEL" file on each Commodore 64 which

is to have access to the IBM compatible's drives.

Part B. Installing the 64NET wedge software.

LOAD and RUN the "64NET WEDGE" file on each Commodore 64 which

is to have access to the IBM compatible's drives.  You will be asked

where in memory you would like the wedge located.  If unsure, or

you have no reason to do otherwise, chose the default location

shown by pressing the RETURN key once.

If you do not wish to accept the default location, then enter your

preferred location as either a decimal, or hexidecimal (preceded by

a "$") number.  The 64NET wedge cannot be loaded above $9800, except

in the range $C000-$C800, due to its size, and the lack of "normal"

RAM which the operating system can easily see.

Please be aware that an inappropriate choice of

location will cause 64NET to fail, and quite likely cause a machine

lockup.

If the location chosen does not cause system conflicts, each

Commodore 64 will attempt to make connection with the PC

server, and then attempt to "auto boot" the "STARTUP n" file from the

server, where n is the parallel port number that the particular

Commodore 64 is connected to, eg "STARTUP 1" for parallel port 1.

If that file doesn't exist, then the

"auto boot" will attempt to load and run a file called

"STARTUP SEQUENCE", if it exists.  The startup files can be any

C64 file which can be loaded and run.  It may contain commands

to set 64NET preferences, load your favourite application, or even

run a game!  Note that this is a perfect spot to alter the

default device number used by 64NET.

Note that both the filenames mentioned above are illegal PC filenames.

They are actually the "real" names of possible files stored under

64NET, which allows full 16 character C64 compatible filenames.

Press the C= key when you RUN the 64NET wedge program to override this

boot feature.

Irrespective of which installation method you used, success will be

shown by your C64 displaying a black border, dark blue background, and

white text.

If the kernel version was chosen you will also notice that a slightly

different cold start message is displayed on the C64's screen.

You should also notice that the status field on the server PC has

altered for one of the displayed ports.  This indicated which parallel

port address you are connected to, NOT which actual parallel port you

are physically attached to.  The reason that what you see is not what

you may have been expecting is that not all PCs map their parallel ports

to the "correct" locations.

64NET checks the following addresses as parallel ports 1 through 4.

Port    Address

1       $378

2       $3BC

3       $278

4       $2BC

Congratulations, you now have 64NET running!  All references to

device 7 (or whatever device you have set the 64NET server to) are now

redirected to the logged drive and directory

on the IBM compatible, and this is also now the default device, just as

the datasette is the default device on a standard C64, and device 8 is

on an SX64 or C65.  All disk drives on the Commodore 64 are still

accessible, unless you have chosen to override them with the 64NET

server.

The RS-232 port is obviously now inaccessable, which is considered a

very minor inconvenience compared to the advantages provided by 64NET!  A

passthrough version of the 64NET cable and software is being considered to

overcome this limitation.

4. Features of 64NET:

Now that you have 64NET physically installed on your system, and have

started the driver software, it is time to cover the many details and

features of 64NET.

As this version (1.68) is a BETA release, some of the facilities may

not always function correctly.  When the public release is completed

you will be sent your update disk (provided your copy of 64NET is

properly registered, ie. you send in your registration form).

64NET attempts to logically emulate the 1541 file system as closely as

possible, thus providing more than just a file transfer facility.  Note

that this does NOT mean emulate a 1541 disk drive, eg. access to 1541

RAM.  All CBM file types are supported, with the exception of REL files.

Sector read and write functions are not supported as the emulated file

system uses common DOS files to store each program or file, to provide

maximum flexability.  This method means that there is no partitioning,

"virtual disks", or other means to emulate the 1541.  The end result is

that subdirectories (8 character names only), and full 16 character file

names are fully supported, and each drive may have well in excess of 664

blocks available, often around 160000 blocks for a 40MB drive (this

author intends to connect his C64 to a PC on a network just to see the

truly awesome site of "7812500 block free" [that's 2 Gigabytes!]) .

On top of the standard 1541 emulation (at KERNAL level), there are

also many other facilities, including 5KB/sec fast loading (up to 12KB/sec

on a Commodore 128, in C64 mode, with burst loading), off board

machine code monitor,

PC to C64 file transfers with exceptional ease, networking facilities

including passwording and differing access levels, improved loading and

saving displays (percentage completed, address ranging or normal).

Further, due to 64NET's disk drive emulation, there is no limit

to the number of files that can be stored on a "disk", subject of course

to the physical size of the actual disk!

64NET allows you to access any physical disk drive on the PC including

floppy drives, hard disks, removable drives, and even CD-ROMs!  Drives

A: to Z: are assigned by default to partitions 1 to 26 under 64NET,

although this is easily reconfigurable if desired.  In addition

to this, any PC subdirectory can also be assigned a logical partition

name or number, making it simple to access that directory directly.

5. 64NET Functions:

Basic Functions

Almost all LOAD and SAVE commands which function on a 1541 will

work correctly, unaltered under 64NET.  64NET however increases the scope

of these commands, with the inclusion of subdirectory support and DOS

file exchange.

i) General differences from a standard C64.

Whereas on a 1541 loading a directory has only a few options, in

64NET there is much more flexibility and choice.  All 1541 directory loading

options are supported.  The default directory

load is a directory output almost identical to a 1541, listing all PRG,

SEQ, USR and REL files (ie. all "CBM" files on the emulated drive), but

adding the DIR files, which are the subdirectories of the logged drive

and directory.  Other filetypes which might be seen include IBM, which

are files that are not definately known to be C64 files of some type.

3 "/64NET         "  64NET

0    "CODE":            DIR

0    "TEMP":            DIR

0    "UTILS":           DIR

0    "GAMES":           DIR

0    "DEMOS":           DIR

0    "EDUCAT":          DIR

0    "DRIVERS":         DIR

0    "BACKUPS":         DIR

0    "PROGRAMS":        DIR

2    "64NET COPIER":    DIR

5    "GEOS BOOT":       PRG

7    "MENU":            SEQ

1    "STARTUP 1":       PRG

2    "STARTUP 2":       PRG

50   "TURBO ASSEMBLER": PRG

0    "TACODE":          DIR

1    "TEST.SRC":        SEQ

0    "TEST.SRC":       *SEQ

67    BLOCKS LISTED.

30107 BLOCKS FREE.

Figure 2, Example directory listing from a 64NET 'LOAD"$ [RETURN]'

There are several differences immediately apparent to that of a

conventional directory listing derived from a 1541.  Examining the

header line you see something like the following, which can easily be

interpreted.

3 "/64NET         "  64NET

^  ^                 ^

|  |                  -- Disk "ID", always 64NET.

|   -- Current logged directory on PC server.

-- Current logged disk drive on PC server.

You may notice that there is no disk header, instead the current path

on the server (in unix style "slashes" forward instead of backward, ie

"/" not "\") is displayed.  Also note the drive unit number (which is

always zero on a 1541). The three in the example corresponds to drive C:

on the PC (0 is reserved for 1541 compatibility).

You might also have noticed that "splat" files (marked with a "*") are

also supported under 64NET!

Here are some things to try:

LOAD"$:/*,*

(note you dont need a ,7 or end quote if you dont want to :)

This will display ALL files of all types (IBM & CBM) in the root

directory of the logged 64NET drive.

LOAD"$:/*,PC

This will display all files in the root directory of the logged 64NET

drive which are not known C64 files.

LOAD"HELP

Get the online help system, self explanatory from there.

The help service is VERY FAR FROM COMPLETE, there are only about

6 man pages in it.

Advanced Functions:

Disabling and re-enabling the 64NET WEDGE.

In order to use certain features of your Commodore 64 you may find it

necessary to disable 64NET temporarily.

To disable the 64NET wedge, just press and hold the RUN/STOP key, and

smartly

tap the RESTORE key.  Your screen should return to the standard power-up

message and colours.

To re-enable the 64NET wedge, type SYS 53242 (if you used the default

location to install 64NET), and press RETURN.  64NET will not

display any messages to indicate that it is active again at this point.

If you believe that 64NET is not behaving correctly at this time, then

entering SYS 53245 (again, assuming the default location) will go through

the full boot sequence again.

Built in LOAD commands.

These give access to the built-in functions of the 64NET server.

LOAD"NETWORK STATUS

Displays the status of 64NET and the C64 you are using.  You

should see something similar to figure X below.

---------------------------------------

64NET V1.68N BETA

4 USER RELEASE.

USER NUMBER: 2

USER NAME:ANONYMOUS

PRIVILEGE LEVEL:HIGH

---------------------------------------

Figure X, result of a LOAD NETWORK STATUS.

LOAD"USER STATUS

This command requires MASTER level access, otherwise a PRIVILEGE

ERROR is returned by the server.  If you have appropriate access then

you will see a display like that below in figure Y.

Figure Y, result of a LOAD USER STATUS.

LOAD"DOS STATUS

RJA     Does nothing on my system, apart from thrashing the hard disk for

a bit.  Then  "09,PRIVELIGE ERROR,03,03" on the server.

LOAD"MON

This is an off board disassember, taking up no additional memory on

the Commodore 64!

@"command" sends "command" to the default disk drive, eg. @$ will

get a directory of the drive.

Note: try @fruit - who said utilities have to be dull?  :)

X or Q will exit from it.

A to assemble,

D to disassemble,

M to view memory in block format,

I to view memory as ascii,

F to fill,

` H to search memory (if you're really lucky!!)

64NET Disk commands:

"S:filespec"        -    Scratch file(s)

"UJ"                -    "Reset drive"

"RDdirectory"      -    Remove empty subdirectory

"MDdirectory"      -    Create a subdirectory

"CDdirectory"       -   Change current subdirectory, on logged drive.

"CPpartition"       -   Change current partition

RJA what directory will you be in?  RJA to check

"I"                 -    "initialize drive" => clear dos message to 00,OK

"NET:LOGOFF"        -    Log out of 64NET as a "network client"

"NET:DEFUSER:class" -    Set security class of non-logged in users

(one of NONE,VISITOR,LOW,HIGH,MASTER)

"NET:FILECLASS:class:filename" - Set security on a file to one of the classes

listed above.

"NET:CLASS:portno:class"-Set the security of another user (only MASTER may do

this)

"O[%-][%+][$+][$-][D+][D-][B+][B-]" - Change various preferences

%+/%-  - Enable/Disable Percentage display on LOAD & SAVE

$+/$-  - Enable/Disable Address display on LOAD & SAVE

D+/D-  - Enable/Disable "directory loading"

B+/B-  - Enable/Disable Burst Loading

6. 64NET Utilities

64NET comes with a number of useful utility programs to enhance your use

of the system.

For the C64

64NETCOPY provides a fast, simple means of copying files from your first

CBM drive to the PC hard disk's logged directory.  Note that it loads the

CBM program into the normal load area for the program, so programs which

load into the same area as the 64NET WEDGE will not be able to be saved

using 64NETCOPY.

64NET FILE COPIER (v2.00.1)

- T/U tag/untag all.

- "space" to toggle all.

- "return" to tag file.

- C to start copy operation.

Has progress meter!

MON is an off board monitor/assembler.  Details can be found (for now)

in the Advanced usage section.

PC Utilities

Ok, now also included in the PC archive are a few other utilities:

C64DMON a bodgey X-Tree style sort of thing for the virtual file system.

64DIR do a directory showing .N64 files a bit better :)

DEHEADER (?) convert .N64 files to raw 64 format.

CNVRT64 will convert several popular PC C64 emulator file formats to

the N64 format used by 64NET.

The C64DMON has a basic listing exporter for the really keen types.

7. Customisation of 64NET

As mentioned earlier, when the 64NET WEDGE starts it loads a file from

the server named STARTUP n, where n is the number of the parallel port

that the particular Commodore 64 is connected to.  This STARTUP program

loads into the start of BASIC memory ($0801) and is RUN.  It can contain

any Commodore 64 BASIC commands, stored as a Commodore BASIC program.

Some examples of what can be done are:

Change screen colours, load and access alternate character sets,

load and run utility programs, load and run games, or just about

anything else you might like to think of!

Permanent customisation can be performed by editing the 64NET.INI file

in the PC's 64NET directory.

The structure of the INI file is as follows:

[customisation item]

customisation details

more details

[another item]

details about that

etc..

To disable out an option temporarily, without deleting it from the .INI

file, place a "#" character at the start of the line you wish to disable.

The data in the .INI file must strictly follow the order shown below,

although no detail fields need be present.  Any detail field not present

will cause 64NET to use the default value shown between brackets after

that detail field.

Specifically, the items that can be customised are:

[Security]

Details are:

DefaultClass ()

Prompt ()

AllowVisitorLoging ()

[Preferences]

Percentage Display ()

Directory Load ()

Burst Loading ()

Help Path ()

[Partition Links] - allows you to assign PC devices and directories to

a particular 64NET partition name or number.

The default is to assign partitions 1 through 26 to

drives A: through Z:

Details:

partition_name/number,path

eg. 7,C;\64net\games assigns logical partition 7 to the games

directory under the 64net directory on the C: drive.  Therefore,

a CP7 command will take you straight to that directory, no matter

which drive and directory you previously had logged..

Ralphey's Directory,D:\TM\DOWNLOAD\CBM\SECRET is also quite

valid.  Therefore a CPRalphey's Directory command will take you

to the specified directory.

Note that a maximum of 255 partitions can be assigned, although

the partition numbers can be from 1 to 65535.  Partition names,

should you wish to use them instead or as well, can be up to 32000

characters long!  The comma "," character may not be used as part

of a partition name.

8. 64NET Limitations

In order for something as wonderful as 64NET to work, certain things

about the Commodore 64 must be changed, or restricted.  For instance,

64NET must occupy some memory on the Commodore 64, and divert operation

through itself.  This leads some potential problems.

Firstly, by occupying memory there will be certain programs which cause

problems by wanting to use that same area of memory.

Secondly, by diverting operation, programs which rely on precise timing

may be affected.

Finally, programs which wish to install fastloaders or otherwise bypass

normal kernel disk operations will undoubtedly fail, as 64NET is NOT a

1541 disk drive!

As programs are discovered which are 64NET incompatible, they will be

noted.  Attempts will then be made to either improve 64NET to remove the

incompatibility, or the offending program's author will be contacted if

possible to update the program.

9. 64NET Trouble-shooting Hints & Tips

Q.  It hangs WHENEVER I try to use it!

A.  Run the software on the PC FIRST, then after you see the 64NET server

screen (see page XX) you can run the C64 wedge.

Q.  64NET hangs on me sometimes.

A.  There's a few things you can do about this:

* If the hanging occurs in the MEMORY MONITOR or HELP SYSTEM, try taking

your PC out of turbo during this time (i dont know why, but it helps!!).

* When in the MEMORY MONITOR or HELP system, dont type too fast (related

to suggestion above, another unknown).

* Use a cable SHIELDED cable, as short as possible, as i believe that

much of the problem it to do with "line talk".  "Short" is no more than

say about 3 metres (although working cables of 25 metres have been tested

these cables experience more crashes and hanging than users of shorter

cables).  The PC parallel port expects to transmit over a distance of

no more than 5 metres, so longer cables may cause your 64NET system to

experience timing problems.

Q.  When I load a file it loads in TWO BYTES UP from where it should.

A.  When the file was converted to .N64 format, the load addr header was not

processed correctly, try the conversion again.

Q.  Some programs "hang" part way through loading, or don't load all parts

of a multipart program.

A.  Your program loads into the area occupied by the 64NET WEDGE (default

$C800-$CFFF).  Try loading the wedge into an alternate area.  This may

take some trial and error until you find

a memory location "safe" from your program.

Another solution is to purchase 64NET-PRO, the plug-in replacement

KERNEL ROM for the Commodore 64, which eliminates most known loading

compatibility problems!  Contact your distributor for further details.

Q.  My program loads okay, but hangs when I run it.

A.  Your program is being confused by the altered vectors that 64NET uses.

The solution is that after the load finishes, reset the vectors by

pressing and holding the RUN/STOP key, then smartly tap the RESTORE key

until you see the standard "blue on blue" power up screen.  Your

program should work fine now.  To reactivate 64NET after your program

finishes, enter SYS 53242, if loaded into the default location.

10. Programming with 64NET.

64NET Extended function set

---------------------------------

Dated:  25 May 1994

Purpose:  To allow FAST usage of the features of 64NET for applications

designed with 64NET in mind.

Calling Convention:

SendChar "X"

SendChar "Function Id"

{

Parameters

}

JMP 64NET_Serve_Loop

Check A for a nonzero value (or ST)

Functions Available:

"R" - Read a block of memory from a file (at the current position)

Parameters:  low byte-high byte of start address and

low byte-high byte of end address.

"W" - Write a block of memory to a file (at the current position)

Parameters:  low byte-high byte of start address and

low byte-high byte of end address.

"P" - Set current position in file

Parameters:  "I" if setting input file

"O" if setting output file

32bit (least significant byte first) value for new

file position.

Appendices

A. Compatibility List

B. 64NET Support

C. The Future of 64NET

D. Constructing the 64NET cable.

E. Differences between the Public Domain and Commercial releases

of 64NET.

F. 64NET File Format.

G. 64NET Error and Status Messages.

H. 64NET Reserved Filenames and their Functions.

I. Files provided on the 64NET system disk.

J. Installing 64NET permanently in your Commodore 64.

K. Glossary.

L. 64NET Registration Form.

M. Recognised PC filetypes.

N. Multi-lingual operation.

A. Compatibility List

Does 64NET work with...

1.  Jiffy DOS

Partly.  the "Ctrl-D" function fails, but most of the loading

procedures work.  Negotiations are underway with CMD to hopefully

enter into joint development.

2.  Action Replay Cartridge

Also partly.  Most of the loading procedures work, but the

directory function only accesses the "real" device 8.

3.  GEOS

Not at this time.  A team of German programmers are working

on a patch for GEOS to make it 64NET compatible.

4.  C-128 & C-65

In C64 mode, yes.  In native modes, not yet.  It is being

worked on.  Note that the C64 wedge source code is available as part

of every 64NET distribution, should you wish to make it compatible

with some device or program of your own.

> 1. Where does the C64 end software load, and what does it get in the way of?

>    What programs do and don't work.

The wedge programme default loads from $c800-$cfff (approximately).

, i am working on

making some replacement "64NET" roms for the C64 that will make it

100% transparent, but with out drive bumping, and with conventional

serial comms also. (possibly at the expense of rs-232 on the user

port coz they cant both be used at the same time (ie switch roms like

an amiga!)

> 2. Does it work on the 128?  If so, see question 1.

No idea! I dont own one.  I would say, however, that the OPEN,CLOSE

wedges would work. LOAD & SAVE would have to be managed through them

(ie goodbye burst loading :( )

> 3. What about CP/M mode on the 128?

Hmm, different CPU!

I'd have to re do the code in Z80, but i cant see why it shouldn't

work, pv i got some info on the file system, and where the wedges

would have to go etc..

B. 64NET Support

The author of 64NET is dedicated to the support of 64NET, and

welcomes your feedback, either in the form of suggestions for

improvements to 64NET, or reports on problems with its use.  Naturally

only registered users can expect any formal response to problem

descriptions.

You should contact the distributor you purchased 64NET from in the

first instance with any questions.  If the distributor cannot answer

your question they will refer it to the author.

Authorised 64NET distributors are:

Australia, East:

Russell Alphey

Australia, Central and West:

Paul Gardner-Stephen

Germany:

Michael Renz

The software author can be contacted in a number of ways -

Voice:	   +61 8 2777479

BBS:	   +61 8 2771361 (answer all questions for instant d/l access)

The BBS carries the latest support files for 64NET, as well

as a "problems" database with solutions.

Internet    pgs@flinders.edu.au

Snailnet:  Paul_Gardner-Stephen@1_Hurst_St.Morphettvile.SA_Australia_5043

(Australia)

The manual author can also be contacted several ways -

Voice:      +61 3 6513057 (BH)

+61 3 4278558 (AH)

Fax:        +61 3 6514073

Internet:   R.Alphey@dce.vic.gov.au (preferred)

ralphey@mirage.aero.rmit.edu.au

Russell.Alphey@p1.f100.n638.z3.fidonet.org

Fido:       Russell Alphey at 3:638/100.1

Snail:      Russell_Alphey@439_Punt_Rd.Richmond.Vic_Australia_3121

C. The Future of 64NET

customise dir separator?

64netpro - ROM?  serial??

128mode

128-cp/m mode

65mode

Pet mode

Amiga version

D. Constructing the 64NET cable.

The 64NET cable requires no special skills to create, except

confidence in your own soldering ability.  A person will rudimentary

soldering skills ought to be able to build a complete, working cable in

under 10 minutes.

Parts required:

1 * CBM User Port female adaptor (mandatory)

1 * DB25 connector, of suitable sex for plugging into your parallel

port - usually male (mandatory)

1 length of 9 wire (minimum) cable [preferably shielded] (mandatory)

note that about 4 metres of cable is advised for maximum

flexibility, but you would should consider your own computer

layout.

solder (mandatory)

1 * CBM User Port adaptor back shell (optional)

1 * DB25 back shell (optional)

The back shells protect the solder joints, by providing a secure

place to grip whilst removing the cable ends from the computers

which reduces the strain on the joints.

The pinout for the cable is:

Userport	PC Printer port

CBM name

-------------------------------

A (GND)		25 (GND)

C (pb0)		10 (an input line)

D (pb1)		11 (an input line)

E (pb2)		12 (an input line)

F (pb3)		5 (data bit 3)

H (pb4)		6 (data bit 4)

J (pb5)		7 (data bit 5)

K (pb6)		8 (data bit 6)

L (pb7)		9 (data bit 7)

It is very simple to make!  Just solder the individual wires to each

adaptor, making sure that each wire is connected to the correct pins

at each end, and that there are no touching wires.

This author was able to make a working cable, first time, on only his

second attempt at soldering.  However, if you do not feel confident

that you will be able to successfully build a 64NET cable, then contact

the distributor from whom you purchased 64NET, and they will provide

a 64NET cable for a nominal fee.

E. Differences between the Public Domain and Commercial releases of 64NET.

Whats in and whats not?

SAVING to the PC is out

LOADING is in (less than about 200 blocks though [as of 1.68])

OPEN is in (for command channel only)

INI parsing is in

BURST-LOADING is out

F. 64NET File Format

64NET v1.50 BETA .N64 File format

---------------------------------

$0000-$0002 -  "C64"

Note: Used to identify file as being a valid 64NET archive

$0003       -  64NET File format version

Note: Current=$01

Current use for version $01 files:

$0004       -  1541 equivalent file type

Valid types are:  $x0=DEL   $x1=SEQ

$x2=PRG   $x3=SEQ

$x4=REL (not implemented)

$x5=DIR   $x6=DIR

Note: Assigning a file as a REL, DIR or IBM type could have

strange side effects and is not reccomended.

$0005-$0006 -  Load Address (for ,8,1 loads) in standard CBM "low byte, high

byte" format.

$0007-$000A -  32bit file length in bytes (NOT blocks).  This is also stored

in low through high byte format.  Therefore the filesize

is actually

($07) + 256 * ($08) + 65536 * ($09) + 16777216 * ($0A) bytes.

It is rather unlikely that any legitimate C64 file will ever

have a value other than 0 (zero) in $0A!

$000B       -  64NET Network security level ($00 = none)

Note: if this value is >MASTER then the file is completely

inaccessible, except through the disk monitor.

$000C-$001E -  Reserved for future enhancement

$001F-$002E -  16 character file name padded with $00's

RJA $002F       -   ???????

$0030-$00FF -  Reserved for future enhancement

$0100       -  Start of actual file body

NOTE:  A known bug is that the first two bytes of the files (its load address)

is NOT RETRIEVABLE through an OPEN and GETCHR combination, resulting in the

possibilty of incompatibilities.  This problem will be solved in 64NET

VERSION 2.

G. 64NET Error and Status Messages.

Conventional 1541 responses are given by 64NET, but these are not

shown here.  Please see your 1541 disk drive manual for a list of

other (standard) status messages which are not listed.

00, OK,00,00                   - OK!

00, WELCOME,00,00              - User logged on with network access

00, GOODBYE,00,00              - User logged off from system

01, FILES SCRATCHED,nn,00

02, NO FORMAT,00,00            - You cannot format a PC drive from the C64

03, NOT 4040,00,00             - The 4040 drive D command (duplicate) is not

supported by 64NET

04, OPTIONS SET,00,00          - Attempt to alter system option succeeded

05, SUBDIRECTORY EXISTS,00,00  - Attempted to create a dir that already exists

06, SUBDIRECTORY CREATED,01,00 - Attempt to create a dir successful

07, SUBDIRECTORY REMOVED,01,00 - Attempt to remove a dir successful

08, DIR NOT FOUND,00,00        - similar to "file not found" for directories

09, PRIVELIGE ERROR,01,00      - Unauthorised access to "system" files

09, PRIVELIGE ERROR,02,00      - Attempt to save with VISITOR access.

09, PRIVELIGE ERROR,03,01      - Unauthorised attempt to use "MON"

09, PRIVELIGE ERROR,03,02      - Unauthorised attempt to use "NETWORK STATUS"

09, PRIVELIGE ERROR,03,03      - Unauthorised attempt to use "USER STATUS"

09, PRIVELIGE ERROR,04,00      - Unauthorised access to network

09, PRIVELIGE ERROR,05,00      - User not allowed access to file

10, SYNTAX ERROR,00,00         - Error in "NET:" disk command

73, 64NET V1.68N BETA 1994,00,00 - Power-up message

H. 64NET Reserved Filenames and their Function.

All of these facilities are accessed by LOADing the filenames listed

below.  Note that if you chose to save files with these names, then

your files are inaccessable from 64NET.  This is to ensure that

the 64NET facilites are always available.  If you do save files

with 64NET reserved names, then you will have to "repair" their names

using the 64DMON program's (R)ename function.

Filename         Function

========         ========

MON            - Offboard memory monitor

HELP           - On-line help system

DIR            - Loads full path and drive information

SYSTEM OPTIONS - Displays (not loads) the current preferences

NETWORK STATUS - Display info about yourself (not load)

USER STATUS    - Display info about all users (MASTER access required)

DOS STATUS     - LOAD the current DOS message (this does not clear

the message however)

I. Files provided on the 64NET system disk.

1. C64 files

64NET WEDGE

64NET KERNEL

PATCH-ROM

2. PC files

64NET.EXE

DEHEADER.EXE

64DMON.EXE

64DIR.EXE

J. Installing 64NET permanently in your Commodore 64.

WARNING!  The process described below will,

a) Void any warranty on your C64,

b) Possibly damage your C64 is done incorrectly,

c) Frustrate you if you would like to install 64NET permanently

in your C64, but can't (for one reason or another), and

d) Possibly make you a little more aware of the insides of your

C64!

At this point in time the author has not been authorised to provide

modified copies of CBM's C64 kernel ROM.  To do so without express

permission would be in breach of CBM's copyright of that ROM, as what

would be distributed is most substantially the work of CBM.

However, relief is at hand!  If you have access to an EPROM reader

and burner, then you can create your own, legal copy of a 64NET compatible

kernel ROM.  Here's how...

Firstly, use the EPROMMER to read your kernel ROM, and save a copy

to disk.  Then, run the "PATCH-ROM" program provided on your 64NET

distribution disk, following the instructions shown on the screen.

Finally, burn a new EPROM with the patched kernel (named KERNEL.64NET).

That's all there is to it!

If you don't have access to an EPROMMER, then your 64NET distributor

*may* be able to assist you in this regard, but we make no guarantees

of this.

K. Glossary.

logged drive - the PC server disk drive that 64NET is currently accessing.

logged directory - the (sub)directory of the PC server that 64NET is currently

accessing.

wedge - a program loaded onto the C64 which is "wedged" into the operating

system in order to alter normal operations.

kernel replacement - a program or ROM chip which is used instead of the

kernel ROM provided with the C64, eg. JiffyDOS.

DOS - Disk Operating System.  Also a commonly used abbreviation for the

operating system used on PCs.

L. 64NET Registration Form.

Please take the time to fill in this form, and return it to

the author.  It will ensure that any updates to 64NET are forwarded

to you, and that in the event of a problem with a distributor, you

will still receive support for 64NET.  Only registered purchasers will

receive update notifications.

Name:

Street Address:

City / Town:

Country:

Post/Zip code:

Your email address, or fidonet node:

Contact phone number (optional): [..] (...) ...-....

[country] (area) local-number

Your 64NET Distributor:

Thank you.  If you could also fill out the following survey questions,

it will help us make 64NET even better!

Where did you hear about 64NET?

What did you hear about 64NET?

What do you like about 64NET?

What do you dislike about 64NET?

Would you recommend 64NET to other C64 owners?

Why/why not?

Thank you again for taking the trouble to answer the survey.  Please

post the completed form to:

Paul Gardner-Stephen

1 Hurst St

Morphettvile

South Australia 5043

Australia

or you can email it to:

pgs@flinders.edu.au

M. Recognised PC filetypes.

As at V1.68beta, 64NET recognises the following "standard"

filetypes on the PC as containing valid C64 files, as part of its

virtual file system (VFS), and can therefore allow you, the user, access

to the files contained therein.

.N64 - 64NET's own, preferred filetype.

(64NET will only save as .N64 filetype)

.T64 - C64S PC disk files.

The .T64s don't have the C64 filename stored in them

correctly in all cases, so the 64NET displayed filename may

not be absolutely descriptive.

.P00

"raw" - ie. files which are plain CBM $0801 loading files.

"red raw" - what are these?

The author is also working at enabling recognition of other known

filetypes, such as .D64, .P00.

N. Multi-lingual operation.

The file 64NETMSG.nnn (where nnn is the country code of the country the messages

have been converted to the native language of) in the PC directory contains almost every message

that 64NET will ever display.  These messages can be altered to display

anything you wish, including in languages other than that which you

originally received it.

Hopefully your distributor has already provided a copy of this

file in your native tongue.  The 64NET team would be pleased to

receive copies of 64NETMSG converted to languages other than English,

but be warned we will make sure that the messages contain the original

meaning, and are not abusive, or offensive!

Files on your distribution disk named 64NETMSG.xxx contain a copy

of the 64NETMSG file already converted to the language indicated by

'xxx'.  Eg., 64NETMSG.GER contains a German (Deutsche) version of

64NET.MSG, and 64NETMSG.SWE contains a Swedish version.

Suffixes adopted by 64NET for the MSG files include the following.

Please be aware that they only indicate the language which is conatined

in the particular MSG file, *IF IT EXISTS ON YOUR DISK*.  It is not an

indication of which languages 64NET.MSG has been converted to!

ENG - English

GER - German (Deutsche)

SWE - Swedish

NOR - Norwegian

FIN - Finnish (Suomi)

ITA - Italian

FRA - French (Francaise)

DAN - Danish (Dansk)

*** Junk to add into manual.

1)

LOAD "VMEM:xxxx:yyyy:zzzz"

SAVE "VMEM:xxxx:yyyy:zzzz"

xxxx = page number of first mem page to load/save ($0000-$ffff)

yyyy = first address to load into/save from

zzzz = last address to load into/save from

This will allow 16MB of Virtual memory!

A partition will have to be allocated as the "swap drive", and this can still

be used for nthe normal VFS also.. The VMEM file will get as big as you swap

into it! and will _NOT_ necessarily be deleted at the end of a session

(wow! - `battery backed' RAM! :-)

2) REL files (finally!)

3) Fakey sector operations if i really feel upto it (fake copy protection ?)

4) "off-board" file copier (bi-directional).

This will require a bit of work to get going well.

It may yet be easier to slack it and write the rest of the one i've

already started.