ComLink V0.96�

(C) 1996 H�rsfalvi Levente

...Maybe, this is just one of the first stuffs released in the new year,

1996. Actually, I was been writing it even on New-Year's Eve... Life over =( .

(Why can't one simply go out and drink, instead of writing a 23674538th

filetransfer- system for PC's and Commodores, after that another existing

23674537?) Hmmmm, that's it. Welcome to the newest (...maybe... ) direct

filetransfer-system, which allows you to send files from Your PC to an old

8-bit Commodore, and even more!

Copyright:

------------

This program, the document and all files included are copyrights of the

Author. It's distributed on freeware-basis, copy and use the package freely.

The only thing I ask that note something like 'Developed using ComLink Vxx.yy

(C) etc. ' in Your stuff if You use it for developing. ...And contact me, I'll

be happy to know that You could use it to anything. Also, no payment may be

charged for it or it's copying. You can even modify and re-compile the source,

in case You need small modifications for the best use. Howewer, the package

may only distributed in it's original form; You're not allowed to spread Your

own versions. By the way, if You decide to add new features please contact me.

From the other hand, I can't accept any responsibility for the possible

damages, neither when assembling and testing the cable used to connect the

computers, nor when simply using the program. The package comes without any

warranty; if You use it or if You even use any information written here, You do

it at Your own risk.

All names used in this document are trademarks of their respective owners.

Table of contents

------------------

1.) Introduction

2.) Files included

3.) System requirements

4.) Installing

5.) More about the package

6.) Switches and parameters

7.) Using makefile

8.) Building the cable

8/a.) Serial cables

8/b.) Parallel cable

- C-64

- Plus/4

- C-16/C-116, building parallel port.

8/c.) Common parallel cable for all systems

9.) Connecting the devices

10.) Using the package

11.) Contact the Author

12.) Thanks and Greetings

Appendixes

A.) Some measuring results

B.) Possible improvements

1.) Introduction

-----------------

As I've said, this is a short filetransfer package, with which You can

download datas and/or code or anything to Your Commodore. Well, this is a

feature that so much transfer stuffs do. Then, why to write an other, new one?

Well. Because I needed a program for a bit different purpose. My primary aim

was to present a program that allows one to develop programs using PC for edit

and compile the source, and then run the code on an original Commodore. Then,

You'll be able to write even hundred K's of sources and compile them in few

seconds, then transfer and try it if it works. If You've ever tried to use C-64

TASM or other assemblers - even using the original BASIC interpreter's text-

and program- editor -, maybe tried to develop programs only with a silly

monitor, You know it's disadvantages; first, You've got only the main memory,

and this way, Your program can't use all of it. And outside this, it's very

easy to let fly away all things, when trying to run a new, untested routine, of

course, containing parts accessing memory. It's the best when You haven't saved

the source, but even if You did, usually it makes one angry; re-load assembler,

re-load source, re-load graphics, tables etc. if any.... So, I suppose it would

be much easier with an other and bigger computer, mainly, in an other memory.

And why to don't make it on a stupid PC, even if it's only a lame XT? The

advantages are convincing; if You do so, You'll have an absolutely clear

Commodore to run Your program, You'll not be angry about long I/O procedures,

compiles, You'll never lose Your sources. Outside these, You'll have at least

a 80 columns screen and some great text editors to edit them.

ComLink was designed especially to support this. It contains some features of

an usual linker. For example, it can read and translate the usual hexadecimal

.OBJ format, besides the normal .BIN and other files. Secondarily, You have the

possibility to write a makefile containing the filenames You wanna transfer

when linking.

In the physical level of the connection, ComLink allows You to use Your old

cables to connect the units, that means that even an X1541 or Disk64E cable

will do it. Of course, the program has it's own cabletype too, for better

transfer-speed. The transfer is quite optimized for speed; about 3.3K/s and

19K/s rates are possible with a normal C-64 and a PC, via serial and parallel

connection. Both the transmitter and receiver routines were written in machine

language.

2.) Files included

-------------------

- comlink.exe      The compiled and executable program

- comlink.pas      The source, written in Turbo Pascal 7.0

- cl64pn.asm,prg   Programs on the other end, coded in 6502 ASM.

cl64pq.asm,prg   '64' and 'p4' means C-64 and Plus/4, 'p' and 's' are

cl64sn.asm,prg   'parallel' and 'serial', 'n' and 'q' are 'normal' and

cl64sq.asm,prg   'quiet'. The codes were compiled with TASM.

clp4pn.asm,prg   The same receivers are used in Plus/4, C-16,C-116.

clp4pq.asm,prg

clp4sn.asm,prg

clp4sq.asm,prg

- clink[vrs].id    Small id-file for BBS'es and FTP-servers.

- comlink.doc      Documentation, You're just reading it.

3.) System requirements

------------------------

- IBM PC, PC/XT, PC/AT or compatible computer

- 640K RAM (some 150-200K of free base-mem would be enough)

- any kind of monitor-controller cards

- 30-40K free HDD or even FDD

- one free LPT-port

- an usual X1541, Disk64E or parallel cable, described later

- MS-DOS 3.3 or better operating system

The program was written and compiled into pure 8086/8088 code, so it should

work on any old coffee-mill :-).

On the other side:

- Commodore 16,116,Plus/4 or 64 computer

- any kind of storage-devices, even Datassette or FDD

This time only Plus/4 series (16,116,Plus/4) and C-64 are supported, because

I only have these; although it would be good to translate to other systems.

Please contact me if You could convert the routines. Mostly, for C-128, VIC-20

and PET series. ...By the way, I guess it would be easy to write own receivers

for not Commodore and/or 6502 based computers.

4.) Installing

----------------

You don't have to do so much installing this package; simply, unpack it - I

guess, You've done it already - and maybe, You only have to delete some

files You don't use, ie. receiver routines which don't match with Your computer

or Your cable.

From the other hand, installing the program on the other side is quite

different and it's not easy at all. The best way is to get an 1541 floppy and

an 1541-handler for the PC as X1541,Trans64,Disk64E or similar stuff. Then, You

can simply save the .PRG files to a disk and load with the other computer. By

the way, it's not as hard as it seems to be at first looking; as I said,

ComLink uses just the same cables as these programs do. This way, if You

assemble a cable, You got only to get or borrow an 1541 or 1571 etc. floppy and

everything is O.K. Moreover, I don't think when somebody owns a PC, he can't

get a floppy, at least for a halfhour. And You can get transfer stuffs anywhere

on the Internet, or if You don't have access for it, at least on BBS'es or

similar places.

Anyway, for developing programs, I suppose to get a floppy drive instead of

that beauty 1530 or 1531 Datassette. Well, if You have a PC then it's not as

necessary as when You don't have it, but I think Your life would be much easier

and more restful not fightin' with this masterpiece of streamers =).

If You can't get a floppy, You can - maybe - use the tape-handling feature of

C64S, programmed by Miha Peternel. But for this, You got 2 have the registered

version of his emulator. Of course, this is for C-64 only.

If everything seems to be out, then You'll have to try Your last chance;

select a routine written for Your computer and Your cable - for example, if You

have a C-64 and a serial - X1541 or Disk64E - cable, select CL64SQ.PRG, list it

on the PC hexadecimally and type it in on the other computer, using a

monitor-program and it's hexadecimal dump feature. Note that these .PRG files

have their start-address in their first places, so if a dump begins with

'00 CF', it means that this routine loads to $CF00 and the following bytes

contain the data, loading to this location. This way, You shouldn't type the

first 2 bytes, but You have to start the typing in this location of the memory.

I suppose to try this method with the quiet versions as they all begin on $CF00

(C-64 versions) or on $05F5 (Plus/4,C-16,C-116), and they're the shortest. If

it's done then You should save the code before You try it. But I still say that

it's not too hard to get an 1541, and You can skip this boring procedure.

5.) More about the package

---------------------------

The main program, comlink.exe is used to transfer the datas and this

transmits them to all kind of computers. It was written and compiled using

Turbo Pascal 7.0 by Borland (...but 6.0, maybe, even 5.0 might be enough to

compile it). The receiver routines depend on the type of Your computer and the

type of the cable; this is because of different memory-mapping and different

place of I/O regs in different computers. From the other hand, I wanted to use

few memory of the computers, and wanted to write the possible fastest routines;

this is the reason of that much version of receivers. I have written even

routines not writing anything on the screen, because of speed and size. All

receivers can load anywhere in the memory, except to their own location.

On C-64, the receiver routines begin on $CF00, except CL64PN which starts on

$CE80 because of it's size. ...Well I should relocate it to under ROM or to

anywhere, having no troubles with long blocks. On Plus/4, all routines start on

$05F5, in the place reserved by the built-in user software package's boot

routines.

The names of the receivers should be decoded as something like this:

- 'CL'          - ComLink

- '64','P4'     - Type of the computer, as C-64 and Plus/4. If You've got a

funny C-16 or C-116, You should use the same routines as

Plus/4.

- 'P','S'       - Connection type, Parallel or Serial. You should use the

'P' version, having parallel cable and 'S' using serial

cables. Both X1541 and Disk64E cables are serial, and

there's no need for different receivers using them; use

the same receiver for both.

- 'N','Q'       - Normal or Quiet version. Using normal, You'll have the

usual messages of the receiver, printed on the screen.

Quiet versions don't write anything on the screen; I've

done them because of smaller size and higher speed, if

anybody needs it so much...

Independently from the version, all of them update $2B-$2C, $2D-$2E pointers

after receiving a block. So, You'll have the right start-end BASIC pointers

when You transfer a program (mostly, games or anything else written already and

transferred to PC) using these. In other words, if You transfer such file, the

computer will seem to be like after loading a program from a conventional I/O

device.

All receiver routines were written using TASM, the Table Driven Cross

Assembler by Speech Technologies Inc.

Transferring datas is not else than sending a set of BLOCKS through the

cable. Each blocks have three words (16 bit) in their begin, start address,

data-size and an address for execute the code in the block, if any. Then, the

block continues with the data-bytes. There's no checksum this time; by the way,

I haven't ever seen nor one bad bit in a transferred block; if so then I did

something weird against the system :-(. The receiver keeps on watching the

lines and waiting for transfer while a block don't have a real executing

address; this way, there's a possibility to transfer more blocks and when all

data was transferred, give an execute command. And this feature is what makes

this stuff a 'linker'. Because, You can transfer all the separated parts of

Your program as blocks, even if they're in binary or object format.

Object files are transferred as separated blocks if they contain data in

different memory-areas.

The maximum block-size implemented is 65526 bytes, it comes from the largest

block-size allocated by GetMem procedure of Pascal. I think it's quite enough

this time. By the way, there's no limitation to the size of object-files, since

they're converted to binary just before transferring, and if You have a block

larger than 63K in an object, the linker transmits it in more steps, although

it doesn't seem to have much sense this time. As, no memory-mapping and only 16

bit addresses are supported.

Some about the block's header.

- The first 2 bytes are always the block's size in bytes. Theoretically, it

could be anything between 0 and $FFFF, but as I said, ComLink doesn't send

blocks larger than 65526 bytes; and while 'size' counts only the data-bytes, it

can be max. 65520.

- Second 2 bytes: run-address. ComLink tries to run the transferred thing

with a JSR to this address after finishing with receiving. It does it, unless

You use one of the two special values - giving $FFFF will cause a jump back to

the begin of the receiver, so it will keep on waiting for more blocks. And

giving $FFFE, the receiver will return to BASIC or to everywhere from You

called it with a JSR, as it will simply execute an RTS.

- Third 2 bytes: Start-address in the memory. The receiver will place the

following data-bytes from this address continuously. There's no limitation for

it's value, even $FFFF is valid.

Physically, the block-transceiver uses 2 handshake lines to detect if a data

was received by the another computer, and to note the valid data on the output.

These lines are CLK and DATA when using parallel and CLK and ATN when serial

connection. The data lines are the D0-D7 lines of the parallel port, and DATA

signal when using serial connection.

ComLink contains a timer function, to detect if no computer is hooked up or

no receiver is running on the other side or if the connection is bad. The time

is one second while it waits for a response from the other computer. It sets

this 'watchdog' function every time waiting for a handshake-bit. By the way, it

uses the 0. timer of the PC, which is used as DOS-clock normally. So, the

system timer will be late after transferring anything. Of course, it doesn't

affect the date and the time of the CMOS setup.

6.) Switches and parameters

----------------------------

ComLink doesn't have a config file or real user interface. This way,

parameters can be given only in DOS parameter-line, or some switches can be

used in the makefile too if You use this feature. But first, let's have a look

to just the DOS-parameters. It looks something like this:

COMLINK [filename||makefile.mak] [-pxxx] [-cx] [-tx] [-sxxxx] [-rxxxx]

Where:

filename    :  The name of the file You want to transfer. It may be either in

binary or object format, even an usual program-file You can find

in several places, BBS'es or FTP servers or transferred by

yourself with a floppy-handler or any similar stuff.

Default: '', so ComLink will give an error-message and exit

when no filename was given.

makefile.mak:  This feature is for people who want to use ComLink's multi-

transfer ability. When developing a program, You can list here

the files You want to link together in the other computer's

memory. If You use it, the extension of the makefile must be

'.MAK' as ComLink identificates makefiles only with this. More

info in 'Using makefile'.

Default: same as the above.

-pXXX       :  The port-address of Your LPT, in hexadecimal format. Only the

first 3 digits. Either $378 (LPT1),$278 (LPT2) or $3BC (LPT3)

are valid.

Default: -P378

-cX         :  Type of the used cable. This time X1541,Disk64E and Parallel

cables are supported, so valid decisions are 'X','D' and 'P'.

Default: -CP

-tX         :  Type of input-file. It may be either 'B'inary or 'H'exadecimal.

Use binary for all filetypes, except for .OBJ's generated by

assemblers. Using 'H', the input-file will be translated into

binary, even to more blocks if it contains incoherent

data-areas and transferred then as binaries.

Default: -TB

-sXXXX      :  Start address in the other computer's memory. Comlink does only

place this number to the 4-5. byte of the transferring block.

When transferring, receiver routines load the data bytes

continuously from this address to the memory. There is just one

exception: If You give $FFFF in the parameter-line, ComLink will

pick the first 2 bytes of the transferring file and place it

here. It's useable for files simply transferred to PC or grabbed

from an emulator's collection.

When transferring .OBJ's, the -s option will not affect the

start address of the transferred datas. It will be picked from

the objectfile.

Default: -SFFFF

-rXXXX      :  ComLink copies this value to the transferring block's 2-3

bytes. Hexadecimal number is needed, of course. After

transferring, the receiver routine will execute a JSR to this

address. Normally, it's no matter that it's not a JMP, but if

You develop a new stuff, it gives You the chance to RTS to the

receiver and waiting for transfer again.

There are 2 special values with different meaning. Using $FFFF,

the receiver will loop to it's begin and waits again. In case

$FFFE it executes an RTS, so if You called it with a SYS command

from BASIC, Your computer will back to the prompt.

Default: Using single-file mode, it's -RFFFE. When using a

makefile, it changes to -RFFFF, giving the chance to send an

executing command only in the last transferred block.

There's another type of executing ComLink:

COMLINK -i [pXXX]

The program won't do anything else than set the LPT's used open

collector I/O lines to high state, giving the possibility to an

other unit to communicate in the cable. Initialize will only

modify the bits of the serial cable's lines, and doesn't operate

with LPT's parallel bits (D0-D7).

Note that no additional option is needed except port-address

and they will not affect anything. This switch may used by

itself only.

7.) Using makefile

-------------------

This feature is why I began this project, of course, besides of using

conventional cables to hook the units together. If You develop a program, the

datas and code and everything are used to be not in just one file and in a

continuous memory-location but in some different files. For example, when

writing a demopart (... 8-), maybe, You will have the graphics, the tables, the

texts etc. not in just one object-file with the code. Well, it's possible to

load them into the Commodore using a floppy drive but... Why to don't transfer

them to PC and send them together with the code to the Commodore, to avoid

loading them from slow tape or 1541 when a crash destroys all datas in the

memory? ...And moreover, it's possible to generate tables or anything else in

PC too, maybe, easier than in 64 with Basic.

ComLink's makefile means something like a listfile, where You describe the

files to be linked together in the other memory. It means that files listed

here will be transferred one after one in blocks to the Commodore.

A valid makefile seems like something this:

SMOOTH.OBJ -TH

SINE.DAT -S2000

C:\PROG\CBM\FACE.PIC -S4000 -R1356

[EOF]

Then, if it was named 'smooth.mak', executing ComLink with

COMLINK SMOOTH.MAK -P378 -CP,

will transfer the areas of SMOOTH.OBJ to their start-address, SINE.DAT to

$2000, C:\PROG\CBM\FACE.PIC to $4000 and executes a JSR $1356. It will use the

LPT1 ($378) port and parallel cable for doing this.

In a makefile, You can use exactly the same parameters as in the command-

line, with few exceptions:

- You cannot use the -i switch.

- The default RUN address is $FFFF, so the receiver will keep on waiting for

new blocks again and again. It's Your matter to write an executing address

to the last, and maybe, only to the last transferring file-name.

- You can even use different port-addresses and cable-types in all rows, but

it seems to have not much sense, hooked only one computer to the LPT. If

You've got more computers in more LPT's, You can use it too. But the

default LPT-address and cable-type will remain until an other command

doesn't change it, while all other defaults reset when ComLink begins to

trace the next row.

- You can't list another makefiles in a makefile. The interpreter logic

doesn't test if a file's extension is .MAK. Well, maybe next time... By the

way, it's only a source of possible troubles. I'm sure, somebody would try

to load a makefile recursively ... :(

It's possible to return to the waiting state of the receiver with a simple

RTS command, as receivers execute the code with JSR. It may be useable, mostly,

if Your code doesn't hang and You want to back to another transfer when testing

the code. Take care about re-setting some default values in the memory before

returning; for example, screen-address, charactermap, screen colors,

memory-mapping etc.

8.) Building the cables

------------------------

8/a.) Serial cables

-------------------

As I said, no special cables are needed using this transfer stuff. If You've

played with PC-1541 disk-handlers, that cable will do, except the cable of

1541.COM which was a capital fail and full with bugs. So I suppose You don't

have that cable. Either X1541 or Disk64E cable are O.K. If You don't have them

yet, here is their description.

You need the following to build the cable:

DIN 6 pin plug (the same type used as video-cables.)

Canon DB-25 connector (male), to the LPT.

DB-25 connector shell

4 or 5 wire cable

solder, soldering iron

Canon DB-25 (male, from the back)

.------------------------------.

13\ ************************** /1

\ ************************ /

25`------------------------'14

DIN (male,from the back)

.. ..

/   U   \

/ 5*     *1 \

I      *6     I

\ 4*     *2 /

\   *3  /

`---'

The connections:

Signal name    DB-25 pin       Signal name             DIN-6 pin

Disk64E     X1541       Disk64E     X1541

STROBE'        1               ATN         CLK         3           4

AUTOFD'        14              CLK         DATA        4           5

{INIT'          16              RESET'      RESET'      6           6}

SEL_IN'        17              DATA        ATN         5           3

GROUND         18-25           GROUND      GROUND      2           2

Connecting INIT' and RESET' is not necessary at all, moreover, it can easily

reset Your computer when re-boot the PC, if You've got an old version of C-64.

In these, external Reset signal could cause reset the whole computer. Newer

ones contain an additional logic to avoid this. By the way, ComLink never sets

INIT' to 0, to reset external devices.

8/b.) Parallel cable

---------------------

Now, some about the other cable which allows You to have better transfer-

rate. It's nothing else than a Disk64E cable, extended with 8 data-lines from

the LPT to an user port's inputs. This means that parallel cables won't be

independent from the system with You use them, as the above ones were.

Usually, You'll need a specific user port-connector outside of the things

listed above at serial cables and 'Disk64E', and additional 8 wires of cable.

The best would be a ribbon cable with 8+4(5) = 12 (13) wires. When assembling

it, You should put a common DB-25 connector to one and two separated (DIN-6 and

the user-port) connectors to the another end. Be sure that You connected right

wires to right pins in the DB-25, before You try to separate the other end of

the cable! :-)

C-64

-----

Signal name    DB-25 pin       Signal name     User port   DIN-6 (serial)

STROBE'        1               CLK             -           4

D0             2               PB0             C

D1             3               PB1             D

D2             4               PB2             E

D3             5               PB3             F

D4             6               PB4             H

D5             7               PB5             J

D6             8               PB6             K

D7             9               PB7             L

AUTOFD'        14              DATA            -           5

{INIT'          16              RESET'          -           6}

SEL_IN'        17              ATN             -           3

GROUND         18-25           GROUND          -           2

C-64 User port connector:

1    2  3  4  5  6  7  8  9 10   11 12

**   ** ** ** ** ** ** ** ** **   ** **

---   --------------------------   ------

**   ** ** ** ** ** ** ** ** **   ** **

A    B  C  D  E  F  H  J  K  L    M  N

Note that 'G' and 'I' letters are missing from the pin-config.

Plus/4

-------

Signal name    DB-25 pin       Signal name     User port   DIN-6 (serial)

STROBE'        1               CLK             -           4

D0             2               P0              B

D1             3               P1              K

D2             4               P2              4

D3             5               P3              5

D4             6               P4              6

D5             7               P5              7

D6             8               P6              J

D7             9               P7              F

AUTOFD'        14              DATA            -           5

{INIT'          16              RESET'          -           6}

SEL_IN'        17              ATN             -           3

GROUND         18-25           GROUND          -           2

Plus/4 User port connector:

1  2  3  4  5  6  7  8  9 10 11 12

** ** ** ** ** ** ** ** ** ** ** **

-------------------------------------

** ** ** ** ** ** ** ** ** ** ** **

A  B  C  D  E  F  H  J  K  L  M  N

Note that 'G' and 'I' letters are missing from the pin-config.

C-16 / C-116

-------------

This is a hard kind of questions, as these computers doesn't have User port.

This way, it seems to be difficult to present parallel connection with them. I

can't believe if it would be possible without modifying the computer

internally. ...Maybe, it would be possible to use the portbits of the Joystick

connectors but it's hard, too; after all, because nobody could get those mini-

DIN's. Then, there's no chance than assembling at least an 8-bit input port or

bus-driver circuit into the computer.

For fixing the problem once and for all, I suppose to use the original 6529B

I/O circuit, used as parallel I/O chip in Plus/4. ...Well, it's not as cheap as

it could be but I think, there's no better cure to the problem. This IC can be

as input as output without setting I/O state of the bits additionally; because,

it's outputs are Open Collector types and the lines are pulled up with internal

resistors to +5V. When You set an output line to '1', it's output transistor

will be open, it means exactly like when a switch would be open and only the

pullup resistor is holding the line. Thus, You can pull down the line

externally, and it can be detected with the 6529B's input- latch.

Instead of it, You can compose an own board substituting this. Well, it

depends on Your demands. If You want to have a simple input or output port, You

can use either 74373,74573,74245 - mostly, from the LS-TTL or CMOS HCT series

as 74LS373, 74HCT573 etc. - as input-latches and 74373,74573 as outputs. If

You need full I/O handle then Your work will be more complicated. For full

compatibility with the original User port parameters, as Open Collector outputs

and no additional setting to I/O state, maybe, You'll have to use an input and

an output latch, with OC driver circuit on the output lines. It means at least

3 or 4 IC's and 8 resistors, to pull up the outputs. And it's recommended to

build a board, as 3-4 chips soldered one on one is very beautiful but not a

very safe effect :-).

As I have a C-16 too, I did a same trying to have a 8-bit bi-directional

port. I've used a 74HC245 and a 74HCT573 as input and output latch, a 74C74 to

store I/O state and a 74LS04 'coz I needed it to invert a line. It works as

input when a reading and output when a writing command occured at last.

...Well, these chips were soldered directly one on one and this whole beast to

the connector I used. Then I needed only some ribbon cables to connect this to

the mainboard =). It was months ago, when I haven't known all things about OC

type of outputs and 6529B. My circuit is not fully compatible with the

original.

By the way, You can use any type of port-circuits, even an Intel 8255 TPI but

it will not be compatible with the original features; this way, Your computer

will not be compatible with programs using Plus/4's 6529B.

If You decided to do the modification in Your computer, then You'll have

to get a 6529B or a functionally compatible circuit, even built from standard

TTL's in a piggy-back board.

AS I'VE SAID ALREADY, I CAN'T ACCEPT ANY RESPONSIBILITY FOR POSSIBLE DAMAGES

OCCURED WHEN INSTALLING MODIFICATIONS WRITTEN HERE. AND I CAN'T GUARANTEE

THAT YOU COULD DO IT IF I DID. IF YOU TRY IT, YOU DO IT AT YOUR OWN RISK!

INSTALLING INTERNAL THINGS IN YOUR COMPUTER WILL VOID WARRANTY! ...Well, if

You bought Your computer these days...

When trying to work inside the computer, You should have a Weller soldering

iron or at least anything which doesn't cause high-voltage peaks on itself.

This is necessary to keep the chips alive; there are much highly integrated

chips inside. And since a 8360 TED can be gotten for the price of 2-3 used

C-16... :-). Also, it's recommended to avoid plastic or nylon etc. kinds of

clothes, because of that voltage peaks.

I think, this circuit might be installed most easily soldering directly to an

existing 6529B chip which is used as keyboard latch. Well, this is not a

beautiful thing but it works and You'll not have to build a board. First, pull

out all cables from the computer, and open it, removing the screws. Then,

disconnect the keyboard and the LED. Open the shield and remove screws from the

mainboard. Take it out from the cover. You'll see a heatsink and a 7805

voltage- stabilizer IC with three pins, and under it the U13 6529B. Bend out

pins 2,3,4,...,9 and 19 on the new 6529B and solder it to the keyport latch

6529B chip with the another pins. Then solder a wire to the bended pin 19 and

solder the another end to pin 16 of U16, 7700 FPLA. ...The FPLA might have an

own type-number, not the original 7700. Maybe, You'll have to solder it to the

down-side of the mainboard, as FPLA was used to assembled in socket. Be sure to

don't short another fields in the board! Then, locate U11, 74LS125 on the board

and cut the field from it's 8th pin. Solder a wire to pin 9 and solder it's

another end to pin 4 of the newly inserted 6529B. This line comes from the CST

SENSE of the tape and it's used for detecting the state of the tape-keys. In

Plus/4, the User port's 6529B is also used for this. In C-16 and C-116, the

74LS125 bus-driver is used but since we've connected it out... O.K. Then,

solder a 8 or rather, 9 wire ribbon cable to pins 2-9 of 6529B and solder them

to the pins of a connector. Solder the 9th wire to GROUND (pin 10 of 6529B is

good) and solder it to an unused pin of the connector. This connector may be

either a Canon DB-9 or a Canon DB-15 or any type of connectors You want.

...Well, it will not be compatible with other systems, only after building an

interface. The best would be the original Plus/4 user-port type but it's an

edge-connector, formed from the mainboard of Plus/4. If You're good in making

boards, maybe You could do it but it's quite difficult as I guess.

Well I shouldn't forget that You'll have to make a hole to the cover, for the

connector. Be cautious and don't do lame work... :-(

After You've soldered everything, then try out Your work immediately. Be sure

that You've turned off the ON/OFF switch before connecting power... Connect RF

cable, turn on the monitor and then, the mainboard. If You see the usual

message 'Commodore Basic V3.5 12277 (or 60671) bytes free' and the prompt, the

unit passed the first test. Type 'LOAD' and if it answers 'press play on tape'

and doesn't blank the screen immediately, the second pass is O.K. too; maybe,

the whole port-circuit works, except if You've done something weird connecting

that ribbon-cable to the new connector. Checking this, type 'MONITOR' and write

'>FD10 FF'. Measure the levels on the new portbits, You should get high level

on all lines. Type '>FD10 00', then try again - the portbits should be on

ground. Type '>FD10 FF' again and short a line to ground. Typing '>FD10' should

give a value with 0 bit on the appropriate place.

If the unit doesn't seem to work, there are several possibilities. If it

begins to boot but it hangs before could print '12277' or '60671', maybe,

You've shorted few wires around the FPLA. Well, if You haven't done something

cruel against Your computer, I'm sure this can be only a short-circuit.

For more troubleshooting, I suppose You to have the schematics of Your

computer.

After doing well everything, put the parts together, in reverse order You did

when opened the cover.

Then, it's the pin-out of 6529B.

.-----\_/-----.

R/W' -I 1       20 I- VCC

P0 -I 2        19 Io CS'

P1 -I 3        18 I- D0

P2 -I 4   MOS  17 I- D1

P3 -I 5        16 I- D2

P4 -I 6  6529B 15 I- D3

P5 -I 7        14 I- D4

P6 -I 8        13 I- D5

P7 -I 9        12 I- D6

GND -I 10       11 I- D7

`-------------'

And it's a sketch of a possible substitution of 6529B

1K

.-----\_/-----.         7407  /-\/\/-> +5V

GND<--  OC'oI 1        20 I- VCC    I\    I

D0<--  D1 -I 2        19 I- Q1 --- I  > -+----> P0

D1<--  D2 -I 3        18 I- Q2     I/

.  D3 -I 4        17 I- Q3  .

.  D4 -I 5        16 I- Q4  .

.  D5 -I 6  74573 15 I- Q5  .           1K

.  D6 -I 7        14 I- Q6  .  7407  /-\/\/-> +5V

.  D7 -I 8        13 I- Q7  .  I\    I

D7<--  D8 -I 9        12 I- Q8 --- I  > -+----> P7

GND -I 10       11 I- CE -\  I/

`-------------'      I    /I 7404

CS' I\  7404                          \-o<  I <------ CS'

--> I  >o-\                                \I

I/    I      .-----\_/-----.

\- OC'oI 1        20 I- VCC

P0 --> D1 -I 2        19 I- Q1 --> D0

P1 --> D2 -I 3        18 I- Q2 --> D1

. D3 -I 4        17 I- Q3 .

. D4 -I 5        16 I- Q4 .

. D5 -I 6  74573 15 I- Q5 .

. D6 -I 7        14 I- Q6 .

. D7 -I 8        13 I- Q7 .

P7 --> D8 -I 9        12 I- Q8 --> D7

GND -I 10       11 I- CE --> VCC

`-------------'

Well I'm not sure in these as I haven't tested them. But some comments. From

74573 I don't think if You should use standard TTL series but use at least LS

or HCT, for less dissipation. Take care about TTL compatibility, at least, with

the receiver one - it shouldn't be C or HC type, else You may have troubles

when You'll have more units hooked up to the same cable. As, I have with my

C-16's 74HC245. 7407 is an OC non-inverting line-driver circuit and it's only

in standard TTL; You'll have to get 2 pieces of them as one chip contains only

6 drivers. 7404 could be either LS or any better type, too. The schematic

mostly seems as 2 independent 74573 with some additional things but I haven't

drawn some lines because of it's visibility. When designing the board, please

connect all lines signed as P0-P7, D0-D7 and CS' in the separated parts

together. And connect all unused inputs of 7407 and 7404 to GND. ...Eh, is it

better to have an original 6529B???

For a partial implementation, You can use just the upper or the lower part

for output or input. By the way, it's enough to have the lower part for the

requirements of ComLink.

I'm waiting for any information of an existing OC I/O chip similar to MOS

6529B. Because I see much possibilities of it's using but I can't do it because

of the price of these chips. Ie., it costs about the half of the price of a

6522 VIA.

8/c.) Common parallel cable for all systems

--------------------------------------------

It was an other project of me, when tried to define a not computer-specific

parallel connection between a Commodore and a parallelled 1541 floppy drive.

The method is quite easy to imagine, well, only a common connector-type is

needed to do it. But assembling this, it needs some modifications in the

computers, to present this connector on them. Now, when trying to work with

ComLink, it means the same as we had only the serial connector on the computers

and the serial cables - I've got easy to do as I hadn't got to assemble

different connector cables for different machines.

This common connector is a Canon DB-15 with 3 rows, exactly the same as the

video-connector of the PC's VGA cables.

(Canon DB-15, 3 rows, male, back)

.--------------------.

5\    **********    /1

10\    **********  /6

15\  **********  /11

`------------'

I've put all lines to this, used when communicating with a parallelled 1541.

This way, it contains all the usual lines of the serial port, and additional

data-lines. Then these data lines can be connected to different port-circuits

in different computers, ie. to the VIA port in 1541, the CIA PB0-PB7 lines in

C-64 and P0-P7 lines of 6529B in Plus/4 and maybe, of a 6529B in a tuned

C-16/C-116.

The description of this connection:

Pin           Signal name     I       Pin numbers         Signal names

I       C-64    Plus/4      C-64    Plus/4

I    CIA2/u.p   6529B/u.p.  CIA2    6529B

1             D0              I     10   C      2    B     PB0     P0

2             D1              I     11   D      3    K     PB1     P1

3             D2              I     12   E      4    4     PB2     P2

4             D3              I     13   F      5    5     PB3     P3

5             D4              I     14   H      6    6     PB4     P4

6             D5              I     15   J      7    7     PB5     P5

7             D6              I     16   K      8    J     PB6     P6

8             D7              I     17   L      9    F     PB7     P7

9             Reserved        I

I       SERIAL CONNECTORS   SERIAL CONNECTORS

10            SRQ'            I             1                 SRQ'

11            ATN             I             3                 ATN

12            CLK             I             4                 CLK

13            DATA            I             5                 DATA

14            RESET'          I             6                RESET'

15            GROUND          I             2                GROUND

I

In the left, it's the connector's pin-numbers and the signal-names. In the

other side, I've listed the pin names and numbers of the connecting

port-circuits of the different computers, also listed the User-port pin names

if somebody want to build an interface, not working inside of the computer.

BTW, You can find all lines outside of the computer too, using the computer's

connectors.

Pin 9 of the cable is reserved for developing in the future; maybe, it should

hold a +5V signal, to present power to an external device, having no power

supply. Or, it should contain a triggered signal like CIA's FLAG' input, but it

would not be good at all, as not all Commodores have such input-line.

By the way, I think it's the real chance for C-16 and C-116 which doesn't

have User port. When assembling Your extension, it's not much harder to solder

5 more wires to the connector and then You'll not have to use more connectors

using ComLink, moreover, You can continue with the extensions on other units

etc. Then You'll have a connector that maybe, compatible with other people's

ones. ...Well, I don't know. It would be good, but I bet, this time only I have

such connection between my units :-(.

The cable, used for connecting PC and this common system

Signal name    DB-25 pin       Signal name     DB-15 pin

STROBE'        1               CLK             12

D0             2               P0              1

D1             3               P1              2

D2             4               P2              3

D3             5               P3              4

D4             6               P4              5

D5             7               P5              6

D6             8               P6              7

D7             9               P7              8

AUTOFD'        14              DATA            13

{INIT'          16              RESET'          14}

SEL_IN'        17              ATN             11

GROUND         18-25           GROUND          15

All connector cables have MALE types on their ends, and all connectors put on

the computers or to the end of the interface-cables are FEMALE.

Assembling the internal connector to a computer, You can get the serial lines

most easily from the old serial connector's pins, in the back of the

motherboard.

If You have more questions about this, contact me. By the way, You can get

some inspirations having the schematic diagram of Your computer.

9.) Connecting the devices

---------------------------

Having the cable, You should connect the units together. Well, You could ask,

how to do this, if You want to use a floppy drive too. It's easy. Connect Your

floppy to the Commodore, and connect the PC to the drive with Your new cable.

It's possible because of the used OC line driver architecture. If You've

assembled that parallel cable, then connect the another part to the user port

besides of connecting to the floppy. Take care about connecting User port

connector WITH RIGHT SIDE ON THE TOP AND ON THE BOTTOM, else You'll burn the

LPT of Your PC. And if You've done the non-specific connectors or at least,

interfaces, then connect the units with them. In my system, there's a computer,

a C-16/60K or a C-64 with that connector, hooked to a parallel 1541/II and to

the PC via the 1541/II's another parallel connector. And it works, however,

I'd better to don't use parallel data-transfer between the Commodore and

1541/II while hooked to the PC. Look out for more at 'Possible improvements'

part.

Important. When booting up the system,

NEVER TURN YOUR PC ON OR OFF WHILE YOUR COMMODORE DEVICES ARE SWITCHED ON!!!

This is by transients that PC could cause when switching on or off. And since

Commodore never took care about real protection against peaks... Outside this,

it's enough to know or think on that at least, in Plus/4 the input serial lines

are connected directly to the processor's own I/O portbits. If You give a peak

to it enough successfully, maybe, You'll have to buy a new 8501 proc.

10.) Using the package

----------------------

Then, maybe, You've got a working system, hopefully, You haven't burnt

anything. You should try if the system is working. If You have a floppy drive,

You can even save the receiver routines to an 1541 disk. Thus, execute for

example, Trans64. You should get it's usual user interface. If not, then Your

Commodore holds a line and You should tell it to leave it. C-64's hold mostly

their CLK line. Set CIA2, 0. reg as $DD00 to 7, this way, the OC output-driver

will open. From Basic,

Poke 56576,7

Maybe, You've just got the right user-interface. Transfer the used receivers

to the disk. O.K.

Then exit the 1541-handler and try to load a receiver from the disk to the

Commodore. Use LOAD"NAME",8,1 as they don't have Basic boot. If the computer

hangs, then Your PC is holding the lines. Execute

comlink -i

to fix this. After loading, run it, either with

SYS 1525

on Plus/4 and

SYS 52864  or  SYS 52992

on 64. 52864 belongs to CL64PN.PRG, because of it's size. All other C-64

receivers start on $CF00 and 52992.

Using the normal versions, You'll see it's message 'waiting for transfer'.

Then, You can transfer anything from the PC.

Important. Transferring blocks use exactly the same lines as used in a normal

serial transmission. This way, there may be troubles when more units are

switched on. I've made the parallel thing that it doesn't use the ATN line so

it doesn't affect any units hooked to the common cable. However, using serial

transmission will cause troubles, as I hadn't got the chance to don't use ATN.

This way, You must switch off all other units when transferring a file from the

PC, to avoid errors while transmission.

Then, all following things depend on You. You can use this fast transfer

system to anything You see it useable. But I said, I wrote it mostly to be a

developer tool.

At last, only one important thing. When transferring, You can cancel it

anytime, pressing a key. But DON'T EVEN PRESS CTRL-C OR CTRL-BREAK while

transferring 'coz Your computer will hang!

11.) Contact the Author

------------------------

If You have any questions, thoughts, suggests etc. You can catch me via

snail-mail at

H�rsfalvi Levente

7200 Domb�v�r

Gorkij 33.

- Hungary -

Or it's better to do it via E-mail. This address should remain at least

until 1997 June or whatever.

TLC@MSZI.PMMF.HU

...But don't event try to make me happy with doz fuckin' T-Bozz, Left Eye and

Chilly 'coz I'll bite You =]. I hate them and I changed my ID as fast as I had

enough from them. But maybe, You'll find some funny stuffs under the name

Beard,  ...well, if I'll ever be at least as active as I was as TLC.

12.) Greetinx and thanx're flying to

-------------------------------------

- To my parents and especially, my mother for the help to get my computers.

...Well, where is just 1988 when I got my first C-16... :-)

- To Marko M�kel�, for his support in hacking.

(...Do You know the album of Suicidal Tendencies: 'Join the Army' ??? :-)

- To all the Plus/4 and C-64 fans, still hacking on these cool computers.

...Well, have You ever seen removed sideborder-effect on Plus/4? I have.

Dun by Larry/WLS... Special greetx to Luca/Fire (...hold on! ...Well, my

letter is coming...), also hello and thanx to Synergy and The Elder for

supporting me with a SID-card. And then, to Larry/WLS (...I was surprised

of Your stuffs... I wanna see a new mega from You, soon... ).

A gigagreet to St. AbcuGLS. ...Poor dude, he's waiting for my letter since

3 months ;( ...

- Greetinx to my (...maybe...) ex team-mates in Coroners design. I don't know

if we'll ever work together again :(.

- Justin O'Hagan, my ex english-teacher. All the bests, guy! I hope we'll

ever meet again.

- To You, well, at least for reading this 'little' document and my terrible

english. By the way, I hope You'll enjoy this little piece of software...

...And then, as GoodByTe:

,        ,

HAJRA FERENCVAROS !!!

I send hello to all thus green hooligans. ...No matter if they got it well

from AFC Ajax and Real Madrid... ;(

APPENDIXES

A.) Some measuring results

---------------------------

I've used a TX486DLC-40 with 256K cache (ISA) with my C-16/60K and C-64.

And, measured all receiver-versions either in normal or in blanked screen

and on the C-16, in blanked screen with switched to NTSC mode. This last is

only for introduction, as I'm sure, nobody will use a PAL machine, with PAL

crystal switched to NTSC mode, having no real screen on the monitor. In this

case, C-16/C-116 and Plus/4 runs at about 2.2Mhz clock, 17.734472/8.

All the measuring results are in KBytes/seconds.

NORMAL      BLANK       NTSC-BLANK

--    C64     --

CL64PN   18.9        19.8        -

CL64PQ   21.32       22.48       -

CL64SN   3.3         3.52        -

CL64SQ   3.4         3.6         -

--   PLUS/4   --

CLP4PN   29.7        43.77       55.45

CLP4PQ   36.16       51.98       63.98

CLP4SN   4.62        6.93        8.66

CLP4SQ   4.72        7.04        8.66

Plus/4 is faster by it's different clock-rate and easier handling of memory-

mapping.

B.) Possible improvements

--------------------------

Well, ComLink is just a quick hack, I wrote it in few days. Then, some

improvements are possible

- The object loader feature. This time, only Intel format objects can be

used. By the way, the loader doesn't do anything with checksums and doesn't

look for additional redundances; it only picks the data-areas from the

hexadecimal file. This way, there may be troubles giving an usual text-file to

the linker, I haven't tried it, maybe, ComLink would hang. Or I don't know.

I'll fix this in 0.97. Moreover, it would be good to have Motorola and MOS

format objects. I've got already their description.

- The common parallel cable, used for hooking up parallelled devices to the

Commodore and used by this program too, may have some more possibilities. This

connection is bi-directional, between Commodore and it's units. When hooking up

to a PC, this will change immediately. LPT's can't switch to input in most

cases, moreover, LPT databits (not controller bits as STROBE, AUTOFEED etc. !!)

always drive the output lines, one can't let it just leave them. This way, no

parallel data transfer is possible between more Commodore devices while they're

connected to the PC's LPT. And even more is true.

DON'T EVEN TRY TO DEFINE PORTBITS OF VIA OR CIA AS OUTPUTS WHILE THEY'RE

CONNECTED TO AN LPT, BECAUSE THEY MAY BURN AWAY!!!

I work on this, maybe, an Open Collector line-driver would do it on the LPT

end of the cable. Moreover, this time, the parallel connection between the

Commodore devices and the PC may be only in one direction - from PC to a

Commodore device. Physically, serial transfer is possible to both directions.

The next step would be, to present a bi-directional, OC input-output port,

using the LPT. As there are not enough input-lines on the LPT for this, I

think on a reading latch with serial output-bits. This way, some OC output line

driver gates (Bill Gates rulez... =)) like 7407 would be needed with some

pullup resistors. And the input-latch would be connected with it's input lines

to the parallel data-lines and the serials - the serial output and the stepper

input - to the LPT's bi-directionals, to an input and to an output. It would

not affect the speed too much, as transfer speed depend mostly on the power of

the Commodore's processor.

Having this parallel connection, some things might be much faster. It would

be possible to write a handler to a parallelled 1541 floppy, or rather, to let

an existing handler to use this. Well, first I should fix this and then ask

somebody, who just have a good working 1541-handler.

That time, maybe, ComLink should be extended to be bi-directional too,

although, I don't see too much improvements in it, as I know some easier ways

transferring a file to PC than loading to a Commodore's memory, and then

transferring to PC.

- Possibility of linking in a PC-file. This time, when You develop a program,

You've got only the possibility to transfer the parts and then if everything is

O.K., save the memory in Commodore to a file, pack it etc. I guess, it would be

better to link the parts together in a PC-file when You've succeed with testing

of the program, pack it on the PC and then having an already packed program,

simply loadable as Basic program (...with a simple SYS xxxx and a depacker

routine linked to it's begin). This way, no argue would be needed with slow

packers on Commodore etc. I'm sure, I'll write this at next ... well, if I'll

have the time. Sorry. My friends could say some things about my behaviour...

:-(

- Receivers. I should write another 2 receiver issues to 64, to support

transferring very large blocks. This time, all receiver routines reside on

$CE80 and $CF00. It's good, because this area is easily accessible. But it

limits the maximum transferrable block-size. Not a hot shot...

- Supporting extended memories. This time, ComLink uses only 16 bit addresses

when transferring, this way, it can handle only 64K of memory, either as block

size or start address. Next step could be to use 24 bit values, this would mean

a theoretically 16M address-range. Then, ComLink could support the C128's

extended memory, the 256K internal memory expansions of C-64, REU etc. But

it's difficult to me as I don't have these yet.

- Maybe, a 1541 emulator stuff should be written, either a simple serial or a

parallel version. This way, the PC would seem as an external unit hooked to the

serial cable, with an own devicenumber. It would help in some affairs. Of

course, I think on a different stuff than ComLink. I see it well at the first

look but there are just two things: 1. I'm too lazy to have the right to

promise anything 2. I need some more info about parallel services and newer

units having more storing space than a simple 1541, to be compatible with them

if it's possible. Any useable information is welcomed about these. I think on

especially 1571, 1581 and CMD's drives, either FDD or HDD.