* hardware

Warning: Be very careful in making the cable and be sure to connect

with the computers turned off!

Don't forget about the diods, they are required too.

If you make a mistake, your CIA in the C64 will surely die.

C64 user port 	    Red o---+---+---------+-------------+----+

------------- 		    |	| 4148	  |		|    | 4148

IC: 7406N  +-----o GND vvv	   ,+,__+__	 ,+,	       ,+, __+__

'-+- 		   | | / \	 | |	       | |  / \

\ /  4148		1k | |,-+-,   1k | |	    1k | | ,-+-,

--+--		   '+'	|	 '+'	       '+'   |

|			    +---+	  |		+----+

Yellow o---+----------------------+-oIC#6,L	  |		|

Grey	 o----------------------------------------+-o IC#2	|

Green  o---+--------------------------------------------------+-oIC#4,K

|

--+--       Red o-----o IC#14,IC#13,IC#11,IC#9,2

/ \ 4148	 J o-----o IC#5

-+- 	 H o-----o IC#3

|		 F o-----o IC#1

GNDo-------o GND

C64 USER PORT          1 2 3 4 5 6 7 8 9 X Y Z

A B C D E F H J K L M N

Red o---+------------+-------------+-------+

PC printer port (LPT1)	     |		  |		|	|

----------------------	    ,+, 	 ,+,		|	|

IC: 7406N			 1k | |       1k | |	       ,+,	|

| | 	 | |	       | |	|

'+' 	 '+'	    1k | |	|

|		  |	       '+'	|

Gray o-------------------------------+-oIC#8,P#13 |		|	|

Green o-------------------------------------------+-oIC#10,P#12 |	|

Yellow o--------------------------------------------------------+-oIC#12|P#11

____	|

P#6o----oIC#3		  P#15 o-|____|-+

P#5o----oIC#1			   1k	|

P#4o----oIC#9			  ____	|

P#3o----oIC#11		  P#10 o-|____|-+

o P#GND,P#20		P#2o----oIC#13			   1k	|

|							  ____	|

GND o----+---o IC#7					  IC#5 o-|____|-+

1k	|

IC#14o--------+

P means the printer ports' pins, IC means the 7406 IC's pins.

The cable must be connected to LPT1 on PC.

Note: It hasn't been tested, but I think these ICs are only needed if you have a

long cable, otherwise it can work with the single cable & diods.

* files included:

C64: PCLINK.PRG

KERNAL.PRG                resident kernal

PINGSERV.PRG

(original binaries)

or    PCLINK (PCLINK.C64)	 transfer program

PING SERVER (PINGSERV.C64) cabletest server

(C64FS files)

PC:  SERVER.EXE executable of C64FS server

PING.EXE   cable & transfer testprogram

INITLPT.EXE initialization of port, must be run before SERVER.