ANSI escape and control sequences used by Flash in ANSI mode

Control-O switch to G0 character set

Control-N switch to G1 character set

ESC (A  Set UK character set to G0

ESC (B	Set ASCII character set to G0

ESC (0  Set graphics character set [line-drawing] to G0

ESC )A  Set UK character set to G1

ESC )B	Set ASCII character set to G1

ESC )0  Set graphics character set [line-drawing] to G1

ESC 7	Save cursor and all character attributes

ESC 8	Restore cursor and all character attributes saved with ESC 7

ESC #8	Fills screen with 'E's [Not particularly useful]

ESC E	Moves cursor to start of next line [index]. Scrolls up if needed

ESC D	Moves cursor down one line. Scrolls text up if needed.

ESC M	Moves cursor up one line [reverse index]. Scrolls down if needed

ESC c	Resets terminal

ESC =	Enter keypad application mode

ESC >	Exit keypad application mode

ESC Z	Send identity sequence

ESC [?1l	Reset cursor key application mode

ESC [?2l	Enter VT52 emulation mode

ESC [?5l	Set screen display to 'normal'

ESC [?6l	Set absolute origin mode

ESC [?7l	Set line wraparound off

ESC	[20l	Set line feed/new line to line feed

ESC [4l		Reset insert mode

ESC [=1l	Set screen lines to 24 [Equivalent to Flash HI OFF]

[This is a private Flash escape sequence]

ESC [?1h	Set cursor key application mode

ESC [?5h	Set screen display to 'reverse'

ESC [?6h	Set relative origin mode

ESC [?7h	Set line wraparound on

ESC	[20h	Set line feed/new line to new line

ESC [4h		Set insert mode on

ESC [=1h	Set screen lines to 48 [Equivalent to Flash HI ON]

[This is a private Flash escape sequence]

ESC	[PnA	Move cursor up Pn lines [Pn is optional]

ESC [PnB	Move cursor down Pn lines [Pn is optional]

ESC [PnC	Move cursor right Pn columns [Pn is optional]

ESC [PnD	Move cursor left Pn columns [Pn is optional]

ESC [PnG	Set cursor to column Pn

ESC	[PnE	Identical to ESC E, but with numeric parameter

ESC [Pne	Identical to ESC D, but with numeric parameter

ESC [PnF	Identical to ESC M, but with numeric parameter

ESC [Pl;PcH	Position cursor to line Pl and column Pc

ESC [Pl;Pcf	Identical to ESC [Pl;PcH

ESC [g or ESC [0g	Clear tab at current column

ESC [3g	Clear all tabs

ESC H	Sets a tab at current column

ESC [Pnd	Moves to row Pn

ESC	[Pn`	Moves to column Pn

ESC [PnP	Delete Pn characters. Shift remaining characters to fill.

ESC [TR;BRr	Set scrolling region from row TR [top] to BR [bottom].

ESC [PnM	Delete Pn lines

ESC [0J		Erase from cursor to end of display

ESC	[1J		Erase from start of screen to cursor

ESC [2J		Erase entire display

ESC [0K		Erase from cursor to end of line

ESC [1K		Erase from beginning of line to cursor

ESC [2K		Erase entire line

ESC [PnL	Insert Pn lines

ESC [PnZ	Backtab Pn times

ESC [c		Sends "What are you" response

ESC [5n		Terminal status report

ESC [6n		Cursor position report

ESC [0m		Set normal graphic rendition

ESC	[4m		Set underline attribute

ESC	[7m		Set reverse vide attribute

ESC	[Pnm	All OTHER values less than 7 set reverse video