A 2.5 Meg Socketed Ram Upgrade for the 1040ST

Copyright 1989

Barry Orlando

A Shareware File by Barry Orlando     Dated: August 10, 1989

Introduction

This shareware text file provides a procedure which details a 2.5

megabyte on board RAM upgrade project. The method described replaces 16

256K dynamic RAM chips with 16 one megabit dynamic RAM chips installed

in chip sockets. Reinstallation of the ST's metal shield cover is also

not impaired.

The beauty of this modification is that pin compatibility is simplified

by the use of sockets instead of directly soldering the 1 meg RAM chips

(DRAMs) to the motherboard and by the fact that the removed bank of

sixteen perfectly reusable 256K DRAMs can be resold by you to persons

upgrading 520STfm computers to 1 megabyte.

If you use this upgrade method, I recommend that you also utilize the

high quality low contact resistance sockets that I've specified below

(and not use standard sockets). These are the type and quality which

are used on expansion boards made for the IBM clones.

I originally wrote this procedure to aid me in upgrading my own

1040ST's memory to 2.5 megabytes because I didn't trust myself not

making mistakes and possibly damaging any of the electronic components,

especially the new DRAMs which I didn't consider cheap. I might note

that my 1040ST's pc-board was marked Rev 4, but I don't believe that

any other recent board revisions (if they exist) for boards with 32

256K DRAMs installed should affect this procedure.

Disclaimer of Liability

I make no claim that this modification will work for you. I can

only say that it worked for me. This modification should not be

attempted by anyone except by someone with experience repairing or

building digital electronic circuits. Performing this modification will

be done at your own risk and may void the warranty on your computer.

1

Background

The 256K dynamic ram chip differs from the 1 meg dynamic ram chip by

having one additional connection, namely the address input A9. The

following chip diagram illustrates this and shows pin layouts:

256K                         1meg

------\__/------             ------\__/-----

A8|1.          16|VSS       Din|1.       **18|VSS

Din|2          *15|CAS        WE|2          17|Dout

WE|3           14|Dout      RAS|3         *16|CAS

RAS|4          *13|A6         NC|4          15|A9

A0|5*         *12|A3         A0|5         *14|A8

A2|6*         *11|A4         A1|6*        *13|A7

A1|7*         *10|A5         A2|7*        *12|A6

VCC|8*         * 9|A7         A3|8*        *11|A5

----------------          VCC|9*        *10|A4

---------------

Truth Table for above chips:

____________________________

A0-A9    Address Inputs

CAS      Column Address Strobe

Din      Data In

Dout     Data Out

RAS      Row-Address Strobe

VCC      Power (+5V)

VSS      Ground

WE       Read/Write Input

NC       Not Connected

*        pc-board pin compatible

**       pc-board pin compatible on most chips

The 2.5 megabyte modification essentially reconnects all pins as

previously connected on the lower of two banks of RAM chips plus adds a

new line from the previously unused A9 pin on the ST's MMU (pin 64 of

U15) to all new 1 meg chip pins A9 via a new 33 ohm resistor. Some of

the Address leads are interchanged, but this has no effect on the

operation of the computer.

Major Tools

1. Desoldering iron (I recommend the spring loaded type that

2

incorporates the heat source.)

2. 15W grounded soldering iron (Radio Shack 64-2051 or similar)

3. Wire wrap clip and stripping tool

4. Exacto knife

5. 3-5 power magnifying glass or jewelers eyepiece

Parts Required

1. One 33 ohm resistor, 1/4 watt, 5%.

2. 5 feet length of 30 gauge wire, Kynar wire wrap solid

conductor (Radio Shack 278-502).

3. 16 double contact low profile dual-in-line I.C. sockets (D.C.

Electronics catalog no. T02-18, cost: $0.12 each)

4. Pack of 10 "Socket Wrap ID" (D.C. Electronics catalog no.

16-ID, cost: $2.05) You can get in touch with D.C. Electronics at

the following location:

D.C. Electronics

P.O. Box 3203

Scottsdale, AZ  85257

(800) 423-0070

5. 16 one megabit dynamic RAM chips, 120ns (I used Hitachi Part

no. HM511000-12, and paid $24.00 each plus state tax and $4.00

shipping from Ed at Computronix on Dec 16, 1988). You can get in

touch with Ed at the following location:

Computronix

2151 A O'Toole Ave.

San Jose, CA 95131

(408) 943-0771

Work Area

Prepare your work area by placing a towel on your flat working

surface and cover it with a layer of aluminum wrapping foil. This

provides padding for the components and a conductor for static

electricity.

Precautions

Since the slightest static discharge could easily damage

components on your computers pc-board, always keep one hand on the

metal foil bonded along the outer rim of the pc-board before and during

touching any part of the pc-board or its components. Avoid working on

3

this project when the humidity is low.

Procedure

1. After removing the pc-board from the insides of your computer,

desolder and remove the front 16 256K chips (U33 through U45 and U16

through U30).

2. Using the Exacto knife, cut the following traces on the top side of

the pc-board:

a. Between U30's pin 1 and U61's pin 1.

b. Between pin 14's of U33 through U45 and U16 through U30 and

other pin 14's of U46 through U61.

c. Between the thru-hole (nearest to pin 1 in center of U32) of

U32 and U46's pin 1. (Note: This thru-hole traces to U32's pin 1

on the underside of the board.)

3. Using the Exacto knife, cut the following traces on the bottom side

of the pc-board:

a. Between U30's pin 1 and the 33 ohm resistor R76.

b. Between U32's pins 2 and 14. Do the same for U33 through U45

and U16 through U30.

c. Between U30's pin 4 and U15's pin 8. (Note: Cut the trace after

it goes through the board to the bottom side near resistor R91,

but before it reappears on the top of the board via another

thru-hole.

d. Between U32's pin 14 and the thru-hole located between U32 and

U33.

4. Preparing New Sockets and Soldering Them Inplace:

a. On all 16 new sockets, remove the internals of pin 4.

b. On all 16 new sockets, bend all pins 1 and 17 flat against

their bottom then up against their outer side.

c. On 14 of the new sockets, bend pin 18's flat against their

bottom and pointing straight out to the side.

d. On the remaining two, bend both pin 18's flat against their

bottom but back and down so it will just reach and fit into the

pc-board hole of it's adjacent pin (pin 17). These two sockets

will be installed and soldered with the socket's pin 18 going into

the old 256K chip's pin 16 hole below the new socket's pin 17.

Trial fit these two sockets at this time.

e. Using a pair of scissors, cut pieces of "Socket Wrap-ID"

plastic markers for sandwiching them between the new sockets and

the pc-board during installation. These little plastic markers

should be cut so as to electrically isolate the socket's pins 1

and 17 from leads and traces on the pc-board, and from pin 18 of

the socket for the two separately prepared sockets above, but also

so as not to interfere with seating the socket's four pads flush

with the top of the pc-board upon installation.

4

f. Solder the two sockets with the bent back pin 18 and markers

into U30 and U33.

g. Solder the remaining sockets with markers into U34 through U45

and U16 through U29. Pin 18 of these socket's must be soldered to

the top of the board at the nearest thru-hole that the pin 18 just

barely reaches. Use plenty of heat as these thru-holes are big

heat sinks.

5. Mounting The New 33 Ohm Resistor:

Install a new 33 ohm resistor on the pc-board by drilling two

small holes from the top at points near capacitor C55 and resistor

R70. Make sure you don't drill through a trace on the other side

by holding the pc-board up to a bright light and marking the spots

with a pencil.

6. Wiring:

Wire the following connections on the top side of the board:

a. Between U32's new socket's pins 1 and 17 and a thru-hole on the

pc-board between U32 and U33. This thru hole is the only one that

has a trace that runs to another thru-hole between U46 and U47 on

the top side of the board.

b. Between U16's new socket's pins 1 and 17, and pin 2 of U54.

Solder the wire directly to the side of the chips pin 2.

c. Between each of the remaining new socket's pin 1 and 17 and a

thru-hole in back of each 256K RAM chip (i.e., U47 through U53 and

U55 through U61). These thru-holes are the ones that you will find

traces back to either pin 2 or pin 14 of U47 through U61 and then

trace over toward U22, U23, U26, and U27. Actually, pins 2 and 14

are tied together on the bottom of the board for every 256K RAM

chip still installed. So that's 14 more wires altogether (i.e.,

each new RAM chip's pins 1 and 17 will be connected to the pins 2

and 14 of the 256K chip directly in back of it.

Wire the following connections on the bottom side of the board

(Note: all pin numbers here refer to the pin numbers on the

pc-board, not the pin numbers of the new sockets or new ram

chips):

d. Between U15's pin 64 and one side of the new 33 ohm resistor.

e. Between all pin 14's of U32 through U45 and U16 through U30,

and the other side of the new 33 ohm resistor.

f. Between all pin 2's of U32 through U45 and U16 through U30, and

the thru-hole near R91 which had its trace cut to U30's pin 4.

This thru-hole traces back on top of the pc-board under the bank

of resistors and over to U15.

g. Between U61's pin 1, U30's pin 4, and the cut trace side of

resistor R76.

h. Between U30's pin 1 and U61's pin 3.

5

7. Inspect all solder joints with the magnifier for solder shorts and

faulty connections. Verify that all connections were made correct.

8. Install the new 1 meg RAM chips into their new sockets.

9.That's it! Clean isn't it? Now sit back and admire your work.

10.Reassemble the computer and reconnect the monitor and power cord and

let it whirl!

Shareware Policy

I am distributing this documentation as a $15.00 Shareware File.

Please feel free to re-post on your favorite Information Service or

local BBS. I ask only that you do so in its original unaltered form.

If you have followed these instructions and everything worked for you,

I'm sure you will appreciate the effort I put into documenting this

project and the money it has saved you from buying one of those

expansion boards on the market or by the time you could have spent

trying to figure such a project out by yourself. Please send your

shareware user registration fee to:

Barry Orlando

1120 Deerfield Dr.

Napa, Ca  94558

All registered users will be sent free of charge, a utility disk and

color photographic prints of the completed project. The utility disk

includes a program to test the new RAM and instructions on how to

further modify the computer to a full 4 megabytes.  The Utility Disk

contains the following support files for the socketed 2.5 meg 1040ST

modification:

1. SYSTAT7.PRG - A handy public domain utility which shows

some neat things going on inside of your computer. Of special

interest is the size of the ram.

2. BANKSTAT.TOS - A program that displays a table of possible

memory configurations which are consistent with those

published in the hardware specification from Atari Corp.

(Developer's Docs). The program also reads into memory and

displays your computer's current memory configuration for

reference.

3. BANKSTAT.C - The C source code for BANKSTAT.TOS.

4. MEMTESTR.TOS - A program that performs a dynamic test of

Bank 0 (the  new memory bank of 1 megabit chips installed by

25MGUPGD.DOC) for data integraty and reports any defective

DRAMs including the specific IC number.

5. MEMTESTR.C - The C source code for MEMTESTR.TOS.

6. 4MGUPGD.TXT - A text file that provides instructions for

6

upgrading the 2.5 meg 1040ST to 4 megs.

The two 8x10 photographic color prints show the completed 2.5 meg

modification on my 1040ST (One print for each side of the motherboard).

These prints may be helpful since they show how the completed wiring

should look.

Good Luck!

7