RATEHD.PRG & RATEHD.DOC

Copyright 1989-1991  ICD, Inc.

This document and the program RATEHD.PRG, may be distributed by

any non-commercial means (may not be sold) as long as the they

are distributed together and no modifications are made.

Language translations are allowed.

The copyright holder is:

ICD, Inc.

1220 Rock Street

Rockford, IL  61101-1437

Voice:  (815) 968-2228

BBS:    (815) 968-2229

FAX:    (815) 968-6888

Orders: (800) 373-7700   NO TECH SUPPORT on this number!!!

RATEHD.PRG was written in Personal Pascal, a product of ICD, Inc.

-----------------------------------------------------------------

RATEHD was written to show performance characteristics of

various hard disk drives.  It was developed at ICD to allow fair

comparison when evaluating hard disk drive mechanisms, embedded

SCSI drives and SCSI controllers when used on the Atari ST.  It

does not go through a hard disk handler (like ICDBOOT.SYS) so it

is not useful in rating the speed of different handlers.

Generally, the ICD hard disk handler provides a significant

speed improvement in actual use due to its caching.

We were a bit apprehensive about releasing this to the public

because of our fears that people would not understand it and

will use it for the wrong purpose.  It is intended as a

comparison test and should not be used to verify a drive or

controller against printed specifications.  Drive manufacturers

test their drives on specialized test equipment and not on an

Atari ST.  Data rates are also usually shown as a burst rate.

What we are measuring is the sustained rate, which is closer to

the top end in real life usage.

Our final decision to release RATEHD was twofold:

1) There is too much misinformation out there when it comes to

computers and hard drives.  There needed to be a uniform

benchmark test for the Atari ST.  Magazine reviews have

typically mislead the public, not maliciously, but because of

the reviewer's ignorance on the subject and lack of

scientific testing methods.  Now you have a benchmark; just

don't abuse it.

2) When we finished the FA-ST Tape Backup, we realized that a

hard drive system would need a certain amount of finesse to

keep up with the streaming tape.  If the tape had to keep

backing up and waiting for the hard drive to catch up, the

backup time would become laboriously long and almost not

usable.  A data rate of about 300 K/s (kilobytes per second)

is required to keep the tape moving without any stopping and

backing up.  As the data rate gets slower, the tape will begin

to backup and restart several times per partition.  A data

rate of less than 200 K/s would probably become unbearable at

attempting a backup.

We have tested many drives and controllers at their optimum

interleaves and found that Adaptec MFM controllers (including

Atari SH204, Megafile 20) run about 300 K/s if they are formatted

at 1:1 interleave.  Adaptec RLL controllers (including Atari

Megafile 30 and 60) run about 315 K/s at 2:1 interleave.  Omti

MFM controllers move along about 360 K/s using 1:1 with Omti RLL

at about 375 K/s at 1:1.  Embedded controller drives are usually

the speedsters with the Seagate ST138N and ST157N cruising at 550

K/s at 1:1.  The quickest we have measured was a Quantum LP105S

which flies at 1197 K/s which is probably fast enough to do real

time video.  It also had 21 ms average access time and had a 105

Megabyte formatted capacity.

THE PROBLEM WITH MOST TEST METHODS

----------------------------------

There are many variables in something as complex as a computer

with a hard disk drive that can make testing a cumbersome task.

The most obvious way to test hard drive speed is to copy files

from a hard drive to a RAMDISK and then back to a clean hard

drive while timing the process.  Although on the surface a test

like this appears sufficient, it cannot give an accurate picture

of what to expect in performance after using the drive for a

while.  Both hard drives would need to be freshly formatted (no

files yet) and with identical partition sizes.  If not freshly

formatted, the DOS fat table delays and fragmentation would enter

a significant amount of error into the rating equation.  Of

course the same DOS and same computer must also be used.

While this seems like a reasonable test method, it would only

show the data rate (assuming both drives were formatted at their

optimal interleave).  Step rate in this type of test would not be

a measurable factor as it would later on when the files on the

drives become fragmented.  Fragmentation occurs as a hard drive

gets used and files are modified.  They increase in size and

when rewritten to the drive, not all the file parts are in

consecutive sectors anymore, probably not even in the same area

of the disk.  The file eventually becomes a group of "fragments."

This is where the movement of the head from track to track (step

rate measured as average access time) really becomes an

important speed factor.  It would not be an easy task to create

two freshly formatted drives of different manufacture with

identical fragmentation on each drive.

Many of the magazine reviews we have seen give no indication of

the test method.  It is almost as if the numbers were arrived at

by magic.  We now offer RATEHD as a solution to all magazine

reviewe