SUPERCHARGER NOTES

The ICD host adapter software fully supports the Supercharger PC

emulator.  You MUST determine the SCSI ID of your Supercharger.

They are shipped as ID 3.  After setting up your hard drive to

auto-boot with HDUTIL, go to the Config option of that program.

Near the bottom of the screen you will see a line that says 'Skip

SCSI ID's:'.  Normally only ID 6 is skipped; this is the ICD real

time clock.  You MUST also set the ID of your Supercharger here,

then save.  This means that all ICD programs will not do any SCSI

commands on this ID.  (This means that you need no longer hold

down the Supercharger's reset button when booting the computer

with the Supercharger attached).

You must also disable write caching when running the

Supercharger, otherwise you will never write to your hard disk.

This can be accomplished in several ways.  First, you can set up

your booter to disable write caching.  Second, you can

permanently enable write caching and turn it off temporarily with

the DESKTOP.ACC or with the CACHEOFF.PRG.  There is also a

CACHEON.PRG to turn the caching back on.  See below for a more

complete explanation.

If using COLDBOOT.PRG you must also configure it with

CFGCBOOT.PRG to skip the Supercharger's ID.

Please note that the Supercharger is fully compatible with old

ICD Host Adapters.  It is NOT compatible with early release

versions of the ICD AdSCSI ST and AdSCSI Plus ST host adapters.

In addition to the software you will need a hardware upgrade kit

which