CTSPATCH.PRG

Copyright (C) 1989

by Bill Penner

26 August 1989

CTSPATCH.PRG  is  a  program  to  fix  the  TOS 1.4 RS-232 RTS/CTS

handshaking  problem.   The  program  can  be run as an autorun program

(placed  in  an  AUTO  folder  and booted) or run when beginning to use

RTS/CTS handshaking.

Outside  of  the Rsconf() command not allowing the RTS/CTS mode to

be enabled, the RS-232 routines in the TOS 1.4 (April 1989) ROMs appear

to  be  bug  free.  The problem with the RTS/CTS routines is due to the

internal  variable  indicating  the  past state of the CTS line and the

active  edge  interupt register for the CTS line improperly set up when

the  system  initializes  at  powerup.   CTSPATCH.PRG  simply  does the

initialization and exits.

Even  with  this program, many programs may not work with RTS/CTS.

This is primarily due to the various programs being written on a system

in  which  the  RTS/CTS  routines  were not working (and causing system

crashes  when  enabled  with  older  ROM's)  and  so many programs will

automatically turn off RTS/CTS handshaking.