TOSLOAD.PRG - The ICD TOS.IMG Autobooter

------------------------------------------

This program (now supplied with our ST Host Adapter format software and

utilities) is used to autoboot any "TOS.IMG" RAM based operating

system from the Hard Disk.  This allows the new TOS to boot very quickly.

A brief amount of floppy access is required and presently there is no way

around this, but it is very short in duration and booting is quite fast.

Related files

-------------

TOSLOAD.PRG - this documentation file

TOSLOAD.PRG - the loader program from ICD to load the TOS image file

TOS.IMG - the the new TOS image file from Atari Corp.

CAUTO.PRG - from Atari Corp... tells the system to use the AUTO folder on C:

How to set up TOS.IMG to autoboot

---------------------------------

The TOSLOAD.PRG program simply loads the TOS image file into memory and

runs it. If TOS is already RAM-based, this program does nothing. This

means that it will work in the AUTO folder of your hard drive!

There is a catch - TOS.IMG (v1.4) will not autoboot from hard drive

once loaded.  Therefore you will need to format a floppy and

place in its AUTO folder the files ICDBOOT.PRG and CAUTO.PRG (in that

order).  CAUTO.PRG will boot the AUTO folder of drive C and cause the

system to act is if it were autobooted from the hard drive.

Step by step procedure

----------------------

1.   Enable your hard drive for auto booting (run HDUTIL).

2.   Copy the lastest version of TOS.IMG to the root directory of

Drive C.  Then copy TOSLOAD.PRG to the AUTO folder of Drive C.

3.   Prepare a floppy with an empty AUTO folder.  Copy ICDBOOT.PRG to the

AUTO folder.  Then copy CAUTO.PRG (Supplied with the TOS.IMG) to

the AUTO folder.  They must be copied in the above order.

4.   Reboot your computer and the new TOS should load.

How it works

------------

The TOSLOAD.PRG program works by loading in TOS.IMG just like

the floppy loader.  When control is passed to the program, control is

never returned because the old system is blown away.  The new system

sets itself up then starts loading from the AUTO folder.  This is on the

floppy and loads in ICDBOOT.PRG and sets up the Hard Disk Drive for

use.  Then CAUTO.PRG is run which redirects the AUTO folder to drive

C and continues the rest of the booting from that drive.  When the

TOSLOAD.PRG program is run again, it will simply return because it

detects that TOS is now RAM based.

To disable the TOS.IMG boot

---------------------------

Rename the TOSLOAD.PRG file on C:  (TOSLOAD.PR is an easy name since you only

need to backspace.)  Then re-boot.

Copyright notice

----------------

This program is copyright (c) 1988 by ICD INC. and *not* in the Public Domain.