- 1 -

A.  #[4mAppendix#[0m_#[4mfor#[0m_#[4mas2650#[0m_#[4mFrankenstein#[0m_#[4mAssembler#[0m

A.1  #[4mPseudo#[0m_#[4mOperations#[0m

A.1.1  #[4mStandard#[0m_#[4mPseudo#[0m_#[4mOperation#[0m_#[4mMnemonics#[0m

End                                END

File Inclusion                     INCL INCLUDE

If                                 IF

Else                               ELSE

End If                             ENDI

Equate                             EQU

Set                                SET

Org                                ORG

Reserve Memory                     RES RESERVE RMB

Define Byte Data                   BYTE DATA DB FCB

Define Word Data                   DW FDB WORD

Define String Data                 FCC STRING

Define Character Set Translation   CHARSET

Define Character Value             CHARDEF CHD

Use Character Translation          CHARUSE

A.1.2  #[4mMachine#[0m_#[4mDependent#[0m_#[4mPseudo#[0m_#[4mOperations#[0m

A.1.2.1  #[4mDefine#[0m_#[4mAddress#[0m_#[4mConstant#[0m

[Label] ACON expression [, expression] ...

The acon statement generates a two byte integer for each

expression in the expression list.  Each expression is

limited in value from 0 to 32767, values outside this range

will result in an "expression exceeds available field width"

error.  There can be up to 128 expressions on the line,

within the line length limit.  The optional label is set to

the address of the first expression.

A.2  #[4mInstructions#[0m

A.2.1  #[4mInstruction#[0m_#[4mList#[0m

Opcode   Syntax                               Selection Criteria

ADDA      ',' REG  expr

ADDA      ',' REG '*' expr ',' REG ',' '+'

ADDA      ',' REG '*' expr ',' REG ',' '-'

ADDA      ',' REG '*' expr ',' REG

ADDA      ',' REG '*' expr

ADDA      ',' REG expr ',' REG ',' '+'

- 2 -

Opcode   Syntax                               Selection Criteria

ADDA      ',' REG expr ',' REG ',' '-'

ADDA      ',' REG expr ',' REG

ADDI      ',' REG expr

ADDR      ',' REG '*' expr

ADDR      ',' REG expr

ADDZ      REG

ANDA      ',' REG  expr

ANDA      ',' REG '*' expr ',' REG ',' '+'

ANDA      ',' REG '*' expr ',' REG ',' '-'

ANDA      ',' REG '*' expr ',' REG

ANDA      ',' REG '*' expr

ANDA      ',' REG expr ',' REG ',' '+'

ANDA      ',' REG expr ',' REG ',' '-'

ANDA      ',' REG expr ',' REG

ANDI      ',' REG expr

ANDR      ',' REG '*' expr

ANDR      ',' REG expr

ANDZ      REG                                 REG1

ANDZ      REG                                 REG2

ANDZ      REG                                 REG3

BCFA      ',' CONDITION '*' expr              COND0

BCFA      ',' CONDITION '*' expr              COND1

BCFA      ',' CONDITION '*' expr              COND2

BCFA      ',' CONDITION expr                  COND0

BCFA      ',' CONDITION expr                  COND1

BCFA      ',' CONDITION expr                  COND2

BCFR      ',' CONDITION '*' expr              COND0

BCFR      ',' CONDITION '*' expr              COND1

BCFR      ',' CONDITION '*' expr              COND2

BCFR      ',' CONDITION expr                  COND0

BCFR      ',' CONDITION expr                  COND1

BCFR      ',' CONDITION expr                  COND2

BCTA      ',' CONDITION '*' expr

BCTA      ',' CONDITION expr

BCTR      ',' CONDITION '*' expr

BCTR      ',' CONDITION expr

- 3 -

Opcode   Syntax                               Selection Criteria

BDRA      ',' REG '*' expr

BDRA      ',' REG expr

BDRR      ',' REG '*' expr

BDRR      ',' REG expr

BIRA      ',' REG '*' expr

BIRA      ',' REG expr

BIRR      ',' REG '*' expr

BIRR      ',' REG expr

BRNA      ',' REG '*' expr

BRNA      ',' REG expr

BRNR      ',' REG '*' expr

BRNR      ',' REG expr

BSFA      ',' CONDITION '*' expr              COND0

BSFA      ',' CONDITION '*' expr              COND1

BSFA      ',' CONDITION '*' expr              COND2

BSFA      ',' CONDITION expr                  COND0

BSFA      ',' CONDITION expr                  COND1

BSFA      ',' CONDITION expr                  COND2

BSFR      ',' CONDITION '*' expr              COND0

BSFR      ',' CONDITION '*' expr              COND1

BSFR      ',' CONDITION '*' expr              COND2

BSFR      ',' CONDITION expr                  COND0

BSFR      ',' CONDITION expr                  COND1

BSFR      ',' CONDITION expr                  COND2

BSNA      ',' REG '*' expr

BSNA      ',' REG expr

BSNR      ',' REG '*' expr

BSNR      ',' REG expr

BSTA      ',' CONDITION '*' expr

BSTA      ',' CONDITION expr

BSTR      ',' CONDITION '*' expr

BSTR      ',' CONDITION expr

BSXA      '*' expr ',' REG                    REG3

BSXA      expr ',' REG                        REG3

- 4 -

Opcode   Syntax                               Selection Criteria

BXA       '*' expr ',' REG                    REG3

BXA       expr ',' REG                        REG3

COMA      ',' REG  expr

COMA      ',' REG '*' expr ',' REG ',' '+'

COMA      ',' REG '*' expr ',' REG ',' '-'

COMA      ',' REG '*' expr ',' REG

COMA      ',' REG '*' expr

COMA      ',' REG expr ',' REG ',' '+'

COMA      ',' REG expr ',' REG ',' '-'

COMA      ',' REG expr ',' REG

COMI      ',' REG expr

COMR      ',' REG '*' expr

COMR      ',' REG expr

COMZ      REG

CPSL      expr

CPSU      expr

DAR       ',' REG

EORA      ',' REG  expr

EORA      ',' REG '*' expr ',' REG ',' '+'

EORA      ',' REG '*' expr ',' REG ',' '-'

EORA      ',' REG '*' expr ',' REG

EORA      ',' REG '*' expr

EORA      ',' REG expr ',' REG ',' '+'

EORA      ',' REG expr ',' REG ',' '-'

EORA      ',' REG expr ',' REG

EORI      ',' REG expr

EORR      ',' REG '*' expr

EORR      ',' REG expr

EORZ      REG

HALT

IORA      ',' REG  expr

IORA      ',' REG '*' expr ',' REG ',' '+'

IORA      ',' REG '*' expr ',' REG ',' '-'

- 5 -

Opcode   Syntax                               Selection Criteria

IORA      ',' REG '*' expr ',' REG

IORA      ',' REG '*' expr

IORA      ',' REG expr ',' REG ',' '+'

IORA      ',' REG expr ',' REG ',' '-'

IORA      ',' REG expr ',' REG

IORI      ',' REG expr

IORR      ',' REG '*' expr

IORR      ',' REG expr

IORZ      REG

LODA      ',' REG  expr

LODA      ',' REG '*' expr ',' REG ',' '+'

LODA      ',' REG '*' expr ',' REG ',' '-'

LODA      ',' REG '*' expr ',' REG

LODA      ',' REG '*' expr

LODA      ',' REG expr ',' REG ',' '+'

LODA      ',' REG expr ',' REG ',' '-'

LODA      ',' REG expr ',' REG

LODI      ',' REG expr

LODR      ',' REG '*' expr

LODR      ',' REG expr

LODZ      REG                                 REG0

LODZ      REG                                 REG1

LODZ      REG                                 REG2

LODZ      REG                                 REG3

LPSL

LPSU

NOP

PPSL      expr

PPSU      expr

REDC      ',' REG

REDD      ',' REG

REDE      ',' REG expr

- 6 -

Opcode   Syntax                               Selection Criteria

RETC      ',' CONDITION

RETE      ',' CONDITION

RRL       ',' REG

RRR       ',' REG

SPSL

SPSU

STRA      ',' REG  expr

STRA      ',' REG '*' expr ',' REG ',' '+'

STRA      ',' REG '*' expr ',' REG ',' '-'

STRA      ',' REG '*' expr ',' REG

STRA      ',' REG '*' expr

STRA      ',' REG expr ',' REG ',' '+'

STRA      ',' REG expr ',' REG ',' '-'

STRA      ',' REG expr ',' REG

STRR      ',' REG '*' expr

STRR      ',' REG expr

STRZ      REG                                 REG1

STRZ      REG                                 REG2

STRZ      REG                                 REG3

SUBA      ',' REG  expr

SUBA      ',' REG '*' expr ',' REG ',' '+'

SUBA      ',' REG '*' expr ',' REG ',' '-'

SUBA      ',' REG '*' expr ',' REG

SUBA      ',' REG '*' expr

SUBA      ',' REG expr ',' REG ',' '+'

SUBA      ',' REG expr ',' REG ',' '-'

SUBA      ',' REG expr ',' REG

SUBI      ',' REG expr

SUBR      ',' REG '*' expr

SUBR      ',' REG expr

SUBZ      REG

TMI       ',' REG expr

- 7 -

Opcode   Syntax                               Selection Criteria

TPSL      expr

TPSU      expr

WRTC      ',' REG

WRTD      ',' REG

WRTE      ',' REG expr

ZBRR      '*' expr

ZBRR      expr

ZBSR      '*' expr

ZBSR      expr

A.2.2  #[4mRegister#[0m_#[4mand#[0m_#[4mCondition#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  The REG and

CONDITION fields in instructions are restricted to only the

values available as reserved symbols. These are r0 - r3, or

R0 - R3 for registers, and plus, PLUS, minus, MINUS, zero,

ZERO, gt, GT, lt, LT, eq, EQ, un, UN, always, and ALWAYS for

conditions.

A.2.3  #[4mDestination#[0m_#[4mRegister#[0m_#[4mfor#[0m_#[4mIndexed#[0m_#[4mAddressing#[0m  If the

destination register, the one adjacent to the opcode, is not

R0 when one of the absolute indexed addressing modes is

used, an error is issued.

A.2.4  #[4mSelection#[0m_#[4mCriteria#[0m_#[4mKeywords#[0m

REG0 REG1 REG2 REG3

Some instructions are restricted to a subset of

the registers.  Only those instructions with the

specified registers are valid.

COND0 COND1 COND2 COND3

Some instructions are restricted to a subset of

the conditions.  Only those instructions with the

specified conditions are valid.

COND0     EQ, or ZERO

COND1     GT, or PLUS

COND2     LT, or MINUS

COND3     ALWAYS, or UN

- 8 -

A.2.5  #[4mApostrophes#[0m  The apostrophes in the syntax field are

a notation used for the parser generator and are not put in

the assembler source statement.

A.3  #[4mNotes#[0m

A.3.1  #[4mLocation#[0m_#[4mCounter#[0m  The Dollar Sign is used as the

location counter symbol in this assembler.

A.3.2  #[4mMemory#[0m_#[4mAddressing#[0m_#[4mError#[0m_#[4mMessages#[0m

ERROR - expression exceeds available field width

This message occurs when :

- The absolute branch address is to an invalid

address, one greater than 32767.

- The absolute or relative address was not in the

present memory page.

- The relative offset was outside the range that can

be represented in 7 bits.

ERROR - instruction crosses page boundary

The first and last bytes of an instruction are on

different sides of an 8k page.

WARNING - Page Boundary

The first byte of an instruction is on a 8k page

boundry.

A.3.3  #[4mPage#[0m_#[4mWraparound#[0m_#[4mfor#[0m_#[4mRelative#[0m_#[4mAddressing#[0m  The

wraparound of an effective address, where a relative address

from an instruction near a page boundary accesses memory at

the other side of the page, is not supported in this

assembler and will result in one or more error messages.

This does not apply to the ZBRR and ZBSR instructions.

Memory destinations for these can range from 0 to $3f and

$1fc0 to $1fff.

A.3.4  #[4mRelational#[0m_#[4mOperators#[0m  The relational operator

keywords GT, LT, and EQ are not available.  The '<', '>',

and '==' special character representations must be used.

A.3.5  #[4mReserved#[0m_#[4mSymbols#[0m

- 9 -

A.3.5.1  #[4mMachine#[0m_#[4mDependent#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  ALWAYS AND

DEFINED EQ GE GT HIGH LE LOW LT MINUS MOD NE NOT OR PLUS R0

R1 R2 R3 SHL SHR UN XOR ZERO always and defined eq ge gt

high le low lt minus mod ne not or plus r0 r1 r2 r3 shl shr

un xor zero

CONTENTS

A.  Appendix for as2650 Frankenstein Assembler...........  1

A.1  Pseudo Operations...............................  1

A.2  Instructions....................................  1

A.3  Notes...........................................  8

- i -