- 1 -

A.  #[4mAppendix#[0m_#[4mfor#[0m_#[4mas6805#[0m_#[4mFrankenstein#[0m_#[4mAssembler#[0m

A.1  #[4mPseudo#[0m_#[4mOperations#[0m

A.1.1  #[4mStandard#[0m_#[4mPseudo#[0m_#[4mOperation#[0m_#[4mMnemonics#[0m

End                                END

File Inclusion                     INCL INCLUDE

If                                 IF

Else                               ELSE

End If                             ENDI

Equate                             EQU

Set                                SET

Org                                ORG

Reserve Memory                     RESERVE RMB

Define Byte Data                   BYTE DB FCB

Define Word Data                   DW FDB WORD

Define String Data                 FCC STRING

Define Character Set Translation   CHARSET

Define Character Value             CHARDEF CHD

Use Character Translation          CHARUSE

A.1.2  #[4mMachine#[0m_#[4mDependent#[0m_#[4mPseudo#[0m_#[4mOperations#[0m

A.1.2.1  #[4mInstruction#[0m_#[4mSet#[0m_#[4mSelection#[0m

CPU string

The instruction set can be specified in the source file with

the CPU pseudooperation.  The string, delimited by quotes or

apostrophes, is scanned for a substring which selects which

instruction set is used.  When the program is invoked, this

operation is performed on the name of the program, then the

-p optional arguement, if any, and then any CPU statements.

The last one selects which subset of the instructions the

assembler will accept.  The instruction set can be changed

at any place in the source file.

Instruction Set          Substrings

68hc05                   hc HC

146805                   4

6305                     3

6805                     05 l05 L05

A.2  #[4mInstructions#[0m

A.2.1  #[4mInstruction#[0m_#[4mList#[0m

- 2 -

Opcode   Syntax                    Selection Criteria

ADC       '#' expr

ADC       expr                     DIRECT

ADC       expr                     EXTENDED

ADC       indexed                  INDEX0

ADC       indexed                  INDEX1

ADC       indexed                  INDEX2

ADD       '#' expr

ADD       expr                     DIRECT

ADD       expr                     EXTENDED

ADD       indexed                  INDEX0

ADD       indexed                  INDEX1

ADD       indexed                  INDEX2

AND       '#' expr

AND       expr                     DIRECT

AND       expr                     EXTENDED

AND       indexed                  INDEX0

AND       indexed                  INDEX1

AND       indexed                  INDEX2

ASL       expr                     DIRECT

ASL       indexed                  INDEX0

ASL       indexed                  INDEX1

ASLA

ASLX

ASR       expr                     DIRECT

ASR       indexed                  INDEX0

ASR       indexed                  INDEX1

ASRA

ASRX

BCC       expr

BCLR      expr ',' expr

BCS       expr

BEQ       expr

BHCC      expr

- 3 -

Opcode   Syntax                    Selection Criteria

BHCS      expr

BHI       expr

BHS       expr

BIH       expr

BIL       expr

BIT       '#' expr

BIT       expr                     DIRECT

BIT       expr                     EXTENDED

BIT       indexed                  INDEX0

BIT       indexed                  INDEX1

BIT       indexed                  INDEX2

BLO       expr

BLS       expr

BMC       expr

BMI       expr

BMS       expr

BNE       expr

BPL       expr

BRA       expr

BRCLR     expr ',' expr ',' expr

BRN       expr

BRSET     expr ',' expr ',' expr

BSET      expr ',' expr

BSR       expr

CLC

CLI

- 4 -

Opcode   Syntax                    Selection Criteria

CLR       expr                     DIRECT

CLR       indexed                  INDEX0

CLR       indexed                  INDEX1

CLRA

CLRX

CMP       '#' expr

CMP       expr                     DIRECT

CMP       expr                     EXTENDED

CMP       indexed                  INDEX0

CMP       indexed                  INDEX1

CMP       indexed                  INDEX2

COM       expr                     DIRECT

COM       indexed                  INDEX0

COM       indexed                  INDEX1

COMA

COMX

CPX       '#' expr

CPX       expr                     DIRECT

CPX       expr                     EXTENDED

CPX       indexed                  INDEX0

CPX       indexed                  INDEX1

CPX       indexed                  INDEX2

DAA                                INSTDAA

DEC       expr                     DIRECT

DEC       indexed                  INDEX0

DEC       indexed                  INDEX1

DECA

DECX

EOR       '#' expr

EOR       expr                     DIRECT

EOR       expr                     EXTENDED

EOR       indexed                  INDEX0

EOR       indexed                  INDEX1

EOR       indexed                  INDEX2

- 5 -

Opcode   Syntax                    Selection Criteria

INC       expr                     DIRECT

INC       indexed                  INDEX0

INC       indexed                  INDEX1

INCA

INCX

JMP       expr                     DIRECT

JMP       expr                     EXTENDED

JMP       indexed                  INDEX0

JMP       indexed                  INDEX1

JMP       indexed                  INDEX2

JSR       expr                     DIRECT

JSR       expr                     EXTENDED

JSR       indexed                  INDEX0

JSR       indexed                  INDEX1

JSR       indexed                  INDEX2

LDA       '#' expr

LDA       expr                     DIRECT

LDA       expr                     EXTENDED

LDA       indexed                  INDEX0

LDA       indexed                  INDEX1

LDA       indexed                  INDEX2

LDX       '#' expr

LDX       expr                     DIRECT

LDX       expr                     EXTENDED

LDX       indexed                  INDEX0

LDX       indexed                  INDEX1

LDX       indexed                  INDEX2

LSL       expr                     DIRECT

LSL       indexed                  INDEX0

LSL       indexed                  INDEX1

LSLA

LSLX

LSR       expr                     DIRECT

LSR       indexed                  INDEX0

LSR       indexed                  INDEX1

- 6 -

Opcode   Syntax                    Selection Criteria

LSRA

LSRX

MUL                                INSTMUL

NEG       expr                     DIRECT

NEG       indexed                  INDEX0

NEG       indexed                  INDEX1

NEGA

NEGX

NOP

ORA       '#' expr

ORA       expr                     DIRECT

ORA       expr                     EXTENDED

ORA       indexed                  INDEX0

ORA       indexed                  INDEX1

ORA       indexed                  INDEX2

ROL       expr                     DIRECT

ROL       indexed                  INDEX0

ROL       indexed                  INDEX1

ROLA

ROLX

ROR       expr                     DIRECT

ROR       indexed                  INDEX0

ROR       indexed                  INDEX1

RORA

RORX

RSP

RTI

RTS

SBC       '#' expr

SBC       expr                     DIRECT

- 7 -

Opcode   Syntax                    Selection Criteria

SBC       expr                     EXTENDED

SBC       indexed                  INDEX0

SBC       indexed                  INDEX1

SBC       indexed                  INDEX2

SEC

SEI

STA       expr                     DIRECT

STA       expr                     EXTENDED

STA       indexed                  INDEX0

STA       indexed                  INDEX1

STA       indexed                  INDEX2

STOP                               INSTSTWA

STX       expr                     DIRECT

STX       expr                     EXTENDED

STX       indexed                  INDEX0

STX       indexed                  INDEX1

STX       indexed                  INDEX2

SUB       '#' expr

SUB       expr                     DIRECT

SUB       expr                     EXTENDED

SUB       indexed                  INDEX0

SUB       indexed                  INDEX1

SUB       indexed                  INDEX2

SWI

TAX

TST       expr                     DIRECT

TST       indexed                  INDEX0

TST       indexed                  INDEX1

TSTA

TSTX

TXA

WAIT                               INSTSTWA

A.2.2  #[4mSelection#[0m_#[4mCriteria#[0m_#[4mKeywords#[0m

- 8 -

DIRECT              The direct addressing mode can be

used if the expression has a value

between 0 and 255. The expression

will be treated as a two byte long

value if it is not defined when the

statement is processed in the first

pass.

EXTENDED            The expression can be a 2 byte long

value.

INDEX0              The Indexed addressing mode with no

offset can be used.

INDEX1              The Indexed addressing mode with a

one byte offset can be used.

INDEX2              The Indexed addressing mode with

two byte offset can be used.

INSTSTWA            The instruction is only available

in the 146805, 6305, and 68hc05

instruction sets.

INSTMUL             The instruction is only available

in the 68hc05 instruction set.

INSTDAA             The instruction is only available

in the 6305 instruction set.

A.2.3  #[4mApostrophes#[0m  The apostrophes in the syntax field are

a notation used for the parser generator and are not put in

the assembler source statement.

A.3  #[4mNotes#[0m

A.3.1  #[4mBit#[0m_#[4mNumbers#[0m  The bit number expression in the BSET,

BCLR, BRCLR, BRSET operations has to have value defined when

the instruction is read in the first pass.  The value must

be between 0 and 7.

A.3.2  #[4mIndexed#[0m_#[4mAddressing#[0m  The indexed addressing mode is

represented in two ways.

First, with no offset, the form ",X" is used.

Second, the form "expression, X" is used for both the one

and two byte offsets modes. The expression will be treated

as a two byte long value if it is not defined when the

statement is processed in the first pass.

- 9 -

A.3.3  #[4mReserved#[0m_#[4mSymbols#[0m

A.3.3.1  #[4mMachine#[0m_#[4mDependent#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  X x

A.3.3.2  #[4mStandard#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  AND DEFINED EQ GE GT

HIGH LE LOW LT MOD NE NOT OR SHL SHR XOR and defined eq ge

gt high le low lt mod ne not or shl shr xor

CONTENTS

A.  Appendix for as6805 Frankenstein Assembler...........  1

A.1  Pseudo Operations...............................  1

A.2  Instructions....................................  1

A.3  Notes...........................................  8

- i -