- 1 -

A.  #[4mAppendix#[0m_#[4mfor#[0m_#[4mas7000#[0m_#[4mFrankenstein#[0m_#[4mAssembler#[0m

A.1  #[4mPseudo#[0m_#[4mOperations#[0m

A.1.1  #[4mStandard#[0m_#[4mPseudo#[0m_#[4mOperation#[0m_#[4mMnemonics#[0m

End                                END

File Inclusion                     INCL INCLUDE

If                                 IF

Else                               ELSE

End If                             ENDI

Equate                             EQU

Set                                SET

Org                                ORG

Reserve Memory                     RESERVE RMB

Define Byte Data                   BYTE DB FCB

Define Word Data                   DW FDB WORD

Define String Data                 FCC STRING

Define Character Set Translation   CHARSET

Define Character Value             CHARDEF CHD

Use Character Translation          CHARUSE

A.2  #[4mInstructions#[0m

A.2.1  #[4mDifferences#[0m_#[4mwith#[0m_#[4mthe#[0m_#[4mTI#[0m_#[4mdefined#[0m_#[4mSyntax#[0m

- 2 -

TI syntax                          as7000 syntax

op % immediate , register          op # immediate , register

op % immed , register, jump dest   op # immed , register , jump dest

op @ label                         op label

op @ label ( B )                   op expr ( B )

op *register                       op [ register ]

movd % immed ( B ) , register      movd # immed (B), register

Constants

?0101010                           %01010101

>89ab                              $89ab

''''                               "'"

Comments

after the last operand             everything following a semicolon

Symbols

R0-R255                            equ with value between 0 and $ff

R0-R255                            0 to 255, Not Recommended !

P0-P255                            equ with value between $100 and $1ff

P0-P255                            256 to 511, Not Recommended !

A.2.2  #[4mInstruction#[0m_#[4mList#[0m

Opcode   Syntax                           Selection Criteria

ADC       '#' expr ',' expr

ADC       '#' expr ',' expr               DSTA

ADC       '#' expr ',' expr               DSTB

ADC       expr ',' expr

ADC       expr ',' expr                   DSTA

ADC       expr ',' expr                   DSTB

ADC       expr ',' expr                   SRCB DSTA

ADD       '#' expr ',' expr

ADD       '#' expr ',' expr               DSTA

ADD       '#' expr ',' expr               DSTB

ADD       expr ',' expr

ADD       expr ',' expr                   DSTA

ADD       expr ',' expr                   DSTB

ADD       expr ',' expr                   SRCB DSTA

AND       '#' expr ',' expr

- 3 -

Opcode   Syntax                           Selection Criteria

AND       '#' expr ',' expr               DSTA

AND       '#' expr ',' expr               DSTB

AND       expr ',' expr

AND       expr ',' expr                   DSTA

AND       expr ',' expr                   DSTB

AND       expr ',' expr                   SRCB DSTA

ANDP      '#' expr ',' expr

ANDP      expr ',' expr                   SRCA

ANDP      expr ',' expr                   SRCB

BR        '[' expr ']'

BR        expr '(' REG ')'                SRCB

BR        expr

BTJO      '#' expr ',' expr ',' expr

BTJO      '#' expr ',' expr ',' expr      DSTA

BTJO      '#' expr ',' expr ',' expr      DSTB

BTJO      expr ',' expr ',' expr

BTJO      expr ',' expr ',' expr          DSTA

BTJO      expr ',' expr ',' expr          DSTB

BTJO      expr ',' expr ',' expr          SRCB DSTA

BTJOP     '#' expr ',' expr ',' expr

BTJOP     expr ',' expr ',' expr          SRCA

BTJOP     expr ',' expr ',' expr          SRCB

BTJZ      '#' expr ',' expr ',' expr

BTJZ      '#' expr ',' expr ',' expr      DSTA

BTJZ      '#' expr ',' expr ',' expr      DSTB

BTJZ      expr ',' expr ',' expr

BTJZ      expr ',' expr ',' expr          DSTA

BTJZ      expr ',' expr ',' expr          DSTB

BTJZ      expr ',' expr ',' expr          SRCB DSTA

BTJZP     '#' expr ',' expr ',' expr

BTJZP     expr ',' expr ',' expr          SRCA

BTJZP     expr ',' expr ',' expr          SRCB

CALL      '[' expr ']'

CALL      expr '(' REG ')'                SRCB

CALL      expr

CLR       expr

CLR       expr                            DSTA

CLR       expr                            DSTB

- 4 -

Opcode   Syntax                           Selection Criteria

CLRC

CMP       '#' expr ',' expr

CMP       '#' expr ',' expr               DSTA

CMP       '#' expr ',' expr               DSTB

CMP       expr ',' expr

CMP       expr ',' expr                   DSTA

CMP       expr ',' expr                   DSTB

CMP       expr ',' expr                   SRCB DSTA

CMPA      '[' expr ']'

CMPA      expr '(' REG ')'                SRCB

CMPA      expr

DAC       '#' expr ',' expr

DAC       '#' expr ',' expr               DSTA

DAC       '#' expr ',' expr               DSTB

DAC       expr ',' expr

DAC       expr ',' expr                   DSTA

DAC       expr ',' expr                   DSTB

DAC       expr ',' expr                   SRCB DSTA

DEC       expr

DEC       expr                            DSTA

DEC       expr                            DSTB

DECD      expr

DECD      expr                            DSTA

DECD      expr                            DSTB

DINT

DJNZ      expr ',' expr

DJNZ      expr ',' expr                   DSTA

DJNZ      expr ',' expr                   DSTB

DSB       '#' expr ',' expr

DSB       '#' expr ',' expr               DSTA

DSB       '#' expr ',' expr               DSTB

DSB       expr ',' expr

DSB       expr ',' expr                   DSTA

DSB       expr ',' expr                   DSTB

DSB       expr ',' expr                   SRCB DSTA

EINT

IDLE

- 5 -

Opcode   Syntax                           Selection Criteria

INC       expr

INC       expr                            DSTA

INC       expr                            DSTB

INV       expr

INV       expr                            DSTA

INV       expr                            DSTB

JC        expr

JEQ       expr

JGE       expr

JGT       expr

JHS       expr

JL        expr

JLT       expr

JMP       expr

JN        expr

JNC       expr

JNE       expr

JNZ       expr

JP        expr

JPZ       expr

JZ        expr

LDA       '[' expr ']'

LDA       expr '(' REG ')'                SRCB

LDA       expr

LDSP

MOV       '#' expr ',' expr

MOV       '#' expr ',' expr               DSTA

- 6 -

Opcode   Syntax                           Selection Criteria

MOV       '#' expr ',' expr               DSTB

MOV       expr ',' expr

MOV       expr ',' expr                   DSTA

MOV       expr ',' expr                   DSTB

MOV       expr ',' expr                   SRCA

MOV       expr ',' expr                   SRCA DSTB

MOV       expr ',' expr                   SRCB

MOV       expr ',' expr                   SRCB DSTA

MOVD      '#' expr '(' REG ')' ',' expr

MOVD      '#' expr ',' expr

MOVD      expr ',' expr

MOVP      '#' expr ',' expr

MOVP      expr ',' expr                   DSTA

MOVP      expr ',' expr                   DSTB

MOVP      expr ',' expr                   SRCA

MOVP      expr ',' expr                   SRCB

MPY       '#' expr ',' expr

MPY       '#' expr ',' expr               DSTA

MPY       '#' expr ',' expr               DSTB

MPY       expr ',' expr

MPY       expr ',' expr                   DSTA

MPY       expr ',' expr                   DSTB

MPY       expr ',' expr                   SRCB DSTA

NOP

OR        '#' expr ',' expr

OR        '#' expr ',' expr               DSTA

OR        '#' expr ',' expr               DSTB

OR        expr ',' expr

OR        expr ',' expr                   DSTA

OR        expr ',' expr                   DSTB

OR        expr ',' expr                   SRCB DSTA

ORP       '#' expr ',' expr

ORP       expr ',' expr                   SRCA

ORP       expr ',' expr                   SRCB

POP        STATUS

POP       expr

POP       expr                            DSTA

POP       expr                            DSTB

PUSH       STATUS

- 7 -

Opcode   Syntax                           Selection Criteria

PUSH      expr

PUSH      expr                            DSTA

PUSH      expr                            DSTB

RETI

RETS

RL        expr

RL        expr                            DSTA

RL        expr                            DSTB

RLC       expr

RLC       expr                            DSTA

RLC       expr                            DSTB

RR        expr

RR        expr                            DSTA

RR        expr                            DSTB

RRC       expr

RRC       expr                            DSTA

RRC       expr                            DSTB

SBB       '#' expr ',' expr

SBB       '#' expr ',' expr               DSTA

SBB       '#' expr ',' expr               DSTB

SBB       expr ',' expr

SBB       expr ',' expr                   DSTA

SBB       expr ',' expr                   DSTB

SBB       expr ',' expr                   SRCB DSTA

SETC

STA       '[' expr ']'

STA       expr '(' REG ')'                SRCB

STA       expr

STSP

SUB       '#' expr ',' expr

SUB       '#' expr ',' expr               DSTA

SUB       '#' expr ',' expr               DSTB

SUB       expr ',' expr

SUB       expr ',' expr                   DSTA

SUB       expr ',' expr                   DSTB

SUB       expr ',' expr                   SRCB DSTA

- 8 -

Opcode   Syntax                           Selection Criteria

SWAP      expr

SWAP      expr                            DSTA

SWAP      expr                            DSTB

TRAP      expr

TSTA

TSTB

XCHB      expr

XCHB      expr                            DSTA

XCHB      expr                            DSTB

XOR       '#' expr ',' expr

XOR       '#' expr ',' expr               DSTA

XOR       '#' expr ',' expr               DSTB

XOR       expr ',' expr

XOR       expr ',' expr                   DSTA

XOR       expr ',' expr                   DSTB

XOR       expr ',' expr                   SRCB DSTA

XORP      '#' expr ',' expr

XORP      expr ',' expr                   SRCA

XORP      expr ',' expr                   SRCB

A.2.3  #[4mSelection#[0m_#[4mCriteria#[0m_#[4mKeywords#[0m

DSTA                The instruction will use a short

form if the A register is used as

the destination register.

DSTB                The instruction will use a short

form if the B register is used as

the destination register.

SRCA                The instruction will use a short

form if the A register is used as

the source register.

SRCB                The instruction will use a short

form if the B register is used as

the source register.

A.2.4  #[4mApostrophes#[0m  The apostrophes in the syntax field are

a notation used for the parser generator and are not put in

the assembler source statement.

- 9 -

A.3  #[4mNotes#[0m

A.3.1  #[4mReserved#[0m_#[4mSymbols#[0m

A.3.1.1  #[4mMachine#[0m_#[4mDependent#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  A B ST a b st

A.3.1.2  #[4mStandard#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  AND DEFINED EQ GE GT

HIGH LE LOW LT MOD NE NOT OR SHL SHR XOR and defined eq ge

gt high le low lt mod ne not or shl shr xor

CONTENTS

A.  Appendix for as7000 Frankenstein Assembler...........  1

A.1  Pseudo Operations...............................  1

A.2  Instructions....................................  1

A.3  Notes...........................................  9

- i -