- 1 -

A.  #[4mAppendix#[0m_#[4mfor#[0m_#[4mas8051#[0m_#[4mFrankenstein#[0m_#[4mAssembler#[0m

A.1  #[4mPseudo#[0m_#[4mOperations#[0m

A.1.1  #[4mStandard#[0m_#[4mPseudo#[0m_#[4mOperation#[0m_#[4mMnemonics#[0m

End                                END

File Inclusion                     INCL INCLUDE

If                                 IF

Else                               ELSE

End If                             ENDI

Equate                             EQU

Set                                SET

Org                                ORG

Reserve Memory                     RESERVE RMB

Define Byte Data                   BYTE DB FCB

Define Word Data                   DW FDB WORD

Define String Data                 FCC STRING

Define Character Set Translation   CHARSET

Define Character Value             CHARDEF CHD

Use Character Translation          CHARUSE

A.1.2  #[4mMachine#[0m_#[4mDependent#[0m_#[4mPseudo#[0m_#[4mOperations#[0m

A.1.2.1  #[4mDefine#[0m_#[4mWord#[0m_#[4mData#[0m  The Define Word Data pseudo-

operations generate the byte reversed form for 16 bit

constants.  The first byte is the low order half, followed

by the high order byte.

A.2  #[4mInstructions#[0m

A.2.1  #[4mInstruction#[0m_#[4mList#[0m

Opcode   Syntax                          Selection Criteria

ACALL    expr

ADD      A ',' '#' expr

ADD      A ',' '@' REG                   R01

ADD      A ',' REG                       R07

ADD      A ',' expr

ADDC     A ',' '#' expr

ADDC     A ',' '@' REG                   R01

ADDC     A ',' REG                       R07

ADDC     A ',' expr

AJMP     expr

- 2 -

Opcode   Syntax                          Selection Criteria

ANL      A ',' '#' expr

ANL      A ',' '@' REG                   R01

ANL      A ',' REG                       R07

ANL      A ',' expr

ANL      C ',' '/' bit

ANL      C ',' '/' expr

ANL      C ',' bit

ANL      C ',' expr

ANL      expr ',' '#' expr

ANL      expr ','A

CJNE     A ',' '#' expr ',' expr

CJNE     A ',' expr ',' expr

CJNE     '@' REG ',' '#' expr ',' expr   R01

CJNE     REG ',' '#' expr ',' expr       R07

CLR      A

CLR      C

CLR      bit

CLR      expr

CPL      A

CPL      C

CPL      bit

CPL      expr

DA       A

DEC      '@' REG                         R01

DEC      A

DEC      REG                             R07

DEC      expr

DIV      AB

DJNZ     REG ',' expr                    R07

DJNZ     expr ',' expr

INC      '@' REG                         R01

INC      A

INC      DPTR

INC      REG                             R07

INC      expr

JB       bit ',' expr

JB       expr ',' expr

- 3 -

Opcode   Syntax                          Selection Criteria

JBC      bit ',' expr

JBC      expr ',' expr

JC       expr

JMP      '@' A '+' DPTR

JNB      bit ',' expr

JNB      expr ',' expr

JNC      expr

JNZ      expr

JZ       expr

LCALL    expr

LJMP     expr

MOV      '@' REG ',' '#' expr            R01

MOV      '@' REG ',' A                   R01

MOV      '@' REG ',' expr                R01

MOV      A ',' '#' expr

MOV      A ',' '@' REG                   R01

MOV      A ',' REG                       R07

MOV      A ',' expr

MOV      C ',' bit

MOV      C ',' expr

MOV      DPTR ',' '#' expr

MOV      REG ',' A                       R07

MOV      REG ',' '#' expr                R07

MOV      REG ',' expr                    R07

MOV      bit ',' C

MOV      expr ',' '#' expr

MOV      expr ',' '@' REG                R01

MOV      expr ',' A

MOV      expr ',' C

MOV      expr ',' REG                    R07

MOV      expr ',' expr

MOVC     A ',' '@' A '+' DPTR

MOVC     A ',' '@' A '+' PC

MOVX     '@' DPTR ',' A

MOVX     '@' REG ',' A                   R01

- 4 -

Opcode   Syntax                          Selection Criteria

MOVX     A ',' '@' DPTR

MOVX     A ',' '@' REG                   R01

MUL      AB

NOP

ORL      A ',' '#' expr

ORL      A ',' '@' REG                   R01

ORL      A ',' REG                       R07

ORL      A ',' expr

ORL      C ',' '/' bit

ORL      C ',' '/' expr

ORL      C ',' bit

ORL      C ',' expr

ORL      expr ',' '#' expr

ORL      expr ',' A

POP      expr

PUSH     expr

RET

RETI

RL       A

RLC      A

RR       A

RRC      A

SETB     C

SETB     bit

SETB     expr

SJMP     expr

SUBB     A ',' '#' expr

SUBB     A ',' '@' REG                   R01

SUBB     A ',' REG                       R07

SUBB     A ',' expr

SWAP     A

- 5 -

Opcode   Syntax                          Selection Criteria

XCH      A ',' '@' REG                   R01

XCH      A ',' REG                       R07

XCH      A ',' expr

XCHD     A ',' '@' REG                   R01

XRL      A ',' '#' expr

XRL      A ',' '@' REG                   R01

XRL      A ',' REG                       R07

XRL      A ',' expr

XRL      expr ',' '#' expr

XRL      expr ',' A

A.2.2  #[4mSelection#[0m_#[4mCriteria#[0m_#[4mKeywords#[0m

R01                 Only the reserved symbols R0 and R1

(or r0, r1) are acceptable as the

REG operand.

R07                 All the reserved symbols R0 thru R7

are acceptable as the REG operand.

A.2.3  #[4mBit#[0m_#[4mOperands#[0m  Bit Addresses can be in two forms.

One form is "expression.constant" where expression gives an

address for a bit addressable register and constant (in the

range of 0 to 7) gives the bit number in the byte.

Expression values must be either in the range of 20h to 2fh,

or the multiples of 8 between 80h and f8h, and defined in

the first pass.

The second form is an symbolic expression with value between

0 and 255.  The value of the expression is placed in the

operand field of the instruction without modification.

A.2.4  #[4mApostrophes#[0m  The apostrophes in the syntax field are

a notation used for the parser generator and are not put in

the assembler source statement.

A.3  #[4mNotes#[0m

A.3.1  #[4mAddress#[0m_#[4mSpaces#[0m  The Four address spaces, Program

Memory, Internal Data Memory, Bit Numbers, and External Data

Memory are not differentiated in the symbol table.  Which

address space a symbol refers to is determined by the

context of the instruction operands it appears in.

- 6 -

A.3.2  #[4mReserved#[0m_#[4mSymbols#[0m

A.3.2.1  #[4mMachine#[0m_#[4mDependent#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  A AB C DPTR PC

R0 R1 R2 R3 R4 R5 R6 R7 a ab c dptr pc r0 r1 r2 r3 r4 r5 r6

r7

A.3.2.2  #[4mStandard#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  AND DEFINED EQ GE GT

HIGH LE LOW LT MOD NE NOT OR SHL SHR XOR and defined eq ge

gt high le low lt mod ne not or shl shr xor

CONTENTS

A.  Appendix for as8051 Frankenstein Assembler...........  1

A.1  Pseudo Operations...............................  1

A.2  Instructions....................................  1

A.3  Notes...........................................  5

- i -