- 1 -

A.  #[4mAppendix#[0m_#[4mfor#[0m_#[4mas8096#[0m_#[4mFrankenstein#[0m_#[4mAssembler#[0m

A.1  #[4mPseudo#[0m_#[4mOperations#[0m

A.1.1  #[4mStandard#[0m_#[4mPseudo#[0m_#[4mOperation#[0m_#[4mMnemonics#[0m

End                                END

File Inclusion                     INCL INCLUDE

If                                 IF

Else                               ELSE

End If                             ENDI

Equate                             EQU

Set                                SET

Org                                ORG

Reserve Memory                     DSB RESERVE RMB

Define Byte Data                   BYTE DCB FCB

Define String Data                 FCC STRING

Define Character Set Translation   CHARSET

Define Character Value             CHARDEF CHD

Use Character Translation          CHARUSE

A.1.2  #[4mMachine#[0m_#[4mDependent#[0m_#[4mPseudo#[0m_#[4mOperations#[0m

A.1.2.1  #[4mDefine#[0m_#[4mWord#[0m_#[4mData,#[0m_#[4mAligned#[0m

[Label]  WORD  expression [, expression] ...

[Label]  FDB   expression [, expression] ...

[Label]  DCW  expression [, expression] ...

The define word statement generates a two byte integer for

each expression in the expression list.  There can be up to

128 expressions on a line, more than the line length will

allow.  The location counter is adjusted by skipping bytes

to make the first location address divisible by 2.  The

generated constants are in byte reversed order, with the low

order byte first, followed by the high order byte.  The

optional label is set to the first location of this area.

A.1.2.2  #[4mDefine#[0m_#[4mLong#[0m_#[4mWord#[0m_#[4mData,#[0m_#[4mAligned#[0m

[Label]  LONG  expression [, expression] ...

[Label]  DCL  expression [, expression] ...

The define long word statement generates a four byte integer

for each expression in the expression list.  There can be up

to 128 expressions on a line, more than the line length will

allow.  The location counter is adjusted by skipping bytes

to make the first location address divisible by 4.  The

generated constants are in byte reversed order, with the low

order byte first, followed by the high order bytes.  The

optional label is set to the first location of this area.

- 2 -

A.1.2.3  #[4mReserve#[0m_#[4mMemory,#[0m_#[4mWord#[0m_#[4mAligned#[0m

[Label] DSW expression

This statement moves the location counter forward by the

number of bytes specified in the expression.  The location

counter is adjusted by skipping bytes to make the first

location address divisible by 2.  The label is set to the

first location of this area.

A.1.2.4  #[4mReserve#[0m_#[4mMemory,#[0m_#[4mLong#[0m_#[4mWord#[0m_#[4mAligned#[0m

[Label] DSL expression

This statement moves the location counter forward by the

number of long words specified in the expression.  The

location counter is adjusted by skipping bytes to make the

first location address divisible by 4.  The label is set to

the first location of this area.

A.1.2.5  #[4mInstruction#[0m_#[4mSet#[0m_#[4mSelection#[0m

CPU string

The instruction set can be specified in the source file with

the CPU pseudooperation.  The string, delimited by quotes or

apostrophes, is scanned for a substring which selects which

instruction set is used.  When the program is invoked, this

operation is performed on the name of the program, then the

-p optional arguement, if any, and then any CPU statements.

The last one selects which subset of the instructions the

assembler will accept.  The instruction set can be changed

at any place in the source file.

Instruction Set          Substrings

80c196                   19

8096                     9

A.2  #[4mInstructions#[0m

A.2.1  #[4mInstruction#[0m_#[4mList#[0m

Opcode   Syntax                                 Selection Criteria

ADD       expr ',' '#' expr

ADD       expr ',' '[' expr ']'

ADD       expr ',' expr ',' '#' expr

ADD       expr ',' expr ',' '[' expr ']'

- 3 -

Opcode   Syntax                                 Selection Criteria

ADD       expr ',' expr ',' expr '[' expr ']'   DIRECT

ADD       expr ',' expr ',' expr '[' expr ']'   EXTENDED

ADD       expr ',' expr ',' expr                DIRECT

ADD       expr ',' expr ',' expr                EXTENDED

ADD       expr ',' expr '[' expr ']'            DIRECT

ADD       expr ',' expr '[' expr ']'            EXTENDED

ADD       expr ',' expr                         DIRECT

ADD       expr ',' expr                         EXTENDED

ADDB      expr ',' '#' expr

ADDB      expr ',' '[' expr ']'

ADDB      expr ',' expr ',' '#' expr

ADDB      expr ',' expr ',' '[' expr ']'

ADDB      expr ',' expr ',' expr '[' expr ']'   DIRECT

ADDB      expr ',' expr ',' expr '[' expr ']'   EXTENDED

ADDB      expr ',' expr ',' expr                DIRECT

ADDB      expr ',' expr ',' expr                EXTENDED

ADDB      expr ',' expr '[' expr ']'            DIRECT

ADDB      expr ',' expr '[' expr ']'            EXTENDED

ADDB      expr ',' expr                         DIRECT

ADDB      expr ',' expr                         EXTENDED

ADDC      expr ',' '#' expr

ADDC      expr ',' '[' expr ']'

ADDC      expr ',' expr '[' expr ']'            DIRECT

ADDC      expr ',' expr '[' expr ']'            EXTENDED

ADDC      expr ',' expr                         DIRECT

ADDC      expr ',' expr                         EXTENDED

ADDCB     expr ',' '#' expr

ADDCB     expr ',' '[' expr ']'

ADDCB     expr ',' expr '[' expr ']'            DIRECT

ADDCB     expr ',' expr '[' expr ']'            EXTENDED

ADDCB     expr ',' expr                         DIRECT

ADDCB     expr ',' expr                         EXTENDED

AND       expr ',' '#' expr

AND       expr ',' '[' expr ']'

AND       expr ',' expr ',' '#' expr

AND       expr ',' expr ',' '[' expr ']'

AND       expr ',' expr ',' expr '[' expr ']'   DIRECT

AND       expr ',' expr ',' expr '[' expr ']'   EXTENDED

AND       expr ',' expr ',' expr                DIRECT

AND       expr ',' expr ',' expr                EXTENDED

AND       expr ',' expr '[' expr ']'            DIRECT

AND       expr ',' expr '[' expr ']'            EXTENDED

AND       expr ',' expr                         DIRECT

- 4 -

Opcode   Syntax                                 Selection Criteria

AND       expr ',' expr                         EXTENDED

ANDB      expr ',' '#' expr

ANDB      expr ',' '[' expr ']'

ANDB      expr ',' expr ',' '#' expr

ANDB      expr ',' expr ',' '[' expr ']'

ANDB      expr ',' expr ',' expr '[' expr ']'   DIRECT

ANDB      expr ',' expr ',' expr '[' expr ']'   EXTENDED

ANDB      expr ',' expr ',' expr                DIRECT

ANDB      expr ',' expr ',' expr                EXTENDED

ANDB      expr ',' expr '[' expr ']'            DIRECT

ANDB      expr ',' expr '[' expr ']'            EXTENDED

ANDB      expr ',' expr                         DIRECT

ANDB      expr ',' expr                         EXTENDED

BMOV      expr ',' expr                         CPU196

BR        '[' expr ']'

CLR       expr

CLRB      expr

CLRC

CLRVT

CMP       expr ',' '#' expr

CMP       expr ',' '[' expr ']'

CMP       expr ',' expr '[' expr ']'            DIRECT

CMP       expr ',' expr '[' expr ']'            EXTENDED

CMP       expr ',' expr                         DIRECT

CMP       expr ',' expr                         EXTENDED

CMPB      expr ',' '#' expr

CMPB      expr ',' '[' expr ']'

CMPB      expr ',' expr '[' expr ']'            DIRECT

CMPB      expr ',' expr '[' expr ']'            EXTENDED

CMPB      expr ',' expr                         DIRECT

CMPB      expr ',' expr                         EXTENDED

CMPL      expr ',' expr                         CPU196

DEC       expr

DECB      expr

- 5 -

Opcode   Syntax                                 Selection Criteria

DI

DIV       expr ',' '#' expr

DIV       expr ',' '[' expr ']'

DIV       expr ',' expr '[' expr ']'            DIRECT

DIV       expr ',' expr '[' expr ']'            EXTENDED

DIV       expr ',' expr                         DIRECT

DIV       expr ',' expr                         EXTENDED

DIVB      expr ',' '#' expr

DIVB      expr ',' '[' expr ']'

DIVB      expr ',' expr '[' expr ']'            DIRECT

DIVB      expr ',' expr '[' expr ']'            EXTENDED

DIVB      expr ',' expr                         DIRECT

DIVB      expr ',' expr                         EXTENDED

DIVU      expr ',' '#' expr

DIVU      expr ',' '[' expr ']'

DIVU      expr ',' expr '[' expr ']'            DIRECT

DIVU      expr ',' expr '[' expr ']'            EXTENDED

DIVU      expr ',' expr                         DIRECT

DIVU      expr ',' expr                         EXTENDED

DIVUB     expr ',' '#' expr

DIVUB     expr ',' '[' expr ']'

DIVUB     expr ',' expr '[' expr ']'            DIRECT

DIVUB     expr ',' expr '[' expr ']'            EXTENDED

DIVUB     expr ',' expr                         DIRECT

DIVUB     expr ',' expr                         EXTENDED

DJNZ      expr ',' expr

DJNZW     expr ',' expr                         CPU196

EI

EXT       expr

EXTB      expr

IDLPD     '#' expr                              CPU196

INC       expr

INCB      expr

JBC       expr ',' expr ',' expr

- 6 -

Opcode   Syntax                                 Selection Criteria

JBS       expr ',' expr ',' expr

JC        expr

JE        expr

JGE       expr

JGT       expr

JH        expr

JLE       expr

JLT       expr

JNC       expr

JNE       expr

JNH       expr

JNST      expr

JNV       expr

JNVT      expr

JST       expr

JV        expr

JVT       expr

LCALL     expr

LD        expr ',' '#' expr

LD        expr ',' '[' expr ']'

LD        expr ',' expr '[' expr ']'            DIRECT

LD        expr ',' expr '[' expr ']'            EXTENDED

LD        expr ',' expr                         DIRECT

LD        expr ',' expr                         EXTENDED

LDB       expr ',' '#' expr

LDB       expr ',' '[' expr ']'

LDB       expr ',' expr '[' expr ']'            DIRECT

- 7 -

Opcode   Syntax                                 Selection Criteria

LDB       expr ',' expr '[' expr ']'            EXTENDED

LDB       expr ',' expr                         DIRECT

LDB       expr ',' expr                         EXTENDED

LDBSE     expr ',' '#' expr

LDBSE     expr ',' '[' expr ']'

LDBSE     expr ',' expr '[' expr ']'            DIRECT

LDBSE     expr ',' expr '[' expr ']'            EXTENDED

LDBSE     expr ',' expr                         DIRECT

LDBSE     expr ',' expr                         EXTENDED

LDBZE     expr ',' '#' expr

LDBZE     expr ',' '[' expr ']'

LDBZE     expr ',' expr '[' expr ']'            DIRECT

LDBZE     expr ',' expr '[' expr ']'            EXTENDED

LDBZE     expr ',' expr                         DIRECT

LDBZE     expr ',' expr                         EXTENDED

LJMP      expr

MUL       expr ',' '#' expr

MUL       expr ',' '[' expr ']'

MUL       expr ',' expr ',' '#' expr

MUL       expr ',' expr ',' '[' expr ']'

MUL       expr ',' expr ',' expr '[' expr ']'   DIRECT

MUL       expr ',' expr ',' expr '[' expr ']'   EXTENDED

MUL       expr ',' expr ',' expr                DIRECT

MUL       expr ',' expr ',' expr                EXTENDED

MUL       expr ',' expr '[' expr ']'            DIRECT

MUL       expr ',' expr '[' expr ']'            EXTENDED

MUL       expr ',' expr                         DIRECT

MUL       expr ',' expr                         EXTENDED

MULB      expr ',' '#' expr

MULB      expr ',' '[' expr ']'

MULB      expr ',' expr ',' '#' expr

MULB      expr ',' expr ',' '[' expr ']'

MULB      expr ',' expr ',' expr '[' expr ']'   DIRECT

MULB      expr ',' expr ',' expr '[' expr ']'   EXTENDED

MULB      expr ',' expr ',' expr                DIRECT

MULB      expr ',' expr ',' expr                EXTENDED

MULB      expr ',' expr '[' expr ']'            DIRECT

MULB      expr ',' expr '[' expr ']'            EXTENDED

MULB      expr ',' expr                         DIRECT

MULB      expr ',' expr                         EXTENDED

MULU      expr ',' '#' expr

- 8 -

Opcode   Syntax                                 Selection Criteria

MULU      expr ',' '[' expr ']'

MULU      expr ',' expr ',' '#' expr

MULU      expr ',' expr ',' '[' expr ']'

MULU      expr ',' expr ',' expr '[' expr ']'   DIRECT

MULU      expr ',' expr ',' expr '[' expr ']'   EXTENDED

MULU      expr ',' expr ',' expr                DIRECT

MULU      expr ',' expr ',' expr                EXTENDED

MULU      expr ',' expr '[' expr ']'            DIRECT

MULU      expr ',' expr '[' expr ']'            EXTENDED

MULU      expr ',' expr                         DIRECT

MULU      expr ',' expr                         EXTENDED

MULUB     expr ',' '#' expr

MULUB     expr ',' '[' expr ']'

MULUB     expr ',' expr ',' '#' expr

MULUB     expr ',' expr ',' '[' expr ']'

MULUB     expr ',' expr ',' expr '[' expr ']'   DIRECT

MULUB     expr ',' expr ',' expr '[' expr ']'   EXTENDED

MULUB     expr ',' expr ',' expr                DIRECT

MULUB     expr ',' expr ',' expr                EXTENDED

MULUB     expr ',' expr '[' expr ']'            DIRECT

MULUB     expr ',' expr '[' expr ']'            EXTENDED

MULUB     expr ',' expr                         DIRECT

MULUB     expr ',' expr                         EXTENDED

NEG       expr

NEGB      expr

NOP

NORML     expr ',' expr

NOT       expr

NOTB      expr

OR        expr ',' '#' expr

OR        expr ',' '[' expr ']'

OR        expr ',' expr '[' expr ']'            DIRECT

OR        expr ',' expr '[' expr ']'            EXTENDED

OR        expr ',' expr                         DIRECT

OR        expr ',' expr                         EXTENDED

ORB       expr ',' '#' expr

ORB       expr ',' '[' expr ']'

ORB       expr ',' expr '[' expr ']'            DIRECT

- 9 -

Opcode   Syntax                                 Selection Criteria

ORB       expr ',' expr '[' expr ']'            EXTENDED

ORB       expr ',' expr                         DIRECT

ORB       expr ',' expr                         EXTENDED

POP       '[' expr ']'

POP       expr '[' expr ']'                     DIRECT

POP       expr '[' expr ']'                     EXTENDED

POP       expr                                  DIRECT

POP       expr                                  EXTENDED

POPA                                            CPU196

POPF

PUSH      '#' expr

PUSH      '[' expr ']'

PUSH      expr '[' expr ']'                     DIRECT

PUSH      expr '[' expr ']'                     EXTENDED

PUSH      expr                                  DIRECT

PUSH      expr                                  EXTENDED

PUSHA                                           CPU196

PUSHF

RET

RST

SCALL     expr

SETC

SHL       expr ',' '#' expr

SHL       expr ',' expr

SHLB      expr ',' '#' expr

SHLB      expr ',' expr

SHLL      expr ',' '#' expr

SHLL      expr ',' expr

SHR       expr ',' '#' expr

SHR       expr ',' expr

SHRA      expr ',' '#' expr

SHRA      expr ',' expr

- 10 -

Opcode   Syntax                                 Selection Criteria

SHRAB     expr ',' '#' expr

SHRAB     expr ',' expr

SHRAL     expr ',' '#' expr

SHRAL     expr ',' expr

SHRB      expr ',' '#' expr

SHRB      expr ',' expr

SHRL      expr ',' '#' expr

SHRL      expr ',' expr

SJMP      expr

SKIP      expr

ST        expr ',' '[' expr ']'

ST        expr ',' expr '[' expr ']'            DIRECT

ST        expr ',' expr '[' expr ']'            EXTENDED

ST        expr ',' expr                         DIRECT

ST        expr ',' expr                         EXTENDED

STB       expr ',' '[' expr ']'

STB       expr ',' expr '[' expr ']'            DIRECT

STB       expr ',' expr '[' expr ']'            EXTENDED

STB       expr ',' expr                         DIRECT

STB       expr ',' expr                         EXTENDED

SUB       expr ',' '#' expr

SUB       expr ',' '[' expr ']'

SUB       expr ',' expr ',' '#' expr

SUB       expr ',' expr ',' '[' expr ']'

SUB       expr ',' expr ',' expr '[' expr ']'   DIRECT

SUB       expr ',' expr ',' expr '[' expr ']'   EXTENDED

SUB       expr ',' expr ',' expr                DIRECT

SUB       expr ',' expr ',' expr                EXTENDED

SUB       expr ',' expr '[' expr ']'            DIRECT

SUB       expr ',' expr '[' expr ']'            EXTENDED

SUB       expr ',' expr                         DIRECT

SUB       expr ',' expr                         EXTENDED

SUBB      expr ',' '#' expr

SUBB      expr ',' '[' expr ']'

SUBB      expr ',' expr ',' '#' expr

SUBB      expr ',' expr ',' '[' expr ']'

SUBB      expr ',' expr ',' expr '[' expr ']'   DIRECT

- 11 -

Opcode   Syntax                                 Selection Criteria

SUBB      expr ',' expr ',' expr '[' expr ']'   EXTENDED

SUBB      expr ',' expr ',' expr                DIRECT

SUBB      expr ',' expr ',' expr                EXTENDED

SUBB      expr ',' expr '[' expr ']'            DIRECT

SUBB      expr ',' expr '[' expr ']'            EXTENDED

SUBB      expr ',' expr                         DIRECT

SUBB      expr ',' expr                         EXTENDED

SUBC      expr ',' '#' expr

SUBC      expr ',' '[' expr ']'

SUBC      expr ',' expr '[' expr ']'            DIRECT

SUBC      expr ',' expr '[' expr ']'            EXTENDED

SUBC      expr ',' expr                         DIRECT

SUBC      expr ',' expr                         EXTENDED

SUBCB     expr ',' '#' expr

SUBCB     expr ',' '[' expr ']'

SUBCB     expr ',' expr '[' expr ']'            DIRECT

SUBCB     expr ',' expr '[' expr ']'            EXTENDED

SUBCB     expr ',' expr                         DIRECT

SUBCB     expr ',' expr                         EXTENDED

XOR       expr ',' '#' expr

XOR       expr ',' '[' expr ']'

XOR       expr ',' expr '[' expr ']'            DIRECT

XOR       expr ',' expr '[' expr ']'            EXTENDED

XOR       expr ',' expr                         DIRECT

XOR       expr ',' expr                         EXTENDED

XORB      expr ',' '#' expr

XORB      expr ',' '[' expr ']'

XORB      expr ',' expr '[' expr ']'            DIRECT

XORB      expr ',' expr '[' expr ']'            EXTENDED

XORB      expr ',' expr                         DIRECT

XORB      expr ',' expr                         EXTENDED

A.2.2  #[4mSelection#[0m_#[4mCriteria#[0m_#[4mKeywords#[0m

DIRECT              The instruction will be generated

with a short form if the last

operand will fit in one byte, and

is defined when the instruction is

processed in the first pass.

EXTENDED            The instruction can be generated

with a long form.

CPU196              The instruction is implemented only

in the 80c196.

- 12 -

A.2.3  #[4mApostrophes#[0m  The apostrophes in the syntax field are

a notation used for the parser generator and are not put in

the assembler source statement.

A.3  #[4mNotes#[0m

A.3.1  #[4mData#[0m_#[4mAlignment#[0m  In the second pass, data address are

checked for alignment.  If the operand field is not

divisible by 2 for word references, or by 4 for long word

references, the error message "expression fails validity

test" occurs.

A.3.2  #[4mGeneric#[0m_#[4mJumps#[0m  The assembler requires that the length

of an instruction be able to be determined at the first

pass, so generalized generic jumps could not be implemented.

A.3.3  #[4mReserved#[0m_#[4mSymbols#[0m

A.3.3.1  #[4mStandard#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  AND DEFINED EQ GE GT

HIGH LE LOW LT MOD NE NOT OR SHL SHR XOR and defined eq ge

gt high le low lt mod ne not or shl shr xor

CONTENTS

A.  Appendix for as8096 Frankenstein Assembler..........   1

A.1  Pseudo Operations..............................   1

A.2  Instructions...................................   2

A.3  Notes..........................................  12

- i -