SIMPLE.DOC

OPERATION

This file is an attempt to create a simplified description of the usage

for rdprep on the HardFrame controller. In order to refresh yourself regarding

the legal option commands type "rdprep -h" or anything else it does not

understand.

-u #\		# is the unit number to configure.

-u# /

-r filename\	Filename is the source for the mountfile.

-rfilename /

-q#		Controls rdprep's verbosity.

0 <default>	Allows operator to "fill in the blanks". Full reports.

1		Allows no operator interaction but has full reports.

2		Allows no operator interaction but has terse reports.

3		Allows no operator interaction and provides errors only.

-t		Skips testing for all possible drives.

-s filename\	Filename is the destination for the saved Mountfile.

-sfilename /

-d device\	Device to prep if boty HardFrame and StarDrive are

-ddevice /	present on the bus. H = HardFrame and S = StarDrive

-i		Allows reading past any RDB errors in attempting to

recover from block errors.

-v		Ends operation after printing suggested disk parameters.

-a		Allows fully automatic operation for installing a single partition.

rdprep can also be run from the workbench. It understands four tooltypes:

UNIT=n		n = unit number to configure

QUIET=n		n = quiet level as above, default is 3.

NOTEST		No parameter. If specified skip testing for all present drives.

MOUNTFILE=f	f = filename in where the mountfile will be found.

SAVEFILE=f	f = filename to which the mountfile will be saved.

DEVICE=d	d = device to prep if both HardFrame and StarDrive are present.

IGNORE		No parameter. If specified ignore RDB read errors.

VALUES		No parameter. If specified print suggested values and exit.

AUTOMATIC	No parameter. Enables automatic single partition mode.

( Note that if you specify a unit for interactive mode operation some of the

questions and prompted values may be a bit odd for the configuration you have.

Notable among these is asking "Is this the highest ID on the bus?" even though

it is not. No testing to determine a suitable estimated answer for this has been

performed. The -u operation bypasses the necessary testing.)

MOUNTFILE DESCRIPTION

The mountfile is made up of disk descriptors and partition descriptors.

These may appear in any order in the file and several different disks can be

described in one file as long as their unit numbers differ. Note that several

values in the descriptions below should not be modified from the defaults given

unless you are well aware of the consequences. These values are alterable

because there are occaisions that demand it. The following is an example of

a mountfile for a very large hard disk with two partitions. (Note that the

partition names do not have to be of the form "DH0" or such.)

/* RigidDiskBlock. */

MICROP  1578-15MB1036509AS09:	disk = HardFrame.device

Unit = 4

BytesperBlock = 512

Cylinders = 1224

Heads = 15

BlocksPerTrack = 35

CylinderBlocks = 525

RDBlow = 3 ; RDBhi = 524

MinCyl = 1 ; MaxCyl = 1217

Interleave = 1

HiLun = FALSE

HiID = FALSE

OnlyDrive = FALSE

Reselect = TRUE

#

/* Partition. */

MySys:	device = HardFrame.device

Unit = 4

Flags = 0

FileSystem = L:FastFileSystem

StackSize = 4096

Surfaces = 15

SizeBlock = 512

BlocksPerTrack = 35

Reserved = 2

Interleave = 0

LowCyl = 1 ; HighCyl = 77

PreAlloc = 0

Buffers = 20

BufMemType = 1

DOSType = 0x444f5301

MaxTransfer = 2097152

Mask = 0xfffffe

GlobVec = -1

Mount = 1

BootPri = 4

Priority = 10

#

/* Partition. */

Mama:	device = HardFrame.device

Unit = 4

Flags = 0

FileSystem = L:FastFileSystem

StackSize = 4096

Surfaces = 15

SizeBlock = 512

BlocksPerTrack = 35

Reserved = 2

Interleave = 0

LowCyl = 78 ; HighCyl = 1217

PreAlloc = 0

Buffers = 20

BufMemType = 1

DOSType = 0x444f5301

MaxTransfer = 2097152

Mask = 0xfffffe

GlobVec = -1

Mount = 1

BootPri = -10

Priority = 10

#

First some generic help with the descriptions. The words that lead off each

descriptor are the disk or partition names. The disk name is for the user's

reference. The partition name is what will be installed on the disk. Each

keyword is followed by an "=" and a value. In the description below the

keywords appear in ALL CAPITALS if they cannot be omited. They appear in mixed

case if they can be defaulted. The default values will be noted. /* Comments are

constructed as in C. They may appear anywhere except in names, values, or

keywords. */ Multiple entries on a single line may be separated by a ";" for

clarity. I recommend it. The "#" mark indicates the end of a disk or partition

description.

In this example the disk drive's reference name is "MICROP  1578-15MB1036509AS09,

which for me means it is the disk description for my Micropolis 1578 drive which

is running SCSI Firmware revision AS09 on a manufacturer's model number "1578-

15MB10360509".

The DISK keyword must follow the disk drive's name. The DISK value is the name

of the controller for the drive. It is used as a reference in searching for

matching partitions. It must be "HardFrame.device".

UNIT is the HardFrame unit number for the drive. It is normally made up of three

decimal digits. Leading zeros can be left out. The left most digit is the number

of the HardFrame card counting (only) HardFrames from the power supply towrds

the left of the A2000 when viewed from the front. Normally this is zero. The

second digit is the SCSI LUN, or Logical Unit Number. This is non-zero in cases

such as the Adaptec 4070A controller which allows two drives for one basic SCSI

address. This value is normally 0 but may range from 0 through 7. The final

digit is the SCSI address. It is the address selected by dip switches, jumper

blocks, or any other means on the drive itself. it may range from 0 through 7

excluding the value you've set for the HardFrame card, which is normally 7. In

this example the UNIT number is 4.

SIZEBLOCK is the number of BYTEs in a block for this filesystem. Currently this

is always 512. It may be changeable in the future.

BYTESPERBLOCK describes how many bytes there are in each physical sector of the

drive. (I believe this must be 512 for the current version of the software.)

CYLINDERS describes how many sets of tracks or cylinders there are on the

physical drive. In this example it is 1218. (It is a big drive.)

HEADS describes how many heads or surfaces or tracks there are for each cylinder

on the physical drive. This may range from 1 to strangely large numbers. In this

example there are 15 heads.

BLOCKSPERTRACK tells us how many user available blocks or sectors there are on

each track. Typically this is the value for the majority of the tracks. For

this drive there really are 36 sectors per track; but, one is reserved for

the drive's sector spareing use. Since each track then has 35 user available

sectors that is the value supplied. However the value of 36 can be used here

if the drive is setup for "cylinder sparing." MiniScribe 8425S drives are

typical of this. For that drive the value here is 17.

CYLINDERBLOCKS describes how many blocks there are on each cylinder that are

available for use. In this case it is HEADS times BLOCKSPERTRACK or 15 * 35. If

36 had been used we could still tell the controller there were only 525 of

them available and things would still work. For the MiniScribe example, though,

the drive reserves one sector per cylinder for sparing. Hence there are only

17 (spt) * 4 (heads) -1 or 67 CYLINDERBLOCKS for an 8425S.

RDBLow defaults to three. It tells rdprep which block on the drive is the first

one available for storing rdprep's special information. It is three in this

example. (This allows room for such things as the Macintosh HD descriptor for

future AMAX compatability if possible.)

RDBHI describes the highest block number available for rdprep RDB,

RigidDiskBlock, data storage. In this example it is one number of the last

sector on the zeroth cylinder or 524.

MinCyl is the lowest cylinder number available for user partitions. It must have

a value large enough that the full range of information can be stored in the

available sectors in the RDB form. 40 blocks is likely enough so usually one

cylinder is used. It is wasteful of drive space; but, it optimizes certain

AmigaDOS activities. It defaults to 1, which is the value in the example, also.

MAXCYL is the highest number cylinder that is available to the user. It is

normally one less than the number of cylinders on the drive. (Devices count

0 through N-1 rather han 1 through N.) In this example, then, it is 1217. Note

that there is no consistancy checking here (or anywhere else to speak of.)

Interleave is a value currently stored for information only. It defaults to 1.

It is the lowest number that the controller will support. At some future time

MDFixer, the MicroBotics hard format and disk fixer utility will use this value

as the default for hard formatting the drive.

HiLUN is true if this drive is the highest LUN at this SCSI address. It defaults

to TRUE.

HiID is true if this will always be the highest numbered UNIT id on the

HardFrame.device. It defaults to TRUE.

OnlyDrive defaults to TRUE. If this drive is the only drive on the

HardFrame.device set it TRUE.

If the drive supports SCSI reselection set ReSelect TRUE. It defaults TRUE as

most drives do support reselect. If yours does not be sure to set this parameter

to FALSE.

The partition descriptor is lifted straight from the device's mountlist with

the proviso that GlobVec wants to be set to 1 if it is used. Rdprep will set

the actual value to -1.

The DEVICE keyword must follow the partition name, "Mama:" in this example. Its

value must be "HardFrame.device".

UNIT is as UNIT above.

FLAGS is not used by the HF controller at this time.

FileSystem is filled in if you wish to use the new FastFileSystem. The example

has this properly filled in as "l:FastFileSystem". FastFileSystem is the

default.

StackSize defaults to 4096 bytes. This default is sufficient for FFS.

SURFACES is the same as HEADS above normally. However to allow cylinder spareing

drives, like the 8425S, work it can be different. For the 8425S we use 1 surface

and pretend all the sectors are on it since we have an "odd" number of sectors.

For the Micropolis in theexample 15 is used. This is the logical number of

surfaces.

BLOCKSPERTRACK describes how many logical blocks there are on each track. For

the 8425S we'd use 67. For the Micropolis in the example the value is 35.

Reserved is normally 2. Unless you have special reason to make it different,

leave this value alone.

Interleave is only used for the old filesystem. Normally it is set to default of

zero.

LOWCYL is the starting cylinder number for this partition. In this example of

only one partition for the whole disk the value is one.

HIGHCYL is the ending cylinder number for this partition. In this example it is

1217. Note that for multiple partitions thse values must not overlap from

partition to partition.

Buffers defaults to a ridiculous 5 buffers. 20 is recommended as a minimum. This

is the initial number of AmigaDOS buffers for the drive. Do not ever set it

below 5.

BufMemType is the type of memory for these buffers. It should be 1 unless you

have some bizarre need to only mount the drive if FAST ram is present (5) or

if it must go into CHIP ram (3). (I cannot recommend highly enough that you

use the (1) default without very good reason. And never use 3 unless you want

to learn how slow an HF can go on a CHIP only system.

If you are running a different file system than the old file system be sure to

enter the correct value for DOSType. It is 8 hexadecimal digits describing the

filesytem for this partition. FFS uses 444f5301. OldFS uses 44f55300. Other file

systems, if any, will use other values. Consult the appropriate manuals for that

file system. The default value is 444f5301 for FastFileSystem.

MaxTransfer can be allowed to default to the 255 blocks or 130560 byte value.

The example uses 2 megs because that is marginally faster and works with this

drive.

GlobVec must be set to one to use the Fast File System. This is the default

value.

Mount defaults to 1 or TRUE. It is set to one if this partition is to be reserved but

not mounted at this time. (HF ignores this flag at this time.)

Boot priority defaults to 0. If you wish to boot from this partition set the

value to the highest value of all your other partitions. A value less than 5 is

recommended due to an interesting bug in the AmigaDOS automount procedures.

Priority defaults to 10 and describes the system priority for this filesystem

task. I suggest leaving it alone except for very special circumstances.