66 Mhz Blizzard 1260.

---------------------

This is a serious hardware hack, so I cannot accept any responsibility to you, your Amiga in any

way what so ever. By doing this incorrectly you can burn out your CPU and knacker you Amiga

1200 beyond Repair.

Just to re-empathise YOU DO THIS AT YOUR OWN RISK.

Now the disclaimer is out of the way. Its really simple and cost less than 8.00 to do.

What do you need.

-----------------

Blizzard1260 Accelerator from Phase 5 Digital products.

66 Mhz Oscillator from Maplins (About 2) (Lately Maplins only have 64Mhz Oscillators)

80486 HeatSink / Cooling Fan from Maplins (About 6) (Product Code VL50)

Steady hands.

Soldering Iron with a sharp point.

A small flat bladed electrical screwdriver.

Some knowledge of Basic Electronics.

History.

--------

Firstly I am not the only person to have done this. Many people have clocked there 1260 cards

successfully and have had no heat / guru problems from them. I have read on the internet that

one such card has been clocked to 80Mhz. However, I personally doubt this and I wouldn't wish

to push a 50Mhz CPU this far.

As you know the 680X0 series has reached the end of the line and  if like me you have got used

to life in the fast lane then this will push your Amiga that bit closer to High Spec PC's.

Why.

----

51 MIPS as opposed to 38 MIPS from a 50Mhz 68060

40 Times faster than a factory standard A1200.

AB3D2 Full Screen 1x1 like you have never seen before (Using TKGPatch).

Nemac4 Full Screen 1x1 is Amazing

Play the demo of Flyin High and compare it to Ridge Racer on the  Playstation.

45 Frames/Sec in XTremeRacing 1x1 Full Screen

Magic64 V1.7 Registered - Original C64 Speed with sound and Framerate 1/1 (6581SID.library)

Windows and some games become usable / playable under PC Emulation. (PCX V1 & PCTask V4)

Shapeshifter graphics almost reach native Mac Speeds.

If you like the sound of the above then read on.

How 2

----------

1.  Remove the 1260 Card from your Amiga.

2.  Remove the SIMM from its Socket.

3.  Remove static by touching a radiator or wear a static protector. (Static kills CPU's)

4.  Insulate the blade of screwdriver using insulation tape.

5.  Place the blade of the screwdriver under the 50Mhz Oscillator

6.  Turn the board over and place it on a towel.

7.  Using the Soldering Iron heat the Oscillator pins and gently lever out the Oscillator using the

screwdriver.

NB

--

1260 Cards can have two types of Oscillator either a Square one or a Rectangular one.

8.  line up the new Oscillator and solder it in.

9.  Refit the SIMM

10. Fit the CPU Cooler to the CPU (you must brake of one of the clips as the clock battery is in

the way) (Further modification to the plastic makes a better fit)

11. Refit your 1260 Card.

12. Wire the fan to a 12V supply in the Amiga (Piggy Back it of the power supply to the internal

disk drive)

13. Cut a Hole in your expansion slot cover to allow air to reach the fan

14. Raise your A1200 at least 4 CM of its desk & Power UP.

NB

--

if your A1200 is in a tower case (Micronik / RAmiga / Eagle)   you may have to cut a hole in

the side of your tower case so that the CPU Cooler can Breath. My Hole is about 4 CM Square.

Please see enclosed Diagrams.

For further information on A1200 upgrades including:

Putting your A1200 in a Zorro 2 Tower

Fitting Internal IDE (Atapi) CD Roms

Fitting Internal HD Diskdrives

Making a 250 Watt A1200 PSU for 25

I can be reached at the following Email address until September 1997

Carl....     BD505910@ntu.ac.uk (Nottingham Trent University)