Custom Services     System Test Utility          Version 8.0

General Information

Section 1.1 Introduction

STU is  an  Amiga system diagnostic.  Its purpose is to

test  disk  drives  and  memory on any model Amiga using the

operating  system functions.  With this program you can test

all   available   memory  and  the  disk  functions  of  the

system.    The  user  has  the  ability  to  test individual

drives  for  errors.   The  tests performed may be random or

serial  in  nature.  All I/O is performed in an asynchronous

manner  allowing  the  system  to  schedule  each  drive  as

needed.   This  allows  faster  drives  to  get ahead of the

slower  ones  and create the best test possible.  Each drive

is  setup and tested individually.  You can test up to seven

drives at one time.

Each Manufacturer  supplies  diagnostics that work with

their  own  products.   One Manufacturer's memory diagnostic

will  not work with another's memory board.  This means that

if  you  have  three  different memory boards in your Amiga,

you  will  have  three  different  test  disks, one for each

expansion  device.   While STU does not replace all of these

diagnostics,   it  does  allow  you  to  test  all  of  your

expansion  memory  boards at the same time!  You can run one

test  that  will  check all memory on your system.  The same

idea  applies  to  disk drives.  Each disk drive has its own

special  requirements  and device drivers.  STU will perform

all  the  basic I/O operations checking each one for correct

operation.   Bad Media, misaligned disk heads, bad blocks on

a  hard  disk  drive  can  all  be found with STU.  Any disk

device  that  AmigaDos  can  recognize and use can be tested

with STU!

STU has  another advantage in that it tests your system

in  a  manner  similar  to  normal operation.  This tends to

find  those  special  problems  the  average  manufacturer's

diagnostic  cannot  detect.    It  helps  to  isolate system

problems and ensures that a fixed problem is really fixed.

Section 1.2 Policy

STU originally  was a commercial product which was sold

through  distributors.  Since the Amiga market has decreased

in  size,  I  have  decided  to release a shareware version.

The registration fee is , a small token amount.

You may   use   STU   for   a   period   of   30  days,

unregistered.   This  will  give  you  time  to  try it out.

After  that  period, you should either register or remove it

from   your   system.    Registering   will  give  you  some

User's Manual            Chapter 1                    Page 1

Custom Services     System Test Utility          Version 8.0

additional   tests  and  email  support.   The  unregistered

version  will  not  verify disk data(very useful for testing

floppy  disks  or  new hard drives).  It will only do one of

the the memory tests.

The programs  and  documentation  are  provided "as is"

without  any  expressed  or  implied warranty including, but

not  limited  to,  the implied warranties of merchantability

and  fitness  for  a particular purpose.  The entire risk as

to  the results and performance of any program is assumed by

you.    Custom   Services  has  no  liability  which   shall

include  damages  for  loss  of  use,  anticipated  profits,

benefits,  or  any  other incidental or consequential costs,

expenses,  or damages, including with limitation any data or

information which may be lost or rendered inaccurate.

Your usage  of this program signifies your agreement to

these  terms  and  conditions.  If  you  lose it because you

mis-used  it,  we  are sorry, but no matter what your lawyer

may say or lead you to believe, we are not liable.

We will   however  make  every  attempt  to  produce  a

program  that  does  what  we  say  it  will.  If you find a

problem,   please   report   it  and  we  will  correct  the

problem(we  are  the  sole  arbitrators  as  to  what  is  a

problem) and issue you a free upgrade.

Section 1.3 Copyright

These programs   and   the  related  documentation  are

copyrighted.   You  may  not  use, copy, modify, or transfer

the   programs,   documentation,   or  any  copy  except  as

expressly provided for in this document.

Section 1.4 License

You have  the  non-exclusive  right  to use STU on your

own  computer. You may not decompile, disassemble, or in any

way  reverse  engineer  the program.  You are not allowed to

modify  the  serial  numbers embedded on the program for any

reason.   All other rights and uses not specifically granted

in  this  license  are reserved by Custom Services.  You may

transfer   your   license   to  another  person,  for  their

exclusive  use,  only  if  you give them all existing copies

and documentation.  You may not keep a copy in this case.

Section 1.5 Warranty Registration and Reward

The required   form  in  in  the  appendix.   You  will

receive  one  free  upgrade  with your registration.  Future

User's Manual            Chapter 1                    Page 2

Custom Services     System Test Utility          Version 8.0

upgrades   will   be  supplied  to  registered  users  only.

Registrations  will be available on The Amiga Zone BBS (609)

953-8159  and  via  regular  mail  for  a   processing  fee.

Additional  upgrades  will  be  available for a small charge

for   postage   and  handling.   Registered  Users  will  be

notified  of  all  updates as they become available.  If you

obtained  this program through other than a normal purchase,

please  send  in a registration form and for a small charge,

you will be able to upgrade to the latest version.

For additional information please contact:

Custom Services

P. O. Box 254

Moorestown, NJ., 08057

User's Manual            Chapter 1                    Page 3

Custom Services     System Test Utility          Version 8.0

Program Operation

Section 2.1 General

STU is  a  system  diagnostic.   It will test your disk

drives  and  memory  using  the  operating system functions.

This   not  only  gives  you  a  "confidence"  test  on  the

hardware,  it  also  checks  the  basic operating system I/O

functions.

This program   is   designed  to  test  the  basic  I/O

functions  of  the  system.   It will read, write, test, and

verify  all  system  I/O.   The user has the ability to test

drives  individually  or  in  groups  for errors.  The tests

performed  may  be  random  or serial in nature.  All I/O is

performed  in  an asynchronous manner allowing the system to

schedule  each  drive  as  needed.   This  will allow faster

drives  to  get ahead of the slower ones and create the best

test possible.

Section 2.2 Limitations

To use  STU,  please  set  the preferences to 80 column

and use an 8 point font.

This will  prevent any problems with the display.   You

do  not  need  external  memory to run STU, it will test and

use   whatever  memory  you  have.   The  address  and  data

displays   on   errors   are   in  Hexadecimal.   Additional

information  on  the  testing  is found in the section "Test

Results".

The best  conditions  for  a test would be to boot your

system  without  a  startup-sequence  file  being  executed.

After  you  boot,  type  in  SETPATCH.  This will give you a

system  with the minimum resources in use.  Then run STU and

do your testing.

This program   is   designed  to  handle  up  to  seven

partitions  or  drives.   It  has  been expanded to not only

test  floppies  but  to test any device mounted except PAR:,

SER:,  RAW:,  and  CON:.   This  means  that  any Hard drive

partition  mounted can be tested.   A drive with the mounted

name  of  "QH0:"  for  example will be listed as one of your

choices  in  the  device  selection  gadgets.  The IBM 5 1/4

drives  are  not  supported  and  will be ignored during the

testing.   STU  will  present  you  with  a  list of devices

available  for  testing  in  the  form  of a gadget.  At the

current  time,  you may test up to seven of these devices at

a  time.  These may be partitions on a Hard Drive, floppies,

or  any  combinations  of  these  testable  devices.  Custom

User's Manual            Chapter 2                    Page 4

Custom Services     System Test Utility          Version 8.0

Services   has   not   tested   every   possible  device  or

combinations.   Our direct testing has been with with A500s,

A1000s,   A2000s,  A2500s,  and  A3000s  using  Quantum  and

Seagate  drives  and  Microbotics  controllers(StarDrive and

HardFrames).   We have also tested with the Commodore A2091,

A2090  and  GVP  Series  II  controllers.  We feel confident

that  if  AmigaDos  can  use  it,  STU can test it.  We will

attempt  to  help  any  registered  user  match STU to their

hardware.   This  is one of the benefits of registration! If

we  have  your product registration on file, you can contact

us  for  direct  support.   We  have not had one user with a

controller we could not test!

Because STU  uses  AmigaDos to do the actual I/O to the

devices,  being  able  to  use  STU assumes that you have at

least  one  drive  or  partition(a  bootable one) that works

well  enough  to load the system and STU.  STU does not take

over  the  system, instead it works with AmigaDos.  AmigaDos

must  be able to run for STU to test.  If you have a problem

where  things  will  not  load from the disk at all, we have

some suggestions to try in the Appendices.

There is  little  difference  between  media errors and

device  errors  except  that errors will not usually go away

if  you  switch disks.  Errors from a device can cause media

problems(such  as  a  head scratching the disk surface).  It

is  important  that  disks  which  have  errors be used only

after   you   have   visually   inspected   the   surface(if

removable).   In  any  case,  do  not  use a floppy diskette

which  has  scratches or indentations on the surface.  A bad

disk  with  a damaged surface can cause a drive to go bad by

causing  the  head  to  bounce on the surface.  This usually

can  be  corrected  by  a  good  cleaning  disk.  It is also

important   to   only   use  Double  Sided,  Double  Density

diskettes  in  your floppy drives.  Use of single sided disk

can  damage  your  disk  heads.  The correct media should be

used  at  all  time.   STU  does  not  fix problems, it just

detects   them.   You  should  always  obtain  service  from

qualified  service centers.  After your system is fixed, run

STU  to  be sure that the problems were fully corrected.  It

sometimes  will  help  to  use the printer option and make a

hard copy report of the testing.

Timings performed  during  tests  are  accurate only to

the  accuracy  of  the  system  clock(plus  or  minus  0.001

seconds).   Other  tasks  running  during  a test will cause

errors  in  the  form of increased run times.  Running tests

on  multiple  drives  while  a good system test, gives lousy

timing  information.   Since each drive runs asynchronously,

the  timing  overlaps.   To get accurate timing of a device,

run  the  tests  on  only  that drive and make sure you shut

down  all  other  system  activity.   Don't  be disappointed

about  the  testing  times.   STU  does alot of things while

testing  and  most  devices  show a lower transfer rate than

the  public  domain  benchmarks.   There are alot of reasons

for  this,  one  of which is the I/O methods used by STU are

User's Manual            Chapter 2                    Page 5

Custom Services     System Test Utility          Version 8.0

not  designed  to  produce  the highest speeds, but the best

tests!   See   the   section  on  device  timings  for  more

information on how to time a drive.

This program  is designed to run from either the CLI or

the  WorkBench.   The  procedure  for running from cli is to

type  in  the  command  STU:STU,   From  the  Workbench just

double  click on the STU icon.  All operations are identical

regardless  of  how  you start up STU.  Due to the nature of

the  tests  performed, multitasking while not disabled, will

be  very limited to programs running from RAM: or drives not

under  test.   This  program  will effectively take over the

system  due  to the testing performed and the amounts of cpu

time  needed.   For best results, this program should be the

only  task  running.   You will find that STU is 100% system

friendly  and  you can test while doing other things with no

problems.   You will find that STU will grab all free memory

while  doing  memory  testing,  this may cause some programs

problems  so  it  is  best  to  run the memory tests with as

little running as possible.

Section 2.3 Functions

STU can   perform   single  or  combined  drive  tests.

Testing  may  be  sequential  or  random  in nature.  Random

testing  will jump all over the disk selecting random blocks

to  read/write(depending on the tests selected).  Sequential

testing  will  start  at track zero and cycle up to the last

track  on  the device.  The tests are based on the functions

available  to  the  drives  which  are: 1) Formatting(always

sequential),  2)  Seeks,  3)  writes,  and  reads.   You are

allowed  to  select  the  type  of  testing on an individual

drive basis.  Each drive is independent of others.

All tests  are  controlled by selecting the appropriate

gadget.  All operations are checked fully for errors.

User's Manual            Chapter 2                    Page 6

Custom Services     System Test Utility          Version 8.0

Operation

Section 3.1 Getting Started

To Start  with  you  will  need  to  backup  the  disks

supplied  with  the  normal  system  command DISKCOPY.  Make

several  backups  of  the  original  disk  to  save you from

accidental  writing  on  the  program  disk.  After you have

make  your  backups  then proceed to create your data disks.

STU  is  supplied  on a disk which has no copy protection(we

do  not believe in it).  If you do accidentally destroy your

only  working  copy,  we  will  replace  it  with the latest

version  if  you  send  us the disk(you pay the normal small

fee for shipping).

Warning, STU  can and will write on your disks.  If you

allow  it  to write on a disk, there is little chance of you

recovering  useful  data.   BE  CAREFUL!  This warning is so

important  I  will repeat myself again! Warning, STU can and

will  write  on your disk devices.  Even with the requesters

warning  you that you are about to write on the disk you may

make  mistakes.  Please be careful!  You CANNOT recover data

from   an  STU  written  disk.   STU  will  write  over  all

information(in  sequential  testing,  and most during random

testing).   This  can be rather deadly to a large hard drive

since  STU  will write data patterns(much like a destructive

search  for  bad  spots)  over every byte of data.  STU will

"pop"  up  a  requester  whenever  a format or write test is

selected  for a drive asking you if you are sure you want to

destroy  your  data.   Please be careful.  If you accidently

start  a  destructive test abort it with a ^C, ^D, or a Q as

soon  as possible, you may be able to salvage "some" of your

data.   Make  sure  you  backup your hard drive on a regular

basis  to  protect  against  "human"  errors  like  this.  A

backup is the only protection against human error.

Section 3.2 Initial Setup Options

When you  startup STU(either from workbench or CLI) you

will   get  a  window  which  contains  the  "Initial  Setup

Options".    You  will  see  an  options  window  like  this

whenever  you  are setting up STU tests.  The "Initial Setup

Options"  window  has  nine  gadgets.   The  first  two will

appear  on every options window.  They are special functions

common  to  all  the  options  windows.  The first one (top,

left  most  one)  is labeled "Done".  The "Done" gadget says

you  have  finished  selecting  your  options  and  wish  to

continue.   On  the  "Initial  Setup  Options", you have the

choices  Test,  Memory,  Disk,  Misc,  Devices,  About,  and

Exit.   You  will select one of these, and then click on the

Done  gadget  to  proceed.   The  reason  for  the  two step

User's Manual            Chapter 3                    Page 7

Custom Services     System Test Utility          Version 8.0

process  is  that you may have multiple gadgets to select on

other  option  windows,  the  Done gadget tells STU when you

have  made  your choice.  The other common option is labeled

"Reset".   The  Reset  will  clear any choices you have made

and  reset  to  the  first  option.   This  will  give you a

default  setup.  In the case of the Initial Setup Options, a

reset  will  select  the  "Test"  gadget as a default.   The

basic  method  of  selecting  an  option  is to click on the

option  you  want, then click on the Done gadget.  This is a

common thread to all selections.

The Test   gadget  tells  STU  to  start  the  selected

tests.   If  you have not selected any memory or disk tests,

it  will  be ignored.  To actually do some tests, you should

first  select  the  devices  and/or  memory  options(See the

Simple  walkthrough section later in the documentation).  If

you  have  selected  tests to run, they will run.  When they

are  completed,  you  will  be given a report of the results

and  will  return  to  the  Initial Setup Options.  Once the

Tests   are   run,  the  devices  and  memory  selected  are

released,  to  rerun  the  tests on disk drives, you have to

reselect  the  drives  again(see  the  Forever options under

disk  drives  if  you  wish to test for a really long time).

After  you  select  the  Test  gadget, you must click on the

Done gadget for STU to proceed.

The Memory  gadget  allows  you  to  select the type of

memory  testing  you  wish.   You will be able to select the

type of memory along with several test options.

The Disk  gadget has two possible actions.  If you have

not  yet  selected  the  Device  gadget  and  selected which

devices  you  wish  to  test(a  device  is a disk drive or a

logical  partition on a disk drive) you will get a requester

that  informs you of that fact and will give you a choice to

Select  or  Exit.   The  Exit  choice  will  exit the Device

selection  option.   The  Select  will do the Devices option

first, then the Disk options.

The Misc  option  will select the Miscellaneous options

which  is a catch all for the options which do not fall into

other categories.

The Devices  option  will  allow  you  to do the device

selection.   This  is  the  option that allows you to select

the partitions or drives to test.

The About  option  gives  a  little  bit  of text about

Custom Services.

The Exit  option  will  cause STU to clean up and exit.

Any memory reserved for testing will be released.

Section 3.3 Memory Setup Options

User's Manual            Chapter 3                    Page 8

Custom Services     System Test Utility          Version 8.0

This controls  the  memory testing that will occur when

you  select  Test  on  the Initial Setup Options.  If you do

not  select this option, no memory will be tested.  Once you

select   this   option,   the  highlighted  memory  will  be

tested(both   Fast   and  Chip  memory)  with  the  selected

tests(all  types of tests).  No test options will show until

you  have selected which devices you are going to test.  You

may  deselect  the  memory tests by clicking on the Fast and

Chip   gadgets(and   may  select  them  again  with  another

click).

The Fast  gadget  selects  whether or not you will test

all  available  fast  memory on your system.  If selected it

will  be  tested.   If  you do not wish to test Fast memory,

deselect it.

The Chip  gadget  selects  whether or not you will test

all  available  chip  memory on your system.  If selected it

will  be  tested.   If  you do not wish to test chip memory,

deselect  it.   Note:  If you elect to test chip memory, STU

will  grab all chip memory on the system.  This will prevent

you  from  opening  windows  and screens with other programs

during  the  tests.   You  may also experience a freezing of

the  mouse pointer and certain other normal system functions

like  re-sizing a window or switching screens for the simple

reason   that   the  system  needs  chip  memory  for  these

functions.   This  is  ok and will not cause a problem.  You

must  wait  for  the  tests to free up the memory.  STU will

test  each  fragment  and  release  it  after  the tests are

completed.   If  you  select  the  Forever  mode,  STU  will

continuously  grab  the  memory,  test  it, release it, then

repeat  the  process.  If you are testing both chip and fast

memory, the sequence will be:

1) grab chip and test it, then release it.

2) grab fast and test it, then release it.

3) if  Forever  mode,  go  back  to  step 1.  This will

allow  you  a  small  bit of time where there is chip memory

available  and  window/screen  options  will  work  at  that

time.   The  only caution is that some programs do not check

to  see  if  the memory they allocated was actually given to

them  and  may  crash  if  ran  at  the  same time as STU is

testing memory.

The WZERO  option  is  a special test that will write a

pattern  of  all ones in the memory and move a single bit of

zero  through all locations.  This detects any failure where

a bit cannot be set to zero.

The WONE  option  is  a  special test that will write a

pattern  of all zeros in memory and more a single bit of one

through  all  locations.   This  detects any failure where a

bit cannot be set to one.

User's Manual            Chapter 3                    Page 9

Custom Services     System Test Utility          Version 8.0

The Address  option  is the fastest test.  It creates a

test  pattern  for each location based on the address of the

location.  This ensures a varying pattern in memory.

The Pattern   option   is  a  complex  series  of  test

patterns  designed  to  detect intermittent failures.  These

patterns  vary  the  bit  patterns  in  ways  that will help

detect  failures  in any memory chip.  There are 128 special

patterns.   If you select the Long test you get all of them,

the Short version uses the first four patterns.

The Forever  option  causes  the testing to repeat over

and  over again.  This is a good option to select(especially

if  you  turned  on  the  option  to  pause  if  an error is

detected  or  selected  the  printer  option  to  print  all

errors)  if  you  wish to test a system overnight and do not

want it to stop.

The Long   option  selects  the  complete  128  pattern

tests.   This option has no effect unless the Pattern option

is selected.

The Short  option  selects  four  pattern  tests.  This

option   has   no   effect  unless  the  Pattern  option  is

selected.

The 8  bit,  16 bit, and 32 bit options select the size

of  the  test data.  It allows you to customize the tests to

the  particular  system.   For the older systems, the 16 bit

option  is  fine(systems  like  the A500, A2000, and A1000),

for  newer  systems  the  32 bit option is best.  The 32 bit

option  has  the  fastest tests.  Sometimes, errors that are

intermittent  show  up  under  different conditions, this is

why you have three choices for the test size.

Section 3.4 Disk Setup Options

This  option will allow you to setup the testing options for

each  disk  drive  you  have  selected.    The  options  are

grouped     at     tests(Format/Write/Read     and    Seek),

ordering(Sequential  or  Random),  size(1 sector, 1/2/3 or 4

tracks),  and  Forever and Data Verify.  The groups are such

that  you may only select one of a group.  Selecting another

will turn off the original selection.

The Format  options  selects  the special write that is

equivalent  to  what  the system Format command does.  For a

trackdisk.device(the  floppies),  this  will cause the write

portion  of  the  test  to  be  in  1 track sizes.  The read

portion  is  what  you select.  This is due to a restriction

of the trackdisk.device.

The Write  option  selects  writing  of  test patterns.

There  is  little  difference  between  the Format and Write

User's Manual            Chapter 3                   Page 10

Custom Services     System Test Utility          Version 8.0

except  you  can  write  any  size.  Both the Format and the

Write  will  cause  an  additional warning requester.  Note:

The   test   patterns   will  write  over  all  sectors  and

effectively  erase  and  contents  of  the disk.  If you are

planning  to test your Hard Drive, make sure you have a good

backup!  All data will be lost on the device in test.

The Read  option  does  a  read  of the selected media.

Reads  also occur when the Format, Write, and Seek tests are

selected.   You may select only one of the Format, Write, or

Read options.

The Sequential   and   Random   options   are  mutually

exclusive.   Sequential  will  start at the beginning of the

selected  device, and read to the end.  The Random will do a

fixed  number  of  I/O'S  randomly  scattered  all  over the

selected  device.   Random  is  more  like normal operation,

sequential is a more complete check of the media.

The size  of  the I/O is selected to be 1 sector, 1, 2,

3,  or  4  tracks  at  a  time.   The actual size of the I/O

depends  on  the  number  of  sectors  per track.  On a Hard

drive  with  35  sectors per track, you can test with 1, 35,

70,  105,  and  140  sector I/Os.  For a quicker test, use a

largest  size,  if  you  do not have sufficient free memory,

STU  will  issue  an error.  Try again with a smaller buffer

size.

The Forever  option  selects  continuous testing.  This

option  will restart the test on the selected device when it

completes.   Note:   If you select the Forever option in the

memory  testing,  and  disk  testing,  the disk testing will

never occur since the memory testing will run forever!

The Data  Verify  is  an  important  option,  when  STU

writes  on a disk, test patterns (much like the memory tests

has  patterns)  are  written to the disk and every sector is

identified  uniquely.   When  reads occur and this option is

selected,  the  pattern  is  checked.  If this option is not

selected,  the  data  read  is  not  checked.   If  you  are

writing(Format   or  Write  selected),  then  turn  on  this

option.   If  you are not writing and are not using STU Data

Disks(see  later  section),  turn  off  this option.  If you

turn  on  this  options  and  have a read test selected, and

have  say  a  standard workbench disk in the test drive, you

will  get  false errors.  This is a good test for you to run

now  while  your  system  is working fine since it will give

you  an  example of what an error looks like.  Also Note: if

you  have  the  Pause  option  selected(See  misc  under the

Initial  Setup  Options), and it is selected by default, you

will  have  to  hit  a  character in the window to continue.

See Keyboard Input for more information.

Section 3.5 Miscellaneous Setup Options

User's Manual            Chapter 3                   Page 11

Custom Services     System Test Utility          Version 8.0

The  Misc  options  are  general  options  that apply to the

whole operation.

The Printer   option  when  selected  will  attempt  to

output  the results of the current test to the printer.  Not

every  output to the screen is echoed, but the error reports

and  other  important information is.  This option is off by

default.

The Pause  option  has  to do with errors.  If an error

occurs,  and  the  pause option is turned on, you must hit a

key(or  click  with the right mouse button) in the window to

continue  the  testing.   If  this  option  is off, STU will

report  the  error  and  continue  as  if it did not happen.

This option is on by default.

The Verbose  option  will  cause  additional output and

will slow down the testing.  This option is on by default.

Section 3.6 Devices

This option  allows  you  to  select  the devices which

will  be  tested.   If  you  have  not  used this option and

select  the  disk  option,  you will be asked if you want to

use this option first.

User's Manual            Chapter 3                   Page 12

Custom Services     System Test Utility          Version 8.0

Testing Suggestions

Section 4.1 Making a Bootable Diagnostic Disk

STU is  not  distributed  as  a bootable disk.  You can

make  a  diagnostic  testing  disk  by making a copy of your

WorkBench  disk and copying the STU files over the Workbench

disk.   Write  protect  the  STU disk at all times.  The STU

disk  has  a  simple  startup  that will minimize the things

running  when the STU disk is booted.  Relabel the disk STU:

and  then  install  it.   STU  should  run  when the disk is

booted.   You  should  modify  the  startup-sequence to do a

SETPATCH and then STU as the only commands.

Section 4.2 Data Disks

Taking a   backup  of  your  STU program disk insert it

into  a  drive and double click the STU Icon.  To create the

special  Data  Disks STU will use in testing your drives you

will  need  to first do a STU version of the FORMAT command.

STU  allows  you to do this on all your floppies at the same

time.   Select  the device.  It doesn't make sense to create

data  disks  information  on  your  hard drive since it is a

destructive  test  and  all  normal  amigados  data  will be

lost.   Select  the  disk drives and enable the format test.

You  will  want  to  select  the Data Verify option for each

drive.   This  option  really only makes sense for floppies.

Drives  like  the  Insite  Flopticals  and Syquest cartridge

drives  have  very  expensive  media  and  it usually is not

worth  the cost to keep a test disk around.  The basic steps

are:

1) Select the devices you wish a data disk.

2) insert  media  into  each  drive(can  be  new or old

media).

3) Select  the  Format  or  Write  tests,  Data Verify,

Sequential, and

1 track for the I/O size.

4) run the test.

5) it  is  suggested  that you label and save the disks

when the testing

has completed.

For your  Hard  Drive,  you  cannot(unless  you  have a

User's Manual            Chapter 4                   Page 13

Custom Services     System Test Utility          Version 8.0

removable   cartridge  drive)  save  a  data  disk.   It  is

suggested  that  you do a full backup of a Hard Drive before

doing  any  testing.   That  means  a  full  backup  of  ALL

partitions.   While STU does test on a partition basis, that

is,  it  will  only  test the area defined by the partition,

leaving  the  rest  of  the  physical  disk  undisturbed,  a

hardware  problem  could cause a total failure of your drive

and  a  backup may be the only way to recover.  When testing

with  a  Hard Drive, you should use the Format or Write test

only  if  you  wish  to  do  a surface test.  DO NOT USE the

Format  or  Write  unless  you have the data backed up.  STU

will  do a destructive pattern test which will overwrite all

sectors  on  a  partition.   You  only  have  to  backup the

partition  under  test,  STU  will  only  access  the  areas

specified  in  your mountlist for the partitions under test.

If  you  do not wish the data overwritten, use only the READ

test and turn off the data verify.

With a  Hard  Drive,  you  have to create the data disk

each  time  you  wish  to  do  intensive testing.  Using the

FORMAT  or  WRITE  tests  will  write  over  any data on the

disk.   When  you format the disk drive using AmigaDos after

testing,  you  will  be removing the STU test data patterns.

If  you are having problems with a particular area of a Hard

Drive,  mount  a  partition that covers the area in question

and  test  only  that  area.   In  any  case,  expect STU to

destroy  any  data  on  that area(or a larger partition that

includes  that  area).   Always backup a hard drive prior to

testing  with  STU.  If you accidentally write test patterns

on  a partition, you will lose all data that was there.  You

will  have  to  do an Amiga dos format on any partition that

STU  has done a Format or Write test on.  If you did not get

errors,  you  can  use  the  Quick  option  on the Amiga Dos

Format to save time.

Section 4.3 Input Summary

During testing,  there are several options you can type

from   the   keyboard  which  will  allow  you  toggle  some

options.  These are:

Option   Tells STU to:

------   -------------

E     Toggle pause on errors during testing

Q     Quit the Testing, or Quit STU,

Control-D, Control-C also work.

V     Be more verbose with output,

You  are  allowed to toggle the "V", and "E", options during

User's Manual            Chapter 4                   Page 14

Custom Services     System Test Utility          Version 8.0

the  testing.   The  "Q, Control-D, or Control-C option will

cause  STU  to stop testing and shut down.  STU may not stop

immediately   since   it  will  complete  the  current  test

operation  before stopping.  This is may take a while during

the  memory  tests  which  only check for this at the end of

each test.

STU will  display  the  options  selected  prior to the

start of the testing.

Note, If  you  do not have enough memory on your system

to  support the buffer size you requested for the disks, STU

will  exit without performing any test.  If this occurs just

restart STU and select a smaller buffer option.

User's Manual            Chapter 4                   Page 15

Custom Services     System Test Utility          Version 8.0

Reference Section

Section 5.1 Testing Walkthrough

Lets assume   you   wish   to   test   memory  and  two

floppies(DF0:  and  DF1:).   The  first  thing  to  do is to

select  the  memory  tests  you  wish.   Since  we suspect a

problem,  we will test both Fast and Chip memory and run all

the  tests.   We have an A2000 with an accelerator(68030) so

we  will  test with the 32 bit option.  Since we do not wish

the  tests  to  take  a  long time, we will select the short

option.   To  do  this,  we  click on the Memory option, and

then  on  Done.   On  the Memory Setup, we see that Fast and

Chip  are  already  selected  along with all tests.  We only

need  click on the 32 bit option then on Done.  Now, we need

to  select  the  two  floppies  for  testing  so we click on

Devices  and then Done.  We click on the desired devices and

then  click  on  Done.  We will see the device list with the

selected  ones  highlighted.   After we hit any key, we then

click  on  Disk to set the test options and then Done.  Each

selected  device  will  appear with a setup option so we can

select  the desired tests.  We click on the settings we wish

and  then  on  Done.   The  display  above the Initial Setup

Options  shows  the test options we have selected.  We check

the  options  and  after  finding everything the way we want

it,  we  click  on  the  Test  and then on Done.  The Memory

tests  will  start  with  the Chip memory, and then the Fast

Memory.   The  Largest blocks of memory are tested first and

will  take the longest.  Once the Chip memory is tested, the

Fast  is  then  allocated and tested.  If at this point, you

wished  to abort the tests, you can type a ^C, ^D, or a Q in

the  window  and  when  the  current  test  is done, it will

stop.   The  memory  tests  will  cycle through the selected

tests  twice  and then pause waiting for you to hit continue

before  going  on.    Since  the default is verbose, we will

see  a  line  displayed for each test ran.  We can turn this

off  at  any  time  by typing in a V.  When done, you should

see  "Memory  Test  Completed:0  errors  found"  and a Pause

message.   Hit  any key and STU will start the disk tests by

asking  you  to  make the devices ready.  At this point, you

could  insert  STU Data Disks if you had them.  Click on the

Continue  button  to  proceed.   If you selected a test that

included   Format   or   Write,   you  will  get  additional

requesters  that warn you of that fact.  This is your chance

to  prevent an error.  Take your time and read the requester

to  make  sure  you  really  wanted to write on that device!

Once  you  have satisfied all the requesters(you get one per

device  that  you  wish to test), the testing will start and

run  to  completion.   When  completed,  you  will  get  the

standard  Pause  message and must hit a key to continue.  At

this  point,  you  will be back at the Initial Setup Options

and  can either select another test, or exit.  When you exit

STU(click  on  Exit and then on Done), you must hit a key in

User's Manual            Chapter 5                   Page 16

Custom Services     System Test Utility          Version 8.0

the  window  for  STU  to close and really exit.  This is so

you  can  read  the contents of the window and close it when

your ready.

Section 5.2 Memory Tests

While most   systems  will  not  have  a  problem  with

memory,  there  are  many  that  do have problems and people

never   find  them  until  they  start  loading  down  their

system.   It  is  particularly  hard  to  find what caused a

problem  when  you  have  4  programs running and the system

crashes!   This  usually  is  caused by some memory that you

rarely  use  which  is bad.  What happens is this.  You have

some  memory  that  you  never  use, or at least hardly ever

use,  it  is  at  the  end  of  some  memory  board.  As you

multi-task,  you  use  more  and more memory and eventually,

you  hit the bad section!  Bingo, a crash.  This can be very

frustrating   since   you  run  simple  things(each  program

separately)  and  no  crashes.   It  is  only  when  you run

several  programs that the system dies..  STU will find that

problem  and  give you an error report that shows what range

of   addresses   is  affected.   STU  collects  the  similar

errors(same   bit   pattern)   and   reports  the  range  of

addresses.   This  can  point  to  the problem in an address

decoder for example, or a shorted address line.

Memory error  Address xxxxxxxx to xxxxxxxxx

Observed: 0000 0000 0000 0000

Expected: 0000 0000 0000 0100

Diff: 0000 0000 0000 0100

The error  shows  up in the difference.  This is a very

powerful  way  to isolate memory problems.  By examining the

difference,   you   see  what  bits  varied  from  the  test

pattern.   This  can point you right to the chip if you have

a  schematic  for your memory board!  Even if you cannot fix

it  yourself,  this  information  will  reduce the time your

service  center  takes to find the problem and fix it!  This

means less costly repairs.

Section 5.3 Basic Drive Test

A basic  Drive test would be the "WRITE/READ/SEEK" test

with  random  I/O.  This will give a general test of all the

basic  functions  of  each  drive.  Make sure the drives are

write  enabled!   Selecting  the Write option will cause the

Write  then  a  Read  to  occur.  For a hard drive, the best

User's Manual            Chapter 5                   Page 17

Custom Services     System Test Utility          Version 8.0

test  to  detect  device  problems  is  to  do  a  READ/SEEK

Sequentially  since this will cause every possible sector to

be  accessed.    You should disable the data verify when not

writing or using STU Data Disks.

Section 5.4 Random Seeks and Reads

By using  the  Random  I/O  option  you create I/O that

will  jump  around over the disk in a random manner.  Drives

will  operate  at  different speeds due to the randomness of

the  seeks(some  drives will get shorter seeks than others).

If  you  do  not  have a data disk(or you are testing a hard

drive)  make  sure  you  disable  the data verify(or it will

generate false errors).

Section 5.5 Writes

Writing occurs  when  either the FORMAT or WRITE option

is  selected  during  the  testing.   The FORMAT option will

always  recreate the STU data Disk and may be applied to any

disk.   This  option  always  performs  sequential I/O.  The

buffer  size  will  be  the  size of one track for a format.

The  WRITE  option  will  allow  either sequential or random

I/O.   STU  always  does  a  read  after the format or write

operation  to verify that it was correct.  If you do a Write

on  a  disk that is not a Data disk, you will probably get a

few  errors  from  the Read portion due to the randomness of

where  you wrote! When doing a Write(or Format), even if you

want  to do a Random Test, do a Sequential one first to make

sure  every  sector  has  the  test patterns!  The way to do

this  is  to  do  a  Sequential Write(or Format) test first,

then a Random Read test.

Section 5.6 Individual Testing

Individual units  may  be  tested with any of the tests

described  in  this  chapter  by  selecting the device.  You

have  to  select  the  devices to test on the device option.

Each  device  can  be setup with it's own test(sequential or

random  i/o,  different  buffer sizes, and different tests).

By  creating  partitions,  you can limit the area of a drive

under  test.   Remember if you testing your Hard Drive, do a

Backup first!

You can  vary  the  tests from unit to unit by changing

each  drive individually.  When doing timing studies of your

system,  you  should  run  each device one at a time.  Doing

timing  studies  on  multiple  drives  at the same time lose

alot of accuracy.

User's Manual            Chapter 5                   Page 18

Custom Services     System Test Utility          Version 8.0

Section 5.7 Random Seeks and Head Cleaning

When a  problem  occurs  the  first defense, before you

attempt  to  service  the  disk  drive is to clean the heads

with  a  good  double sided cleaner.  STU will actually help

you  clean  your drive by moving the heads around.  This has

the  effect  of  extending the life of the cleaning disk and

your drives.

The best  way  to  do  a  cleaning is to setup a Random

Seek  of  1  sector  without  data verify.  Then, insert the

cleaning  disk  and start the test.  The Seeks will move the

heads  all  over  the  surface  and  do  a  very good job of

cleaning.   This  method  will  also  extend the life of the

cleaning  disk  since  the  random  distribution of the head

movement  will  spread  the  dirt over the whole surface and

not just one area.

It is  a  good idea to clean the heads on all drives on

a   regular   schedule.   While  cleaning  is  important  it

normally  is  not  needed  more than once a month.  Any time

read  or  write  errors occur it is a good idea to clean the

drive  just  to be sure.  If errors occur on a regular basis

you  may  need  to  either change the types of diskettes you

are  using  or  replace the drive.  Drives do wear out, even

the best ones will eventually have to be replaced.

Section 5.8 Fast I/O Tests

For quick  tests  of your drives the options specifying

a  larger  buffer can be used to decrease the time taken for

the  test.   When  a  particular  drive shows errors it is a

good  idea  to  rerun the test with the smallest buffer size

to  obtain  an  accurate  idea of where the error is.  Note,

certain  forms  of  copy  protection will deliberately place

bad  tracks  or sectors on the disk to prevent coping of the

disk.    These  disks  will  generate  various  errors  when

tested.    See  the  section  on  testing  other  disks  for

information on checking non-stu data disks in your drives.

Section 5.9 Device Timing

You may  wish  to  time  the access to a device.  There

are  two  ways  to do so.  You may run other normal tasks on

your  system  and  let  STU multi-task getting a timing that

represents  the device speed under loaded conditions, or you

can  shut  down  all  the  utilities,  PD software and other

tasks  and  run  the  test getting the most accurate fastest

time  possible.   The  buffer  size for a test will vary the

time  it  takes  to  do  an I/O, the smaller the buffer, the

faster  the  I/O but the more time it takes to read a device

User's Manual            Chapter 5                   Page 19

Custom Services     System Test Utility          Version 8.0

from  beginning  to  end.  For the fastest possible test you

should  shut  down  all  other  activity  and  test a single

device  at a time with the largest buffer that is a multiple

of  sectors  that  will  exactly  read  the  whole device or

partition.   One  track's worth of sectors is a good choice,

or  two  tracks if there are an even number of tracks in the

device.   A change in the characteristics of a device timing

may  indicate  a  problem.   If you use a small buffer size,

you  will  show a lower transfer rate than a large size(less

overhead  per  byte).  Realize that STU will probably show a

lower  number  than  what  you will get from most benchmarks

since  it  is testing the data and the drive while doing the

I/O.   This  extra work is the reason.  The numbers from STU

are  really  more  representative of the true to life access

times.

Section 5.10 Testing other disks

You may  test  any disk not just the data disks written

by  STU.   What  this  will tell you is where a disk has bad

spots.   STU  may  be  used  in  this  manner  if  you  do a

sequential  READ  test with no data verifying.  This command

will   cause   STU  to  read  every  sector  on  the  drive,

displaying  all  sectors  it  cannot read.  Just because STU

can  read  a sector does not indicate that it is correct, it

just  says  that  it  is  a  readable  sector.   Disks  with

deliberate  bad  spots  may generate read errors during this

test.

Make sure  you  have  the  "READ"  option  showing when

testing  a  non  STU  disk.   If you accidentally select the

"WRITE"  or  "FORMAT" option you will get a requester making

sure  you want to write on the disk.  Any disk written on by

STU  will  TOTALLY lose the original data, not even DiskSalv

can  recover  it.   This  applies  to Hard Drives as well as

floppies!

User's Manual            Chapter 5                   Page 20

Custom Services     System Test Utility          Version 8.0

Other items of interest

Section 6.1 Future Enhancements

This program  will  not  remain  static.   Just  as new

devices  are  added  to  the  system  for you to use, Custom

Services  will  be  adding  new capabilities to STU help you

find  problems.   The  areas  which  will  be addressed will

depend  on what is asked for from you the user.  Please fill

out  the  registration and suggestion form found in Appendix

A.  The Shareware fee is .

User's Manual            Chapter 6                   Page 21

Custom Services     System Test Utility          Version 8.0

Appendix A - Registration

PRODUCT: STU   REGISTRATION:  [yes] [no] (check one)

DATE:____/____/____

RECEIVED FROM:_______________________

______________________________

______________________________

Your  Name:____________________________

Email address:____________________________

Address:_______________________________

_______________________________

_______________________________

Who told you about this product:___________________

___________________________________

___________________________________

What improvements would you like:___________________

___________________________________

___________________________________

User's Manual            Chapter 7                   Page 22

Custom Services     System Test Utility          Version 8.0

Appendix B - Typical Testing times

Section 8.1 General

The times  here  represent some measurements on a A2000

with  5 Mbs of memory(1 MB Chip, 2 MBs 16 bit fast, 2 MBs 32

bit  fast),  2  hard  drives  (  40  MB Quantum with 2-20 MB

partitions,  a  52  MB  Quantum  with  a  single partition).

These  timings  were  done  with all the devices at the same

time.   All  times  are  in  milliseconds.   The  system  is

equipped with a VXL-30 68030 40 Mhz accelerator.

Operation      Minimum   Maximum     Rate    Device

Seek              74      1539             20 MB partition

Read             146      1494     490Kb/s 20 MB partition

Read             199      1384     360Kb/s  2 MB partition

Read             126      1077     796Kb/s 52 MB hard drive

Read             496      1409      22Kb/s 880K floppy

Write           1092      1620      10Kb/s 880K floppy

Write            908     14449*     61Kb/s 20 MB Insite Floptical

* -  The  Insite floptical was being tested with a disk

that  had  a  know bad spot.  This caused the drive to retry

attempting  to write the data patterns.  STU'S test patterns

were  causing  an  error  due to weak media.  This caused an

extremely long time for that I/O.

Note: The  floppy  I/O  was  2  tracks(22 sectors), the

Hard  drives  were  doing  4 tracks(140 sectors).  These are

representative  times,  yours may vary.  Also, these results

were  recorded  to  the printer which also can create delays

if  the printer buffers are full(like with verbose printing)

and  STU  has  to  wait  for  the printer.   This is another

reason  why  single  device  testing  is  better  for timing

tests.

These are  similar tests on the same drives.  Each test

was done on one device at a time.  Notice the difference.

Operation      Minimum   Maximum   Rate    Device

Write           747      4410    74Kb/s  20 MB Insite Floptical

Read            708      1122    78Kb/s  20 MB Insite Floptical

User's Manual            Chapter 8                   Page 23

Custom Services     System Test Utility          Version 8.0

Write          2010      2270    11Kb/s  880K floppy

Read            961      1525    23Kb/s  880K floppy

Read             67       307  1500Kb/s  52 MB Hard drive

You will  notice  that  the slow device(the floppy) did

not  improve much by being tested alone.  The faster devices

did improve dramatically!

User's Manual            Chapter 8                   Page 24

Custom Services     System Test Utility          Version 8.0

Appendix C - Suggestions

The best  suggestion  for  floppies and other removable

media  is to use a cleaning diskette.  A standard 3 1/2 inch

cleaning  disk  will  work  fine on floppies and drives like

the  Insite  floptical which have the save form factor.  For

Hard  drives,  the  problem  becomes  more complicated.  The

typical  problems with drives are the Media, power and cable

problems.   The  first  step  in  locating the problem is to

isolate  the symptoms.  Random errors scattered all over the

surface  tends  to  indicate  a  controller or cable problem

where  a series of errors grouped in the same area indicates

a media problem.

User's Manual            Chapter 9                   Page 25

Custom Services     System Test Utility          Version 8.0

Appendix E - Errors

Section 10.1 STU Messages

Error Text

Unable to allocate a buffer            Not enough memory

Observed xxxxx Expected at offset ddd  data error, disk read problem

or was not a data disk.

Closed, finished                       The drive has finished the

requested tests.

Section 10.2 SYSTEM errors

Unable to get a Port                   System problem getting started

Unable to get a pointer                System problem getting started

Unable to set break control            System problem getting started

I/O request Aborted                    Control C was used to abort

***These are the various disk errors which may occur***

Error couldn't be determined

Couldn't find sector header

Error in sector preamble

Error in sector identifier

Header field has bad checksum

Sector data field has bad checksum

Incorrect number of sectors on track

Bad sector header

Disk is write-protected

Disk has been changed or not in drive

Seek error verifying position

User's Manual            Chapter 10                  Page 26

Custom Services     System Test Utility          Version 8.0

Out of Memory for this operation

Bad Unit number for drive

Bad drive type, not an AMIGA drive!

Drive is busy, another task has it

***Drive: dd Unknown Error Code=dd***

With any  of  the  above  errors  the general procedure

would  be  to  clean the drive, retest, switch media, retest

before  suspecting  a  hardware  problem! If a cleaning does

not  eliminate  the  problem,  you should seek an authorized

service center to help you correct the problem.

User's Manual            Chapter 10                  Page 27

T a b l e   o f   C o n t e n t s

Chapter 1 General Information       ..................    1

Section 1.1 Introduction        ...................    1

Section 1.2 Policy        .........................    1

Section 1.3 Copyright        ......................    2

Section 1.4 License        ........................    2

Section 1.5 Warranty Registration and Reward     ..    2

Chapter 2 Program Operation       ....................    4

Section 2.1 General        ........................    4

Section 2.2 Limitations        ....................    4

Section 2.3 Functions        ......................    6

Chapter 3 Operation        ...........................    7

Section 3.1 Getting Started       .................    7

Section 3.2 Initial Setup Options      ............    7

Section 3.3 Memory Setup Options      .............    8

Section 3.4 Disk Setup Options      ...............   10

Section 3.5 Miscellaneous Setup Options      ......   11

Section 3.6 Devices        ........................   12

Chapter 4 Testing Suggestions       ..................   13

Section 4.1 Making a Bootable Diagnostic Disk    ..   13

Section 4.2 Data Disks       ......................   13

Section 4.3 Input Summary       ...................   14

Chapter 5 Reference Section       ....................   16

Section 5.1 Testing Walkthrough       .............   16

Section 5.2 Memory Tests       ....................   17

Section 5.3 Basic Drive Test      .................   17

Section 5.4 Random Seeks and Reads     ............   18

Section 5.5 Writes        .........................   18

Section 5.6 Individual Testing       ..............   18

Section 5.7 Random Seeks and Head Cleaning    .....   18

Section 5.8 Fast I/O Tests      ...................   19

Section 5.9 Device Timing       ...................   19

Section 5.10 Testing other disks      .............   20

Chapter 6 Other items of interest     ................   21

Section 6.1 Future Enhancements       .............   21

Chapter 7 Appendix A - Registration     ..............   22

Chapter 8 Appendix B - Typical Testing times   .......   23

Section 8.1 General        ........................   23

Chapter 9 Appendix C - Suggestions     ...............   25

Chapter 10 Appendix E - Errors     ...................   26

Section 10.1 STU Messages       ...................   26

Section 10.2 SYSTEM errors       ..................   26

-XXVIII-