************************************************************

*                                                          *

*      Enhanced DBLNTSC display: 704x480 31.55kHz/60hz     *

*                                                          *

************************************************************

What it's good for:

This archive solves the main problems associated with

AGA DBLNTSC/DBLPAL monitor drivers, which are:

(1) The scan rates aren't really compatible. VGA monitors

really require a 31.5kHz horizontal scan rate and a

minimum 60Hz refresh rate. This archive provides

exactly that.

(2) It's hard to get the screen centered on my monitor.

The HSSTRT and HSSTOP tooltypes have been set to give

a display that is centered in the same place a PC

VGA card would put it. MonEd is enclosed if adjustments

need to be made.

(3) There are large gaps around the display, which doesn't

seem wide enough to cover the display without severe

restrictions in display quality. The hacked Overscan.prefs

file gives 704 across (rather than 640) which leaves only

very small borders.

(4) Some error screens aren't displayable on VGA monitors.

The BootControl program solves this problem.

How to install it:

(1) Unarchive the DBLNTSC31.lha file in a temporary directory.

(2) Copy the MonEd files to C:.

(3) Copy the Overscan.prefs file to ENVARC:SYS

(4) Copy the DBLNTSC.info file to Devs:Monitors. Make sure that

the DBLNTSC executable file is already there as well.

(5) Copy the BootControl files to C:

(6) Add the following line to the top of your startup-

sequence:

BootControl MODE=VGA INSTALL BOOT GFX=BEST LOADVGA PATCHALERT QUIET READICON >NIL:

Note that the centering on the bootscreen will not be right

until the HSSTRT and HSSTOP values get loaded, but it will be

visible at least. The BootControl program will make the DBLNTSC

mode as the default mode.

(7) Use the ScreenMode Prefs program to set the display to 704x480.

I like to use 128-color mode.

(8) Reboot you computer. The computer will boot twice as BootControl

installs the DBLNTSC monitor driver as the default display.

(9) You should then be treated to a much-enhanced DBLSCAN display!

(10) I recommend reading the docs to MonEd and BootControl.

Send thanks, ideas, bug reports, complaints, etc. to bitbrker@ionet.net