Asm05

Fast Assembly Tool for the MC68HC05 series microcontrollers

Asm05 is an assembler set up to turn an ascii text file into an 8192 byte

binary image ready for chip programming.

usage: Asm05 <source file name> [flag] [flag] [flag] [flag]

The source file name would normally have a ".asm" extension. If no errors are

found during assembly an image file is created with a ".img" extension.

The first optional flag signifies the target processor. The default is the

MC68HC705C8.

flag "a" MC68HC805C4 -  4k of flash memory (EEPROM)        -  8K image.

"b" MC68HC705C8 -  7k of EPROM memory (window or OTP) -  8k image.

"c" MC68HC705C9 - 15k of EPROM memory (window or OTP) - 16k image.

The main purpose of this flag is to sort out the size of the binary image

that will be generated. Also the MC68HC705C8 has an unusual binary image that

needs to be padded with $00 bytes rather than the normal $FF. There is a

wide range of processors in the 6805 family but they all conform to one

of the three variations above as far as assembling binary images.

The optional flag "s" instructs the assembler to generate a symbol list.

This symbol list is a text file with a .sym extension. This file lists the

address in hex of each lable. This may be necessary for debug purposes.

The optional flag "d" instructs the assembler to generate a debug file. The

debug file is used by various realtime debug commands to find out the address

of variables. This binary file is stored in the temporary directory (usually

on the RAM disk) as T:DebugV.bin. This file must be present and up to date

before any realtime debug commands can be used.

The optional flag "v" instructs the assembler to generate a variable list

The variable list is useful for checking what memory locations have been

allocated to variable by the auto allocator

The optional flag "p" instructs the assembler to assemble a "project". A

project is simply a list of assembler files. So insted of putting all your

code into one file you can split it up into modules. Make a list of

the files in your project. Assemble this list with the "p" option. The

project list must have each file name (and path if necessary) on a seperate

line with no comments.

As an example to assemble say three source modules into a binay image ready

for programming a MC68HC705C9 chip. And generate a symbol list. And generate

an error text file. At the CLI ..

join file1.asm file2.asm file3.asm as file.asm

Asm05 >errors file.asm c s

If all is successful this will generate file.img, file.sym and errors a

text file with status information.

This is a non standard assembler that uses a mix of 6805 6502 and Intel

assembly syntax. All opcodes are 3 bytes long. This approach simplifies

reassembly on different processors with other FAST assemblers. Don't

panic the assembly syntax closely follows factory 6805 syntax so it is easy

to convert if necessary (see convert.doc).

Instruction Format

.uses typical assembler format

label field	op code field	address field	comment field

loop		LDA		$51		;A = value at loc $51

ADC		#$01		;A = A + 1

BBS		$6C.2 loop	;if bit 2 at $6C set

;  goto loop

Rom Labels

for nameing locations in ROM for subroutine calls branching etc.

.Romlabels MUST begin at the first character on a line

.rom labels can be of any length but only the first 8 characters are

distinguished

.upper and lower case characters ARE distinguished

.rom labels MUST begin with a lower case letter a..z

.rom lables must not begin with an upper case letter

this is reserved for declaring variables (see below)

.rom lables must not begin with a numeric character

they is reserved for branch lables (see below)

.you can use rom labels with branch instructions

.all rom labels are global ie they can be accessed from any module

example

JSR doit

...

doit	LDA #$00

...

table   $00,$01,$02

Branch Labels

this type of lable is reserved for branch instructions (relative addressing)

and is useful for making code more modular ie you don't have to dream up

lots of different label names.

.branch labels consist of a single numeric char 1..8

.branch labels are referenced foward or back with arrow operators

.be careful the assembler picks up the first lable it finds foward

or back ie don't try & branch over a label with the same name

example

1	LDA table,X	;

STA buffer,X	;

DEC X		;

BNE 1<-		;

BRA ->2		;

NOP		;

2	NOP		;

Op Code Field

.all codes consist of 3 characters followed by whitespace (space,tab,rtn)

.upper and lower case IS NOT distinguished

.op code field must not start at the begining of the line

.Motorola 4 char & 5 char codes have been converted to 3 character  format

substitute 3 character codes

Motorola	FAST6805	Function

BHCC		 BFC		Branch if half carry Flag Clear

BHCS		 BFS		Branch if half carry Flag Set

BRSET	 BBS		Branch if Bit Set

BRCLR	 BBC		Branch if Bit Clear

BSET		 SMB		Set Memory Bit

BCLR		 CMB		Clear Memory Bit

STOP		 STP		stop

WAIT		 WAI		wait

INCX		 INC X		note the address field is seperated

DECX		 DEC X

RORX		 ROR X

additional codes allowed (but not recomended)

INX			Increment X register short for INC X

DEX			Decrement X register short for DEC X

Pseudo Ops

Pseudo Ops direct the assembler during assembly, they place no code in the

map file. This assembler has only two.

psop		example			comment

ORG		ORG $0100		set origin of code in memory map

default is $0100

more than one ORG may appear in file

code overlapping is not checked

END		END			denotes end of processing

end is compulsory

Data Tables

Data tables are used to load bytes directly into memory. For the sake

of portability to other assemblers it is recomended you use the "," as

a seperator.

.unlike other assemblers db pseudo op is not used

.data tables must start in op code field

.only hex format recognised

.valid separators are comma blank or tab

example

$8C,8c,$ff,$FF,$CC,$CC,$CC

$8C 8c $ff $FF $CC $CC $CC 	;comment

Ascii Text

To load text directly into ROM just enclose the text in inverted

commas on a seperate line in the opcode field.

example

ORG $2000

"this text will go into ROM @ loc $2000"

Vector Addresses

It is sometimes required to place the value of a label (an address) directly

into memory. An address table is required for the interupt and reset vectors

for example.

.use the # symbol to tell the assembler to place a label value

.vector addresses appear in the op code field

.only one entry is allowed per line

example

ORG $1FF4		;vectors are here in the HC05C8

#spi_int		;spi inteface interupt vector

#sci_int		;sci inteface interupt vector

#timer_int		;timer interupt vector

#irq_int		;interrupt request vector

#sw_int			;software interupt vector

$01 $40			;reset fixed at loc $0140

Address Field

.must be separated by white space from op code field

.12 addressing modes supported

name		example		comment

Inherent		INC		A register is implied

INX		X register is specified in op code

MUL		A & X register implied

RTS		control instruction

Register A		INC A		same code generated as inherent

Register X		ROR X		"X" must appear in address field

Immediate		LDA #$00	only hex supported

LDA BUFFSIZE	predefined constant - no # like C

Direct		STA $51		write directly into page zero

STA Variable	write directly to a predefined variable

Extended		LDA $0100

LDX label

Indexed		ADD ,X		"," compatable with motorola syntax

ADD @X  	"@" is easier to spot

Indexed Page Zero	ADC $58,X	no lables allowed with RAM addressing

Indexed Extended	JSR $0F00,X

EOR table,X

Relative		BRA loop	lables only with relative

BSR doit

BEQ ->1		local branch foward

BNE 8<-		local branch back

Bit set clear	SMB $8C.0	set least significant bit at loc $8C

CMB $8C.7	clear most significant bit at loc $8C

CMD MyFLAG	yes you can define names for each bit

Bit tst & br	BBS $5F.3 lbl	branch to label lbl if bit 3 at loc

$5F is set

BBC $5F.0 lbl	branch to label lbl if bit 3 at loc

$5F is clear

BBC MyFLAG lbl	branch on a bit field variable

BBC MyFLAG ->1	branch fwd on a bit field variable

Comment Field

.denoted by ";" or "*" character

.everything to the right of the comment character is ignored

.comments can begin at the first character on the line

Variables

.variables are names given to locations in RAM memory

.variable must be declared before they are used

.variable MUST begin with an uppercase letter A...Z

.variable are declared by placing their name at the first charater position

on the line (the label field)

examples

Index	$50		; this variable declaration names RAM loc $50 as Index

RcvBuff	$0100		; this variable is in extended memory

LDX Index	; set index reg to value at loc $50

STA RcvBuff,X	; write A reg to a buffer at $0100 + index

Bit Field Variables

.all rules for the variable above apply

.the assembler does not care what name is given as long as

the first letter is uppercase.

.it is recomended you use the naming convention used in the examples

to avoid confusion with normal variables (ie bit name in uppercase

byte name like above).

examples

TcrIEDG   $12.1  ; this names the timer control register input edge bit

TcrTOIE   $12.5  ; and timer control timer oveflow interupt enable

; note these 2 use the Motorola names exactly

MyFLGA    $60.0  ; this names an A flag in a byte

MyFLGB    $60.5  ; this names a B flag in the same byte

Constants

.all rules for the variable declarations (above) apply

.the assembler does not care what name is given as long as

the first letter is uppercase.

.it is recomended you use the nameing convention used in the examples

to avoid confusion with variables (ie all uppercase).

.NOTE when using the constant you don't need the # symbol like other

assemblers. Assembles normally don't have constants & variables but

use a text substitution using the EQU pseudo op. Fast is more like

C in this regard.

examples

SIZE   #$10       ; declare a constant (before you use it)

LDA SIZE   ; use it

STA Index  ;

Auto Allocation of Variables

to allow code to be more modular this assembler will allocate variables

on your behalf. there are three types of variables that can be declared

for auto allocation, Page Zero, Extended and Bitfield.

.you must declare one page zero variable and one extended variable

to tell the assembler where to start.

.bitfield variables will be allocated to the last Page Zero variable

declared. after 8 bits you must declare another Page Zero variable.

ie always declare a Page Zero parent variable before declaring Bit

Field variables

.you can reserve more than one byte by placing a HEX value inside

brackets when declaring Page Zero or Extended variables.

.you can always see what the assembler has allocated buy assembling

with the "v" flag

examples

; allocated to

ExtMem		$0100		; $0100

Spare		$50		; $50

Count		PgZ		; $51

Flags		PgZ		; $52

FlgBITA		BtF		; $52.0

FlgBITB		BtF		; $52.1

FlgBITC		BtF		; $52.2

Buffer		PgZ[0A]		; $53 ... $5D

Varible		PgZ		; $5E

ExtBuffer	Ext[10]		; $0101 ... $0111

ExtVar		Ext		; $0112

Traps for young players...As the autoallocator uses the last declared

"fixed" variable as the starting point you must declare all your fixed

variables (eg hardware ports) in your first main assembler module.

Error Messages

message				cause

invalid op code			op code field syntax error

invalid address field			address field syntax error

or wrong addressing mode

undefined label			label in address field not matched

duplicate label			label declared more than once

address out of range			relative address out of range

data table format			syntax error in data table

unexpected end of file  FATAL		no END psuedo op found

too many errors  FATAL		more than 128 errors

output map overflow  FATAL		writing to loc outside ROM

too many labels max 512  FATAL	exceeding limit on labels

FATAL errors cause termination of processing