FAST 68HC05 DEVELOPLEMENT SYSTEM

This is a developement system for the Motorola 6805 series of

microcontrollers. The system consists of several tools or "Commands"

that run from the CLI. The commands work in conjunction with a monitor

and real time debugger, which are programmed into the chip along with

your 6805 code.

COMMANDS

Asm05   the assembler (see Asm05.doc for full details)

Pgm8C4  programs empty 68HC805C4 devices

Pgm7C8  programs 68HC705C8 devices

Pgm7C9  programs 68HC705C9 devices

PgmIC   reprograms a 68HC805C4 device in circuit

Img2s19 converts ROM images to motorola S record files

rm      Read Memory

rv      Read Variable

rpz     Read Page Zero

cm      Change Memory

cv      Change Variable

go      Run

stp     Stop

DEBUG CODE

monitor.asm   basic 6805 reset monitor

dbug.asm      serial port real time debugger

To use the system you need a 68HC805C4 device. This device has 4K of

EEPROM (or Flash) memory for program storage. This device can be

erased and reprogrammed in the target circuit via its serial port. It

is an ideal device for program developement as it can be reprogramed

any number of times without removing it from the test circuit (unlike

EPROM based devices).

The assembler, monitor and debugger can be used to target any 6805

family device.

The Devlopement cycle goes like this

1) write some code

2) join the code to the monitor code using the amigados join command

3) assemble this with the assembler

4) program an empty 68HC805C4 using the Pgm8C4 command (empty

devices require a simple bootstrap circuit).

5) place the chip in the target circuit

6) apply power (including VPP) then issue the go command

7) when it doesn't work hit reset

8) now use rpz, rm or rv commands to view the insides of the chip

9) fix the bug

10) now assemble but use the PgmIC command to download the new code

in circuit

11) re issue the go command

12) if it doesn't work goto 7) again

13) when you have a fully working device burn an EPROM or OTP

device using the Pgm7C8 command.

14) sell the design to a large corporation for a large sum of money

15) send me 10%

COMMANDS IN DETAIL

Asm05 <sourcefile> [flag] [flag] [flag]

Flags:

Select MCU

"a" MC68HC805C4 -  4k of flash memory (EEPROM)        -  8K image.

"b" MC68HC705C8 -  7k of EPROM memory (window or OTP) -  8k image.

"c" MC68HC705C9 - 15k of EPROM memory (window or OTP) - 16k image.

default b = 68HC705C8

Symbol List - s = create text file listing labels

Debug File  - d = create binary file for debugging

Variable List - v = create variable list

Project File - p = use a project source file list

This is the assembler. It Creates a ROM image file with a .img

extension or outputs an error listing. See Assembler.doc for full details

Pgm7C8 <ROM image file name>

This command progammes the 68HC705C8 EPROM or OTP device via the

serial port using the Motorola bootstrap circuit.

Pgm7C9 <ROM image file name>

This command progammes the 68HC705C9 EPROM or OTP device via the

serial port using the Motorola bootstrap circuit.

Pgm8C4 <ROM image file name>

This command progammes the 68HC805C4 EEPROM device via the

serial port using the Motorola bootstrap circuit.

Img2s19 <ROM image file name>

This utility converts ROM images to motorola S record files.

ebf <rom image file name> <start hex addr> <end hex addr> <origin>

Extract Binary File

This CLI tool extracts a binary data from a binary file and writes

a smaller file with a .bin extension. The primary use of this tool

is to extract blocks binary of data/code from the rom image files generated

by the Fast Assemlers. However, this tool is not limited to handling rom

image files, you can cut out data from any file up to 64k in size.

Notes

.all arguments must be given

.when extracting from rom images origin is set to zero

.start, end & origin are limited to word size. ie you can only use this tool

on binarys up to 64K in size. It was written with 8 bit CPU's in mind.

The following commands work in conjunction with the monitor (see below)

for in circuit emulation program development. They require a 4MHz XTAL

plus VPP on the target circuit with the chips serial port connected to the

Amiga (with RS232 drivers).

PgmIC <ROM image file name>

Program In Circuit

This command programmes the 68HC805 in circuit via the serial port.

Note that all bytes are verified during programming as part of the

hanshake protocol.

VfyIC <ROM image file name>

Verify In Circuit

This command will read the ROM from the 68HC805 in circuit via the

serial port. Then compare each byte to a ROM image file.

rpz

Read Page Zero

This command dumps the contents of page 0 ($0000...$00FF) inside the

805 to the Amiga screen. And consequently can be redirected to

a file.

rm <hex address>

Read Memory

This command reads 16 bytes from anywhere in memory and displays them

on the screen.

rv <variable name>

Read Variable

This command findes the current value of a variable you have declared

in your code. You must assemble with the "d" flag for this command to

work.

go

Start executing

This command causes the monitor to jump into the users program. The

monitor clears to $FF all of the RAM in page 0. Use a hardware reset

to get back to the monitor.

The following commands work only in conjunction with the real time

debugger & monitor (ie you must join monitor.asm & debug.asm to your code

before assembly).

cm <hex address> <hex data byte> [data] [data] ....max 16

Change Memory

This command changes the contents of any series of RAM locations inside

the chip. Note that this command re reads the chip to verify.

cv <variable name> <new data>

Change Variable

This command changes the contents of memory using your variable names.

ny series of RAM locations.You must assemble with the "d" flag for this

command to work. When changing individual named bits use either 0 or 1

to clear or set bits. Note that this command re reads the chip to verify.

stp

Stop

This command stops execution of your program & hands control back

to the monitor. It saves you the bother of hitting reset. The

contents of the chip are then frozen so you can examine the damage.

Then reprogram the chip.

THE MONITOR

This file (monitor.asm) is a simple reset monitor that you join to the

END of your code insted of the normal ROM vectors (vecC8.asm). When

installed in an EEPROM device like the 68HC805C4 it will allow in

circuit developement. The monitor communicates with the commands PgmIC,

rpz, rm, rv & go.

THE DEBUGGER

This file (dbug.asm) is a realtime debugger, ie it operates while your

program is running. It takes over the serial port so you must write any

serial port modules just using the reset monitor. Somewhere at the start

of your code you must "JSR dbg_open" ie open the debug port. The

debugger lets you read variables while you program is running (very

handy). Better still you can change the value of any RAM memory

location. The debugger communicates with the commands rpz, rm, rv, cm,

cv, & stp.

GENERAL NOTES

Use script files

Amigdos scripts are a handy way to automate the developement cycle.

I normally write a simple script to assemble, reprogram the chip and

isue the "go" command. Using this technique you can edit any file with

your text editor, save it then re-run the script. A new version of the

chip will then be running and ready for testing in a few seconds.

Large Projects

Usually large projects are broken up into seperate assembler modules.

The FAST assembler is fast enough that all modules can be re-assembled

every time elimanating the need for a linker and make utilities. Utilise

the assemblers ability to assemble files from a list of source files.

The monitor is placed last in the file list as it supplies the "END"

assembly op code.

ROM Images & S Records

The FAST series of assemblers generate binary ROM images. These files

can be burned directly into ROM/OTP/EPROM/EEPROM as is. S records are a

Motorola standard file format for representing ROM images in HEX format

(ie they are text files). Some developement systems only support S

records. Use the Img2S19 utility to convert the output of the assembler.

If you use batch files this can be done automatically.

Bootstrap Programming Hardware

The 68HC705 & 68HC805 devices have self programming algorythms built

into ROM. To access these you need to set up a special RESET circuit and

some switches to one of the chips IO ports. See Page 3-1267 in the

MICROCONTROLLER DATA book. To get the FAST system going you need to

connect the serial port on the chip to the Amiga serial port via RS232

drivers. 705 type devices require 14.5V on VPP. 805 devices require 19.5

via a 10K resistor on VPP.

In Circuit Programming Hardware

To reprogram devices "in circuit" you must first load the monitor into

a blank 68HC805 device using the above bootstrap method. Once the

monitor resides in memory you can place the device in the target

circuit. The serial port must connected to the Amiga again via RS232

drivers. Also the 19.5V via 10K resistor must be present on the VPP pin

(pin 3). Once powered up you will be able to communicate with the

monitor via the debug commands. When the chip is to be reprogrammed a

small program is loaded into RAM which erases the entire chip then

reprogrames the new code via the serial port. As the entire chip must be

erased each time the monitor must be re-assembled and uploaded along

with the program under test. If this reprogramming process interupted

the monitor can be lost and the device will have to be bootstraped from

scratch.

Good Luck