FAST05 Disassembler

Dis05 is a disassembler for the MC68HC05 series Microcontrollers.

The tool runs from the CLI and takes the form.

Dis05 <binary file> [control script]

Dis05 outputs a assembler file of the same name but with a ".asm"

extension.

An optional control script can be used to control the disassembly

process. You can set start & end points within the binary file

and also declare data sections to avoid the generation of

garbage. The control script has four commands

o <hex addr>             set origin

e <hex addr>             set end

l                        add a memory listing to the comment field

d <hex addr> <hex addr>  declare a data section

The command character must be the first chr on the line.

No comments are allowed.

You can declare as many data sections as you like.

for example

o 0100

e 01FF

l

d 0114 01FF

this script

.starts disassembly $0100 (256) bytes from the begining of the

binary file

.ends disassembly at byte $01FF (511).

.A comment will be added to each line of output listing the address

of the line

.binary data from loc $0144 to loc $01FF will be disassembled into

data tables. binary data from loc $0100 to loc $0113 will be

disassembled into code.

All output can be reassembled as is on the Asm05 assembler. When

disassembling rom image files (from Asm05) you will need a control

script to avoid disassembling large sections of unused rom.