COVERTING Asm05 SOURCE CODE TO MOTOROLA FORMAT

The Asm05 assembler is not compatable with standard 68HC05 assemblers. It

borrows syntax from Intel & 6502 assemblers. Listed here are the

differences.

1) Motorola 4 char & 5 char codes have been converted to 3 character

format

Motorola	FAST6805	Description

BHCC		 BFC		Branch if half carry Flag Clear

BHCS		 BFS		Branch if half carry Flag Set

BRSET	 BBS		Branch if Bit Set

BRCLR	 BBC		Branch if Bit Clear

BSET		 SMB		Set Memory Bit

BCLR		 CMB		Clear Memory Bit

STOP		 STP		stop

WAIT		 WAI		wait

2) When refering to registers the Fast assemblers expect the register

name in the address field not the opcode.

Motorola	FAST6805

INCX		 INC X

DECX		 DEC X

RORX		 ROR X

ASLX		 ASL X

CLRX		 CLR X

COMX		 COM X

LSLX		 LSL X

LSRX		 LSR X

NEGX		 NEG X

TSTX		 TST X

INCA		 INC A

DECA		 DEC A

RORA		 ROR A

ASLA		 ASL A

CLRA		 CLR A

COMA		 COM A

LSLA		 LSL A

LSRA		 LSR A

NEGA		 NEG A

TSTA		 TST A

3) The bit field instructions take a different format

Motorola			   Asm05

BSET 1,$50			SMB $50.1

BCLR 1,$50			CMB $50.1

BRCLR 1,$50,label		BBC $50.1 label

BRSET 1,$50,label		BBS $50.1 label

4) In standard assemblers data tables uses db in front of each line

of the table.

Motorola			   Asm05

db $FF,$77,$33,$EE		$FF,$77,$33,$EE		; data table

dw #label

5) In standard assemblers ROM vectors uses dw in front of each

vector to signify a word size.

Motorola			   Asm05

dw #label			#label

To convert a file do the following

1) convert opcodes 	- replace "BBS"     	with "BSET"

2) convert registers	- replace " X"      	with "X"

3) convert bit fields 	- replace " $**.*"  	with " *,$**"

4) convert tables	_ replace "<tab>$"	with "<tab>db $"

To Convert variable declarations

1) - replace "<Name>"  with "<Name> EQU"

To Convert files containing named bits

1) replace "<bitname>" with "#,<bytename>"

Do It The Easy Way

When converting files if all the bit fields are named then a simple

search & replace file is all that is required. The utility "TxtRep"

takes a replace file (a list of search & replaces) and a source file

and produces a new source file compatable with Motorola syntax. Just

make up a ".rep" file listing all the named bit fields & their Motorola

replacements (ie NameBIT|#,Name). Use the file "ISet.rep" to convert the

instruction set. Duplicate your  header file with the bit fields missing

and EQU added to variable  declarations. Use the alternate header when

assemblig via motorola.

I usually make up a batch file which

1) makes a new copy of my Asm05 source (or sources)

2) uses TxtRep with ISet.rep to fix instruction set

3) uses TxtRep with a rep file to fix any bit fields

4) uses TxtRep with a rep file to add EQU to my variable declaration

files