******************************************************************************

NOTE: This doc was written in one night, when I decided to get this very, very

incoplete prog. to you and say ..now you're beta tester..

.. so use at your own risk !!

All tips & suggestions are welcomed so don't be lazy and write crazy !

******************************************************************************

In this archive should be included:

-----------------------------------

PICPgm

PICPgm.info

pic.doc

pic.doc.info

pic_kit09.readme

bin.info

BIN/

picasm

examples.info

EXAMPLES/

LED_Busting.p84

LED_Busting.hex

rexx.info

REXX/

pic.ged

hw.info

HW/

hw.iff

hw.eps

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

This archive may be distribued as long as all the files listed

above stay together and'll left unchanged.

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

PIC Developer's kit v 0.9 beta.

===============================

Please, don't think that this document is manual for PIC microcontrollers.

NO !! This isn't DataBook and never'll be. So more infos about PIC controllers

are in official Microchip materials and especially in [1].

What it's ?

===========

This package include PIC 16C5x, PIC 16C84 cross assembler and PIC 16C84

programmer. To allow program those stuffs you'll need a piece of HW too. So

little pretty piccy of very basic programmer is included. This version'll works

with this programmer but all newer versions that'll able to program PIC 16C5x, PIC

16C7x and other will need a bit special programmer. Still low cost, but more

expandable (like EPROM 2716..27256 etc.). This is done for you that you can

develop and debug software for 16C84 which has ( as one) 1Kb of EEPROM. And in

this case is suitable for developing. In fact I got to you quite powerful tool

for development in price of ICs you need to construct the programmer.

All never versions will be shareware that'll not be compatible with this

one ( programmer). But maybe I'll release newer value version for 16C84 (like

this) again.

NOTE:   All future developing is in your hands. Please, if you are interested in

this project, give me a sign that'll give me power to do more for Amiga.

Amiga isn't only TOY, so take a care for her Jobs !! :-)

Instalation:

============

Copy all file is one directory ( for example  DH1:Apps/PIC_Kit) and add assign

"Assign >NIL: PIC: DH1:Apps/PIC_Kit". This assign is necesary only if you want to use

arexx macro for GoldED.

An arexx macro is added for using this kit with GoldED, so if you want to use

it, you have to install this in your GoldED. Simply by adding this into the menu

strip as an arexx script.

assembler v0.9

================

implemented:

- HEX16 code support

- full PIC 16c84 support

- both serial and parallel programming mode

non implemented

- PIC 16C5x support

This is only limited in this version (anyway - available)

- INHX8 (merged, split)

don't cry will be soon as possible (tommorow's late)

If you think that there is missing something you can live without please let

me know and I will do maximum to satisfy you.

Short info about gadgets:

"INFO" area

-----------

INFO gadet  - i think there's nothing special

"Actions" area

--------------

Action "Program" - Allows you to select specific operations like program,

read, removing code protection etc.. See [1].

Action "Type select" - Now you can select 16c84 or 16c5x, but 16c84 only

works.

Action "Oscilator type" - LP, HS, XT, RC type. See [1].

"CFG Word" area:

----------------

Ext - If selected external configuration word is used from a HEX file.

PW - PWRTE Power On Reset Timer Enabled see [1].

WD - WDTE Watch Dog Timer Enabled see[1].

CP0, CP1 - Code protection flags for future compatibility with P16C7x

is PC1 flag added too, anyway see[1].

"Prog area" area

----------------

PM - When program action is selected, this gadget determines if program

memory'll be programed.

EM - If type = p16c84 this gadget determines if Data EEPROM will be

programmed.

"ID" area:

----------

Here you can enter 4 hexadecimal numbers that are ID nubers for your

products. In fact you can program eight bits, but when code protection is

selected and upper 4 bits are not set to zero you can't read this number

corectly, so in this programmer you can enter only lower 4 bits of each 4

bytes that represent hexadecimal number. ID is then readed corectly

independent on code protection.

"Method" area

-------------

You can select serial or parallel, but remember that RemoveCP, EraseDM,

ErasePM can be used only when parallel mode is selected.

"Format" area

-------------

Sets current input and output format.

Now is supported INHX16 ( HEX16) only.

"GO"

----

Starts selected actions read, program etc.

"LOAD" & "SAVE" & "EXIT"

------------------------

No comment

------------------------------------------------------------------------------

Literature:

-----------

[1] Microchip Data Book 1994

(c) 1994 Microchip incorporated

To contact:

-----------

E Mail:

beyer@jackal.cis.vutbr.cz

Post:

Tomas Beyer

Slavkovska 983

Bucovice 685 01

Czech Republic

Tel.(Fri - Sat):

+42 - 0507 912 983