IBMKEY v2.5 (27.1.97)

#####################

Copyright  1997 Stephen Marsden

Connect a PC keyboard to any Amiga.

Inspired by Eric Rudolph.

If people wish to sell this hack they are free to do so.  It won't

make you rich but it may help tower case manufacturers.

INTRODUCTION

############

This is a small hack I made many months ago and is the original reason

for writing Epic.  It is difficult for me to remember how it works

but I will try my best.  Here goes:

Many A1200 owners have expanded their machines with CD-ROMS and

hard-drives, the only problem being where to put them.  The option of

putting everything in a tower case has appealed to many people but the

most difficult aspect is incorporating a detachable keyboard.  It's

not simply a matter of connecting a A1500 keyboard (you would have to

find one first), but it also needs interfacing to the main motherboard.

Owners of big box Amigas may also be interested in finding a cheap

replacement for a broken keyboard or maybe they would prefer having

a 'clicky' keyboard.

Anyway, this hack allows you to add one of the vast array of cheap PC

keyboards currently available.  I hope someone finds it useful, but

don't blame me if your motherboard goes up in smoke.

KEY ASSIGNMENTS

###############

Many of you will be interested in how the keys are mapped on the PC

keyboard as there are a number of differences to the Amiga's.  So

here goes:

PC                      AMIGA                   NOTE

##                      #####                   ####

L-Ctrl                  L-Amiga                 =Ctrl in Win95 mode.

R-Ctrl                  R-Amiga                 =Ctrl in Win95 mode.

Insert                  R-Amiga & 7             CygnusED

Home                    Shift & Left-Cursor     CygnusED, Reqtools.

End                     Shift & Right-Cursor

Page Up                 Shift & Up-Cursor

Page Down               Shift & Down-Cursor

F11                     \

F12                     Help

Pause/Break             Ctrl-C                  Break CLI commands.

Scroll Lock             )  Keypad

Print Screen            R-Amiga & P             CygnusEd etc.

Num Lock                (  Keypad

Left Win Key            L-Amiga                 Win95 keyboard only.

Right Win Key           R-Amiga                 Win95 keyboard only.

Menu                    L-Amiga & M             Swap Screen (Win95).

Caps Lock               Ctrl                    When 2 keys are pressed.

e.g. Caps & Q=Ctrl & Q

(102-key mode only)

If you have a Win95 keyboard then the keyboard reset is the usual

3 keys but remember that 'W' keys replace the 'A' keys.

With a normal 102-key PC keyboard you press CAPS-CTRL-CTRL to get

the same effect.

In normal 102-key mode the Caps-Lock key works as normal but when

used in combination with another key it will act as the Ctrl key.

Because some of the keys on a PC keyboard have different legends on

their keytops, I have created a keymap to make them work correct with

an Amiga.  This problem is mainly with shifted characters such as

@'~# etc.  Included are keymaps for English and German keyboards.

Just copy them to your 'c:devs/keymaps/' directory.  And select the

one you want using 'c:prefs/input'.

I'm not sure the German keymap works as I don't have a German keyboard.

If someone creates keymaps for other nations then please uuencode me

a copy so that I may include it in further releases.

CONFIGURATION

#############

Once connected, if you press the '*' key on the numeric keypad

20 times you will be able to set up a couple of features.  For this

setup mode to work you will need to be in a text editor as the

keyboard will try to communicate to you by writing to the screen.

The settings are stored in non-volatile memory so you only have

to do this once.

Okay, load up CED or your favourite editor.  Press '*' key 20 times

and you should see:

----------------------------------------------------------------

Notes:

*******************             -> '*' key Pressed 20 times

SELECT TYPE OF KEYBOARD

1= UK 102 KEY                   -> Use 1 & 2 on main keyboard

2= UK 105 KEY WIN95

OK                              -> Make your selection

SELECT TYPE OF RESET

1= FAST RESET

2= SLOW RESET

OK                              -> Make your selection

----------------------------------------------------------------

If you get garbage on the screen then it may be that you pressed

a key while the chip is writing to screen.

The keyboard option allows you to use the new Window '95 keyboards

which have 3 extra keys.  This is probably the best option as the windows

keys are direct replacements for the special Amiga keys.

The slow reset option has been added to cure the booting problem

WB3.0 has with some hard-drives.  When power is switched on to the

Amiga, the slow reset option will allow the hard-drive an extra few

seconds to spin upto speed.  It does this by holding the reset line

low for a few seconds when the machine is first switched on.  This

is not required for WB3.1 ROM's which cures the problem.

INTERFACE (see keyboard.iff)

#########

I've tried to make this as simple as possible but it does require a

skilled hand at soldering.  The number of components is minimal so the

cost should be less than 10 and are readily available from most

electronic stores.

This design does not require any PCB tracks or chip legs to be cut.

Two resistors are used to prevent conflicts with the original keyboard

CPU.  This means that you can still use the original A1200 keyboard

at the same time as a detachable one.  It must be made clear that this

is a naughty hack but it works fine here.  If the motherboard keyboard

CPU is taken offline then the two interface resistors are not

required.

It is possible to use a 44-pin PLCC to sit on top of 8520 CIA chip.

This is the method I used but a soldered joint will be more reliable

as the socket tries to jump off.

Although this circuit has been made for an A1200 it can be easily

adapted for an A500, A3000, A4000 etc.  Just find the correct pinout

and connect it up as required (47R & 330R resistors not used).  The

big box Amigas may need an external reset switch as I understand

they have reset timer.  Unfortunately I can't test this as I only

have an A1200.

The keyboard requires a 0V and +5V supply which can be taken off the

internal floppy power connector.

If you have mounted your A1200 in a tower case then you may wish to

connect the reset switch between 0V and the motherboard reset.  This

is probably more reliable.

Keep the wires short between the Amiga and PIC because noise on the

signal lines can cause problems.

If you get lots of ' characters then you probably have the CLK and

DAT lines swapped on the motherboard.

Amiga keyboards are meant to send a reset warning code to the

computer but I don't think I got it to work properly.  I can only

think of one program that uses it anyway.  Maybe I'll buy the RKM's

one day to see how it works.

Good Luck!

AUTHOR:     Stephen Marsden

#######

If you want to contact the author of this program then please write

to the following addresses:

email:      Ste@smarsden.demon.co.uk    (Home)

malobs@oslo.expl.pgs.com    (Work)  Subject: Ste

I work away from home for long periods so try either address and don't

expect a speedy reply.

snail-mail:

Stephen Marsden

9 Lumley Road

Redcar

Cleveland

TS10 2BD

ENGLAND