New Upgrade features:

SYSIHack Problems->Read notes

o WideTermination check

As a lot people don`t seem to have read the manual correctly

and use Wide/Narrow bridge without upper Widebus termination

which leads to not working reselection.

(CDROM is reselection by default)

(Syndrom: 44.2 works perfectly with your devices but only slowly)

If the check detects such broken SCSI configuration the device

completely disables reselection for all units.

o BootMenu which can be invoked by holding

the <ESC> key during the system boot.

Then you see a menu with the 2 entries

o SCSI

o List with 8 or 16(WideSCSI) Units.

The entries show the current settings for the UnitID.

The Controller UnitID entry is marked differently.

If you click on the UnitID you get into a simular menu

like Unitcontrol`s Option menu.

If you click on the controller UnitID entry you get into

a simular menu like Unitcontrol`s Prefs menu.

Unit prefs:

o NO LUN

1 LUN

2 LUNs

3 LUNs

4 LUNs

5 LUNs

6 LUNs

7 LUNs

8 LUNs

-> This Entry can be used to disable the scan for the whole

unit for more Luns which can speed up booting a lot.

o Offset

Here you specify the synchron handshake.

o Period

Here you specify the synchron clock.

o ASynchron/Synchron

ASynchron

Synchron

Here you enable the synchron mode, so you

can change Offset and Period now.

The ASynchron/Synchron mode means that the

synchron mode is activated if specified by

the RDB for example.(The AUTO mode)

o Auto Buswidth

8-Bit

16-Bit

Here you specify the buswidth for a unit.

Well..anything more than Auto Mode isn`t really

needed.

o Auto Reselection

No Reselection

Reselection

Disabling reselection for units with problems is

possible here. Auto Mode means that reselection

is activated if the RDB flag is set or if it has

no RDB but is a normal unit like a cdrom.

Scanners and other esotheric units don`t use

reselection in AUTO mode.

Controller Prefs:

o BusID

Controller BusID..normally 7.

->Read the Unitcontrol documentation

o SCSIResetDelay

->Read the Unitcontrol documentation

o ErrorDelay

->Read the Unitcontrol documentation

o RemovableDelay

->Read the Unitcontrol documentation

o ErrorDelayRetry

->Read the Unitcontrol documentation

o ErrorRetry

->Read the Unitcontrol documentation

o TimeOut

(not yet used)

->Read the Unitcontrol documentation

o SYSTEM

o MemorySpeed 70ns

MemorySpeed 60ns

->Read set60nsmode documentation.

o No MapRom

MapRom

MapRom+MacPatch

MapRom+SCSIPatch

MapRom+Mac/SCSIPatch

->Read cybermap documentation.

o SuperScalar No

SuperScalar Yes

->Enables superscalar mode to speed up booting.

This mode is activated with the 68060.library later

anyway.

o BranchCache No

BranchCache Yes

->Enables the branchcache to speed up booting.

This mode is activated with the 68060.library later

anyway.

o WriteBuffer No

WriteBuffer Yes

->Enables the writebuffer to speed up booting.

This mode is activated with the 68060.library later

anyway.

o SAVE | USE | CANCEL

- SAVE saves the settings into the flashrom which

means that these settings are used for every coldboot

or warmboot if not changed or *NEW FLASH UPDATE* is

done. With the help of the 2nd joystickport`s button

you can completely disable scsi device and settings

if you have for some reason a problem to get into the

new bootmenu after playing around with special settings.

So you SHOULD HAVE the PPCUpdate/MK3Update on a disk so

you can quickly reinstall it without a hd which is

connected to the CyberStorm.

- USE uses the current changed settings for the current boot.

The settings aren`t saved and are lost in the next reset.

- CANCEL doesn`t use the settings.