HOW TO CORRECTLY SET UP A SCSI TERMINATION

The termination must be connected on the first AND on the last

scsi device.

Term.                UnTerm.              Term.

Controller --------- [n] Device --------- Last Device

Example:

Term.                UnTerm.         Term.

Controller --------- Cd-Rom -------- HardDisk

Only one exception is allowed:

If you have an internal and an external scsi chain, you can leave the

internal chain unterminated if the chain is shorter than 10 cm.

Example:

Internal Chain                  External Chain

UnTerm.   8cm   Term.            UnTerm.              Term.

HardDisk ------ Controller ----- Cd-Rom ------ HardDisk

Different combinations may work, but is better if your scsi chain

works following the scsi standard.

The need of an extra termination is a symptom of a bad scsi chain...

Better you substitute your old scsi devices with new ones.

SOME OTHER SCSI TECHNICAL RESTRICTIONS

The maximum length of a SCSI chain is 6 meters.

If you want to have a T ramification you should follow some rules:

min 30cm

|------------------------------|

Cable ----------T------------------------------T-------------....

| -                            |

| |                            |

| | 10 cm max                  |

| |                            |

| |                            |

| -                            |

Device                         Device

The minimum distance between two parts of the main cable is 1.27mm

(OkOk...it may also work if you leave only 1.26266267 mm :-) )

Example:

Cable -----------------+ min  +---------- .....

| 1.27 |

| mm   |

|      |

+------+

The impendance of the cable must be between 90ohm AND 140ohm,

better if is 132ohm.

Don't connect cables with different impendances.

A little note: Controller isn't the right term,

better you use Host Adapter.

Alessandro Zummo

P.S. Sorry for my english....