;----------------------------------------------------------------------------

; $VER: MCControlModule.Doc 1.40 (19.02.2000)

;----------------------------------------------------------------------------

;This is not an documentation how to use the .mcm modules in your own

;software! Its an documentation about how to write modules for other

;card reader.

;----------------------------------------------------------------------------

TABLE OF CONTENTS

MCControlModule/General

MCControlModule/Module_Info

MCControlModule/Module_Open

MCControlModule/Module_Close

MCControlModule/Module_AccessDirectFrame

MCControlModule/Module_AccessDirectPage

MCControlModule/Module_AccessRawFrame

MCControlModule/Module_AccessRawPage

;----------------------------------------------------------------------------

First the main structure:

Module_Identifier       = "MCCM"

Module_VersionNumber    = 1

RSRESET

Module_Code                 rs.w 1 ;MUST $4e75 (RTS)

Module_Version              rs.w 1 ;Must be Module_VersionNumber

Module_ID                   rs.l 1 ;Must be "MCCM"

Module_Flags                rs.l 1 ;see below (unused flags must be zero)

Module_Exec_Base            rs.l 1 ;Filled by MCControl

Module_Dos_Base             rs.l 1 ;Filled by MCControl

Module_Intuition_Base       rs.l 1 ;Filled by MCControl

;--- Card Data

Module_Reserved1            rs.l 14 ;Reserved (must be ZERO)

;--- Jump Table

Module_Info                 rs.w 3

Module_Open                 rs.w 3

Module_Close                rs.w 3

Module_ReservedFunctions2   rs.w 3*7 ;Reserved

Module_AccessDirectFrame    rs.w 3

Module_AccessDirectPage     rs.w 3

Module_AccessRawFrame       rs.w 3

Module_AccessRawPage        rs.w 3

Module_ReservedFunctions3   rs.w 3*6 ;reserved

Module_SIZEOF               rs.b 0

;----------------------------------------------------------------------------

;--- Enable transfer routines:

BITDEF Module,AccessDirectFrame,3  ;Driver supports DirectFrameAccess

BITDEF Module,AccessDirectPage,7   ;Driver supports DirectPageAccess

BITDEF Module,AccessRawFrame,6     ;Driver supports RawFrameAccess

BITDEF Module,AccessRawPage,8      ;Driver supports RawPageAccess

;--- Enable gadgets within the settings window:

BITDEF Module,ConfigMultiPage,5    ;Driver requires MultiPage setup

BITDEF Module,Reserved2,0          ;Must be Zero

BITDEF Module,ConfigDevice,1       ;Driver requires an Device/Unit selector

;--- Enable special functions:

BITDEF Module,EnableMultiPage,2    ;Turn on MultiPage support

BITDEF Module,EnableMultiSlot,4    ;Turn on MultiSlot support

BITDEF Module,EnableModuleInfo,9   ;Turn on ModuleInfo support

;----------------------------------------------------------------------------

Module_Error_NoError       = 0

Module_Error_OpenDevice    = 1 ;Opening the given device fails!

Module_Error_NotCompatible = 2 ;If hardware supports identification!

Module_Error_NoTimerDevice = 3 ;If you need the timer.device and opening fails

Module_Error_NoParallelPort= 4 ;No ParallelPort resource

Module_Error_NoResources   = 5 ;Use for missing signals, CIA Interrupts,...

Module_Error_OutOfMemory   = 20

;----------------------------------------------------------------------------

;--- For AccessDirectFrame (Mode)

Module_AccessDirect_Read    = 0

Module_AccessDirect_Write   = 1

;--- For AccessDirectPage (Mode)

Module_AccessDirect_Up      = 0

Module_AccessDirect_Down    = 1

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

NOTES

The exec, dos and intuition  bases  are  for  free  usage.  This  saves

memory,  time and of cource code size. The card data is for free usage,

too.

Beside the arguments delivered for  each  function  youll  receive  an

pointer on the module itself in register A4!

Make sure to backup a2-a5 and d2-d7 just like the Amiga-OS is doing!

You must implement port and device allocation! Dont try to  play  with

the  hardware without owning the right to do so. If possible try to use

parallel.device or serial.device. (The  user  defined  device  will  be

given during Module_Open().

Its not required to create an real  jumptable,  by  using  JMPs.  Feel

free, to use an BRA followed by an NOP. (See RamCard.mcm)

Implement the code as fast as possible, because its used for many times

during reading and writing memory cards. Try to avoid overhead by using

Module_Open.

If you need help then ask me and Iam telling you what to  do.  If  you

dont  know how to code a driver then send me your information and Ill

do my very best.

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

MCControlModule/Module_Info

NAME

Module_Info -- open and inits the module

SYNOPSIS

Error   = Module_Info(Buffer, BufferSize)

D0                    a0,     d0

(ULONG) = Module_Open(*UBYTE, ULONG)

FUNCTION

Give MCControl special information about your driver,

the hardware version detected or something like that.

INPUTS

Buffer:     Destination buffer for your message

BufferSize: Size of the destinaton buffer! Make sure you

dont write more than these numbe of bytes!

RESULT

0 = No Error

Other results are currently not allowed!!

NOTES

Please support this routine! Even if there isnt any

important to say, then just report the version strings

like shown in the example sources.

Buffer is garanted to be 100 bytes at minimum. If you are

not sure your routine may create longer strings, then use

BufferSize to not step over boundaries. The visible gadget

space isnt very big, so take an eye on this, too!

This function requires the flag: ModuleF_EnableModuleInfo!

Otherwise MCControl is ignoring this routine.

SEE ALSO

Module_Close()

;----------------------------------------------------------------------------

MCControlModule/Module_Open

NAME

Module_Open -- open and inits the module

SYNOPSIS

Error   = Module_Open(Device, Unit)

D0                    a0,     d0

(ULONG) = Module_Open(*UBYTE,ULONG)

FUNCTION

Open the required device and alloc additional memory

if needed.

INPUTS

Device: Pointer on device name (STRING IS READ ONLY!!!)

Unit:   Unit of the given device

RESULT

0 = No Error

1 = Open device error

2 = Not compatible

3 = time.device open error

4 = No parallel resource

5 = No resources (no signals, CIA interrupts)

20 = Out of memory

Other results are currently not allowed!!

NOTES

Device and Unit are only valid if the ModuleF_ConfigDevice

flag within the module structure (Module_Flags) is set.

SEE ALSO

Module_Close()

;----------------------------------------------------------------------------

MCControlModule/Module_Close

NAME

Module_Close -- free all data and close all devices.

SYNOPSIS

Module_Close()

FUNCTION

Close all devices and free all allocated memory .

NOTES

Module_Close must be save to be called more than

one time. This means you must detect an closing of

an just closed module. In this case you must return

without any action.

SEE ALSO

Module_Open()

;----------------------------------------------------------------------------

MCControlModule/Module_AccessDirectFrame

NAME

Module_AccessDirectFrame -- Handles a comlete frame at once

SYNOPSIS

Error   = Module_AccessDirectFrame(Frame,Mode,Data,Slot)

D0                                 D0,   D1,  A0  ,A1

(LONG)  = Module_AccessDirectFrame(WORD, LONG,*Byte,WORD)

INPUTS

Frame: Frame number to read or write!

Mode:  Module_AccessDirect_Read or Module_AccessDirect_Write

Data:  Pointer on Buffer!

Slot:  Number of card slot (0-x)

RESULT

0 = Ok

-1 = ERROR

Other results are currently not allowed!!

FUNCTION

Read or write the given frame (128 bytes)!

NOTE

Data must be 128 bytes long!

This function requires the flag: ModuleF_AccessDirectFrame!

Otherwise MCControl is ignoring this routine.

;----------------------------------------------------------------------------

MCControlModule/Module_AccessDirectPage

NAME

Module_AccessDirectPage -- Handle page swapping

SYNOPSIS

Error   = AccessModule_DirectPage(Offset,Mode,Slot)

D0                                D0,    D1  ,A1

(LONG)  = Module_AccessDDirectPage(WORD, LONG, WORD)

INPUTS

Offset: Number of pages to jump!

Mode:   Module_AccessDirect_Next or Module_AccessDirect_Prev

Slot:   Number of card slot (0-x)

RESULT

0 = Ok

-1 = ERROR

Other results are currently not allowed!!

FUNCTION

Change the active card page of an multipage MemoryCard!

NOTE

This function requires the flag: ModuleF_AccessDirectPage!

Otherwise MCControl is ignoring this routine.

;----------------------------------------------------------------------------

MCControlModule/Module_AccessRawFrame

NAME

Module_AccessRawFrame -- Raw handling of frame access

SYNOPSIS

Error   = Module_AccessRawFrame(Data ,Size,Flags,Slot)

D0                              a0   ,D0  ,D1   ,A1

(LONG)  = Module_AccessRawFrame(*Byte,LONG,LONG,WORD)

INPUTS

Data:     Pointer on raw buffer

Size:     Number of bytes in buffer

Flags:    Currently always NULL

Slot:     Number of card slot (0-x)

RESULT

0 = Ok

-1 = ERROR

Other results are currently not allowed!!

FUNCTION

Youll get a real playstation message for direct raw

card access. Just take the first byte, put it to your

reader and store the result where the source byte came

from. Do this for each byte (size times). Thats all!

MCControl handles the error checking for you!

NOTE

This function requires the flag: ModuleF_AccessRawFrame

Otherwise MCControl is ignoring this routine.

;----------------------------------------------------------------------------

MCControlModule/Module_AccessRawPage

NAME

Module_AccessRawPage -- Raw handling of page access

SYNOPSIS

Error   = Module_AccessRawPage(Data ,Size,Flags,Slot)

D0                             a0   ,D0  ,D1   ,A1

(LONG)  = Module_AccessRawPage(*Byte,LONG,LONG,WORD)

INPUTS

Data:     Pointer on raw buffer

Size:     Number of bytes in buffer

Flags:    Currently always NULL

Slot:     Number of card slot (0-x)

RESULT

0 = Ok

-1 = ERROR

Other results are currently not allowed!!

FUNCTION

Youll get two messages mixed together. Your job is to

put the first byte via CMD line into the card an the

second byte at the same transfer via DAT. As you can see

its required to output both information lines, so there

is no need to store any result. The card is thinking its

hearing an PSX and an PSX-Controller talking and browses

through the pages. Size (Size) delivers the total number

of bytes in buffer and you transfer 2 bytes at the same

time, so just half this value to get the number of loop

required for transfer.

NOTE

This function requires the flag: ModuleF_AccessRawPage

Otherwise MCControl is ignoring this routine.

;----------------------------------------------------------------------------