1

GEOD

USER'S MANUAL

version 1.1

(c) Copyright 1992-1993

H. S. Aurand

All rights reserved

4 June 1993

GETTING STARTED

If you are viewing this document on the screen of a personal computer,

you have already decompressed GEODLHA.EXE to obtain the GEOD.DOC which

contains this manual.  You probably have done this by executing the

installation file, INSTGEOD.EXE.  In any event you should have the

following files in a single directory.

INSTGEOD.EXE  Installation file

GEODLHA.EXE   Self-extracting compressed file containing:

GEOD.EXE

GEOD.MSG

GEOD.MIX

GEOD.DOC      User's Manual (this file)

README.DOC    Shareware information

VENDOR.DOC    Vendor Information

FILE_ID.DIZ   BBS Information

If these files are not present in a desired directory, you should

install them in such a directory as follows:

Place the GEOD distribution disk containing INSTGEOD.EXE and

GEODLHA.EXE in a floppy drive.  If the letter for that drive is n,

type n:instgeod <Enter>.  Answer the questions (there may be only

one), and you will be at the initial GEOD screen.  Please read the

information there.  Then press any key, and you will be at a screen

that permits input and execution as described in this document.  At

any menu the different help screens and the background and explanation

screens that make up this document can be viewed by pressing F1 or F8

respectively.  If want to save disk space you may delete all but

GEOD.EXE, GEOD.MSG and GEOD.MIX, but be sure to have INSTGEOD.EXE and

GEODLHA.EXE saved on a backup floppy disk.

2

GEOD - BACKGROUND AND EXPLANATION

GEOD is a program to solve shortest-distance (geodetic) lines on an

ellipsoidal earth.  It also compares these results with three

different spherical earth solutions.

Standard ellipsoids stored and used in the program are the Bessel-

Encke(1852), International or Hayforth (1910) and Clarke (1866).

Other ellipsoids may be entered and used.  Also several different ways

are provided to calculate the earth's mean radius for these

ellipsoids.

Different units may be specified for length and are changed

automatically throughout the program.  Those available are meters,

kilometers, feet, statute miles and nautical miles.  Nautical miles

have no standard definition.  A value of 10800 feet is used in this

program.

There are seven variables that are defined by a line between two

points on the earth's surface - the latitude and longitude of one

point, the latitude and longitude of the other, the distance between

them and the azimuths of the two points from each other.  Four of

these variables must be specified in order to solve for the other

three.

Of the several cases that ensue, the two most interesting, and the

ones solved by this program, are the vector, wherein a point and the

distance and azimuth to the other are specified, and the inverse,

wherein the two points are specified and the azimuths and distance

between them determined.

In addition to single solutions for each of these two cases, there is

another mode in which multiple solutions are provided for the vector

case only.  In this mode the distance or the azimuth can be varied

between two values in specified increments.  Output to a printer can

also be provided.

Comparisons with three different spherical solutions are made in the

single solution case.  In the first, the geodetic latitudes are used

as they are specified.  This is called the geographical solution.  In

the other two, the specified geodetic latitudes are altered before

calculation.  After calculation the inverse alteration is performed on

a calculated latitude.  These two cases are called the geocentric and

reduced respectively.

In the geocentric spherical solution a latitude is changed to one

specified by a line running from the center of the meridional ellipse

to the point.  In the reduced spherical solution a latitude is altered

by reducing it in accordance with a geometrical construction used in

calculating the meridional ellipse.

3

It is beyond the scope of this brief note to explain and justify these

methods of calculation, but the user can run the program and see that

by altering the latitudes in the geocentric and reduced spherical

calculations a closer approximation to the geodetic line can often be

achieved.  To understand this and other aspects of geodetic

calculations, the user is referred to a general text on geodesy such

as "Geodesy", G. Bomford, Oxford, 1971.

The author is not schooled in geodesy, but is an engineer.  On

occasion I have had the need to calculate geodetic lines.  In

reviewing the literature, I found that the several formulas were

predominately special purpose approximations that minimized

computational effort, but could be used only for limited ranges of

distance.  Before the advent of computers this is understandable.

However, I also noticed references to Helmert's method.  These

references stated that his solutions could be made as exact as desired

for any distance, albeit at the expense of computational effort.  I

found this to be so in his book, "Die Mathematischen und

Physikalischen Theorieen der Hheren Geodsie", F. R.  Helmert,

B. G. Teubner Verlagsgesellschaft, Leipzig, 1880, republished by

Minerva GMBH Frankfurt/Main, 1962.

This book contains an elegant solution for the vector case which

relies on strongly convergent series and is ideally suited for use on

small computers.  Helmert's solution for the inverse case not only has

this feature, but uses a strongly convergent iteration.  The

algorithms looks like they were developed with computer solutions in

mind!  Helmert's methods are used in this program.

Helmert also addresses the case where the two points are antipodal or

near antipodal.  The results are counter-intuitive.  In the vector

case the locus of points having a constant geodetic distance is not a

continuous curve around the antipodal point, but has four well-defined

cusps.  This can be clearly seen by using the multiple solution case

varying the azimuth from 0 with a value of eta of .5.  In the

inverse case there are two solutions for the geodetic line for the

near antipodal case.  In this program only the shorter is calculated

and displayed.

There is another counter-intuitive aspect of the geodetic line on an

ellipsoid that should be mentioned.  In general the geodetic line is

not contained in a plane containing the vertical at one end point and

the other end point.  Such a plane does not even contain the vertical

at the other end point (as does a great circle on a sphere).  In fact

the geodetic line is not contained in a plane at all.  Beware, there

are other counter-intuitive aspects of geodetic lines.  For further

explanation consult a geodesy text such as Bomford's.

In this program there are two levels of accuracy that can be selected.

The first, labeled Helmert, follows Helmert's book closely except that

fifteen significant figures instead of seven are used.  The second,

labeled Helmert extended, in addition to this greater number of

significant figures, adds to the number of terms in his series,

4

decreases the epsilon at which iteration terminates in the inverse

case and adds an additional iteration to this case in which Newton's

Method is used to adjust for greater accuracy.

This program has been tested for accuracy in two ways.  First it has

been checked against the examples given in Helmert's book.  Second it

has been checked by running the vector and inverse cases sequentially

and observing the change from the initial values.  In this particular

attention has been paid to singular cases such as polar and antipodal

ones.  Nevertheless, bugs and mistakes are bound to slip through.

This version, 1.1, is a considerable improvement in this regard to

version 1.0 which saw only limited release.  The first registered user

who reports a non-trivial bug or error will be provided with a free

upgrade.

This program is written in Microsoft C, version 6.0.  Double precision

is used for all calculations.  These have been encapsulated in a few

functions separate from the user interface.  Copies of the source code

can be licensed along with a mathematical explanation of the methods

used to extend the accuracy of the calculations beyond that given by

Helmert.  For information please contact the author at the address

listed below.

Your comments, critique and suggestions would be appreciated.

Henry S.Aurand

734 Camino Catalina

Solana Beach, CA 92075

Telephone:  619-481-0781

5

EXPLANATION OF MENUS

MAIN MENU

Provides access to all other menus.

Use the highlighted and capitalized letter key or move the pointer

using the up and down arrow keys to the selection of your choice, then

press ENTER.

The selections are:

Quit -              Exits the program

Spheroid -          Permits the selection of ellipsoids

Radius of sphere -  Permits choice of several means for determining

the radius of an approximating sphere used in

calculating spherical solutions

Units -             Permits the selection of the units used for the

ellipsoid axes, distance and spherical radius

Accuracy -          Permits the selection of Helmert or Helmert

extended accuracy along with selection of the

epsilon for error termination and maximum number

of iterations

sIngle execute -    Goes to Single/Input Output Menu where data can be

entered and a solution obtained for either vector

or inverse cases

muLtiple execute -  Goes to Multiple Input/Output Menu where data can

be entered and solutions obtained for the vector

case.  Azimuth or distance can be varied in steps.

Output is to the screen and may be sent to a

printer

Zero variables -    Zeros all seven input/output variables.  Does not

affect ellipsoid parameters

View descriptive information - Displays several pages of background

6

Keys which operate at all menus are as follows:

F1 and H - Help

F2 -       Go to single execution input/output menu

F3 -       Go to multiple execution input/output menu

F4 -       Go to spheroid menu

F5 -       Go to radius of approximating sphere menu

F6 -       Go to units menu

F7 and Z - Zeros variables.  Zeros all seven input/output variables.

Does not affect ellipsoid parameters.  Z is only effective

in the two Input/Output menus.

F8 -       View descriptive information.  Displays five pages of

background

F9 -       Go to Accuracy menu

7

SPHEROID MENU

Permits the selection of different spheroids.

Use the highlighted and capitalized letter key or move the pointer

using the up and down arrow keys to the selection of your choice, then

press ENTER.

Quit -             Quits program

Main -             Return to Main Menu

Radius of sphere - Goes to Radius of Sphere Menu

Units Menu -       Goes to Units Menu

Single Execute -   Goes to Single/Input Output Menu where data can be

entered and a solution obtained for either vector

or inverse cases.

muLtiple execute - Goes to Multiple Input/Output Menu where data can

be entered and solutions obtained for the vector

case.  Azimuth or distance can be varied in steps.

Output is to the screen and may be sent to a

printer

Bessel -           Changes to the Bessel standard ellipsoid

haYforth -         Changes to the Hayforth standard ellipsoid

Clarke -           Changes the Clarke standard ellipsoid

Other -            Permits direct entry of semimajor and semiminor

axes of any ellipsoid

8

RADIUS OF SPHERE MENU

Permits the selection of the radius for the approximating sphere.

Use the highlighted and capitalized letter key or move the pointer

using the up and down arrow keys to the selection of your choice, then

press ENTER.

Quit -                Quits program

Main menu -           Return to Main Menu

Spheroid menu -       Goes to Spheroid Menu

Units Menu -          Goes to Units Menu

Single execute -      Goes to Single/Input Output Menu where data can

be entered and a solution obtained for either

vector or inverse cases

muLtiple execute -    Goes to the Multiple Input/Output Menu where

data can be entered and solutions obtained for

the vector case.  Azimuth or distance can be

varied in steps.  Output is to the screen and

may be sent to a printer

Geometric mean -      Sets the radius of the approximating sphere

equal to the geometric mean of the axes of the

ellipsoid used

Equal volume -        Sets the radius of the approximating sphere

equal to that of a sphere having the same volume

as the ellipsoid used

Equal area -          Sets the radius of the approximating sphere

equal to that of a sphere having the same

surface area as the ellipsoid used

Other proportional -  Sets the radius of the approximating sphere

equal to c * a0 ^ rho  *  b0 ^ (1-rho).  c and

rho must be entered.  a0 and b0 are the

semimajor and semiminor axes respectively of the

ellipsoid used

Enter -               The radius of the approximating sphere is

entered by the user

9

UNITS MENU

Permits the selection of units for all distance variables and

parameters.

Use the highlighted and capitalized letter key or move the pointer

using the up and down arrow keys to the selection of your choice, then

press ENTER.

Quit -             Quits program

Main menu -        Return to Main Menu

sPheroid -         Goes to Spheroid Menu

Radius of sphere - Goes to Radius of Sphere Menu

Single Execute -   Goes to Single/Input Output Menu where data can be

entered and a solution obtained for either vector

or inverse cases.

muLtiple execute - Goes to Multiple Input/Output Menu where data can

be entered and solutions obtained for the vector

case.  Azimuth or distance can be varied in steps.

Output is to the screen and may be sent to a

printer

When one of the following is selected, the units for all variables and

parameters are changed to the value selected.

Kilometers

Nautical miles

Statute miles

mEters

Feet

10

SINGLE INPUT/OUTPUT MENU

Permits the entry of input and displays output for a single solution.

The display indicates whether the calculation is vector or inverse as

well as the spheroid, the units, the radius approximation and the

accuracy used.  In the inverse case the number of iterations required

is also shown and if extended accuracy is used the number of

iterations for this purpose is also shown.

Use the highlighted and capitalized letter key or move the pointer

using the up and down arrow keys to the selection of your choice, then

press ENTER.  When you make a selection requiring input of an angle,

you must press ENTER after each entry to the "deg?", "min?" and "sec?"

prompts.  For latitude an "n" or "s" and for longitude a "w" or "s"

are required to complete the entry.  If out of range entries are made,

the display so indicates.  To return to inputting press any key.

Quit -                       Quit the program

Main menu -                  Return to Main Menu

mUltiple execute -           Goes to MULTIPLE INPUT/OUTPUT MENU

Type of calculation toggle - Toggles between vector and inverse

calculations

Go -                         Performs calculation after input data

has been entered.  The  results are

displayed alongside the selections and at

the bottom of the screen.  The computed

results of the three spherical

approximations are compared with the

ellipsoidal results.  See the Main Menu

selection, "View descriptive

information" or press F8, to see an

explanation of these approximations.

In the following you are permitted to enter only the variables needed.

1 Enter Lat1 -        Enter the latitude of the first point

2 Enter Long1 -       Enter the longitude of the first point

3 Enter Lat2 -        Enter the latitude of the second point

4 Enter Long2 -       Enter the longitude of the second point

5 Enter Az1,2 -       Enter the azimuth, clockwise from north, of the

second point from the first

6 Az2,1 -             Azimuth, clockwise from north, of the first

point from the second.  Never can be entered,

results of calculation only are displayed.

7 Enter distance, s - Enter the distance between two points in the

units shown

11

MULTIPLE INPUT/OUTPUT MENU

Permits the entry of input and displays output for a range of

solutions.

Use the highlighted and capitalized letter key or move the pointer

using the up and down arrow keys to the selection of your choice, then

press ENTER.  When you make a selection requiring input of an angle,

you must press ENTER after each entry to the "deg?", "min?" and "sec?"

prompt.  For latitude an "n" or "s" and for longitude a "w" or "s"

only are required to complete the entry.  If out of range entries are

made, the display so indicates.  To return to inputting press any key.

Quit -            Quit the program

Main menu -       Return to Main Menu

Units -           Goes to Units Menu

Single execute -  Goes to SINGLE INPUT/OUTPUT MENU

Variable toggle - Successively selects Az1,2, distance and distance

parameter, eta, to be varied

Go -              Calculates vector case in accordance with beginning,

ending and difference variables entered.  Outputs

the results below these and scrolls the display up

as required.  Outputting can be stopped for viewing

by pressing any key except ESC.  Pressing any key

except ESC will resume outputting.  Pressing ESC

will return to the menu

12

In the following you are permitted to enter only the variables needed.

If selected,

1 Enter Lat1 -        Enter the latitude of the first point

2 Enter Long1 -       Enter the longitude of the first point

3 Lat2 -              Latitude of the second point

4 Long2 -             Longitude of the second point

5 Enter Az1,2 -       Enter the azimuth, clockwise from north, of the

second point from the first.  No entry permitted

if you are varying this azimuth

6 Az2,1 -             Azimuth, clockwise from north, of the first

point from the second.  Cannot be entered,

results of calculation only are displayed

7 Enter distance, s - Enter the distance between two points in the

units shown.  s and eta cannot be entered if you

are varying either.

8 Enter eta -         Enter this dimensionless parameter, called

offset ratio, that sets distance.  Used

primarily for antipodal or near antipodal cases.

When eta is 0, the distance, s, is set at pi

times the semiminor axis, b0.  When 1, s is set

at pi times the semimajor axis, a0.  When .5, s

is set to the average of these two values. This

is just slightly smaller than the meridional

distance from equator to equator.  The correct

value for this distance can be determined using

the inverse case of the single input/output case

9 Print toggle -      When "on" outputs to standard printer in the

same format as the output to the screen.

In the following, input is made to a variable determined by the

Variable toggle set above.  The three types are: Az1,2, distance and

offset ratio.

A Enter starting (variable name)

B Enter ending (variable name)

C Enter delta (variable name)

Pressing F7 or 'Z' zeros the seven variables, but not the ellipsoid

parameters

13

ACCURACY MENU

Permits the selection of variables that affect the accuracy of

calculations.

Use the highlighted and capitalized letter key or move the pointer

using the up and down arrow keys to the selection of your choice, then

press ENTER.

Quit -             Quits program

Main menu -        Return to Main Menu

sPheroid -         Goes to Spheroid Menu

Radius of sphere - Goes to Radius of Sphere Menu

Units menu -       Goes to Units Menu

sIngle Execute -   Goes to Single/Input Output Menu where data can be

entered and a solution obtained for either vector

or inverse cases.

muLtiple execute - Goes to Multiple Input/Output Menu where data can

be entered and solutions obtained for the vector

case.  Azimuth or distance can be varied in steps.

Output is to the screen and may be sent to a

printer

Accuracy toggle -  Toggles between Helmert's method and extended

accuracy achieved by extending his series and

adding a Newton's Method approximation in the

inverse case.  Resets the epsilon used in

terminating iterations in the inverse case. Uses

1.0e-9 for Helmert and 1.0e-15 for Helmert extended

accuracies.

enter Epsilon -    Permits the user to select the epsilon used in

terminating iterations.  Note that this value is

overwritten by the Accuracy toggle.

enter max inVerse iterations -  Permits the user to select the

maximum number of iterations used in the inverse

case.  Default value is 100

enter max inverse iterations for Newton's method -  Permits the user

to select the maximum number of iterations used in

the inverse case when Helmert extended accuracy

is used.  Default value is 100.