PrintCAD 1.30

printer driver for IBM/DOS CAD packages

including AutoCAD and Microstation

(C) Copyright Ravitz Software Inc. 1991,1993

Ravitz Software Inc.                              BBS/FAX 606-268-0577

P.O. Box 25068

Lexington, KY  40524-5068

USA                                  Compuserve Cary Ravitz [70431,32]

Introduction ....................................................... 2

License and Registration                                             3

What Is New ........................................................ 3

Operation                                                            4

Installation ....................................................... 5

Syntax                                                               6

Example Setup Instructions ......................................... 7

Overview of Options                                                  8

/1, /2, /3, /4, /5 - Alternate Configurations ...................... 8

/A - Location Option                                                 8

/B - Tiling Option ................................................. 9

/C - Pen Color Option                                                9

/D - Output Destination Option .....................................10

/F - Output Format Option                                           11

/I - Scaling Point Location Option ................................ 13

/J, /K - Printer Code Prefix, Suffix Options                        13

/L - Page Layout Option ........................................... 14

/M - Magnification Option (Scale, Size)                             15

/O - Origin and Orientation Option ................................ 15

/P - Intercept Mask Option                                          15

/Q - Number of Copies Option ...................................... 16

/R - Internal Resolution Option                                     16

/S - Pen Shading Option ........................................... 16

/T - Temporary File Option                                          16

/W - Pen Width Option ............................................. 17

/X - Control Option                                                 17

/Y - D/Y and S/E Settings, ADI Handler Option ..................... 18

/Z - Chord Angle and Enhanced Font Option                           18

Large Paper ....................................................... 19

Color Processing                                                    20

DESQview and Windows .............................................. 20

Answers                                                             21

Useful Prefix and Suffix Codes .................................... 22

Output Devices                                                      23

Compatibility and Technical Information ........................... 28

User Support                                                        28

p-2

Introduction ---------------------------------------------------------

PrintCAD is a pen plotter emulator that acts as a printer driver for

most IBM/DOS based CAD packages. It lets you print drawings directly

from CAD programs, via the CAD program's pen plotter driver - either

AutoCAD's ADI plotter driver or an HP-GL driver.

PrintCAD  works  with  AutoCAD,  AutoSketch,  Microstation,  Cadkey,

Generic CADD, Draft Choice, PC-Draft-CAD, and other CAD programs. It

will  print on Epson  and IBM 9 pin  printers,  Epson, Fujitsu, IBM,

NEC, and Toshiba  24 pin printers, HP LaserJet,  DeskJet,  PaintJet,

QuietJet,  HP-GL/2  devices, and HP-GL plotters, Canon  LBP, BJ, and

BJC printers, HI JetPro  printers,  IBM  LaserPrinter, ExecJet,  and

Quietwriter  2  and  3,  Kodak  Color  4, PDP  ProTracer,  Tektronix

ColorQuick, and PostScript printers, and it will generate PCX files.

PrintCAD  prints  faster than  many native  CAD printer  drivers and

provides  additional  flexibility, such as pen  width,  shading, and

color   (with  color   mixing),   and   paper   size,   orientation,

magnification, and  positioning.  PrintCAD  uses  the  best graphics

modes available for each printer that it supports.

PrintCAD  is a swapping resident program that uses 5K of base memory

and  400K of EMS or XMS memory.  Either 400K of EMS or XMS memory is

required.  EMS and  XMS are expanded and extended  memory management

protocols that are typically set  up with memory management software

such as HIMEM/EMM386, QEMM386, or  386MAX.  If you have  a  computer

with extended  or expanded memory hardware, you need to install such

a driver to use it as EMS or XMS memory.

For  use with DOS extender based  programs such as  AutoCAD  386 and

Microstation  4, a VCPI handler  needs to be present.  VCPI handlers

are built into MS-DOS  5's  EMM386.EXE and  recent  versions of QEMM

386, 386MAX, and other 386 memory managers.  If PrintCAD notes "VCPI

not  present"  then you  need  to  upgrade  your  memory  management

software.

PrintCAD  has  been tested in  a variety of  environments.  However,

because it  is a  swapping resident program, unusual  situations may

cause  it to crash the system.  Please  test  it  thoroughly on your

system before using it with unsaved data.  PrintCAD  will  generally

work  with multitasking  systems  such as DESQview and  Windows, but

this is not guaranteed. See DESQview and Windows for information.

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to  make  sure  that  the  shareware

principle  works   for  you.   If  you   are  unable  to  resolve  a

shareware-related  problem  with  an ASP  member  by  contacting the

member directly, ASP may be able to help. The ASP Ombudsman can help

you resolve a  dispute or problem  with an  ASP member, but does not

provide technical support for members' products. Please write to the

ASP  Ombudsman at  545  Grover Road,  Muskegon, MI 49442  or send  a

CompuServe message via CompuServe Mail to ASP Ombudsman 70007,3536.

p-3

License and Registration ---------------------------------------------

PrintCAD is  provided  as is.  There are  no warranties expressed or

implied.  PrintCAD  is  a  swapping resident  program that may  have

problems with unusual environments.

PrintCAD  is  copyrighted  by  Ravitz Software  Inc.  You  may  copy

PrintCAD only for use under this license agreement.

PrintCAD  is distributed as shareware.  You may use PrintCAD without

charge on a trial basis to determine its suitability for you. If you

continue to  use  it  after  your  evaluation,  you must  purchase a

registered copy for $49.  This registration  covers use  by a single

person (on multiple computers) or installation on  a single computer

(may be used by a group of people).  There is no cost to use a later

1.** version after registering any 1.** version.

You may distribute PrintCAD as a stand alone product if you keep the

entire   package  together,  unchanged,   including   this   license

explanation, and do not charge more than $10. You may not distribute

PrintCAD as part of another product.

To purchase a registered copy of PrintCAD, send your name,  address,

and $49 check to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

You will  be sent  a  printed manual  and  disk (always  the  latest

version).  Please specify 3.5  or 5.25 inch disk.  If you previously

registered  PrintCAD,  you may send $15 to  get a current  disk  and

manual.

The price for multiple licenses is $49 for the first copy (with disk

and  manual) and thereafter $40 per copy with disk and manual or $25

per copy without disk or manual.

For users outside the USA, please make checks payable in US dollars,

and except  for  Canada,  please  use  a  check  from  a US bank  if

possible. Don't forget to include your country in your address.

If you register through a business, be sure to include your name.

What Is New ----------------------------------------------------------

V 1.30 - Version number matches PrintGL. Supports Canon BJC880,

Canon LBP 8 Mk4, Canon BJC800 native mode, IBM 4029

LaserPrinter 600 dpi PPDS upgrade. PrintCAD can take binary

ADI data. The PCADDV function is built in to PCAD.EXE.

V 1.18 - Supports HP DeskJet 500C, PaintJet XL 300, LaserJet 4. /XN

inhibits swapping for improved compatibility with DOS

extender based programs.

p-4

Operation ------------------------------------------------------------

PrintCAD is a resident program that intercepts plot data output from

CAD software, processes the data, and prints the resulting  picture.

When PrintCAD is first run it installs itself.  Subsequent runs look

for the resident copy  and  send control information to it, updating

the parameters.

PrintCAD must  be installed before running the CAD software.  Do not

install it from a DOS shell.  PrintCAD automatically loads high if a

5K DOS or XMS upper memory block is available.  Do not use LOADHI or

LOADHIGH.

PrintCAD can only function as  a  plotter - it cannot  function as a

ADI printer/plotter or as a rendering driver.

PrintCAD  can intercept data from two  sources, DOS file  writes and

AutoCAD/Sketch  ADI 4.0  interrupts.  With  the  option  /PADI,  ADI

interrupts  are intercepted.  With /Pmask, for example  /P*.PLT, DOS

file writes are intercepted.

PrintCAD can interpret two  kinds of data, HP-GL  and AutoCAD/Sketch

ADI  4.0 binary plotter data.  HP-GL data is always interpreted.  If

you use /PADI or /YA then ADI plotter data is also interpreted.

To use PrintCAD as an HP-GL interpreter, use /Pmask to tell it  what

files to intercept.  Use /P*.PLT, /P*.DPF, /P*.HPG, /P*.000, etc. to

match the names that the CAD software uses when writing HP-GL files.

Configure the  CAD software for HP-GL (HP 7550 or  7585) output to a

file.  Then plot (do  NOT print).  PrintCAD  intercepts the data and

prints it.

To  use PrintCAD with AutoCAD  or AutoSketch as an  ADI  interpreter

that intercepts ADI binary files use /P*.PLT.  Configure AutoCAD for

an  ADI 4.0 plotter, binary data output to a file, up to forty pens,

1016 steps/inch, and six non-solid line types. Then plot.

To use PrintCAD  with AutoCAD or  AutoSketch as an installed ADI 4.0

plotter  driver  use  /PADI.  After  installing  PrintCAD  configure

AutoCAD for an ADI 4.0 plotter, installed at interrupt hex 78, up to

forty  pens,  1016 steps/inch, and  six non-solid  line types.  Then

plot.

After making PrintCAD resident, you can verfiy the installation with

"pcad /xq". You can verify that it is functional by copying an HP-GL

(or  ADI  if you  use /YA)  file to  "pcad.plt"  with  the  DOS COPY

command.

For COLOR output, the CAD package should be configured for a plotter

with multiple pens, the CAD package's pen assignment table should be

set up match screen colors to pen numbers, and PrintCAD's  /C option

should set up to match.

PrintCAD  causes a  high/low beep  the  first time that data is sent

after a file open. If you turn on sound (/XS), it causes a high beep

after swapping in its code and a low beep before swapping it out, to

let you monitor its processing.

p-5

When PrintCAD  is  running,  Esc stops processing at  the next  good

stopping point.  Ctrl-B stops processing immediately and will likely

leave the printer in need of a reset.

Internally PrintCAD uses two  byte coordinates  on a 1016  dots/inch

grid. This allows for plots with coordinates of over 30 inches - far

larger  than  an HP  7475  can handle.  However, if  you  are  using

PrintCAD to emulate  a large plotter, you may  overflow the internal

coordinate system.  This condition is not checked, and it will cause

garbage output.  You can change the  internal resolution to 508 dots

per  inch (giving  60 inch maximum  plot sizes) with the  /R  option

(/R508).  This only helps  store the original  plot internally.  The

next  step in PrintCAD converts these  coordinates  to the requested

size in the resolution  of the output device,  and these coordinates

must also fit in the two byte coordinate list.

Installation ---------------------------------------------------------

Copy the file PCAD.EXE to your  working  disk or directory.  This is

the PrintCAD  executable  file  and  is  all  that you  need  to run

PrintCAD.

In addition  to PCAD.EXE, these  files are included in  the package,

and must be included if you distribute PrintCAD as shareware:

PCAD.DOC   documentation

ADD.DOC    additional documentation since last release

ORDER.FRM  order form

README.*   quick information

p-6

Syntax ---------------------------------------------------------------

The calling syntax of PrintCAD is:

PCAD /option1 /option2 ...

Spaces between parameters are not required.

Except for the /X and  /P options, if a given option is entered more

than once, only the last entry is used.  And if an option is invalid

(but not incorrect), it is ignored. Only one /X and one /P option is

allowed.

If you do  not enter any  parameters, PrintCAD  displays  its option

reminder screen.

The DOS environment variable PRINTCAD is examined by PrintCAD and if

it is nonblank  then  it  is  put  in  front  of  the  command  line

parameters.  When you  need to exceed the 127  character DOS command

line limit, put some of the parameters in the PRINTCAD variable, for

example

SET PRINTCAD=/CRGB... /S... /W...

PCAD /P*.PLT ...

Below, [  and ] enclose optional suboptions and | separates mutually

exclusive suboptions. The options and suboptions may be in any order

except  that  multiple  numeric suboptions are  ordered  and numeric

suboptions must follow character suboptions where the characters may

be digits.  "c"  refers to  a suboption character,  "x" and  "y"  to

suboption  numbers,  "i"  and "j" to suboption  integers, "b"  to  a

suboption byte, and "f" to a DOS file or device name.

For numbers (x, y), a decimal point  is allowed but is not required,

and scientific notation is not allowed.  "-" is allowed but not "+".

For  integer  input (i, j),  only  base ten  integers from -32768 to

32767 are allowed. "+" is not allowed. A byte (b) must be a base ten

integer from 0 to  255 or a hexadecimal  number prefixed with $ from

$00 to $FF.  Further range checking is done for most options to flag

unreasonable input.  If an option  has multiple  numeric suboptions,

these may be separated with commas or blanks.

* It  is  very  important  to  choose  the right  output  format (/F

* option).  It is likely that the default output format will work on

* your  printer,  but it is equally likely that it will  not produce

* optimal results.  Please  look carefully  at  the  entire list  of

* output formats before choosing one, and do not assume that because

* a format works that it is the best one to use.

p-7

Example Setup Instructions -------------------------------------------

Below are instructions for  simple setups to  help  you get started.

After you have PrintCAD running, there  are many additional  options

that might be useful.

To  use  AutoCAD (or AutoSketch)  with  an HP DeskJet  500C or 550C,

install PrintCAD with  "pcad /padi  /fa!  /l7.9,10,0,.3  /crygcbmk".

Configure  AutoCAD  for  an  ADI  4.0  vector  plotter  installed on

interrupt 78h,  7  pens,  1016  steps/inch,  plot  area  10x7.9,  no

calibration.  Set up  the AutoCAD pen  assignment table so  that the

seven pens are assigned red, yellow, green, cyan, blue, magenta, and

black. Set skip perforation off on the printer by setting DIP switch

1-8 up. Then plot.

To  use a Canon BJC800 or 820 with 11x17 paper with AutoCAD, install

PrintCAD  with  "pcad  /p*.plt  /ya   /f&  /l10.5,16,0,0  /crygcbmk.

Configure  AutoCAD for an ADI 4.0  vector plotter with binary output

to  a   file,  7  pens,  1016  steps/inch,  plot  area  16x10.5,  no

calibration.  Set up  the AutoCAD pen  assignment  table so that the

seven pens are assigned red, yellow, green, cyan, blue, magenta, and

black. Then plot.

To  use  a Canon BJ330 with cut sheet  paper  with  AutoCAD, install

PrintCAD with:

pcad /ppcad#.plt/f7 1,1.01 /l8,10,0,.4 /j27,67,0,11

pcad /2/f7 1,1.01 /l10.5,16,0,.4 /j27,67,0,17

pcad /3/f7 1,1.01 /l13.5,21,0,.4 /j27,67,0,22

Configure AutoCAD for an HP  7585 plotter with output to a file.  To

print on 8.5x11 paper set the plot area to 10x8 and output  the plot

to file PCAD1.  For 11x17  paper set  the  plot area to 16x10.5  and

output to  PCAD2.  For 17x22  paper set the plot area to 21x13.5 and

output to PCAD3.

To  use the BJ330 with another CAD package, replace the  ".plt" with

the  default  plotfile  extension  for  that  package.  AutoCAD  and

AutoSketch  use  ".plt",  Microstation  uses  ".000",  GenCADD  uses

".dpf",  and EasyCAD  and  FastCAD uses  ".hpg".  DesignCAD  has  no

default  extension - use ".plt" and enter  the entire file name when

plotting (PCAD1.PLT for example).

To  use a  LaserJet with Microstation,  install PrintCAD with  "pcad

/p*.000  /fl /l7.9,10,0,.1".  Configure Microstation for an  HP 7585

plotter, set  the plot  area to 10x7.9, and  plot.  Instead of  /fl,

choose the best LaserJet driver from this list:

/FL  any LJ

/FL! any LJ2 and any LJ3 with less than 1.5 Meg

/FI  any LJ3 with 1.5 Meg or more and the LJ4L

/FI+ any 600 dpi LJ4

For   manual  feed  legal  paper  use  "/p*.000  /fl   /l7.9,13,0,.1

/J27,38,108,50,72,27,38,108,51,65" and set the plot area to 13x7.9.

p-8

Overview of Options --------------------------------------------------

PrintCAD's options fall into several categories.  The first is input

and output  choices.  The input option  (/Pmask) lets you  choose an

intercept filename mask, or you may select ADI interrupt 78 by using

"adi". The output options are /F, the output format (printer model),

and /D, the destination port, device, or file.

The  second  category is the page layout.  /L controls  the size and

location  of the print window (hard  clip limits) and the option  to

box  the  window, not send a form feed, and reverse the image color.

Tiling is set with /B and copies with /Q.

The third category defines  how the  plot  looks  within  the  print

window.  /M,  the  magnification,  sets  the size  of  the plot.  /A

determines the location of the plot within the print window. /O sets

the  origin  and  orientation, and  is used to rotate or  mirror the

plot. /I sets the plotter's initial scaling points.

The  fourth category defines the pens, /W for the width,  /C for the

color, and /S for the shade.

The fifth category determines how PrintCAD processes the plot.  This

includes the temporary file  /T, the default and minimum chord angle

/Z, the plotter Y/D and S/E switch position and ADI handling /Y, the

internal  resolution /R, and prefix and suffix printer  codes /J and

/K.

The  last category is PrintCAD setup, /P sets the intercept mask, /X

sets  up  the initial configuration, /1,  /2, /3, /4, and /5 set  up

multiple configurations.

/1, /2, /3, /4, /5 - Alternate Configurations ------------------------

These options select up to 5 alternate configurations, labeled 1..5.

/1 is  the  default.  The configuration used  for printing is chosen

based on the file name and mask (/P option).

/A - Location Option -------------------------------------------------

/A[O][x,y] - location - default /A

The A option determines the location of the plot in the print window

via a point on  the plot that is  aligned with a point in  the print

window.  You can specify the plot alignment point with x,y in inches

from the  plot's origin (before  PrintCAD applies magnification), or

let  it default to  the plot center.  For the print window alignment

point, you can default to the center or specify the origin with O.

/A      puts the center of the plot at the print window center

/AO0,0  puts the plot 0,0 at the print window origin

/A0,0   puts the plot 0,0 at the print window center

p-9

/B - Tiling Option ---------------------------------------------------

/B[i,j,x] - tiling parameters - default /B1,1,0

/B  lets  you  specify tiling options that break a  large  plot into

pages. Each page is defined by the page layout option (/L).  I and j

specify  the number of tiles (pages) horizontally and vertically.  X

specifies the overlap in inches.

/B2,2,.5  breaks the plot into 4 pages with .5 inch overlap

/B        is the same as /B1,1,0 and produces no tiling

/C - Pen Color Option ------------------------------------------------

/C[O|T]c.. - color - default /CK

/C specifies the color of each pen. The colors are B for blue, C for

cyan,  G for green, K for black,  M  for magenta,  R  for red, W for

white, and Y  for  yellow.  Unspecified  pens use the  last selected

color  (/CRGB  is the same as /CRGBBBBBB).  For  black/white  output

formats, all nonwhite colors become black.

Additional colors can be created with shade mixing. These colors use

shading patterns  to  mix the eight  pure colors.  The  format for a

mixed color is  .??  where  each ?  represents any of the eight pure

colors.  The  first  color is printed with the pen's shading pattern

(/S  option)  and the  second is printed  with the  inverse  shading

pattern.  If the shade  value is 0 (solid) it is replaced by 2 (50%)

which produces the most  uniform  mixed  colors.  On many  printers,

using a shade of 1 (75%) or 3 (25%) gives a better color.  Here  are

some useful mixed colors for inkjet printers. Use them with shade 1.

color  description     color  description

.cb    light blue      .yr    orange

.mb    violet          .rk    brown

.cg    blue-green      .ck    teal

.yg    yellow-green    .kr    black on DeskJet 500C

Mixed colors need line  widths  of at least  2 (4  for 2 dot shading

patterns) to look good, and they may have  visual artifacts on edges

near a 45 degree angle.  Multipass dot matrix formats may not handle

color mixing well.

By default, colors are  overwritten into the print,  so they  appear

opaque.  You can  switch to translucent colors (ored into the print)

with the  T suboption, and then back to opaque with the O suboption.

Make  this  mode switch in front of the pen that you want to change.

For PostScript devices and plotters, the color processing is done by

the device, not PrintCAD.  PostScript  devices  use opaque lines and

plotters use translucent lines.

" means repeat the previous color.

/COKR.MB""T.KY  sets pen 1     to opaque black

pen 2     to opaque red

pen 3..5  to opaque magenta on blue

pen 6..40 to translucent black on yellow

p-10

/D - Output Destination Option ---------------------------------------

/D[+]f - destination - default /D1

The D option specifies the output printer port, device, or file. The

+ suboption causes files to  be appended rather than overwritten (it

has no effect on device output). 1, 2, and 3 select the BIOS printer

routines.  X1, X2, X3, and X4 select xon/xoff controlled serial port

drive.  H1,  H2,  H3, and H4 select hardware controlled serial  port

drive. You can also use the DOS devices LPT1, COM1, etc.

PrintCAD cannot set up the serial ports. This is usually done in the

AUTOEXEC.BAT with MODE (probably MODE COM1:9600,N,8,1,P).

If you  specify a three digit number  for a file name extension (for

example .000), and multiple pages are output via the multiple copies

option or tiling, then rather than  appending the destination  file,

additional files  are  created  with sequential extensions.  This is

useful for PCX output.

/D2          sends output to the second parallel port via BIOS

/DLPT3       sends output to the DOS device LPT3

/D+TEMP.PRN  appends output to file TEMP.PRN

/DOUTPUT.000 sends output to OUTPUT.000, OUTPUT.001, ...

p-11

/F - Output Format Option --------------------------------------------

/Fc[-|+|*][!|�][~][`][x,y] - output format - default /FN~

The F option specifies the output format. -, +, and * are resolution

modifiers,  !  and  �  select compression levels, ~ turns off  color

processing, and ` switches four  plane color  processing on  or off.

These suboptions are allowed even if they have no effect.

For HP-GL (/F8),  HP-GL/2 (/F0), PostScript (/FS), and PCX (/FZ) you

can append  the  nominal  device  resolution.  Matching the  nominal

resolution to  the printer  may yield improvements in print quality.

For PCX the nominal device resolution may be one or two numbers.

For  output  formats  other  than  PCX,  you  can  append  the  true

horizontal  and   vertical  resolution  of  the  output   device  in

dots/inch.  This overrides the normal  output device resolution, but

does not affect  the structure of the output.  These numbers  should

follow all other suboptions. This lets you compensate for print size

error. For example, a 360x360 dpi printer consistently under indexes

so that a 6 inch high box comes  out  5.94 inches.  Using  360,363.6

(6/5.94*360  = 363.6) removes the error.  For print size correction,

you  also specify a number from .5 to 2 which is interpreted as  the

nominal  resolution  times  this number.  For  example on  a 360 dpi

printer, 1.0,1.01 is the same as 360,363.6.

Each printer driver has a default level of  data compression, chosen

to give maximum compatilibity among the different  printers that the

driver supports.  Many  drivers have  an optional enhanced  level of

compression, chosen with  !.  This  will be incompatible  with  some

printers.  And  some drivers let you turn all compression off with �

(ASCII 173).  This is rarely of any use.  Where these suboptions are

effective, they are marked in the table below.

Drivers  marked below with  ~  can handle color.  Using ~ suppresses

color processing, substituting black for all colors except white.  `

switches between 4 and 3 plane color (see Color Processing).

/F0[i]       ~ HP-GL/2 ................................ 600x600, ixi

/F1          ~ IBM 9 pin                                      120x72

/F1+         ~ IBM 9 pin (2 pass) .......................... 120x144

/F1*         ~ IBM 9 pin (4 pass)                            240x144

/F2            IBM Quietwriter 2 ........................... 240x240

/F3            IBM Quietwriter 3                             240x240

/F4-       !�  IBM LaserPrinter (PPDS mode) ................ 150x150

/F4        !�  IBM LaserPrinter (PPDS mode)                  300x300

/F4+       !�  IBM LaserPrinter (PPDS mode) ................ 600x600

/F5        ! ~ IBM 24 pin alternate 1                        180x180

/F5+       ! ~ IBM 24 pin alternate 1 (2 pass) ............. 360x180

/F5*       ! ~ IBM 24 pin alternate 1 (4 pass)               360x360

/F6        !   IBM Proprinter X24 .......................... 180x182

/F6+       !   IBM Proprinter X24 (2 pass)                   360x182

/F6*       !   IBM Proprinter X24 (4 pass) ................. 360x364

p-12

/F7         �  Canon BJ IBM mode, IBM ExecJet .............. 360x360

/F7+        �  Canon BJ IBM mode, IBM ExecJet (2 pass)       360x360

/F8[i]         HP-GL ................................ 1016x1016, ixi

/F9        ! ~ NEC 24 pin                                    180x180

/F9+       ! ~ NEC 24 pin (2 pass) ......................... 360x180

/F9*       ! ~ NEC 24 pin (4 pass)                           360x360

/F&          ~ Canon BJ/BJC native mode .................... 360x360

/F@          ~ Canon BJC CaPSL mode                          360x360

/FA-       ! ~ HP DeskJet 500C, Mannesmann Tally MT92C ..... 150x150

/FA        ! ~ HP DeskJet 500C, Mannesmann Tally MT92C       300x300

/FB         �~ Canon BJ/BJC Epson mode ..................... 360x360

/FB+        �~ Canon BJ/BJC Epson mode (line overlap)        360x360

/FD-       !�  Canon LBP (ISO/CaPSL mode) .................. 150x150

/FD        !�  Canon LBP (ISO/CaPSL mode)                    300x300

/FD+       !�  Canon LBP (ISO/CaPSL mode)................... 600x600

/FF        ! ~ Fujitsu 24 pin                                180x180

/FF+       ! ~ Fujitsu 24 pin (2 pass) ..................... 360x180

/FF*       ! ~ Fujitsu 24 pin (4 pass)                       360x360

/FG         �  PDP Protracer (IBM mode) .................... 360x360

/FI-           HP LaserJet 3, DeskJet                        150x150

/FI            HP LaserJet 3, DeskJet ...................... 300x300

/FI+           HP LaserJet 4                                 600x600

/FJ            Canon BJ BJ130 mode ......................... 360x360

/FK          ~ Kodak Diconix Color 4                         192x192

/FL-       !�  HP LaserJet ................................. 150x150

/FL        !�  HP LaserJet                                   300x300

/FN          ~ Epson 9 pin .................................. 120x72

/FN+         ~ Epson 9 pin (3 pass)                          120x216

/FN*         ~ Epson 9 pin (6 pass) ........................ 240x216

/FO        ! ~ Toshiba 24 pin                                180x180

/FO+       ! ~ Toshiba 24 pin (2 pass) ..................... 360x180

/FO*       ! ~ Toshiba 24 pin (4 pass)                       360x360

/FP        ! ~ HP PaintJet ................................. 180x180

/FQ            HP QuietJet (PCL mode)                        192x192

/FR          ~ Tektronix ColorQuick ........................ 216x216

/FS[i]       ~ PostScript                               600x600, ixi

/FT        ! ~ Epson 24 pin ................................ 180x180

/FT+       ! ~ Epson 24 pin (2 pass)                         360x180

/FT*       ! ~ Epson 24 pin (4 pass) ....................... 360x360

/FW          ~ Epson 9 pin alternate                          120x72

/FW+         ~ Epson 9 pin alternate (3 pass) .............. 120x216

/FW*         ~ Epson 9 pin alternate (6 pass)                240x216

/FX        ! ~ IBM 24 pin alternate 2 ...................... 180x180

/FX+       ! ~ IBM 24 pin alternate 2 (2 pass)               360x180

/FX*       ! ~ IBM 24 pin alternate 2 (4 pass) ............. 360x360

/FZ[i[,j]]   ~ ZSoft PCX                           100x100, ixi, ixj

p-13

/I - Scaling Point Location Option -----------------------------------

/IA|R|W[N] - IP point location - default /IA

This  option is not useful with AutoCAD or any software that outputs

HP-GL that  does not use  scaling commands  or that initializes  the

plotter or sets the scaling points.

The  I option  sets the initial scaling points (P1, P2 - the scaling

points  may  be set  in  the HP-GL  file  with  the IP  command).  A

(absolute) sets the points  to the default  for  the paper size (see

below).  R fits P1 and P2 to the print window with at least .25 inch

margins  and an x/y ratio of 10/7.2 (the same ratio as  the HP  7475

with small paper).  W fits P1 and P2  to the print window with a .25

inch margin. N makes the margin 0 for R or W and is ignored with A.

The  default scaling points depend on the print window  (/L  option)

with the implied paper size and the plot orientation (/O option).

paper    options            default scaling pts  with RO90 command

7440 A   /O1 /L 7.54,10.14  250,279 10250, 7479  279,250  7479,10250

7440 A4  /O1 /L 7.54,10.74  250,279 10250, 7479  279,250  7479,10250

7475 A4  /O1 /L 7.84,10.20  250,596 10250, 7796  154,244  7354,10244

7475 A4  /O1 /L 7.60,10.88  603,521 10603, 7721    0,610  7200,10610

7475 B   /O1 /L10.20,16.38  522,259 15722,10259  283,934 10283,16134

7475 A3  /O1 /L10.88,15.90  170,602 15370,10602  607,797 10607,15997

7440 A   /O2 /L10.14, 7.54  250,279 10250, 7479  279,250  7479,10250

7440 A4  /O2 /L10.74, 7.54  250,279 10250, 7479  279,250  7479,10250

7475 A   /O2 /L10.20, 7.84  250,596 10250, 7796  154,244  7354,10244

7475 A4  /O2 /L10.88, 7.60  603,521 10603, 7721    0,610  7200,10610

7475 B   /O2 /L16.38,10.20  522,259 15722,10259  283,934 10283,16134

7475 A3  /O2 /L15.90,10.88  170,602 15370,10602  607,797 10607,15997

anything else               250,279 10250, 7479  279,250  7479,10250

/O3 and /O4 gives the same default scaling points as /O1 and /O2.

/IA   uses the default scaling points

/IWN  sets the scaling points to fit the print window

/J, /K - Printer Code Prefix, Suffix Options -------------------------

/J[b,b,..] - printer code prefix - default /J

/K[b,b,..] - printer code suffix - default /K

/J lets you send codes to the  printer before  PrintCAD's usual data

is  sent and /K lets you send codes  after PrintCAD's usual data  is

sent.  This lets  you  set  up  and reset the  printer  for  special

situations. For PostScript and HP-GL output, the /J codes follow the

initialization commands.

The  codes  are  specified  with  byte  values  (0..255 or  $0..$FF)

separated by blanks or commas. There is a limit of 80 characters for

any option, so long sequences must be handled outside of PrintCAD.

/J27 79   sets skip perforation off  to allow plots  larger than one

page on most continuous form printers

p-14

/L - Page Layout Option ----------------------------------------------

/L[B][F|N][R][x,y[,x,y]] - page layout - default depends on printer

The L option defines the page layout.

B boxes the plot at the print window edge using pen 40.

F (the default) and N turn form  feed on  or off.  These are ignored

for bit maps and display output.

R  reverses the  colors on displays, bit maps, and output formats 4,

D, I, and L  (Canon,  HP, and IBM  page  printers) and is  otherwise

ignored. This gives the equivalent of a photographic negative.

The first optional x and y are the print window width  and height in

inches.  These define  the hard  clip  limits.  These values are not

checked against the printer's capabilities.  If you specify  a print

window  that  exceeds  the  output  device limits, the  results  are

unknown.  You may need to reset the printer's margins or page length

for large plots (see Large Paper). The size needed for a given width

and height exceeds x and y by 8 dots to allow for line widths.

The  second  optional x and y are the left and top margins, measured

from the printer's  left margin and current vertical position to the

print window. The margins for HP-GL and PostScript are measured from

0,0 (left, bottom).  Margins are ignored for display output  and bit

maps.

THE PRINT WINDOW PLUS THE MARGINS MUST FIT WITHIN THE PRINTABLE AREA

OF  THE PRINTER.  To find the largest printable area turn on the box

and form feed  options  and set a  window and margins of 7.5,10,0,0.

Any plotfile will do - the  only  concern  is the box.  Increase the

window until you have the largest  box that fits on the paper.  This

is the best that you can do. To improve centering you can reduce the

print window and increase the corresponding margin.

The default print window is 7.54x10.14 for printers and bit maps and

10.14x7.54 for HP-GL and displays, corresponding to HP 7440 A paper.

The  default  margins are printer  specific to compensate for  paper

handling.

/L7.5,3,.5,0   print window 7.5x3, left margin .5, top margin 0

/L7.54,10.14   sets window equivalent to HP 7440 with A  paper

/L7.54,10.74   sets window equivalent to HP 7440 with A4 paper

/L7.84,10.20   sets window equivalent to HP 7475 with A  paper

/L7.60,10.88   sets window equivalent to HP 7475 with A4 paper

/L10.20,16.38  sets window equivalent to HP 7475 with B  paper

/L10.88,15.90  sets window equivalent to HP 7475 with A3 paper

p-15

/M - Magnification Option (Scale, Size) ------------------------------

/M[F]x - magnification - default /M1

The M option  sets the magnification.  F fits  the plot to the print

window.  The default, /M1, prints at the same size as an HP 7440.

/MF.9  prints at 90% the largest size that fits in the print window

/M1    plots at the same size as an HP 7475

/O - Origin and Orientation Option -----------------------------------

/O[1|2|3|4[L]] - origin, orientation - default /O

The O  option  sets  the print  window origin  and orientation.  The

origin  is  one of four corners numbered 1..4  for upper left, lower

left, lower right, and upper right, and implies a plot rotation. The

L suboption specifies a left handed coordinate system, which mirrors

the plot.  If no origin is specified then it is chosen so that the x

axis is the longer dimension.

Here are pictures of PrintCAD's view of the output media compared to

an HP 7475's view. O marks the default origin.

HP 7475                               PrintCAD

A size           B size               printer        plotter

y��������Ŀ    0,0��������Ŀy       ul o����Ŀur   ul ��������Ŀur

�        �       �        �         1 �     �4     1 �        �4

�        �       �        �           �     �        �        �

0,0����������x      �        �           �     �     ll o���������lr

�        �           �     �      2           3

�        �        ll �������lr

�        �         2        3

x����������

/O1  puts the print window's origin in the upper left corner

/O2  puts the print window's origin in the lower left corner

/P - Intercept Mask Option -------------------------------------------

/PADI|mask - intercept mask

/PADI sets up PrintCAD as an AutoCAD ADI plotter driver. /Pmask lets

you  choose  a DOS file mask as PrintCAD's intercept mask.  The mask

should not include disk  or directory information - PrintCAD ignores

this information when looking at  file names.  The mask may  contain

the character #.  This is the same as ?, but when a file matches the

mask   the  character  that  matches  the  #  selects  the  PrintCAD

configuration - 1..5  selects configurations 1..5  and anything else

selects configuration 1.

/Padi        sets up PrintCAD to take data from AutoCAD interrupt 78

/P*.plt      intercepts data written to *.plt

/Ppcad#.plt  intercepts data written to pcad1.plt, pcad2.plt, etc.

p-16

/Q - Number of Copies Option -----------------------------------------

/Qi - number of copies - default /Q1

/Q  sets the number of copies of  each printed page.  The default is

one.  Page printers  produce multiple  copies  faster using  the  "N

copies" prefix code (see Useful Prefix and Suffix Codes).

/R - Internal Resolution Option --------------------------------------

/Ri - internal resolution - default /R1016

/R sets the internal grid resolution  in dots/inch.  It is  normally

1016 and rarely needs  to be changed.  To fit very large plots  into

the   -32768..32767  coordinate  limit,  use   /R762.   Setting  the

resolution to a multiple  of the output device  resolution times the

magnification may yield a small improvement in print quality.

/S - Pen Shading Option ----------------------------------------------

/Sc.. - pen shading patterns - default /S0

/S assigns a shading pattern to each pen.  Unspecified pens use  the

last  specified  shade.  The shade values  are 0..8  and  A..F.  The

percent coverage for  each shade is list below.  Narrow line  widths

may not work well with some shade patterns.

0 100%    1 75%      5 75%   coarse pattern    A 6.2%    D 87.5%

2 50%      6 50%   coarse pattern    B 3.1%    E 62.5%

3 25%      7 25%   coarse pattern    C 1.6%    F 37.5%

4 12.5%    8 12.5% coarse pattern

Some printers, especially dot matrix printers  in  multipass  modes,

wash out shading because they have a dot size that is large relative

to the dot spacing.

/S024  sets pen 1 to solid, pen 2 to 50%, pen 3..40 to 12.5%

/T - Temporary File Option -------------------------------------------

/T[+]f - temporary file - default /TPLOT.TMP

PrintCAD  uses  a temporary file when it does not have enough memory

to handle  a plotfile.  You can use /T to put it on a RAM  disk  for

extra speed.  You can force the use of a temporary file, if the plot

has over  1024 data points, with +.  If you use a RAM  disk this may

speed printing.

p-17

/W - Pen Width Option ------------------------------------------------

/Wc.. - pen widths - default /WA

The W option assigns pen line widths. Each width may be 0..9, .10 ..

.24 dots, A..I for 2..10 units of 4/1016 inch (.1  mm), or N to turn

off the  pen.  A..I widths are  approximations, based  on the device

technology. Odd widths of 9 or above are modified to the next higher

(even) width. Unassigned pens use the last assigned width.

For nonsquare  matrix printers, PrintCAD  uses pen points  optimized

for  an  h/v  ratio of .67,  1, or  1.5.  Widths  are figured in the

direction of higher resolution, and are  not accurate  in  the other

direction.

Line widths  of over eight dots are  distorted at  the print  window

edge to fit within the four dot border.

/WA      sets pen 1..40 to 0.2 mm

/WB2.16  sets pen 1 to .3 mm, pen 2 to 2 dots, pens 3..40 to 16 dots

/X - Control Option --------------------------------------------------

/XQ - queries PrintCAD installation

/XR - removes PrintCAD if allowed

/X[E|N|X][A][M][L][W][P][S][-|+] - controls PrintCAD's setup

/XQ  checks to see  if  PrintCAD is installed.  /XR removes PrintCAD

from memory, if allowed. /XQ and /XR disallow all other options.

The other  suboptions  control setup,  and  are  effective only  for

initial setup. X specifies XMS for swapping - this is the default. E

forces the use  of  EMS  for  swapping instead  of  XMS.  N inhibits

swapping.  This forces the  entire program to load in low memory and

it will  take a large chunk of  DOS  memory.  In return,  all of the

compatibility problems of  swapping TSRs  are removed.  This  can be

very useful with DOS extender based CAD programs.

A sets  an  alternate  handle  (use  with PC-Draft-CAD,  Drafix, and

DesignCAD).  M sets an alternate mouse handler. These suboptions may

improve compatibility.

L forces PrintCAD to load its resident  part low.  W forces PrintCAD

to load its swapping work area low.  P causes program prefix segment

checking.  These  suboptions may  improve compatibility  in  unusual

situations, but are generally not needed and may make things worse.

+ or  -  increases  or  decreases  the PrintCAD  memory  space,  for

improved speed or reduced resource usage.

S adds progress beeps. This is not advised with the N suboption.

/XM  sets up PrintCAD with its alternate mouse handler

p-18

/Y - D/Y and S/E Settings, ADI Handler Option ------------------------

/Y[D|Y][S|E][A] - D/Y and S/E  setting, ADI handler - default /YDS

The Y option sets the plotter's D/Y  switch to  D  or Y and the  S/E

switch to S or  E.  Y starts  with the  plotter off and responds  to

esc.(, esc.Y, esc.), and esc.Z commands.  D  starts with the plotter

on and ignores these commands. S sets the standard interpretation of

IW (clipping window) commands.  E sets 7550 enhanced  interpretation

of IW commands, so that they use scaled coordinates.

The A  suboption  turns on  PrintCAD's ADI  handler.  In  this  mode

Autodesk  ADI  4.0  vector plotter binary  commands  are interpreted

(along with  HP-GL).  The ADI  driver  should  be  set  up  at  1016

steps/inch with up to forty pens and up to six non-solid line types.

This is not needed when you specify /PADI.

/YD  puts the YD switch in the usual D position

/Z - Chord Angle Option ----------------------------------------------

/Z[E]i,j - default, min chord angle, enhanced font - default /Z10,5

This option is  not useful with AutoCAD or any software that outputs

HP-GL  that draws circles,  arcs, and  text as line segments  rather

than using HP-GL circle, arc, and label commands.

The  Z  option  specifies the  default and minimum chord angles  for

circles and arcs. The HP 7475 defaults to a chord angle of 5 with no

minimum.  Using chord angles of less than 10  returns  little  print

quality  improvement  and adds to processing  time.  The default  is

/Z10,5. For better emulation use /Z5,2.

The  E suboption  says to use the enhanced font,  which  doubles the

number of chords on curves in text, improving the  curve smoothness.

This is only effective on character heights above .5 cm.

/Z5,2  sets the default chord angle to 5 degrees, the minimum to 2

p-19

Large Paper ----------------------------------------------------------

PrintCAD can handle printing on  paper larger than the usual 8.5x11,

but  this  takes  some  extra  work.  First  you  need  to  increase

PrintCAD's print window to cover the large paper.  This is done with

the /L option.  The  print  window plus the  margins  (/L)  must fit

within the printer's printable area.

And  then you must make sure that your printer recognizes the larger

paper  height.  For  continuous forms  paper  this is best  done  by

setting skip perforation to  off,  either by printer switch or  with

PrintCAD prefix codes.  For Canon, Epson, Fujitsu, IBM, and NEC line

printers (dot matrix, ink jet, thermal) use /J27 79.

For  cut  sheet  paper,  on Canon  (except BJ  native  mode), Epson,

Fujitsu, IBM, and  NEC line printers use /J27 67 0 n, where n is the

page length in inches.  For the PDP ProTracer use /J27 67 n, where n

is the page length in lines, at six lines per inch.

And for some printers you need to increase the right margin. For the

Canon BJC  in  Epson  mode  use  /J27 81 114.  For the HI V50/100 in

printer mode use /J27 91 88 2 0 1 255.

For page  printers, legal size  paper  may  be specified  with these

printer code prefixes: HP-PCL  printers  /J27 38 108 51 65  (/K27 38

108 50 65 to  reset to letter), Canon  LBP  /J27 91  51 50 59 59 112

(/K27 91 51 48  59 59 112 to reset), IBM LaserPrinter /J27 91 70 5 0

3 1 2 1 (/J27 91 70 5 0 3 1 1 1 to reset).

To  get a  10x16 print  window on 11x17 paper with a  Canon,  Epson,

Fujitsu, IBM, or NEC line printer, use these options: /L10 16 .4 .25

/J27 67 0 17.  You will probably need to adjust the margins (.4 .25)

for best centering.

The  maximum  width  of  the print  window on  most  narrow carriage

printers is 8  - 8/(horizontal dpi).  For 300 dpi printers,  this is

7.97.  At 120 dpi it is 7.93.  The maximum width of the print window

on most wide carriage printers is 13.6 - 8/(horizontal dpi).  At 180

dpi this is 13.55.  At 360 dpi it is 13.57.  The  horizontal  margin

should be set to 0 to get this width.

p-20

Color Processing -----------------------------------------------------

Color graphics devices use a variety of color technologies. PrintCAD

supports these basic types.

CMY or  cyan/magenta/yellow is used by inkjet printers with no black

ink. This uses three color planes and give eight pure colors.

CMYK or  cyan/magenta/yellow/black  is used by inkjet and dot matrix

printers.  This uses  four color planes  and give eight pure colors.

Using black ink instead  of  a cyan/magenta/yellow mix gives a  more

neutral black color.

RGB or red/green/blue is used by  bit  maps.  This uses  three color

planes and give eight pure colors.

RGBI  or red/green/blue/intensity is  used and bit  maps.  This uses

four color planes. All simple colors except black are intense.  When

you  specify any color mixed with black at  shade 0, then instead of

mixing the  colors at  50%, the intensity is turned off.  This gives

fifteen pure colors.

CMYK and RGBI are used by default when they are appropriate. You can

switch to CMY or RGB with  the  ` suboption of /F.  For  dot  matrix

printers this  may speed  printing  by  reducing  memory  usage  and

eliminating the use of the black ribbon.

/FA!  (300  dpi color  HP-PCL) uses  CMY  color  by  default.  /FA!`

switches to CMYK color to let you  access true  black on the DeskJet

550C.

DESQview and Windows -------------------------------------------------

PrintCAD  can generally be  run  inside  a  window  of  DESQview  or

Windows. In this case it can be accessed only from that window.

PrintCAD  can  generally be  run  under (installed  before starting)

DESQview  or Windows.  In  this  case it  can be accessed  from  any

window, but while running all other tasks will be stopped.

p-21

Answers --------------------------------------------------------------

To access PrintCAD from a CAD package you must PLOT. Do NOT print.

If you  are getting system crashes or nothing from PrintCAD, try the

options  /XA, /XM, /XE,  and all  combinations of these (/XAM, /XAE,

/XME, ...).  Try /XN for DOS extender based software.  Try /XL, /XW,

and /XLW if none of the above work.

If  your plot is messed up, with wraparound lines covering the page,

you have probably  exceeded the PrintCAD  internal coordinate limit.

This happens when the HP-GL coordinates exceed 32 inches. Reduce the

internal resolution  with /R762 for  coordinates up to 42  inches or

/R508 for coordinates up to 64 inches.

If  you  are  having  trouble getting  the  print  size or  location

correct, check your  /L  option.  The print window  plus  the  print

margins  must define an area that fits in the printable area  of the

paper. Printers cannot print over the entire area of the paper. They

usually require .25 to .5 inch margins.

To get color output from PrintCAD, you must:

Define a multipen plotter in your CAD package.

Set up the pens in your CAD package so each color is assigned to a

pen.

Use the /C option to assign the same colors to PrintCAD's pens.

Plot spoolers are  not generally compatible with PrintCAD, and would

be  of little value if they were.  Do  not  bother  with one.  Print

buffers/caches are generally compatible. You must install the buffer

first. A print buffer may be very useful.

If your prints are coming out not quite to scale:

For ADI plotters,  check  that your  plotter  is  defined  as 1016

steps/inch.

Check that no calibration is being done by the CAD package.

See /F - Output Format Option for print size correction.

For the Canon BJC-800 in Epson mode, the right margin  defaults to 8

inches. Use /J27 64 27 81 114 to set it to 11.4 inches.

For the  Canon BJ-330 and most wide  carriage printers,  the maximum

print window  width is 13.57  inches.  This is a printer  limitation

that you cannot get around.

If your  printer ejects the paper  before the plot is  finished, you

probably needed to specify a paper length prefix code. For most line

printers use /J27 67 0 n where n is the page length in inches.

p-22

Useful Prefix and Suffix Codes ---------------------------------------

Below,  n  represents any number 0..255, nA represents any number in

ASCII format (1 becomes 49, 2 becomes 50, 43 becomes 52 51, etc.).

Canon,      Reset                    27 64

Epson,      N inch paper             27 67 0 n

Fujitsu,    17 inch paper            27 67 0 17

IBM, NEC    Right margin n columns   27 81 n       (not for IBM)

dot matrix, Skip perforation off     27 79

inkjet      Unidirectional printing  27 85 1

HP PCL,     Reset                 27 69

DeskJet,    Portrait mode         27 38 108 48 79

LaserJet,   Top margin 0          27 38 108 48 69

PaintJet    Skip perforation off  27 38 108 48 76

11 inch paper         27 38 108 50 65

14 inch paper         27 38 108 51 65

11x17 inch paper      27 38 108 54 65

Manual feed           27 38 108 50 72

N copies              27 38 108 nA 88  (page printers)

HP DJ500C   High quality, slow    27 42 111 49 81

No ink depletion      27 42 111 49 68

HP PJXL300  Low quality, fast     27 42 111 45 49 81

HP QJ       HP-PCL mode           27 37 65

DIP switch mode       27 37 64

IBM LP      11 inch paper, tray 1, n copies  27 91 70 5 0 3 1 1 n

14 inch paper, tray 1, n copies  27 91 70 5 0 3 1 2 n

14 inch paper, manual, n copies  27 91 70 5 0 1 1 2 n

N copies                         27 91 70 5 0 0 0 0 0 n

Can BJ nat  Page length, right margin x 10   27 40 103 3 0 n 1 n

Can CaPSL   Reset                  27 99

11 inch paper          27 91 51 48 59 59 112

14 inch paper          27 91 51 50 59 59 112

Manual feed            27 91 49 113

Automatic feed         27 91 48 113

Full page memory mode  27 59 27 91 50 38 122

N copies               27 91 nA 118

Cursor to top of page  27 91 48 100

Diablo mode            27 58

ISO mode               27 59

PostScript  N copies      nA 32 88 67

Paper tray n  nA 32 88 80

ProTracer   N line paper  27 67 n

HI V50/100  Max margins   27,91,88,2,0,1,255

p-23

Output Devices -------------------------------------------------------

Canon BJ and BJC Printers, IBM ExecJet

PrintCAD has five basic drivers for Canon  BJ and  BJC and the IBM

ExecJet.  /F7  is the IBM mode driver.  It  works with the BJ130e,

BJ5,  BJ10/10e/10ex, BJ20, BJ200,  BJ300/330,  and  ExecJet.  /F7+

gives dark print with line to line overlap.

/FJ is the BJ130 mode driver.  It works with all Canon BJ printers

in IBM or native mode. The printer is set to 10 pitch text.

/FB is  the  Epson  mode  driver.  It  works  with  the BJ200  and

BJC800/820.  Color is supported.  /FB+ gives sparse  line to  line

overlap.  For the BJC800/820  the  prefix code 27  81 114 sets the

print width to 11.4 inches.

/F&  is the  native mode  driver.  It works  with  the  BJ200  and

BJC800/820 in any mode. Color is supported.  Epson/IBM mode prefix

codes are not compatible and are not needed.

/F@ is the CaPSL mode driver for the BJC880. Color is supported.

Some  of these printers have  an image density option (DIP  switch

and/or  front panel switch).  Low density  eliminates every second

dot. To get the full 360 dpi resolution, use high density.

Canon LBP Printers

/FD works with any Canon LBP printer.  If the printer is in Diablo

mode,  it is switched to ISO mode.  At 300 dpi it  takes up to one

meg of printer memory to print a full page of graphics.

The ! suboption switches to CaPSL 4 data compression. /FD+! is the

best driver for the LBP 8 Mk4 at 600 dpi.

You might need to set up the printer with /J27 59 27 91 50 38 122.

To switch back to Diablo mode on completion, use /K27 58.

Epson, IBM 9 Pin Printers

/FN, /FW, and /F1 cover a wide range of Epson/IBM compatible 9 pin

dot matrix printers.  You need a  cyan/magenta/yellow/black ribbon

to get color prints. The * drivers use 1/240 graphics. /FN+, /FN*,

/FW+,  and  /FW*  use  1/216  indexing  and  do  three  vertically

interleaved passes.  /F1+  and /F1* use  1/144 indexing and do two

vertically interleaved passes. /FW sets line feeds to 1/6 inch.

For  Epson  printers  and  other  printers  with  1/216   indexing

hardware,  /FN  is  preferred.   For  some  not  quite  compatible

printers,  /FW works.  For IBM Proprinters and other printers with

1/144 indexing hardware, /F1 gives better results.

p-24

Epson, Fujitsu, NEC, Toshiba 24 Pin Printers

/FT  covers  all  Epson LQ printers and  many  compatible printers

(Panasonic in LQ mode and others).  /FF works with  Fujitsu 24 pin

printers in Fujitsu mode.  /F9 works with NEC 24 pin printers. /FO

works  with Toshiba 24  pin printers.  /FO  resets the  line  feed

distance to 1/6 inch on completion.

For all of  these  drivers, you  need  a cyan/magenta/yellow/black

ribbon to get color prints. The + and * drivers use 1/360 graphics

mode,  and  the * drivers use 1/360  (1/120 for /FO*) indexing and

reset the line  feed distance to 1/6 inch on completion.  A  clean

paper path with  equal tension on  each side  is  needed  for good

360x360 graphics.

Many 24 pin printers are not capable of 360x360 graphics and a few

cannot handle 360x180 graphics.

The compression mode drivers  (! suboption)  may or  may not be an

improvement with these printers.

Houston Instruments JetPro V50/100

You can  drive  the  HI V50/100  as an  HP-GL/2  device.  Set  the

resolution to 360 (/F0  360) and reverse the  width and height  in

the print window, for example use 10x8 instead of 8x10  for 8.5x11

paper. The maximum print window height is 15.25.

In printer mode the HI V50/100 acts as a Canon BJ IBM mode printer

(/F7).  The  maximum  print width is 15.25 and you need the prefix

code 27,91,88,2,0,1,255 to get beyond 13.6 inches.

HP DeskJet, HP PaintJet XL300, Mannesmann Tally MT92C

/FI works with any black/white DeskJet in portrait mode.

/FA drives the  DeskJet 500C, PaintJet XL300, MT92C, and any color

300 dpi HP-PCL printer that supports data transfer mode 2.

/FA  has three modes and covers different  levels of 300 dpi color

HP-PCL printers.  /FA uses RGB color  and  works with the  DeskJet

500C and Mannesmann  Tally MT92C.  /FA!  uses  CMY color which can

speed printing  in some  cases.  This works with the DeskJet 500C,

DeskJet 550C, and PaintJet XL300.  /FA!` uses  CMYK  color to give

give better blacks on the DeskJet 550C.

p-25

HP LaserJet

/FL works with any LaserJet compatible printer. /FL! works with LJ

2 (PCL 4) printers.  /FI works with LJ 3 (PCL 5) printers. /FI+ is

a 600 dpi driver for the LaserJet 4.

All  of these drivers work in portrait mode, which may be set with

/J 27 38 108 48 79 or you can reset the printer with /J 27 69.

LaserJets with under one meg of memory may not be able to handle a

full page of graphics. /FL!  uses less printer memory than /FI, so

it may be preferred on LaserJet 3s with limited memory.

HP PaintJet

/FP drives 180 dpi PaintJet printers.  The compression mode driver

(/FP!)  uses  data  transfer  mode 2 (instead of 1) and works only

with a PaintJet XL.

/FA! (the DeskJet 500C CMY driver) works with the PaintJet XL300.

HP QuietJet

/FQ works with QuietJets in HP-PCL mode.

HP-GL, HP-GL/2, HP and IBM Pen Plotters

/F8  outputs low level  HP-GL, converting text,  arcs,  fill,  and

clips to  vectors.  The output  is HP 7220 and HP-GL/2 pen plotter

compatible, containing  IN,  IP, SC, SP, IW,  LT,  PA, PU,  and PD

commands.  PG is used for form feeds.  If you specify no form feed

then multiple plots may be appended to the same page.

/F0  drives  HP-GL/2  raster  devices.  Use  /F8  for HP-GL/2  pen

plotters.

You  can specify the nominal resolution  of  HP-GL  or  HP-GL/2 by

appending it to the /F option (for example /F8 300).  When you set

the nominal  resolution of HP-GL to other  than 1016,  enhanced IW

command interpretation (HP 7550 enhanced mode) is needed to handle

dashed lines correctly.

For HP-GL output  the  width parameter  (/W)  is  used  as  a  pen

selector, so for each pen in the original HP-GL, you can specify a

new  pen  in  the  output.  The  default  /W  option  for  /F8  is

/W12345678. Shading and color are ignored.

IBM LaserPrinter

/F4 drives any LaserPrinter in PPDS  (native) mode.  /FL will work

for HP LaserJet mode. At 300 dpi it takes up to one meg of printer

memory to print a full page of graphics.

The !  suboption switches  to 4029 data compression.  /F4+!  works

with the 4029 with 600 dpi PPDS upgrade.

p-26

IBM 24 Pin Printers

PrintCAD has three drivers for Proprinter X24 compatible printers.

Use  native mode, not AGM.  /F6 is for all IBM X24s  and the  24P.

These printers do  1/144 inch indexing and using them  at  180 dpi

gives horizontal white  streaks every  2/3 inch.  The  /F6 drivers

compensate  for this by indexing 19/144 per line, giving  182 dpi.

To work  properly the printer  must start on a 1/72 boundary so it

is best to keep line feeds at a multiple of 1/72 (1/6, 1/8, 1/9).

/F5 is referred to as X24 alternate 1 and is for printers that are

X24 compatible  except for  1/180 inch indexing.  This is the case

with  most X24 compatible printers, including the IBM  Quickwriter

and IBM 2390 and 2391. /F5* works only with printers that do 1/360

indexing, including the 2390 and 2391 but not the Quickwriter.

/FX  is referred to  as X24  alternate 2  and is  the same  as /F5

except for the  vertical units command.  /FX  works with Panasonic

printers in X24 mode.

The  compression  mode drivers (!  suboption) may or may not be an

improvement.

IBM Quietwriter 2 and 3

/F2 and /F3 drive  the Quietwriter 2 and 3.  Do not use  1 for pen

widths because  Quietwriters do not  print consistent 1 dot lines.

These  drivers  reset  the  line  feed distance to  1/6  inch upon

completion.

Kodak Diconix Color 4

/FK drives the Color 4.

Pacific Data Products ProTracer

/FG drives the ProTracer (base model) in IBM Proprinter mode.  The

maximum print window width is 14.97. Use the prefix code 27,67,144

for cut sheet paper longer than 11 inches.

PostScript Printers

/FS outputs PostScript.  All characters, arcs, fill, and clips are

converted to simple vectors.

PrintCAD's pen  color and shading parameters are  mapped into gray

levels or RGB color levels.

You can specify  the  nominal  resolution  of  the  PostScript  by

appending it to the /F option (for example /FS 300).

You can  specify  the  number  of copies with a  prefix code  that

includes the number of copies followed by " XC". For example /J 50

32 88 67  makes 2 (ASCII  050) copies.  You  can specify the paper

tray with the XP macro.

p-27

Tektronix ColorQuick

/FR drives the ColorQuick.

ZSoft PCX and Fax

Use /FZ to output a ZSoft PCX bit map. The horizontal and vertical

dots/inch may  be appended  to override  the default 100x100,  for

example /FZ80,80.  Such output  should generally be directed to  a

file with the /D option.

Since  most  fax  software can  take PCX files  as  input, this is

useful for  faxing  drawings.  Use /FZ200,196  to  match  the  fax

resolution.

p-28

Compatibility and Technical Information ------------------------------

PrintCAD will run on any IBM compatible  DOS computer  with 400K  of

EMS or XMS memory and a base memory region of 304K.  Because it is a

swapping   resident  program,  it   may   have  problems   in   some

environments.  PrintCAD can handle  VCPI  (Virtual  Control  Program

Interface) based  DOS extenders,  but DPMI based programs  and stand

alone DOS extenders may cause problems.

When  using a DOS extender  based program  such  as  AutoCAD  386 or

Microstation   4,  use  an  extended/expanded  memory  manager  that

provides VCPI, such as MS-DOS 5 EMM386/HIMEM, QEMM 386, or 386MAX.

PrintCAD  is  a TSR that takes over interrupt hex 21 and watches for

DOS 2.0+ file management open functions.  If the filename on an open

command matches either "pcad#.plt"  or  the mask  specified with /P,

then file handle dec 99 (dec  19  if /XA  is used)  is returned  and

PrintCAD is awakened. It then responds to write, device information,

close, commit, seek, and set date functions.

When awake, PrintCAD stores HP-GL commands (from  the write command)

until its 16K buffer is filled  and then it swaps in its 304K memory

space, processes the data, and swaps the original memory back.

PrintCAD  was written  by Cary Ravitz and  compiled  with  Borland's

Turbo Pascal 6.0. PrintGL Printer Driver is the graphics engine.

User Support ---------------------------------------------------------

Correspondence about PrintCAD may be sent via the  support  bulletin

board, fax, Compuserve, or mail.

BBS:        606-268-0577  1200..9600,N,8,1  24 hours/day

To ask a question, prepare an ASCII text file with the details  of

your question.  Then call the bulletin board and choose the upload

question option.  XMODEM  file transfer  protocol  is required for

this.  Your  question will  be assigned a  number (such as 1015) -

remember  this number.  An answer file (with the  assigned number)

will be posted, usually within  24 hours.  To get an answer,  call

the  bulletin board, choose the  download option,  then the answer

option, and then enter the answer number.

Compuserve: Cary Ravitz [70431,32]

Fax:        606-268-0577

If  you hand dial,  the BBS  will answer  and  hang up  (about  20

seconds), then the call is routed  to the fax.  Faxes from outside

the  USA may be answered via  letter.  The  fax may be unavailable

sometimes.

Mail:       Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA