S A F E P A C K

===============

The SAFE Disk Defragmenter

User Manual - Version 1.5

CONTENTS

========

Introduction  . . . . . . . . . . . . . . . . . . . . . . . .    2

Why Safety is Important in a Defragmenter . . . . . . . . . .    3

Requirements & Limitations  . . . . . . . . . . . . . . . . .    4

Usage Procedure for SAFEPACK  . . . . . . . . . . . . . . . .    5

Swap Files . . . . . . . . . . . . . . . . . . . . . . .    6

SAFEPACK Options  . . . . . . . . . . . . . . . . . . . . . .    6

Sorting Options  . . . . . . . . . . . . . . . . . . . .    7

Individual File vs Sub-directory Based Sorting . . . . .    8

Sorting Optimizations  . . . . . . . . . . . . . . . . .    8

Directory Sorting  . . . . . . . . . . . . . . . . . . .    9

Immobile Files . . . . . . . . . . . . . . . . . . . . .    9

OS/2 Control . . . . . . . . . . . . . . . . . . . . . .   11

Running from the Command Line Only . . . . . . . . . . .   11

Miscellaneous Options  . . . . . . . . . . . . . . . . .   12

Disclaimer - Agreement  . . . . . . . . . . . . . . . . . . .   13

Appendix 1 - SAFEPACK Error Messages  . . . . . . . . . . . .   14

Appendix 2 - Disk Recovery  . . . . . . . . . . . . . . . . .   16

Copyright (C) 1993 D.J.Harvey.  All Rights Reserved.

--- SAFEPACK Manual --- Page 1 ---

Introduction

============

SAFEPACK reorganizes the files on a disk, moving them around as

necessary to ensure that each file occupies a single, contiguous

block of disk space, improving the effective performance of the

drive.  Apart from this improvement in speed, many users will

benefit from the collection together of all the free space on the

disk, allowing the creation of large, contiguous special files.

An example of this is a Windows 3 permanent swap file, and the

Windows manual suggests that a disk defragmenter be run before

creating a permanent swap file.  No such program is supplied,

however, and SAFEPACK is designed to fill this gap.  For the user

who wishes to fine-tune the organization of the disk, a large

number of options are provided to control the final arrangement of

the files.  Those using both DOS & OS/2 will benefit from

SAFEPACK's ability, safely, to defragment the very fragmented

hidden files that OS/2 creates on DOS type disks.

As its name implies, SAFEPACK has been designed with data security

as the most important principle, and it manipulates the disk file

structures such that they are all valid at all times.  This means

that no loss of data should occur if the program is interrupted

while running, whether this be due to user break-in, machine

malfunction, or even power failure.

Why May Your Disk Need to be Defragmented?

==========================================

If you are fully conversant with the problem of disk

fragmentation, and know exactly why your disk needs to be

defragmented, you can skip this section, and go to the following

section on safety.

In order to understand why a program such as this is required, it

is necessary to understand just a little about how DOS actually

stores files on a disk.  Although users generally refer to files

simply by their path & filename, DOS must allocate space for each

file on the disk, and be able to keep track of where each file is.

Is does this by dividing the disk into a number of 'Allocation

Units' or 'Clusters', normally of 512, 1024, 2048 or 4096 bytes,

and when a file is written, DOS allocates as many unused clusters

as are necessary to accommodate it. Where possible, DOS uses

consecutive clusters, as this improves the speed of both reading

and writing, but this is not always possible, especially after a

disk has been in use for some time, as the unused clusters may be

scattered around the disk in locations previously occupied by

files which have since been deleted.

--- SAFEPACK Manual --- Page 2 ---

An example may make this clearer:

Suppose a very small disk with 18 clusters has 6 files, named F1-

F6 on it, and that they are located as in the following map:

Cluster:  |1  2 |3  4  5 |6  7 |8  9 10 11 12|13 14|15 16|17 18

File:     | F1  |   F2   | F3  |      F4     | F5  | F6  | FREE

If files F1, F3 & F5 are now deleted, the map becomes:

Cluster:  |1  2 |3  4  5 |6  7 |8  9 10 11 12|13 14|15 16|17 18

File:     |FREE |   F2   |FREE |      F4     |FREE | F6  | FREE

There are now 8 free allocation units, and if a file requiring 7

units called F7 were to be written to this disk, it would have to

be put into the free space, which is spread over the disk, making

the new map:

Cluster:  |1  2 |3  4  5 |6  7 |8  9 10 11 12|13 14|15 16|17|18

File:     | F7  |   F2   | F7  |      F4     | F7  | F6  |F7|FREE

File F7 is now said to be fragmented, making access to it

significantly slower than if it were in one contiguous section.

Over time, similar fragmentation occurs to many of the files on a

disk, reducing the effective performance of the drive.  This

process happens slowly, however, so the gradual degradation in

drive performance is likely to go un-noticed for a long time.

SAFEPACK reorganizes the files on the disk, amalgamating the

scattered parts of each file, restoring the disk to its original

speed.

Why Safety is Important in a Defragmenter

=========================================

As described above, SAFEPACK moves most of your data from one part

of your disk to another, and this takes quite a long time

(typically 2-30 mins depending on machine, disk size etc).  During

such an interval there is clearly the possibility of a failure of

either the power supply or of the machine, and it is important to

consider what would happen to the valuable data on the disk if

this were to happen.  Some commercial defragmenters can wreck the

file structure of a disk if they are interrupted, but SAFEPACK has

been designed, as its name implies, to be tolerant of

interruptions, and should leave your disk in a fully usable

condition, even if switched off in the middle.  SAFEPACK achieves

this by ensuring that the file structures are valid at all times

during the run, and though this may slow the program down

slightly, what is an extra few minutes running time, compared with

having to reload your hard disk from scratch?

--- SAFEPACK Manual --- Page 3 ---

That said, however, Murphy's law of computers dictates that

something unforeseen will always happen somewhere at some time,

and despite my belief in the safety of this program, I would not

recommend that anyone defragments their hard disk with ANY

defragmenter, without first taking a full backup.  Backups should

be taken occasionally anyway, and as there is rarely any great

urgency to defragment a disk, it would make sense to run SAFEPACK

immediately after the backup.  Further aspects of data security

are mentioned in the detailed usage instructions below.

Requirements & Limitations

==========================

SAFEPACK requires and supports all versions of MS DOS between 2.0

and 5.0, and all DR DOS up to 6.0.  There is no fixed memory

requirement other than the size of the program itself, but

SAFEPACK uses as much conventional memory as possible, and the

number of files it can handle is proportional to the amount of

memory available.  Given the normal space of about 550k, SAFEPACK

can keep track of about 5000 files.  If it does run out of memory

it will terminate, but in doing so will leave the disk data

intact.

It should be possible to defragment any true DOS disk, whether

floppy or hard (including partitions), and even RAMdisks or non-

standard media such as 8" floppies, but the following types of

disk MUST NOT be packed:

a) Network drives

If it is required to pack a DOS network drive, this should be done

locally, with the network disabled.

b) Substituted drives

These are virtual drives, which are in fact sub-directories of

another drive.

c) Drives with proprietary compression methods

Examples of this are Stacker or Superstor, and although

compression of such disks would normally be safe, it would not be

of any benefit, and will not be performed by SAFEPACK.

--- SAFEPACK Manual --- Page 4 ---

Usage Procedure for SAFEPACK

============================

Although it should be possible to run SAFEPACK safely at any time,

the following procedure is recommended for maximum data security.

1)   Run CHKDSK to ensure that there are no internal errors in the

File Allocation Table (FAT) for the drive to be defragmented.

The format of this DOS command is:

CHKDSK drive:

If all is reported as being well, go on the next stage, but

if errors are found, they must be corrected before running

SAFEPACK.  Note that many common errors can easily be

corrected by running CHKDSK with the /F (for Fix) switch.

2)   The AUTOEXEC.BAT & CONFIG.SYS files, which normally contain

device drivers and other resident programs should be

disabled, either by temporarily renaming them and then

rebooting, or by booting from a floppy drive.

Although not always necessary, this step ensures that there

are no background programs, or TSRs, which could try to

access the disk during defragmentation, as this could lead to

data corruption.  The absence of TSRs will also, normally,

increase the memory available to SAFEPACK. If you have a disk

greater than 32 Mbytes, you may receive a warning about

loading SHARE for large media - this can be ignored for

running SAFEPACK.  Experienced users may wish make temporary

modifications to their startup files by placing REM at the

start of any offending lines.

3)   Run SAFEPACK as a command from the DOS command-line.  It must

NOT be run from within Windows, or any other multi-tasking

environment.

The format of the command is

SAFEPACK drive: [options]

where drive is the drive letter of the disk to be

defragmented, and options are as described below.

Default options exist, which are designed to be suitable for

the majority of users, including those maximizing their free

space prior to making a Windows swap file.  The following

example command line would therefore be appropriate for

defragmenting drive C:

SAFEPACK C:

--- SAFEPACK Manual --- Page 5 ---

Before SAFEPACK commences, a screen will appear, showing the

drive and options selected, and you are asked to confirm that

these are correct before defragmentation starts.  This stage

may be by-passed by use of the /Y- option if you wish to run

SAFEPACK unattended in a batch file.

While SAFEPACK is running, it will give an indication of its

progress, but this is intended as a guide only, and is far

from linear, so do not become alarmed if it appears not to

change for several minutes.  After two minutes, SAFEPACK

replaces its normal display by a simplified version, which

moves around the display area.  This is in order to prevent

damage to your screen during a prolonged run, and the normal

screen may be restored at any time by pressing N.

The program may be interrupted at any time by pressing Ctrl-C

or Ctrl-Break.  If this is done, a message will appear

acknowledging the break, but the program will tidy-up the

disk before terminating a few seconds later.

4)   After running SAFEPACK, confirm that the disk's file

structure remains correct by running CHKDSK again.

Note that some disk caches can become confused after running a

defragmenter, as not all respond properly to SAFEPACK's request to

reset the disk.  Therefore, if CHKDSK or a directory listing

indicate a problem, it can probably be cured by rebooting the

computer, but if this fails, then the file RECOVER.DOC should be

consulted for advice.

Swap Files

----------

SAFEPACK will automatically leave the swap files created by

Windows 3 or similar programs alone, and if you do not intend to

change the size of any such files, their presence may be ignored.

If, however, one of the objects of defragmenting your disk is to

increase the size of a swap file, then it should be deleted before

running SAFEPACK, and re-created (with its new size) afterwards.

SAFEPACK Options

================

The command line options available are described in detail below,

but a quick on-screen summary may be obtained by running SAFEPACK

with the /H switch.  Most options may be entered on their own to

activate them, or followed by - to suppress, the default varying

from one option to another according to likely usage. For example:

/D  would activate directory sorting (default)

/D- would suppress directory sorting.

--- SAFEPACK Manual --- Page 6 ---

Sorting Options

---------------

Subject to the various restrictions/modifications on sorting

described below, there are five basic methods used determine the

final order of the files on the disk.  By default, the files are

sorted alphabetically, but the method used may be selected by use

of the following switches:

/N :  Name sorting;

This is the default if no other sorting criterion is specified.

Files are sorted alphabetically by filename and extension,

regarded as a single 11 character string.

/E :  Extension sorting

Files are sorted in alphabetical order of extension only.

/L :  Length Sorting

Files are sorted by length, with the longest being placed first.

This is quite a useful option, as it can help to reduce future

fragmentation. Sub-directories, which do not have a length, are

placed before the files.

/A :  Age Sorting

Files are sorted by their ages, the oldest being placed first.

Again, this can help to reduce future fragmentation, as it is

generally newer files that are more likely to be deleted, thereby

creating free space at the end of the disk rather than at the

start.

/C :  Current position Sorting

Files are sorted such that the order in which their first cluster

appears on the disk is as it was before the pack.  This could be

useful if files are regularly accessed in a particular order, and

may be faster than the other options.

/R :  Reverse sort

This switch works in conjunction with any of the above sorting

methods, to reverse the resulting order.

e.g. SAFEPACK C: /L /R

would arrange the files such that the shortest were sorted to the

start of the disk.

--- SAFEPACK Manual --- Page 7 ---

Individual File vs Sub-directory Based Sorting

----------------------------------------------

/F :  Filesorting

The normal mode of operation of SAFEPACK is to place the files on

the disk in an order that reflects the hierarchical directory

structure, each sub-directory being sorted as if it were a file,

followed by the files it contains.  This method has the advantage

of speeding access to complicated sub-directory structures, as the

disk head has to move only short distances between directories and

the files they contain.  It does, however, mean that the options

above, such as placing the longest files at the start of the disk,

are not fully effective, as a long file may end up at the start of

a directory which is itself towards the end of the disk.  The /F

(Filesort) switch is therefore provided to sort files on the disk

by their own characteristics, rather than by their parent

directories.  Note that if this switch is used with sorting by

filename, then only the 11 characters of the basic filename &

extension are used, not the pathname.  This option would probably

be of most use with the /L (length) or /A (age) options.

Sorting Optimizations

---------------------

Generally speaking, the closer a file is to the start of the disk,

the more quickly it can be accessed, and as some files are

accessed more often than others, it would make sense to place

these files towards the start of the disk. In the absence of

precise information on the frequency with which files are

accessed, three assumptions can be made:

1)   Sub-directories are accessed more commonly than individual

files within a directory.

2)   Executable files (.EXE & .COM), are accessed more frequently

than other files.

3)   Files in sub-directories defined in the DOS PATH list, are

accessed more commonly than others.

By default, all of these assumptions are used in sorting the

files, so directories in the PATH list are placed towards the

beginning of the disk, and sub-directories are placed at the start

of the directories.  These are followed by files with .EXE & .COM

extensions, followed in turn by the other files, each category

being arranged according to the main sorting mode selected.

--- SAFEPACK Manual --- Page 8 ---

These assumptions may be disabled if required as follows:

/X :  Executables/Directories

If this option is set to off (/X-), then files are sorted without

any priority being given to executable files (.EXE & .COM) or

directories.

/P :  Path

If set to off (/P-), then the DOS path is ignored.

Note that any sorting order for sub-directories may be selected by

altering the DOS PATH before running SAFEPACK, as it does not have

to be your normal PATH.  See the DOS manual for details of how to

change the PATH. This option is automatically disabled if the

filesorting (/F) switch is used.

Directory Sorting

-----------------

/D :  Directory sorting

SAFEPACK is able to sort the directory entries to reflect the

order in which files are placed on the disk, and in so doing also

'packs' the directories to eliminate space occupied by deleted

files, marginally increasing the speed at which files are

accessed.  This process is quite distinct from the reorganization

of the files themselves, and though it is performed by default, it

can be disabled if required by using the switch /D-.

Immobile Files

--------------

Normally, a program accessing a disk file neither knows, nor cares

about the exact position of a file on the disk, providing that it

can be found through its directory entry.  There are, however, a

few exceptions to this rule, and in these cases SAFEPACK can leave

the file untouched.  Most files to be left alone are, effectively,

labelled as immobile by the programs that created them, as they

have their 'system' and 'hidden' attributes set, and SAFEPACK will

not attempt to move any files in this category.  Examples of such

files include the main DOS files IBMBIO.SYS & IBMDOS.SYS (or

similar) that contain the main operating system, and 386SPART.PAR

created by Windows when making a permanent swap file.

Apart from these, other necessarily immobile files may have been

created on the disk as part of copy-protection schemes etc.  The

vast majority of these will be marked as either hidden or system

--- SAFEPACK Manual --- Page 9 ---

(or normally both), and should not, therefore cause any problems,

but if any others are known, the following options will allow  you

to indicate any that must remain static.

/I :  Immobile files by name

An option of the form /Ifilename ensures that any files on the

disk with the name 'filename' will not be moved.  Only the actual

filename, without a pathname, should be quoted, and this will

immobilize files of that name in any directory.  If no extension

is given, it is assumed to be blank (i.e. 3 spaces), but

simplified wildcards are allowed, in that either the filename, or

extension may be replaced entirely by *. Thus the following

options are valid:

/IMYFILE.TXT   immobilizes MYFILE.TXT

/IMYFILE.*     immobilizes all files called MYFILE whatever the

extension.

/I*.SYS        immobilizes all files in all directories with an

extension of SYS

/IMY*.TXT would not, however, be valid, as partial wildcards are

not allowed.

Several /I options may appear on the command line, to immobilize

different files.

/S :  Static files by attribute

By default, files that are have either their hidden or system

attributes set are regarded by SAFEPACK as being immobile, but any

set of attributes may be selected as immobile using the /S option.

Note that files which are both 'hidden' AND 'system' will not be

moved, irrespective of these options.

The format is /Sx[-].., where x is one of the following:

D: Sub-directories

H: Hidden files

S: System files

R: Read-only files

A: Files with the 'Archive' bit set

Examples:

/SD       immobilizes all sub-directories

/SH- /SR  allows hidden files to be moved, but immobilizes all

read-only files

--- SAFEPACK Manual --- Page 10 ---

The above mechanisms should cope well with most varieties of copy

protection schemes, but in the event of problems with a

particularly badly behaved program, it should be possible, as a

last resort, to de-install the program before defragmentation,

then re-install it after.

OS/2 Control

------------

If you do not use the OS/2 operating system, this section may

safely be skipped.

OS/2 can use disks formatted in the MS-DOS manner (called FAT

volumes), but in order to do so, has to create hidden files

(called "EA DATA. SF" & "WP ROOT. SF") to hold extra information

about files for which there is no space in the main directory.

The first of these files tends to become excessively fragmented,

and its presence, if left unmoved, would impede the

defragmentation of the remainder of the disk.  SAFEPACK can

therefore regard these files as exceptions to the rule that hidden

and system files are left alone, and this exception is controlled

by the /o switch.  By default, this switch is on enabling the

files to be defragmented, so there is normally no need to use it,

but should you wish the OS/2 hidden files not to be touched, then

this can be achieved by adding /o- to the command line.

Running from the Command Line Only

----------------------------------

/Y :  User Confirmation On/Off

If this option is on, as it is by default, then a screen appears

before the defragmentation starts, asking the user to press Y to

confirm the drive and options selected, and a key-press is also

required at the end of the program.  If this option is disabled,

using /Y-, then no user input is required during the run, making

the program suitable for running in a batch file, and in this

manner, a number of drives may be defragmented with a single

command.  If this option is used, any errors cause immediate

termination of the program, rather than prompting for Retry/Abort,

as would be the normal practice. Those using SAFEPACK in a batch

files may find the following exit codes useful, as they can be

tested using the errorlevel function:

0 : Normal completion

1 : Terminated by user interrupt

2 : Terminated due to an error

--- SAFEPACK Manual --- Page 11 ---

Miscellaneous Options

---------------------

/Q :  Quick pack

The normal mode of operation of SAFEPACK is to move all files,

irrespective of their current positions, to positions based on the

sorting order as defined by the above options. This method not

only ensures optimal arrangement of the files, but also ensures

that all the free space is contiguous, as is advisable before

creating swap files.  The /Q switch, however, invokes a quicker

method whereby only those files that are currently fragmented are

moved.  It is suggested that the /Q switch should not be used for

the initial packing of a disk, but should be used for most

subsequent defragmentation, a complete reorganization, without the

/Q switch being performed every few months.

/B :  Bypass Multitasking Checks

Before starting to defragment a disk, SAFEPACK checks to see

whether it is being used from within a multi-tasking environment

such as Windows, MS DOS 5 taskswitcher, or DR DOS TaskMAX, as

these could be unsafe for defragmentation. Later versions of

FASTOPEN are also detected. SAFEPACK will normally refuse to

proceed if one of these environments is identified, but for those

users who know what they are doing, this option is provided to

bypass such checks, and to carry on anyway.  Note that some system

software provides its own protection against such risky procedures

and may refuse to let SAFEPACK write to the system areas of the

disk, even if this option is used.

This option is not for the novice - you have been warned!

/V :  Verify

DOS has an option to verify all disk writes, by reading back the

data after it has been written. This procedure increases data

security at the expense of speed, and is normally controlled by

the DOS VERIFY command.  By default, SAFEPACK leaves the verify

switch unaltered, but verification can be selected independently

for SAFEPACK with the this option, /V turning verification on, and

/V- turning it off.  In either case, SAFEPACK returns DOS to its

original status at the end of the program.

/M :  Monochrome screen

This option sets the screen colours so that they are more legible

on monochrome screens.

--- SAFEPACK Manual --- Page 12 ---

Disclaimer - Agreement

======================

Users of SAFEPACK must accept this disclaimer of warranty:

"SAFEPACK is supplied as is.  The author disclaims all warranties,

expressed or implied, including, without limitation, any

warranties of merchantability and/or fitness for any purpose. The

author assumes no liability for damages, direct or consequential,

which may result from the use of SAFEPACK."

This copy of SAFEPACK is licensed for use on any one computer at

any one time.  You must treat this software just like a book.  An

example is that this software may be used by any number of people

and may be freely moved from one computer location to another, so

long as there is no possibility of it being used at one location

while it's being used at another, just as a book cannot be read by

two different persons at the same time.   Site-Licenses are

available to those wishing to use SAFEPACK on more than one

machine, and are very reasonably priced, e.g. 50 Pounds Sterling

for up to 20 machines, or 200 Pounds for unlimited site licence.

For further details, please contact the author, D J Harvey at

PO Box 14, Cardiff CF4 6XW, UK.

All the registered trademarks used herein are registered to

whoever it is that owns them. This notification is given in lieu

of any specific list of trademarks and their owners.

--- SAFEPACK Manual --- Page 13 ---

Appendix 1 - SAFEPACK Error Messages

====================================

The majority of SAFEPACK error messages are self-explanatory, and

most refer to user errors on the command-line such as requesting

defragmentation of a non-existent disk.  Some, however, require

more explanation.

Multi-tasking environment XXX or FASTOPEN detected

--------------------------------------------------

It is not safe to defragment a disk from within a multi-tasking

environment such as Windows, and SAFEPACK contains checks for the

more common of such environments.  If you encounter this message

and are running a multi-tasking environment, terminate the multi-

tasker before re-running SAFEPACK.  If FASTOPEN is found, then it

is necessary to reboot after removing FASTOPEN from the CONFIG.SYS

or AUTOEXEC.BAT file. If you really wish to defragment under such

circumstances and understand what you are doing, or if this error

occurs without obvious reason, then the checks may be by-passed

with the /B option.  If this error is reported, no writing to the

disk will have occurred.

Error in the File Allocation Table

----------------------------------

SAFEPACK performs a stringent test of the disk's file structure

before starting to defragment it, and will abort with this message

if any errors are found, however minor.  Most common errors may be

fixed by running CHKDSK with the /F switch  - see the DOS manual

for details.  If this error is reported, no writing to the disk

will have occurred.

Insufficient memory for the program to continue

-----------------------------------------------

This error is most likely to occur if there are more than about

5000 files on the disk, or if you have less than 500k of free

memory.  Under some circumstances, it may be possible to run

SAFEPACK again with the same options, with success.  The disk

should remain fully usable after this error, which is a

'controlled exit'.

--- SAFEPACK Manual --- Page 14 ---

Disk read/write errors

----------------------

These may be reported under a variety of names such as write

protect, drive not ready, data error (CRC), seek error, sector not

found, write fault, read fault, or general failure. If any of

these errors occur, then there is likely to be a hardware or

magnetic surface error on the disk concerned, and SAFEPACK will

ask whether to retry or to abort - the normal ignore option is not

provided, as misreads of directory sectors could have disastrous

results.  If you have a utility to check &/or repair bad sectors

then this should be used, but otherwise it may be best to reformat

the disk, and restore your backup.  Note that errors of this type

are not normally detected by CHKDSK.

SAFEPACK Internal errors

------------------------

SAFEPACK contains several internal error checks to ensure that it

is functioning correctly, and if any of these fail then it aborts,

quoting an error number.  These errors should never be seen by

users, but can be produced if another program is accessing the

disk at the same time as SAFEPACK, as can occur if it is used

under a multi-tasking environment with the /B option.  Should an

internal error be reported, please check your disk with CHKDSK,

and contact the author, providing as much detail as possible about

the circumstances.

--- SAFEPACK Manual --- Page 15 ---

Appendix 2 - Disk Recovery

==========================

I sincerely hope that you will never need to use the information

in this section, but include it just in case any mishaps occur

whilst SAFEPACK is running.

Due to the way that SAFEPACK is written, disruption of the program

at any stage should not affect the integrity of your data, but may

cause minor disruption to the organisation of the disk, such that

it should be tidied up before further use.  This file contains

information on how do perform this tidying, and an explanation of

the error messages that may be encountered.

In the vast majority of cases of interruption to the program,

including all cases of user break-in with ctrl-break, the disk

should remain in a perfectly acceptable, usable condition, causing

DOS no problems.  It may well be even more fragmented than before,

but that shouldn't cause any problems that another run of SAFEPACK

wouldn't cure!  In rare cases, however, if the interruption occurs

while the directories are being written, two types of 'error' will

be noticed by CHKDSK, namely:

1) Invalid sub-directory

------------------------

Despite sounding serious, the form of this error left by SAFEPACK

is actually very minor, and refers to the fact that the .. entry

at the start of a sub-directory may not point correctly to the

parent directory.  Since the parent's entry pointing at the

sub-directory is correct, CHKDSK can fix this fault easily and

automatically if run with the /F switch, i.e. CHKDSK drive: /F.

2) 'Lost' clusters

------------------

This error can occur as moved clusters at the start of a file are

allocated together into a chain before the directory entry

pointing at them is updated, and the originals are only

de-allocated after the directory is updated.  If, therefore, the

defragmentation is interrupted in the middle of this process, then

one or other of the chains will be reported by CHKDSK as 'lost'.

As the contents of the chains are identical, either chain is

equally valid, and the 'lost' clusters may safely be freed, making

their disk space available for re-use.  This is most simply

accomplished by running CHKDSK with the /F switch, and then

answering NO to the question about whether to convert lost

clusters to files.  Should you have any worries about doing this,

however, you can easily answer YES to this question then copy the

files of the form FILExxxx.CHK (all in root directory), to another

sick before deleting them.

--- SAFEPACK Manual --- Page 16 ---

If other errors are reported by CHKDSK, it would be unwise to

trust the data on the disk, and it is probably best to re-format

the disk and restore the backup you took before running SAFEPACK.

If any such problems occur, please contact the author, with as

much detail as possible about the circumstances.

I hope that this section does not scare anyone from using

SAFEPACK, as I have yet to know it fail, but only a fool would

believe himself to have created a perfect program, and I believe

that an honest discussion of potential problems, however unlikely,

is better than pretending that they do not exist.

If nothing else, I hope that users will be persuaded to perform a

full backup of their disk before running ANY defragmenter,

especially one without SAFEPACK's safety features!

--- SAFEPACK Manual --- Page 17 ---