A caution about the PANASONIC 24 Pin printers:

With the 24 pin Panasonic printer you will have to set it for the

EPSON 2500 mode and make sure that you have added the extra

memory chip to the printer (the "KX-P43 Buffer" at an extra

cost!). Mr. Jim Sweeney of Laguna Beach, CA. writes:

"The Panasonic 24-pin printers are actually not that expensive to

upgrade for downloadable characters. All that is required is a

commonly available 256K SRAM (32Kx8, probably 200 ns or under). I

bought a chip at a local swap meet for $9. It isn't immediately

obvious how to open the printer up (it snaps together; the front

panel comes off first) but once it's open an empty socket

presents itself."

Bob Fink adds the following:

"I used the Digi-key SRM 20256LC 10 chip @ $18, but better buys

are on the street are available.  Jim found any of these chips will

work:

GM/RCA CDM62256

Hitachi HM62256

Motorola MCM60256

NEC yPD43256

The chip is CMOS so ground the protective container to

your printer frame (and you) before inserting."

For the KXP-1124 printer Fink gives these instructions:

1. Hold Twin tangs togethr at the middle of the Front Top and

pry Top center outward.

2. Insert screwdriver at one side of Front and pry Forward to remove

Tang. Repeat on other side  to Drop Front off.

3. Remove 3 Phillips Head Screws from inside Front. Top will rotate

up and back like a suitcase.

4. Three sockets will become visible at the rear with the socket

on the left being empty.  That is where the chip is to go.

When everything is set and the printer closed hold down the function

switch and turn on the Printer to enter Set-up mode. Change Download

buffer to "Enable"

Purchasing the chip through a dealer can get the price up by

about $70 so it is a good idea to try and 'do it yourself.'

If you try to download to the Panasonic without the extra memory

chip you will get a lot of curious looking junk, formfeeds and

dinging of bells. Since the printer has no place to store the

font data it prints out exactly what it is getting from the font

file.

[Jimmy Paris Software]