3UTILS.DOC --  CTRL 1.1,  DVORAK 1.3,  KEYCODE 1.3

February 4, 1993

CTRL 1.1   This update of CTRL and DVORAK facilitates higher interrupt 15h

function calls in the Phoenix BIOS.

A tiny memory resident utility that switches the left control key and the

caps lock key.  Useful on the enhanced keyboard.

A>CTRL           to install

A>CTRL  /U       to uninstall

The program checks to see if it is already loaded.  It will only install

once.  CTRL makes use of interrupt 15h, service 4Fh, so computers with a

BIOS older than January, 1986, may not be able to use it.  It will not work

in programs that access the keyboard directly, or in a Windows environment.

* * * *

DVORAK 1.3

Takes only 128 bytes of memory.

A>DVORAK           to install

A>DVORAK  /U       to uninstall

The program checks to see if it is already loaded.  It will only install

once.  When installed it will change the Qwerty keyboard layout to the

Dvorak layout as given in the following diagrams.  These may be printed out

and used as a reference.

When Scroll Lock is toggled, the Qwerty layout is active.  When Scroll Lock

is off, the Dvorak layout is active.

QWERTY KEYBOARD

_____________________________________________________

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | _ | = | \ |

|___|___|___|___|___|___|___|___|___|___|___|___|___|

| Q | W | E | R | T | Y | U | I | O | P | [ | ]  |

|___|___|___|___|___|___|___|___|___|___|___|____|

| A | S | D | F | G | H | J | K | L | ; | ' |

|___|___|___|___|___|___|___|___|___|___|___|

| Z | X | C | V | B | N | M | , | . | / |

|___|___|___|___|___|___|___|___|___|___|

DVORAK KEYBOARD

_____________________________________________________

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | [ | ] | \ |

|___|___|___|___|___|___|___|___|___|___|___|___|___|

| ' | , | . | P | Y | F | G | C | R | L | / | =  |

|___|___|___|___|___|___|___|___|___|___|___|____|

| A | O | E | U | I | D | H | T | N | S | _ |

|___|___|___|___|___|___|___|___|___|___|___|

| ; | Q | J | K | X | B | M | W | V | Z |

|___|___|___|___|___|___|___|___|___|___|

Note that A and M are the same in each layout.  In all, 33 keys are

translated.  There are minor Dvorak variants dealing with the quote, slash,

and semicolon keys.

DVORAK makes use of interrupt 15h, service 4Fh, so computers with a BIOS

older than January, 1986, may not be able to use it.  It is intended for

use with DOS.  Those with Windows already have an included Keyboard Layout,

US-Dvorak.

Key redefinition programs that make use of interrupt 16h, such as my own

DDKEY, can be used on top of this program in their normal fashion.

* * * *

KEYCODE 1.3

KEYCODE returns the scan code and the Ascii code, in that order, for any

key pressed on a keyboard.  First both values are given in hex, then they

are repeated in decimal.

The decimal values may be used in redefining keys with one of the ANSI

family of utilities.  KEYCODE is also useful for patching such programs as

DDKEY.  In entering hex codes with Debug, word (two byte) values, including

addresses, are reversed.  First enter the Ascii value, then the scan code,

reversing the hex KEYCODE display.

KEYCODE checks whether an enhanced BIOS is present, and issues key calls

accordingly.  To restrict to the nonenhanced BIOS, start up with

A>KEYCODE /O     or

A>KEYCODE -O

In particular, use this option to check macro output from DDKEY when you

have used DDKEY /O to install.

David M. Dibble

5388 Dahlia Lane

Riverside, CA  92507

U.S.A.