HANDS ON  Remote Access   HO.DOC  manual      Page 1

Version 5.00    Updated September, 1994

Easy Up and Running Release 1

This Product is Shareware, if you continue to use it

Please Register to Legalise your usage

_______

____|__     |               (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    Member

Table of Contents                                          page #

1.0    Introduction

2.0    What HANDS ON Remote Access Will Do

3.0    What HANDS ON Will NOT Do

4.0    Why Use HANDS ON?

4.1   Access to Local Area Network from Home

4.2   Can use common DOS software.

4.3   Software Support

4.4   Library CD-ROM Access

5.0    HANDS ON Host Installation

5.1   Select Serial Port

5.2   Change Password

5.3   Change Modem Initialization String

5.4   Change Modem dialing Format String

5.5   Select Maximum "Baud" Rate

5.6   Use modem CARRIER DETECT signal

5.7   Flow control

5.8   Generate INT 9 on each key

5.9   Respond to modem RING signal

5.10  Reboot on loss of carrier

5.11  Synchronize "baud" rate

5.12  Send wakeup every 2 minutes

5.13  Private mode

5.14  Printer redirection

5.15  Maximum call length

5.16  Create consistent .HO setup file

HANDS ON  Remote Access   HO.DOC  manual      Page 2

5.17  Abandon Changes

5.18  Save Changes and Leave Installation Program

6.0    Remote Installation

6.1   Setting the serial port

6.2   The baud rate

6.3   The command interpreter

6.4   Video settings

6.5   File Transfers

6.6   Printing

6.7   Setting the bell or beep

6.8   Sizing file transfer blocks

6.9   Break

6.10  Block wait

6.11  Block delays

6.12  Lost characters

6.13  Passwords

6.14  Inter-computer links

6.15  Flow control

6.16  Send Breaks?

6.17  Keyboard settings

6.18  PROMPT when shelled

6.19  Hotkeys

6.20  Sending CONNECT

6.21  Sending disconnect

6.22  Modem response strings

7.0    How to Use HANDS ON

7.1   Host Computer

7.2   Remote Computer

8.0    HANDS ON Utility Programs

8.1   HOREMOVE          - memory ripper

8.2   HOHOSTC           - flying changes

8.3   HOTALK            - chat

8.4   HOEXTS and TREXTR - links to DSZ

8.5   HOMODSET          - modem setting

8.6   HOCHECK           - time monitor

8.7   HOASK             - batch query tool

8.8   1STHAND           - multiple tools

9.0    SUPPORT and the ASP

10.0   Warranty

APPENDIX A - Configuration Scenarios

A1    Consultant/Client via Modem

A2    Remote/Shared via Modem

A3    Laptop/Desktop via Direct Cable

A4    Remote/Shared via Common Mini-computer

A5    Home/Office via Modem

APPENDIX B - Sample File Transfer Session

APPENDIX C - Modem Cables

HANDS ON  Remote Access   HO.DOC  manual      Page 3

APPENDIX D - DESQview Compatibility

APPENDIX E - Acknowledgments

APPENDIX F - ShareWare Vendor Notice (if any)

1.0 Introduction

To Get HANDS ON Remote Access Up and Running Quickly...

Consider just reading the QUICK.DOC first, then go through

this manual.  QUICK.DOC is for QUICK STARTS and QUICK HELP.

If you will be using modems then read the modem setup

suggestions at the start of appendix A.  Then read whichever

of the following sections most closely matches your own

application:

Consultant/Client via Modem

One consultant wishing to connect via modem to many different

clients in order to take over the clients' computers.

Remote/Shared via Modem

One remote access PC to be taken over by many different users

via modem.

Laptop/Desktop via Direct Cable

A simple direct connection with no modems.

Remote/Shared via Common Mini-computer

Instead of a direct link between the two PCs, each PC will be

logged into a common UNIX system.

Home/Office via Modem

A dedicated office PC to be taken over from a home PC.  This

option also contains some advice about configuring high speed

V32bis/V42bis modems (USR Courier V32bis or similar).

Please note that over 95% of our support calls, between July

and September 1994 (after the initial widespread release of

version 5.00) involve people who have incomplete copies of

Hands On.  Our response is to 'shrink' the default BBS

HANDS ON  Remote Access   HO.DOC  manual      Page 4

distribution and to put 'extras' on additional disks and

'other' locations like Compuserve.  This means that you will

repeatedly read of software that may not be present, but it

also means that we have included the necessary core materials

to get you going.  Nobody that we know of uses the entire

package.  We all use portions.  Locally, we install various

portions depending on specific needs, and we expect that you

will do the same.

2.0 What HANDS ON Remote Access Will Do

HANDS ON will allow you to connect two IBM PC-compatible

computers together (via telephone lines and modems, or via a

direct cable, or via a common mini- or main-frame computer),

and arrange for input to be accepted from either keyboard, and

for both displays to appear identical.  In this document the

computer which is running normal programs is called the host

computer, and the other computer is called the remote

computer, since its display is a remote copy or replica of the

host computer's display.  (Or, you sit in front of the

'remote' computer because it is remote from the program being

run)

HANDS ON will do a reasonable job of optimizing the screen

changes transmitted, so that (for example) moving a

highlighted menu bar will appear almost immediately even at

1200 bps (provided the call is relatively local).  (Try a

cross-continent call at 1200 baud sometime.)

HANDS ON always updates the remote/replica computer's screen

to match the current host screen.  This means that if several

commands are issued without waiting for the display to catch

up, then the remote/replica screen will be updated to show the

final host screen, without wasting time transmitting the

intermediate screens.

HANDS ON uses a password of up to 39 characters, which means

an intruder would take an astronomical time to guess a well

chosen password!  Since this password is transmitted

automatically, there is no need to keep it short and easy to

remember - once HANDS ON is installed, you will never need to

enter the password manually.  You can use various external

encryption tools for file transfers, and outfits like PSL

include a wide range of security tools in their catalogs and

disk libraries.  The passwords are stored in individual .HO

files that can easily be encrypted and archived as you like.

HANDS ON  Remote Access   HO.DOC  manual      Page 5

HANDS ON will allow the host site to call the remote/replica

site, or the other way around.  For consultants this means you

can make your clients pay the 'phone bills!

HANDS ON will automatically detect the "baud" rate being used

by the remote/replica computer, and set the host computer's

"baud" rate to match.  This means that users with different

speed modems can all access a single host.

HANDS ON uses just over 10K of memory on the host computer,

which means there should be enough memory left to run most

applications.

HANDS ON can be installed to "type" a command in response to

the 'phone ringing, which allows better control than merely

using the modem's auto-answer capability.  It can also send a

command directly to the modem.  See the WIZARD.DOC for

information on the Wizard and completely automating sessions.

You can also use tools such as CED to make sessions much more

interactive.  Use tools that are familiar to you.

HANDS ON will trap anything printed to any printer on the host

PC, and redirect it to the chosen printer at the Remote PC.

This redirection takes place only when a session is active, so

at other times the host can be used to print normally even if

HOHOST is still loaded.

HANDS ON will allow the host PC to run with screen blanked and

keyboard dead for improved privacy.

HANDS ON will support file transfers using external protocol

programs such as DSZ, which provide crash recovery and cope

better with poor lines than HANDS ON's built-in protocol.  The

Hands On file transfer design mimics DSZ in many ways, so the

products can work well together.

HANDS ON can limit the duration of a single session, to

prevent one user from "hogging" the line.

HANDS ON can automatically locate the COM ports (including

non-standard IRQ lines) currently available in a PC, including

IRQ8-15.

HANDS ON is "DESQview-Aware".  See Appendix D for details.

Portions of the Hands On distribution actually alter other

.exes in the distribution.  This sharply reduces RAM memory

HANDS ON  Remote Access   HO.DOC  manual      Page 6

usage and .EXE file size while increasing speed, but you may

find virus checkers that go bonkers.  Also, if you pklite or

diet the .EXE's to make them smaller, you will be hiding vital

configuration information in the individual .EXE, and will

knock the program 'off its rails' and cause wonky behaviour.

The initial installation uses the HOMNU.exe menu program and

associated batch files.  You are free to move the whole suite

to your favourite menu program.  You will need to customise

and finish the installation to meet your needs in any case.

Source code is available to registered users.  Contact us for

licensing information.

3.0 What HANDS ON Will NOT Do

HANDS ON will not manage remote graphics displays.  If the

user at the Remote computer runs a program that displays

graphics, then the Remote display may show (textual) garbage.

Keystrokes will still be transmitted to the host computer, but

the user must remember which keys are necessary to get out of

graphics mode again!  HANDS ON gives a warning that graphics

mode has been entered, but by then it is too late to avoid

it.  However, in an emergency HANDS ON can instruct the host

computer to reboot, so provided HOHOST is run from

AUTOEXEC.BAT this provides a means of recovery.

HANDS ON will only work with an 80 column by 25 line display.

HANDS ON will only work with the standard keyboard buffer.

Check the WIZARD.DOC for information on how the Wizard uses

the keyboard buffer.

Where HANDS ON is being used with a modem, it will work best

if the modem and modem cable provide carrier detect, data

terminal ready and ring indicator signals in addition to the

minimum transmit data, receive data and ground signals.

HANDS ON will cause bad "snowing" on some older CGA display

cards.

HANDS ON requires DOS 3.0 or later.

HANDS ON will not work particularly well on a noisy 'phone

line.  File transfers are error-checked, but normal screen

HANDS ON  Remote Access   HO.DOC  manual      Page 7

updates are not.  Normally this will not be a problem, since a

corrupted screen will be visually obvious and in any case

HANDS ON will usually detect the problem and automatically

request a full refresh.  However, if errors happen very

frequently then HANDS ON can spend all its time continually

re-refreshing the display!  Printer capture also is not

error-checked, so line noise will garble the printout.  [If

this proves to be a problem then the best solution would be to

use error-correcting (MNP4 or V42) modems.]

4.0 Why Use HANDS ON?

4.1 Access to Local Area Network from Home

HANDS ON will allow you to connect into your office computer

from home.  This means you do not need to take all your data

files home with you, and that you do not need to license

additional copies of your software (since your home computer

is not running the software, only replicating the screen of

your office computer).  If your office computer is part of a

local area network, then you automatically have access to all

the network facilities from home.

4.2 Can use common DOS software.

HANDS ON works fine with such programs as WordPerfect,

WordPerfect Office (especially mail), WordStar, xBASE,

spreadsheets, all the Novell NetWare utilities, and local area

networks are regularly backed up after hours via HANDS ON!

The only programs we have come across that could not be used

remotely either:

a)  used graphical displays (e.g.  WordPerfect preview); or

b)  took over interrupts needed by HOHOST (e.g.  comms programs);

4.3 Software Support

HANDS ON will allow you to take over your client's computer,

so that you can see what is going wrong.  In many cases this

will avoid the need for an trip to the client's site.

4.4 Library CD-ROM Access

HANDS ON is becoming quite popular for allowing remote (dial

HANDS ON  Remote Access   HO.DOC  manual      Page 8

in) access to CD-ROM databases running on PCs in libraries.

The chief attraction is that the library only needs to

register once for each CD-ROM PC that will be simultaneously

available, and the users do not need to register at all;  the

library can give them the software freely.  However, another

significant factor is that the CD-ROM database software often

requires a very large amount of memory, and HANDS ON uses very

little memory on the host PC.

Of course companies could also offer similar catalogs and

technical information to clients and customers too.  All of

the major software developers provide on-line information

services.

For details of these and other applications, see the scenarios

in Appendix A.

5.0 HANDS ON Host Installation

There are two ways to install the Hands On package:

1) Use the install program.  It copies the files and starts up

the 1STHAND.exe program, which provides a simple, interactive,

installation, setup, and learning process (see its description

below).

The following description provides an alternate 'lower level'

more detailed description of the installation.

2) manually copy the following files and follow the

instructions below:

To manually install HANDS ON at the host computer, first copy

the files into a directory (say \HO) on the host computer.

Then enter the command:  HOHOSTIN

(Please note that the 'menus' shown are simplified from the

original screens so you can print the manual on any printer.)

Your display should show a menu similar to:

HANDS ON   Host 5.00 Installation  (C)1994 P. C. Softsmith

A. Select serial port (currently COM1)

B. Change password (currently "password")

HANDS ON  Remote Access   HO.DOC  manual      Page 9

C. Change modem initialization string (currently:

"\rat &c1 &d2\r")

D. Change modem dialing format string (currently:

"ATDT %s\r")

E. Select maximum "baud" rate (currently 19200)

F. Use modem CARRIER DETECT signal (currently YES)

G. Flow control (currently NONE)

H. Generate INT 9 on each key (currently NO)

I. Respond to modem RING signal (currently NO)

J. Reboot on loss of carrier (currently NO)

K. Synchronize "baud" rate (currently YES)

L. Send wakeup every 2 minutes (currently NO)

M. Private mode (currently NO)

N. Printer redirection (currently HOST)

O. Maximum call length (currently unlimited)

P. Create consistent .HO file

Q. Abandon changes

S. Save changes and leave installation program

Selection:

Consider each of these options in turn:

5.1 Select Serial Port

Another menu will appear:

HANDS ON   Host Program Serial Port Selection

0. Completed, no (further) change

=> 1. COM1 port (base 0x03F8, interrupt IRQ4)

2. COM2 port (base 0x02F8, interrupt IRQ3)

A. Custom base (currently 0x03F8)

B. Custom interrupt (currently IRQ4)

X. Search for hardware

Selection:

Initially, the ports listed will be those reported by the

computer's BIOS during its initialization at power-up.  Since

the BIOS provides no information about interrupt numbers, the

IRQ numbers will simply be the most common (IRQ4 for COM1 and

COM3, IRQ3 for COM2 and COM4).  If you only use COM1 and COM2

then this may be all you need.  If you are not sure, you can

look at the back of the machine to check what is attached to

who.

HANDS ON  Remote Access   HO.DOC  manual      Page 10

However, if you use COM3 or COM4, especially with non-standard

IRQ numbers, you should select option X to have HOHOSTIN

examine your hardware and see what ports are really there.

[This is not done automatically since the searching process

could conceivably lock up the computer.] After selecting X,

the menu will be modified to reflect any additional ports

discovered;  for example:

HANDS ON   Host Program Serial Port Selection

0. Completed, no (further) change

=> 1. COM1 port (base 0x03F8, interrupt IRQ4)

2. COM2 port (base 0x02F8, interrupt IRQ3)

3. COM3 port (base 0x03E8, interrupt IRQ5)

A. Custom base (currently 0x03F8)

B. Custom interrupt (currently IRQ4)

X. Search for hardware

Selection:

All the existing ports are now listed.  To select one of these

ports, just type the port number.  If you have a non-standard

base address (one that is neither in the BIOS table nor one of

the common "standard" values), then use A and B to specify

exactly what you want [it may be worth selecting the base

address, and then choosing X again, so that HOHOSTIN will

check for the right IRQ line for that base and add it to the

menu].  When you are happy with the settings, select 0 to

return to the main menu.

Hands On has not been tested with any system which uses the

16-bit AT IRQ numbers (8-15) cards.  Also, you should avoid

having two programs simultaneously active on ports which are

listed with the same IRQ number - for example a mouse on

COM1/IRQ4 and a modem on COM3/IRQ4.  Mice do not provide good

screen images for HANDS ON to display for you.  Actually we

will provide free registration to anyone that successfully

uses Hands On to capture screen images from their mouse on

either the host or remote computer!

5.2 Change Password

The string editing menu will appear:

HANDS ON  Remote Access   HO.DOC  manual      Page 11

HANDS ON  Host Program Password Selection

Edit existing string:

password

Whenever you see the "Edit existing string:" prompt, you can

use the left and right arrow keys to move the cursor through

the string, and the backspace and delete keys to delete

characters to the left of or under the cursor respectively.

Any normal characters you type will be inserted at the cursor

position.  If the first key you press is a normal printable

character then the string will be cleared automatically.

Since you will only need to enter the password once during

installation, and never in normal use, it's a good idea to

make the password near the maximum length and quite random.

The main thing is that nobody should have any chance of

guessing the password.  Of course it has to be the same on

both ends of the connection.

5.3 Change Modem Initialization String

The string editing menu will appear again, this time asking

for the modem initialization string.  This string is

transmitted to the modem at the maximum "baud" rate when

HOHOST runs for the first time.  If you are using an automatic

power switch that powers up the host computer when the 'phone

starts ringing, then the initialization string may need to

answer the 'phone (see scenario A2).  There may also be

various operating modes that cannot be stored permanently in

your modem's non-volatile memory, and which must therefore be

established each time HOHOST starts.  In many applications,

where nothing needs to be changed, this string may be empty.

If the original installation did not provide a suitable

general modem initialization string in the modem database, you

might still go back and look for something similar.  You can

also check the settings in the software that came with your

modem.  Otherwise, you will probably need to refer to the

instruction manual for the host modem to work out what if

anything is required.

As well as normal characters, you may need to include some

special characters such as carriage return.  Carriage returns

can be encoded as \r (see the main menu at the start of

HANDS ON  Remote Access   HO.DOC  manual      Page 12

section 4).  Alternatively, any character can be entered in

hexadecimal as \0x followed by two hex digits (for example,

the escape key is \0x1B).  To get a \ character, use \\.

It may also be necessary to include some kind of delay.  For

example, some modems go off into a world of their own for a

second or so after a reset command.  Delays are entered in

milliseconds as \D followed by the delay (for example, \D1000.

[1 second] or \D500.  [half a second]).  Delays are only

accurate to the nearest 55 ms, so the number you enter may be

modified slightly to reflect the nearest possible actual

delay.

5.4 Change Modem dialing Format String

This string determines what the Hands On dialer HODIAL will

send to the modem to dial a number for Host made calls.  The

usual use of HODIAL will be something like HODIAL 221166, so

normally the dialing format string should contain a %s to

insert the number.  For example, if the dialing string is

"ATDT %s\r" and the user enters the previous dialing command,

then what will be sent to the modem is:  ATDT 221166 followed

by a carriage return.  Your modem manual will remind you that

ATDT is for Tone dialing and ATDP is for Pulse dialing.

Alternatively, if only one number is used, then the dialing

format string may include the actual number, and HODIAL can be

used without a number parameter (you just type in "HODIAL").

Note that (when CARRIER DETECT is being used) HODIAL will only

send anything to the modem if there is NO carrier detected -

once a carrier had been established the modem would not act on

the command in any case, but would send it through to the

other end.  When HOHOST is installed to ignore CARRIER DETECT,

HODIAL will send data to the modem in any case.

Other dialing choices:

The easiest choice is to use the PHONE menu entry in

1sthand.exe to setup some .HO entries with phone numbers and

use the same "1STHAND CALL" or "1STHAND DIAL" scheme that the

HOMENU.BAT menu setup uses.  1STHAND has lots of on-line help

(please yell loudly if you think the help is too big or too

small or whatever).

Many people use HOMODSET to set the modem and to dial.  You

HANDS ON  Remote Access   HO.DOC  manual      Page 13

can also fill in the entries in the bottom of your xxx.HO

configuration files to add automatic dialing.

We do not automatically setup any of these choices for some

reasons.  First, only you can best judge which best meets your

needs.  Second, its quite possible to setup automatic dialing

that always dials the 'wrong' number and we prefer that you

setup your own controls.  Its quite common to setup to only

dial one phone number, but this becomes a problem when you

change the number and cannot find or remember where it is

hidden in a configuration file.  Third, many people that dial

multiple numbers will have their own database and may have

their own dialer, or can choose which dialing method best

meets their needs.

Finally, most people call from the Remote to the Host, so all

of these options are best considered in that light.  It the

Host is simply receiving calls then the attention string

"ATA\r" can replace "ATDT %s\r".

5.5 Select Maximum "Baud" Rate

Another window will appear:

HANDS ON Host Program Maximum "Baud" Rate Selection

Enter "baud" rate:

You should enter the maximum "baud" rate that the host modem

is capable of using.  Usually this will be 2400 or 19200.

Note that HOHOST can automatically adjust to half this speed,

a quarter of this speed, or one eighth of this speed.  Thus if

you choose 19200 bps, HOHOST will be able to work at 19200,

9600, 4800 or 2400 bps (provided the host modem can handle

these speeds).

If you will be using a direct cable between two PCs, then

19200 should work on all systems and 38400 will work on faster

computers.  If your computers have 16550 type UART chips

instead of the standard 8250 chips then 115200 should work.

We are currently successfully using 19200 over 8250 chips

between an XT and a 386-25.  We normally use 115200 for

development and testing.  We also prefer 115200 when using

Hands On to transfer software from one computer to another

during computer repairs, and for various upgrades and

HANDS ON  Remote Access   HO.DOC  manual      Page 14

backups.

5.6 Use modem CARRIER DETECT signal

Selecting this option will determine whether or not HOHOST

will use the carrier detect signal provided by the modem.  If

you are using a real modem, then you should set this to YES

and make sure the modem is initialized so that it reports the

true CD status from the 'phone line.

If you are using a direct cable between two PCs, or connecting

PCs via a mini-computer, then set this option to NO.

5.7 Flow control

Selecting this option will bring up the following menu:

HANDS ON Host Program Flow Control Selection

0. Completed, no (further) change

=> 1. No flow control

2. Hardware (RTS/CTS) flow control

3. Software (XON/XOFF) flow control

4. Both hardware and software flow control

5. Set inter-character gap time (currently 330 ms)

Selection:

HANDS ON can use the following flow control systems:

NONE No flow control.  Use this with a direct cable or with

modems that require no speed adjustment.

CTS Hardware flow control using RTS (request to send) and CTS

(clear to send).  Use this in preference to software flow

control if it is available.

XON Software flow control using XON and XOFF (/).  Use

this if flow control is needed and hardware flow control is

not available.

BOTH Use both software and hardware flow control.  I can't

imagine why anyone would want this...

Normally HANDS ON will not need to use any flow control even

HANDS ON  Remote Access   HO.DOC  manual      Page 15

when it is available.  However, some modems communicate with

the PC at a different speed than that at which they

communicate with other modems, so that it is possible for the

PC to "get ahead" of the modem and hence lose data.  In these

cases it is necessary for the modem to tell the PC when to

wait a bit before sending the next byte.

A side effect of software flow control is that HANDS ON will

not use ANY control characters:  it will not send any, and it

will ignore all control characters received apart from XON and

XOFF.  This may be useful for networks that respond to certain

control characters;  however, binary file transfers will be

slightly slower since 64 characters (control characters with

and without the parity bit) are not used.  Note that both HO

and HOHOST must therefore either use or not use software flow

control If only one end uses software flow control, then no

communication will be possible!

When HOHOST is receiving characters (normally keys that have

been pressed at the remote PC - each key is encoded as two or

three characters) it waits a certain length of time after

receiving the first character before giving up and ignoring

the key.  This is important, since otherwise a single byte

introduced by noise on the line could cause all subsequent

keys to be misinterpreted (the second character of one key

being treated as the first character in the next key and so

on).  HO always sends all the characters which constitute one

key as close together as the communication speed will allow,

so usually a gap of even 0.1 seconds means that something has

gone wrong and that HOHOST should start looking for a new key

again.  However, if the link between HO and HOHOST uses

packets, or for any other reason introduces significant gaps

between two characters that were sent together, then HOHOST

may need to be adjusted to tolerate longer gaps between

characters which should be together.  Selecting option 5 will

bring up the following screen:

HANDS ON Host Program Inter-character Gap Time Selection

Longest gap to be tolerated (ms): 330

Any time up to about 10 seconds (10000 ms) can be entered.  Be

aware that if this time is set to 10 seconds, and line noise

does introduce a stray character, then you will need to wait

10 seconds without typing anything at the remote PC to get

HOHOST back into synchronization!

HANDS ON  Remote Access   HO.DOC  manual      Page 16

5.8 Generate INT 9 on each key

When HOHOST receives a key from HO, it will usually just place

it in the keyboard buffer to be found by any program that

wants to read a key.  However, some applications monitor the

keyboard interrupt (INT 9) and only read the keyboard input

buffer after there has been a keyboard interrupt!  To keep

such programs happy, HOHOST can generate an INT 9 after it has

placed a key into the buffer.  However, this can cause

problems for other programs, particularly when the host and

remote keyboards may both be in use at the same time (e.g.

HOTALK).  Leave this option OFF unless it is really

necessary.

5.9 Respond to modem RING signal

Selecting this option will determine whether or not HOHOST

will respond to the RING signal which the modem provides to

tell the PC that the 'phone is ringing.  If this option is

turned on, then the menu will be extended with the following

sub-options:

G.  Respond to modem RING signal (currently YES)

---|- 1. too short  220 ms

|- 2. too long  5000 ms

|- 3. delay    10000 ms

|- 4. reset    10000 ms

|- 5. "type" response

|- 6. response

"\0x1Btrauto\r"

If the 'phone only rings for a very short time, you may wish

to ignore the ringing completely - otherwise just knocking the

'phone off the hook can trigger a HOHOST response!  (It

happens and scares people).

If the 'phone rings for a long time, then this may indicate

that the caller is a person who should not be greeted by an

ear full of chatty modem!  If HOHOST is intended to respond

before the 'phone stops ringing, then this time should be set

the same as the delay below.

After a certain length of time (measured from when the 'phone

starts ringing) HOHOST will check that the amount of ringing

was within the specified range, and if so, will respond to it

(see point 5).

HANDS ON  Remote Access   HO.DOC  manual      Page 17

If the 'phone rang for too long, then HOHOST must know when to

start looking for another call.  If the 'phone has not rung

for a certain length of time then HOHOST assumes any ringing

must be a new call.

The response string may be either "typed" (that is, inserted

into the keyboard buffer as if a user at the host PC had just

typed it in), or it can be sent direct to the modem.  The

advantage of "typing" is that you can run a batch file to

check times and do all sorts of other things.  The advantage

of sending direct to the modem is that the entire operation

can take place in the background while an application

continues running.

You may specify a response of up to 39 characters.  If you

expect the host PC to be at the DOS prompt when a call comes

in, then it is a good idea to start with an escape key, so

that DOS discards anything already typed and treats what

follows as a new command.

If all this sounds too complicated, but you still think you

want to enable ring response, then have a look at the

configuration scenario called "Home/Office via Modem" in

section A5.

5.10 Reboot on loss of carrier

Selecting this option will determine whether or not the

computer will reboot when the modem reports that the carrier

has been lost (presumably because the other end hung up).  See

scenario A5 for an example of an application taking advantage

of this feature.

The PC will not reboot unless there has been a HANDS ON

session active since HOHOST was loaded.

5.11 Synchronize "baud" rate

Selecting this option will determine whether or not HOHOST

will attempt to match the "baud" rate being used by HO at the

other end.  This is useful if several users will all dial into

the same host, but some users have slower modems than others.

See scenario A2 for an example of this.

5.12 Send wakeup every 2 minutes

HANDS ON  Remote Access   HO.DOC  manual      Page 18

Selecting this option will determine whether or not HOHOST

will send a NULL character whenever two minutes has passed

with no other activity.  A single NULL is ignored by HO, but

may serve to keep a connection to a mini-computer link.  See

scenario A4 for an example of this.

5.13 Private mode

In private mode, the screen of the host PC will be blank, and

the keyboard of the host PC will be inactive (including

CTRL-ALT-DEL).  This is useful to enhance privacy when the

host PC is in a public location.  Note that the screen

blanking has only been tested on VGA, EGA, CGA and Hercules

display cards, and only in text modes.  It has worked

correctly with all the applications tested, but you should

test it with your own applications and display cards before

relying on it!

5.14 Printer redirection

When a HANDS ON session is active, it is possible to redirect

printer output (which would otherwise be printed at the host

PC) so that it is printed at the Remote PC instead.  When no

session is active, the host will print normally even though

HOHOST is still loaded.

5.15 Maximum call length

In some cases, perhaps where multiple users are competing for

a limited number of phone lines or a toll-free number is being

used, it will be desirable to enforce some upper limit on the

connection time.  After this time, HOHOST will drop DTR for 2

seconds, which will cause a (correctly configured) modem to

hang up.

One minute before the timeout HOHOST sends a warning message

to HO, so that the user will have a minute in which to exit

gracefully from whatever is happening.  Note that this will

not happen during a file transfer, so you should not start a

file transfer unless you are sure there will be time to

complete it before the host hangs up!

Due to the way this timer is implemented, there is a limit of

3598 seconds (just under an hour).  If I get enough people

complaining that they really need limited length sessions, but

the limit needs to be over an hour, I will consider changing

HANDS ON  Remote Access   HO.DOC  manual      Page 19

the implementation...

Note that DTR will also be dropped before rebooting if a

maximum call length has been specified, so there is no (known)

way to defeat the mechanism.  However, it might be a good idea

to restrict access to programs like HOHOSTIN and HOHOSTC which

could be used to modify the time limit!

5.16 Create consistent .HO setup file

Once HOHOST has been installed correctly, HOHOSTIN can create

a configuration file for HANDS ON that is consistent (same

"baud" rate, password, use of carrier detect etc).  It will

normally still be necessary to edit this file to do some fine

tuning, but the work required will be less than if you started

with the generic HO.HO file distributed with HANDS ON.  Of

course you can use HOHOSTIN to re-create a generic HO.HO that

better meets your needs.

5.17 Abandon Changes

If you have made a mistake, or if you only wanted to see what

the current settings were, then this option allows you to quit

without making any changes.

5.18 Save Changes and Leave Installation Program

This is the normal way to exit this program when you have made

all the required changes.  The relevant files (HOHOST.EXE,

OMIGET.EXE, OMIPUT.EXE HOTALK.EXE, HODIAL.EXE, HOEXTR.EXE,

HOEXTS.EXE and HOMODSET.EXE) will all be hunted down and

modified appropriately.  HOHOSTIN whines when it cannot find

these files to modify them.

Note that the executable files are actually modified, so

beware of virus protection programs which compute checksums

for executable files - they are likely to report possible

virus infections after installation!  The alternative (loading

configuration information from a disk file) was rejected due

to the additional memory required for the file processing

code.

6.0 Remote Installation

HANDS ON  Remote Access   HO.DOC  manual      Page 20

There are two ways to install the Hands On package on the

Remote computer:

1) Use the install program.  It copies the files and starts up

the 1STHAND.exe program, which provides a simpler, more

interactive, installation and learning process.  You will

normally finish the installation with some tailoring or

customizing details such as the dialing options for the host

and the remote.

The following description provides a 'lower level' more

detailed description of the installation.  It is useful to

read it before finishing the final tinkering for the initial

installation.

2) manually copy the following files and follow the

instructions below:

HO.EXE, HO.HLP, and HO.HO (or better still the .HO file that

was created during host installation) into a directory (say

HO).  Then use the 1sthand PHONE menu or a text editor to look

over and possibly edit the .HO file.  This file should look

something like:

HANDS ON Configuration File

Do NOT change the order of these parameters or delete lines!

"COM1"       Communications port ("COMn" or base/interrupt)

9600         "Baud" rate (typically 2400 or 19200)

"COMSPEC"    Path for DOS Command Interpreter (or "COMSPEC")

07           Attribute for normal text (such as help screen)

47/70        Attribute for file transfer monitoring

4F/0F        Attribute for pop-up windows

"omiput %s\r" Host command to send (download) files

"omiget\r"    Host command to receive (upload) files

0            Printer (normally 0, 1 or 2)

440          Frequency of "bell" tone (Hz)

100          Duration of "bell" tone (ms)

256          Block size for file upload (1-256)

8            Number of block buffers for upload (1-8)

111          Length of "break" (ms)

3000         Time to wait for first char before timeout (ms)

500          Time to wait for subsequent chars in block (ms)

56           Time to wait between "typing" keys (ms)

"password"   Password to log in to host computer

"MODEM"      Type of link (MODEM, DIRECT, or OTHER)

"NONE"       Flow control (CTS, XON, BOTH or NONE)

HANDS ON  Remote Access   HO.DOC  manual      Page 21

"YES"        Send BREAKs to host computer

"YES"        Use BIOS for keyboard input

"[OMI] %s"    DOS shell prompt format string

"LRCA"       Local command shifts (Left, Right, Ctrl, Alt)

""           Connect command for modem

""           Disconnect command for modem

Most of these parameters will not need to be changed.

You can add the actual path to OMIPUT/OMIGET to ensure that

they can be found from anywhere on your system, as in

"c:\\HO\\omiput %s\r" Host command to send (download) files

However,Its generally more useful to add the HO directory to

the machine's PATH.

Each of the remaining lines must contain 3 strings and a

number.  The first string is the trigger string to search

for.  The second string is the response to be sent after the

trigger.  The third string is the response to be sent after

timeout.  The number is the time to wait before timeout (in

ms).

"DISCONNECTED"      "\D250\r"          "\r"             5000

"Enter RESOURCE ? " "\D200G9000\r"     "\r"             5000

"CONNECTED TO"      ""                 ""               5000

"\rlogin: "         "\D200doug\r"      "\B\r"           2000

"\nlogin: "         "\D200doug\r"      "\B\r"           2000

"Password:"         "\D200secret\r"    "\r"            10000

"Selection: "       "\D100d\r"         ""              40000

"/users/"           ""                 ""              40000

For a 'generic' .ho file leave the 'Connect' and 'Disconnect'

lines unchanged.  If you want to automatically connect/dial

you can fill in the 'Connect command for the modem' with the

appropriate string.  You can also peruse at the INIT file

created to match your modem during the initial installation.

You can insert it and something like "ATDT433-5189\r" to

automatically dial the number.  (However, its easier to use

the PHONE option in 1sthand.exe later.)

from:       ""                Connect command for modem

to:       "AT&F0&C1&D2S95=44S7=70 ATD 123 4567\r"

Connect command for modem

(actually all on one line, and use your modem's INIT string)

If you just want to dial the phone do something like the

following:

HANDS ON  Remote Access   HO.DOC  manual      Page 22

from:       ""                Connect command for modem

to:       "AT DT 123 4567\r" Connect command for modem

Note that including the ATDT and the phone number locks you

into dialing that number each time you run the software.

6.1 Setting the serial port

The serial port may be one of the standard ones ("COM1",

"COM2", "COM3", or "COM4") or you may specify the base address

and interrupt number [e.g.  COM1 would be 03F8/IRQ4].  For

compatibility with previous versions of HANDS ON, you may also

use a hex interrupt vector number, so that a third way to

specify COM1 is 03F8/0C.  Include those double quotes "".

6.2 The baud rate

The "baud" rate should be set appropriately for the modem.

Its often useful to set modems to a higher rate than their

actual speed.  For example, we normally use 19200 for a 14400

baud fax modem.

6.3 The command interpreter

The DOS command interpreter should normally be taken from the

COMSPEC environment variable.  However, if this does not work

(or does not select the desired command interpreter) the full

path name of a command interpreter can be specified (e.g.

"C:/DOS/COMMAND.COM" - or "C:\\DOS\\COMMAND.COM" - the \\ is

needed since \ introduces an escape sequence).

6.4 Video settings

Video attributes are two hexadecimal digits:  the first is the

background color, and the second is the foreground color.

Optionally a second attribute may be specified (e.g.  4F/70) -

the first attribute is used for color displays and the second

is used for monochrome systems (this allows a single control

file to work without change with different displays).

6.5 File Transfers

When the user running HANDS ON requests a file transfer, HANDS

HANDS ON  Remote Access   HO.DOC  manual      Page 23

ON "types" the necessary command at the host PC.  These two

lines should not be changed except possibly to fill in a full

path name if OMIGET.EXE and OMIPUT.EXE are not in the path of

the HOST PC.

6.6 Printing

When host printing is being redirected to the Remote

computer's printer, HANDS ON needs to know which printer to

use.  LPT1:  is printer 0, LPT2:  is printer 1, and LPT3:  is

printer 2.  Serial printers are not directly supported.

Note that it is not necessary or possible to specify which

host printer is to be redirected;  anything sent to LPT1:,

LPT2:  or LPT3:  will be redirected.  This was done to save

the user the trouble of working out which printer the host was

using.

6.7 Setting the bell or beep

The frequency (or pitch) and the duration of the "bell" can be

modified to suit your personal preferences (and the volume of

your PC's speaker!).  The "bell" is used to indicate that some

corruption of the data being received by HANDS ON has

occurred.

6.8 Sizing file transfer blocks

The size of file transfer blocks and the number of buffers

should not need to be changed, except on a very poor 'phone

line (in which case smaller blocks and fewer buffers may

improve matters).  However, if the line is this bad then HANDS

ON will probably be unusable anyway...

6.9 Break

The length of "break" required is not likely to need to be

changed - unless perhaps a mini-computer requires longer?

6.10 Block wait

During a file transfer, HANDS ON will wait a certain time for

the start of each block.  If nothing has been received within

HANDS ON  Remote Access   HO.DOC  manual      Page 24

the time, then HANDS ON will request the block be sent again

(since presumably it got lost).

6.11 Block delays

Once the first character in a block has been received, the

rest should follow immediately - any gap probably means

problems.  However, when using an overloaded mini-computer

link (or using a packet network that may introduce delays

between packets, or using a modem with error correction,

compression or bps translation) this parameter could need to

be longer.

6.12 Lost characters

Since on some PCs (mainly ones with CGA displays) interrupts

are disabled for LONG periods when writing to the screen, it

may be necessary to make HANDS ON pretend to type quite

slowly, especially when sending things like file transfer

commands.  If you find you are losing characters under these

circumstances, then make this delay longer;  if you have a

smart BIOS and/or version of DOS and/or command interpreter

and/or display card then you may be able to get away with a

shorter delay (even 0 ms).  On all the systems used to date,

the time taken to send two characters at 2400 bps or below is

sufficient, so this delay is probably only needed above 2400

bps.  (You may say "who still uses CGA?", but we provide free

support to non-profit people that use whatever we scrounge

together for them.)

6.13 Passwords

The password must match the password installed in HOHOST at

the host computer.  The Hands On system can also be configured

to use secondary passwords, so you can use this automatic

password setting and supplement it with a second one.  For

example, the RUNT program also includes password capability.

6.14 Inter-computer links

There are three kinds of link that may be used between two

PCs:

MODEM Use this if you are working with a modem that provides

HANDS ON  Remote Access   HO.DOC  manual      Page 25

CARRIER DETECT (note that you will require an appropriate

cable and possibly some initialization of the modem [see the

start of Appendix A]).  You can also use this if you are using

a null modem cable that provides a CD line.

DIRECT Use this if you are working with a three-wire null

modem cable, or if you are using a modem/cable that cannot

provide accurate CD information.

OTHER Use this if you are working with a mini-computer link

(see scenario A4), or for initial modem setup (see the start

of Appendix A).

6.15 Flow control

HANDS ON can use the following flow control systems:

NONE No flow control.  Use this with a direct cable or with

modems that require no speed adjustment.

CTS Hardware flow control using RTS (request to send) and CTS

(clear to send).  Use this in preference to software flow

control if it is available.

XON Software flow control using XON and XOFF (/).  Use

this if flow control is needed and hardware flow control is

not available.

BOTH Use both software and hardware flow control.

See the discussion of flow control under host installation for

further details.

6.16 Send Breaks?

Normally HANDS ON should send BREAKs to HOHOST, but see

scenario A4 for use with a mini-computer link.

6.17 Keyboard settings

Normally HANDS ON will use the BIOS for keyboard input.

However, many BIOSes filter out keys from enhanced keyboards

(e.g.  <F11>), and if use of these keys is important then it

may be necessary to NOT use the BIOS, but rather to use the

keyboard input buffer directly.  Note however that some

HANDS ON  Remote Access   HO.DOC  manual      Page 26

keyboard enhancing programs (such as Newkey) will only work

with the BIOS setting.  This is because they intercept the

keyboard interrupt and store keys away in some place other

than the standard buffer area, so that HANDS ON cannot find

them!

6.18 PROMPT when shelled

The DOS prompt for when you are in a HO shell can be edited to

suit your individual preferences - the %s inserts the original

prompt.

6.19 Hotkeys

If you don't like the default <Ctrl-Alt-LeftShift-RightShift>

key combination to access local commands in HO, then you can

change this to any other combination of these four main shift

keys ("CAL" would mean <Ctrl-Alt-LeftShift> for example) - see

scenario A3 for an application of this).

6.20 Sending CONNECT

The connect command for the modem is sent when HANDS ON starts

running (this will often be a dial command).  The modem

command strings here have the same format as those described

above for the host computer installation (with the addition of

a \B escape sequence that causes HANDS ON to transmit a

BREAK).  Make sure you leave the double quotes (") around each

string!

6.21 Sending disconnect

The disconnect command is sent when HANDS ON terminates (using

exit option 3 only).  "ATH0\r" is a standard 'hangup' string.

6.22 Modem response strings

The automatic response strings at the end of the file can be

used to respond to modem status messages.  For example, if the

Remote computer's modem is not set to auto-answer, then HANDS

ON may need to respond to a "RING" message from the modem by

answering the 'phone - see scenario A5 for an application.

These strings can also be used to achieve an automatic login

HANDS ON  Remote Access   HO.DOC  manual      Page 27

to a mini-computer - see scenario A4 for an example.  The

trigger strings are only active before a HANDS ON - HOHOST

link is established.  They are also disabled once the final

trigger string has been detected.

Please note that since this file contains the password, anyone

who obtains a copy of this file can potentially access the

host computer!  You might consider encrypting the individual

.HO files.  One option is to store them in an encrypted .ZIP

file and bring them out as needed.  Consider truncating such

files to zero length before erasing them afterwards so they

cannot be easily undeleted.

7.0 How to Use HANDS ON

The following instructions assume that HANDS ON has been

correctly installed (see sections 4 and 5).  Only general

guidelines are given here;  more details are provided in the

scenarios in Appendix A.

7.1 Host Computer

First run the HOHOST program.  If the Remote computer is to

originate the 'phone call, then this may be all that is

necessary.

If the host computer is to originate the call, then use the

command:  HODIAL or HODIAL 999-9999 depending on how the

dialing format string was installed.

Once HOHOST is resident and the call is established, the host

computer can be used normally.  The only difference is that

everything that appears on the screen is also being repeated

on the Remote computer's screen.

The best way to remove HOHOST from memory is to run HOREMOVE.

It is also possible to use utilities such as MARKNET/RELNET.

Note that some common mark/release utilities can NOT be used,

because they do not restore the interrupt mask register.  You

will find the RS232OFF.EXE program in the distribution.  It

can be used to reset your machine's com ports.  We use it 'all

the time'.

7.2 Remote Computer

Use the command:  HO [use HO.HO configuration file]

HANDS ON  Remote Access   HO.DOC  manual      Page 28

or  HO filename      [use filename.HO configuration file]

If the Remote computer was to originate the call, then the

modem initialization string in the configuration file will

usually include a dial command to call the host computer, but

it is NOT automatically included in the installation.  See the

INIT file for the initialization string that matches your

modem.  Add something like ATDT 433-5189 for dialing the

specific phone number.  You can use the 1STHAND program to

setup for calls to multiple numbers with various passwords.

Check out the HOMENU.BAT menu files for usage clues.

If the host computer was to originate the call, then the

initialization string may contain an answer 'phone command.

Note that in this case it is important that the HANDS ON

program not be run until after the 'phone starts ringing!

(Alternatively, there may be no initialization string, but

instead there may be a trigger string to respond to RING from

the modem - see scenario A7.)

If the configuration file contains no modem initialization

string, then the user at the remote computer can enter modem

commands manually.  This may be convenient for a consultant

wishing to dial many different clients.  You can also use

HODIAL or HOMODSET for modem setup and dialing.  This gives

you the most flexibility.  (Before a carrier is detected,

HANDS ON will behave much like a normal communications

program.)

Once both HANDS ON and HOHOST are running and connected to

each other, they will automatically adjust the host computer's

"baud" rate (if this feature is enabled) and the host computer

will check the password provided by the Remote computer.

Provided the password is correct, the Remote computer will

effectively be able to take over the host computer, or

alternatively to observe whatever the host computer is doing.

Any key typed on the Remote computer will be acted upon by the

host computer, and anything that appears on the host

computer's screen will also appear on the Remote computer's

screen.

It is sometimes necessary to execute a command on the Remote

computer, rather than transmitting the keystroke to the host

computer.  This is achieved by holding down some combination

of left shift, right shift, ctrl and alt all at the same time

(the exact combination is specified in the .HO file currently

in use).  When you do this a list of all the possible commands

will be displayed, and you can then type the required command

HANDS ON  Remote Access   HO.DOC  manual      Page 29

key:

F2 simulate a carrier signal (useful if the modem does not

provide this information) [link type should be OTHER to use

this command] Ctrl-F2 simulate NO carrier signal (opposite of

F2)

F6 send break, restart session [use this if HANDS ON locks up

- very rare!]

F8 reboot the host computer (useful if host computer "hangs"

for any reason, but make sure HOHOST is run from

AUTOEXEC.BAT!)

F9 shell directly to DOS, return to HANDS ON later by typing

"exit"

F10 exit HANDS ON [optionally shell out and return later]

Home refresh display from HOST [use this if a bad 'phone line

messes up the screen]

PgUp upload file using OMNI protocol [transfer file(s) from

Remote to host]

PgDn download file using OMNI protocol [transfer file(s) from

host to

Remote]

Note that before performing a file transfer, the host computer

must be at the DOS prompt, and OMIGET.EXE and OMIPUT.EXE must

be accessible in the current path.  Alternatively the .HO file

must contain full paths, not just OMIGET but rather

C:/HO/OMIGET - note the forward slashes so the software will

not eat them.  If you vastly prefer backslashes you will need

to double them.  If OMIGET/OMIPUT cannot be found, then the

host will display a "command not found" error message, but HO

will go on waiting for a response from OMIGET/OMIPUT!

Pressing any key will end this waiting and bring up an

appropriate error message.

If a file transfer needs to access a file in another

directory, it may be convenient to shell out of HO, change

directories, and then exit from DOS back into HO to start the

file transfer.  HO will transmit any path that you specify, so

if the two PCs have equivalent directory structures it is easy

to transfer files to the corresponding directory on the other

PC (see Appendix B for a sample file transfer session).  If

HANDS ON  Remote Access   HO.DOC  manual      Page 30

you automate file transfers with the Wizard and the Wizard

Feeder, the Feeder can automatically change directories for

you.

8.0 HANDS ON Utility Programs

8.1 HOREMOVE - memory ripper

This will search for a copy of HOHOST loaded in memory.  If

one is found, then it is checked to see whether it is

currently in use (if so, HOREMOVE will ask for confirmation

before removing it).  If it is not in use, or if it is to be

removed anyway, all interrupts, the interrupt mask register

and the serial chip state are restored to their initial state

when HOHOST started running, and HOHOST's memory is freed.

Please note that as with most TSR programs, this may not work

unless HOHOST was the most recent TSR to be loaded.  In

general TSR programs should be unloaded in the reverse order

they were loaded.  In this case "not working" means that any

TSR loaded after HOHOST may cease to function.  If this is not

possible then the use of MARKNET/RELNET (or some other

mark/release utility that preserves the interrupt mask

register as well as the vectors) is recommended.

It is sometimes convenient to be able to keep a modem

connection even though HOHOST is being unloaded.  Normally,

HOREMOVE will restore the serial port state to its original

state when HOHOST was first run.  However, it is possible to

force the DTR line to be left active by entering:  HOREMOVE ON

and DTR can be forced off by entering:  HOREMOVE OFF If either

ON or OFF is specified, then HOREMOVE assumes you know what

you are doing and does not ask for confirmation even if a link

is active, allowing HOREMOVE to be used in a batch file on the

host PC without pausing for input.

8.2 HOHOSTC - flying changes

This program works much like HOHOSTIN, except that instead of

modifying files on disk it modifies the copy of HOHOST

currently active in memory.  Not all parameters may be altered

"on the fly" - for example, the COM port cannot be altered

mid-session.  The anticipated use for this program is to do

things like turning private mode on or off for just part of a

session...

HANDS ON  Remote Access   HO.DOC  manual      Page 31

Note that the copy of HOHOST.EXE on disk is not modified.

8.3 HOTALK - chat

HOTALK will allow the users at the host and Remote computers

to "talk" to each other.  Anything that is typed at the host

computer is displayed in the top half of the screen, and

anything that is typed at the Remote computer is displayed in

the bottom half of the screen.  This is also known as a CHAT

feature.

HOTALK sends a special command to HO to tell HO that talk mode

is required, so that HO will send keys in a way that can be

distinguished from the host keys.  If this command is lost,

then what is typed at either end will appear in the top

window!  If this happens, simply exit HOTALK (Alt-X) and run

it again.

HOTALK also sends a special command to HO to tell HO to end

talk mode.  If this command is lost, then HO will behave

normally unless a program that uses shift-key status is run,

which will cause problems because in talk mode HO does not

send shift status information!  To correct this problem, use

local HO command <F2> (simulate carrier), since a side effect

of this command is to end talk mode.

Please note that this distribution includes the telemenu,

which also has chat capability.

8.4 HOEXTS and TREXTR - links to DSZ

These programs are normally called from HOSZ.BAT and HORZ.BAT

respectively.  They allow a clean interface between HANDS ON

and external file transfer programs such as DSZ, and

hydracom.

These programs are an exception to the rule that HOHOSTIN

installs only programs used at the host end;  HOEXTS.EXE and

HOEXTR.EXE both need to run at both ends, so they must be

installed for the correct modem port etc on the Remote as well

as the host.

Once HOHOSTIN has installed these programs correctly on both

the host and the Remote, and HORZ.BAT, HOSZ.BAT and the

external protocol program (such as DSZ.EXE) are in some

directory on the path of both the host and Remote, file

HANDS ON  Remote Access   HO.DOC  manual      Page 32

transfer is quite simple:

To download a file from the host PC to the Remote PC, simply

enter the following command on the host PC:  HOSZ <filename>

...  and HO will automatically run HORZ on the Remote without

you doing anything!

To upload a file from the Remote PC to the host PC, first

ensure that the host PC is at the DOS command prompt.  Then

shell out of HO (using local F9).  Finally enter the following

command on the Remote PC:

HOSZ <filename> ...  and HOHOST will automatically run HORZ

on the host.

HORZ.BAT, HOSZ.BAT, HOEXTS.EXE and HOEXTR.EXE all pass

parameters on to the external protocol program, so parameters

other than file names can be used if desired.

HOEXTS.EXE and HOEXTR.EXE recognize one special parameter:

#portx#

which will substitute in the DSZ portx <base>,<IRQ> command.

This allows HORZ.BAT and HOSZ.BAT to remain unchanged even

when HANDS ON is installed for a different port.  See the

supplied HORZ.BAT and HOSZ.BAT for examples of this.

8.5 HOMODSET - modem setting

Usage:  HOMODSET [-fr] [filename]

HOMODSET will send all the "enclosed" commands in HOMODSET.DAT

(or in filename.DAT if specified) to the modem, waiting for an

"OK" response to each command.  This is useful to set up very

complicated parameters necessary for some of the more

sophisticated modems.  It assumes a modem that uses the Hayes

AT command set (at least at&f to restore factory defaults and

OK in reply to a good command).  Yes, you can use it to dial

the phone.

The -r command line option will cause HOMODSET to start with

an "at&f" command to reset the modem to factory defaults, just

in case a previous program has left the modem in a peculiar

state.  The "at&f" is sent before HOMODSET starts monitoring

for command echo or "OK" responses.

If a carrier is present when HOMODSET runs then it will exit

without doing anything unless the "-f" (for "force") command

line argument is present.  This allows HOMODSET to be run from

HANDS ON  Remote Access   HO.DOC  manual      Page 33

AUTOEXEC.BAT without disrupting an existing connection if the

host PC needs to be rebooted.

Note that HOMODSET is installed by HOHOSTIN, so if you want to

use HOMODSET on the Remote PC you will need to install the

host programs on the Remote PC (once installed, the other host

programs may be deleted, but they must be present for HOHOSTIN

to run).

Note also that you cannot use HOMODSET to set the modem to "no

echo" or "quiet" modes, since HOMODSET monitors that commands

are being echoed and that the modem is responding with "OK" to

each command.

8.6 HOCHECK - time monitor

HOCHECK checks to see if the current time is within a

specified range, and returns the DOS errorlevel set to 0 if

within the range and 1 if outside the range.  The following

code might appear in an AUTOEXEC.BAT file on a host computer:

HOCHECK 9:00:00 17:00:00

if not errorlevel 1 goto done

HOHOST

:done

The effect of this is to first check to see if the time is

between 9AM and 5PM.  If so, then errorlevel is set to 0, and

nothing further happens.  However, if the time is not between

9 and 5, then HOHOST will be run.

This has proved valuable when HOHOST must run on an office

computer outside normal working hours, but the computer must

be used normally during the day.  See also scenario A5 for a

further application.

8.7 HOASK - batch query tool

Usage:  HOASK <timeout> <prompt> <answers>

HOASK works much like a normal batch file enhancing ASK

program, except that it has a timeout.  This is useful when a

computer will usually be used as a HANDS ON host, but

sometimes must be used locally.  Rather than having to boot

from a different disk, the following code might appear in an

AUTOEXEC.BAT file on a host computer:

@echo off

path c:\\HO

HANDS ON  Remote Access   HO.DOC  manual      Page 34

HOASK 5 "Do you want to load HOHOST? (y/n)\a\a: " "yn"

if errorlevel 1 goto no

HOHOST

:no

The effect of this is to run HOHOST if the user presses the

'y' key or if no key has been pressed after 5 seconds.  If a

user is at the computer and answers 'n' then HOHOST will not

be loaded.  See scenario A5 for an example of this.

HOASK returns an errorlevel that depends on the <answers>

string:  the first letter in this string (or a timeout)

returns error level 0, the second returns error level 1 and so

on.  Case is not significant unless the <answers> string

contains the same letter in both upper and lower case.

HOASK may be distributed freely and separately.

8.8 1STHAND - multiple tools

You first use 1STHAND during the installation and

configuration, but it has ongoing uses as follows:

The most common ongoing use will likely be for call/connection

setup, including the various people and phone numbers.

The 1STHAND.exe program provides quick on-line access to the

manuals, a simple DOS menu, a simple equipment display, the

modem database and other configuration settings, and provides

easy access to the hohostin.exe and hohostc.exe programs.  It

has its own context sensitive user changeable help.

1STHAND accepts command line switches or parameters as in:

1STHAND call      Provides the overall "Phone" menu that

includes selecting, editing, and new entries.

1STHAND dial      Gives you a list of telephone numbers and

names for you to select from before it dials.

It actually uses a text file called PHONE.TEL

for the 'directory information'.

When you use "1sthand dial" you are presented with the list of

people and places to call that you have previously setup.  Selecting

one actually creates a batch file called "MAKECALL.BAT".  Run

makecall.bat to actually dial the number.  The HOMENU menu scheme

automates this for you.  Naturally, you are free to swap the whole

scheme over to your favourite menu program.  Its designed to be flexible.

HANDS ON  Remote Access   HO.DOC  manual      Page 35

1STHAND CALL      The overall phone menu.

1STHAND DIAL EDIT To edit the PHONE.TEL file and matching .HO file.

1STHAND DIAL NEW  To add new entries to PHONE.TEL and new .HO files.

1STHAND EDIT      To choose and edit text files.

1STHAND setup     Guides you through a directed Hands On setup.

1STHAND modem     Directly takes you to modem selection.

1STHAND xyz.doc   To browse a .DOC file enter its name as a parameter.

The file's name must end with .DOC.

1STHAND contains its own setup under the SETUP menu

selection.

9.0 SUPPORT and the ASP

Please consider reporting the circumstances, preferably in

written form, to technical support when you discover problems

with the software.

For technical support, either call (604) 433-5189, or write to

Suite 606 - 6455 Willingdon Avenue, Burnaby, British Columbia,

Canada, V5H 4E8.  Both registered and soon to be registered

users currently get the same support without any arbitrary

time limit.  You may also try leaving a Compuserve message at

75020, 2664, but be aware that messages rapidly 'scroll off'

Compuserve.  The best electronic message area (also available

from Compuserve) is via the internet at:

INTERNET:vic_williams@mindlink.bc.ca.

P. C. Softsmith Suite 606 6455 Willingdon Ave Burnaby, B. C.,

Canada V5H 4E4

portions Copyright (C)1994 P. C. Softsmith All Rights Reserved

ASP Introduction:

Please also carefully read the accompanying SHR_WARE.DOC

document.

"This program is produced by a member of the Association of

Shareware Professionals (ASP). ASP wants to make sure that the

shareware principle works for you. If you are unable to

resolve a shareware-related problem with an ASP member by

contacting the member directly, ASP may be able to help. The

ASP Ombudsman can help you resolve a dispute or problem with

an ASP member, but does not provide technical support for

HANDS ON  Remote Access   HO.DOC  manual      Page 36

members' products. Please write to the ASP Ombudsman at 545

Grover Road, Muskegon, MI 49442 or send a CompuServe message

via CompuServe Mail to ASP Ombudsman 70007,3538."

10.0 Warranty

THE DOCUMENTATION AND ASSOCIATED SOFTWARE ARE DISTRIBUTED AND

SOLD WITH NO WARRANTIES, EITHER EXPRESS OR IMPLIED, REGARDING

ITS MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE.

THE INFORMATION IN THIS DOCUMENTATION IS SUBJECT TO CHANGE

WITHOUT NOTICE AND DOES NOT REPRESENT A COMMITTMENT ON THE

PART OF THE VENDOR.

OTHER PRODUCT AND BRAND NAMES MENTIONED ARE TRADEMARKS OR

REGISTERED TRADEMARKS OF THEIR RESPECTIVE HOLDERS.  FOR

INFORMATION ONLY, PORTIONS OF THIS DOCUMENT AND THE SOFTWARE

ARE LICENSED COPYRIGHT MATERIAL COURTESY DOUGLAS THOMSON

APPENDIX A - Configuration Scenarios

The following are specific situations in which HANDS ON has

been used.  The descriptions of the situations, and the

resulting configurations of HANDS ON, are presented in detail.

It is hoped that seeing the way HANDS ON was installed in some

typical applications will help when it comes to installing HO

for your own situation.

Note that unless otherwise stated, all modems are assumed

initially to be in the HANDS ON default state, to be connected

to the PC via a 25-way straight-through modem cable, and to be

Hayes compatible.  To get a Hayes compatible modem into the

HANDS ON default state the following commands should be sent

to the modem using any normal communications program (or using

HOMODSET):

at &f      restore modem to "factory default"

at &c1     modem should report true carrier status

at &d2     modem should take notice of DTR to hang up 'phone

at x4      (optional) give detailed status messages etc

at &w      save current state for future use (the state is

saved in non-volatile RAM, so that this setup only needs to be

done once unless some other program interferes)

To ensure that HANDS ON is talking to the modem, I recommend

the following sequence (at both the host and the Remote PC):

HANDS ON  Remote Access   HO.DOC  manual      Page 37

1.  Run HOHOSTIN and select the COM port and IRQ line that are

connected to your modem (use the X option first to have

HOHOSTIN search your hardware for available ports and active

IRQ lines).

2.  Set the "baud" rate to the appropriate speed for your

modem.

3.  Save the configuration.

8.  Run HOMODSET. This not only sets the modem into a

reasonable state for HANDS ON, but in doing so it will confirm

(if everything is OK) or deny (if it reports an ERROR) that

you have indeed selected the right address and interrupt for

your modem.  There is no point in proceeding further until

HOMODSET will run and report everything OK.  Rerun HOHOSTIN

and change COM ports if all you get is ERRORS (or turn the

modem on, ... or see QUICK.DOC for more help).

A1.  Consultant/Client via Modem

Here the consultant is running a 2400 bps modem connected to

COM1, and the client (whom we will call Mary) is running a

similar modem also connected to COM1.  The consultant will be

performing the HOHOST installation for the client, and then

sending the client the installed version ready to run.

When the client wants help, she will call the consultant (a

normal voice call).  When both parties are ready HANDS ON must

take over the call.

First the consultant must run HOHOSTIN, and make the changes

necessary to achieve the following display:

HANDS ON Host 5.00 Installation   (C)1994  P. C. Softsmith

A. Select serial port (currently COM1)

B. Change password (currently "Fred's password")

C. Change modem initialization string (currently:

"")

D. Change modem dialing format string (currently:

"ata\r")

E. Select maximum "baud" rate (currently 2400)

F. Use modem CARRIER DETECT signal (currently YES)

G. Flow control (currently NONE)

H. Generate INT 9 on each key (currently NO)

I. Respond to modem RING signal (currently NO)

HANDS ON  Remote Access   HO.DOC  manual      Page 38

J. Reboot on loss of carrier (currently NO)

K. Synchronize "baud" rate (currently YES)

L. Send wakeup every 2 minutes (currently NO)

M. Private mode (currently NO)

N. Printer redirection (currently HOST)

O. Maximum call length (currently unlimited)

P. Create consistent .HO file

Q. Abandon changes

S. Save changes and leave installation program

Selection:

Refer to section 4 for the meaning of each option. Since both

modems run at the same speed it does not matter whether "baud"

rates get synchronized or not.  Since Mary will only run

HOHOST when she wishes to get help from the consultant, the

password is not important.  The modem dialing format string

simply answers the 'phone - or more accurately, since the

'phone will already be in use for the voice call, it causes

the modem to take over the call in "answer" mode.

Next the consultant selects option P from the main menu, and

enters the name of the configuration file he wishes to use for

Mary:

HANDS ON Host Program .HO File Name Selection

Edit existing string:

Mary

Finally the consultant exits HOHOSTIN by selecting option S to

save the changes.  The consultant can now send Mary a disk

with all the installed HANDS ON files on it (note that the

consultant should also send Mary an unaltered original copy of

the HANDS ON files in case Mary wants to pass them on to

anyone - see the LICENCE.DOC file for details).

Now the consultant needs to edit the file named Mary.HO to

make a few alterations (HOHOSTIN only provides a starting

point...).  In particular, the following line must be

changed:

""                Connect command for modem

to:

"at d\r"          Connect command for modem

This command tells the modem to go "off hook" and take over

HANDS ON  Remote Access   HO.DOC  manual      Page 39

the call in "originate" mode.

This completes the installation stage.

When Mary wants some help, she will run HOHOST and then 'phone

the consultant and explain the problem.  When the consultant

is ready, he will enter the command:  HO Mary and then

instruct Mary to enter the command:  HODIAL and hang up the

'phone as soon as she (Mary) hears the modem making a

"screaming" noise.  After giving these instructions (and thus

while Mary is still typing) the consultant will hang up his

'phone.  Mary will finish entering the HODIAL command (or more

likely selecting correct menu selection), and hang up when she

hears the screaming noise.  HANDS ON will then take over the

call, synchronize "baud" rates, exchange passwords, and give

the consultant control of Mary's computer.

When the consultant is finished, he will exit HO using option

3, which will cause the modem to hang up and terminate the

'phone call.

Mary can either leave HOHOST running (it will do no harm, and

uses only a little over 10K of memory), or else use HOREMOVE

to uninstall HOHOST.

A2 Remote/Shared via Modem

Here a remote user wishes to connect into a local area

network, and one node in the LAN is dedicated to this purpose.

The main problem is that since the LAN node will be

unattended, all it takes is one user to run a program which

"hangs" the computer, and then no-one will be able to dial in

until someone pushes the reset button on the node PC!

However, in this scenario there is a modem-controlled switch

box which turns on power to the node PC as soon as the 'phone

starts ringing, and leaves the power on until either a minute

or so has elapsed with no carrier being established, or until

the carrier is lost (which will happen when the remote user

hangs up).  This means there is no problem if a user "hangs"

the computer;  just hang up and then dial in again, and the

computer will have been powered down and come up again clean.

Since many users must share the remote access PC, the duration

of a session may need to be limited so that one user cannot

tie up the line for more than 30 minutes.

The node PC has a 9600 bps modem connected to COM2, and the

HANDS ON  Remote Access   HO.DOC  manual      Page 40

remote user is running a 2400 bps modem connected to COM1. The

node PC is equipped with a CGA color display card (with no

monitor required).  The remote PC is equipped with a Hercules

monochrome display.

The LAN manager will be performing the HOHOST installation for

the node PC, and then providing all the remote users with a

disk that is almost ready to go.

The 'phone number of the node PC is 123 4568.

First the LAN manager must run HOHOSTIN, and make the changes

necessary to achieve something like the following display:

HANDS ON Host 5.00 Installation  (C)1994 P. C. Softsmith

A. Select serial port (currently COM2)

B. Change password (currently "sdggsdjgxvwept9m,.mvikjdsdh")

C. Change modem initialization string (currently:

"\rata\r")

D. Change modem dialing format string (currently:

"\r%s\r")

E. Select maximum "baud" rate (currently 2400)

F. Use modem CARRIER DETECT signal (currently YES)

G. Flow control (currently NONE)

H. Generate INT 9 on each key (currently NO)

I. Respond to modem RING signal (currently NO)

J. Reboot on loss of carrier (currently NO)

K. Synchronize "baud" rate (currently YES)

L. Send wakeup every 2 minutes (currently NO)

M. Private mode (currently YES)

N. Printer redirection (currently Remote)

O. Maximum call length (currently 1800 s)

P. Create consistent .HO file

Q. Abandon changes

S. Save changes and leave installation program

Selection:

Refer to section 4 for the meaning of each option.  Note that

the modem initialization string answers the 'phone - this is

nice in the case of timed call charges, since the remote user

does not pay for the time it takes the PC to boot.  Since some

remote users will dial in at 2400 bps, HOHOST must be

installed for 2400;  but since other remote users will dial in

at 1200 bps it is essential that "baud" rates get

synchronized.  Since anyone could try to get into the LAN via

the node PC the password is very important - notice that it is

both long and quite random.  The LAN node PC is in a public

HANDS ON  Remote Access   HO.DOC  manual      Page 41

location, so the screen is blanked and keyboard locked for

privacy.  Printer output is sent back home to the users.  The

maximum session length allowed is set to 1800 seconds (30

minutes).

Next the LAN manager selects option P from the main menu, and

enters the name of the configuration file he wishes to use for

the node PC:

HANDS ON Host Program .HO File Name Selection

Edit existing string:

lan1

Finally the LAN manager exits HOHOSTIN by selecting option S

to save the changes.  He can now make up a disk with all the

installed HANDS ON files on it to give to the remote users,

but first he must edit the LAN1.HO file to install the node

PC's 'phone number.  This means changing

the line:   ""                Connect command for modem

to:         "at d 123 4567\r" Connect command for modem

so that when remote users run HO the number will be dialled

automatically.

Also, since modems more often use COM1 than COM2, the LAN

manager should probably edit the line:  02F8/IRQ3 (COMn or

base/interrupt) to become:  "COM1" (COMn or base/interrupt)

A2.3 Now the remote user needs to edit his personal copy of

the file named LAN1.HO to work at 1200 bps. The line

containing:  2400 "Baud" rate (typically 2400 or 19200) must

be changed to:  1200

This completes the installation stage.

When the remote user wishes to connect into the LAN, he will

enter the command:  HO LAN1

This will cause the modem to dial the node PC, which will

cause the modem switch box to turn on the power to the node

PC, which will boot and then run HOHOST, which will make the

node PC's modem answer the call.  HANDS ON will then

synchronize "baud" rates, exchange passwords, and give the

remote user control of the node PC.

When the remote user is finished, he will exit HO using option

3, which will cause the modem to hang up and terminate the

HANDS ON  Remote Access   HO.DOC  manual      Page 42

'phone call.  The node PC will then be powered down until the

next call.

A3 Laptop/Desktop via Direct Cable

A3.1 Here someone wishes to connect a laptop computer to a

desktop computer via a three-wire null modem cable. The reason

for this is that the desktop only has a 5.25" disk, and the

laptop only has a 3.5" disk, and it is necessary to transfer

files between the two systems.  The desktop computer is a

20MHz 80386, and the laptop is a 12MHz 80286.  The cable is

connected from COM1 on the desktop to COM1 on the laptop.  The

desktop has a VGA color display;  the laptop has a gas plasma

display that emulates EGA.

It does not matter which PC is the host, but for the sake of

this example we will assume the desk PC will be the host.

The laptop computer has its <Del> key right next to

<RightShift>, so the user does not wish to be using

<Ctrl-Alt-LeftShift-RightShift> for fear of accidentally

rebooting the computer. <Ctrl-Alt-LeftShift> seems a safer

combination.

First the user must run HOHOSTIN, and make the changes

necessary to achieve something like the following display:

HANDS ON Host 5.00 Installation   (C)1994 P. C. Softsmith

A. Select serial port (currently COM1)

B. Change password (currently "password")

C. Change modem initialization string (currently:

"")

D. Change modem dialing format string (currently:

"")

E. Select maximum "baud" rate (currently 38400)

F. Use modem CARRIER DETECT signal (currently NO)

G. Flow control (currently NONE)

H. Generate INT 9 on each key (currently NO)

I. Respond to modem RING signal (currently NO)

J. Reboot on loss of carrier (currently NO)

K. Synchronize "baud" rate (currently NO)

L. Send wakeup every 2 minutes (currently NO)

M. Private mode (currently NO)

N. Printer redirection (currently HOST)

O. Maximum call length (currently unlimited)

P. Create consistent .HO file

HANDS ON  Remote Access   HO.DOC  manual      Page 43

Q. Abandon changes

S. Save changes and leave installation program

Selection:

Refer to section 4 for the meaning of each option. Notice that

the "baud" rate is set to 38400 for fast file transfers

(experiment if you happen to find a failing system).

Obviously, the password is not important here.

Next the user selects option P from the main menu, and enters

the name of the configuration file he wishes to use for the

desktop:

HANDS ON Host Program .HO File Name Selection

Edit existing string:

desk

Finally the user exits HOHOSTIN by selecting option S to save

the changes.

Next the user needs to edit the file named DESK.HO to use the

shift key combination required for the laptop. The line

containing:

"LRCA"  Local command shifts (Left, Right, Ctrl, Alt)

must be changed to:

"LCA"   Local command shifts (Left, not Right, Ctrl, Alt)

The user can now copy the installed files onto a 5.25" floppy

disk, take the disk to a computer that has both 5.25" and 3.5"

inch drives, and copy the file onto a 3.5" disk for the

laptop.

This completes the installation stage.

When the user wishes to connect the two computers, he will

enter the command:  HOHOST on the desktop, and the command:

HO DESK on the laptop. This will cause HANDS ON to exchange

passwords and give the user control of the desktop computer

from the laptop. Files can now be transferred at almost 3.8K

per second!

When the user is finished, he will exit HO using option 3,

which will take the laptop back to normal, but leave HOHOST in

the desktop's memory (see A1.5 for further discussion of

HANDS ON  Remote Access   HO.DOC  manual      Page 44

this).

A4 Remote/Shared via Common Mini-computer

Here a remote user wishes to connect into a local area

network, and one node in the LAN is dedicated to this purpose.

There is no direct link between the PCs;  instead, both the

node PC and the remote user must log into the same

mini-computer and run a link program (such as HOFIFO) to

establish a connection via the mini-computer.

One problem is that since the LAN node will be unattended, all

it takes is one user to run a program which "hangs" the

computer, and then no more remote users will be able to

connect to the LAN until someone pushes the reset button on

the node PC!  (Note that HANDS ON does support rebooting the

host PC, but this can only work if interrupts are not disabled

and if the relevant code has not been overwritten. As a rule,

HANDS ON can get out of any situation that <Ctrl-Alt-Del>

could get out of, but if the hardware reset button was needed

HO would be helpless.)

Another potential problem is that if one user exits HO while

still logged in to the LAN, then the next user could pick up

the session!  This is avoided by having the link program send

a "reboot" command to the node PC when the link is broken.

HOFIFO cannot pass BREAKs from one end to the other;  instead,

BREAK is used to terminate the execution of HOFIFO. This means

that HO must not send a BREAK unless it wants to kill the

link.

To make life complicated, the mini-computer will automatically

disconnect any terminal that does no input or output for 5

minutes. However, the node PC must remain connected to the

mini-computer all day waiting for remote users to connect in.

The mini-computer links are set at 9600 bps, and cannot be

changed. Both computers are connected to the mini-computer via

COM1, and both computers are running color displays (however,

the dedicated node PC requires no monitor).

The LAN manager will be performing the HOHOST installation for

the node PC, and then providing all the remote users with a

disk that is almost ready to go.

First the LAN manager must run HOHOSTIN, and make the changes

HANDS ON  Remote Access   HO.DOC  manual      Page 45

necessary to achieve something like the following display:

HANDS ON Host 5.00 Installation   (C)1994  P. C. Softsmith

A. Select serial port (currently COM1)

B. Change password (currently "sdfddffgjkiou77hgsduh875sdh")

C. Change modem initialization string (currently:

"")

D. Change modem dialing format string (currently:

"")

E. Select maximum "baud" rate (currently 9600)

F. Use modem CARRIER DETECT signal (currently NO)

G. Flow control (currently NONE)

H. Generate INT 9 on each key (currently NO)

I. Respond to modem RING signal (currently NO)

J. Reboot on loss of carrier (currently NO)

K. Synchronize "baud" rate (currently NO)

L. Send wakeup every 2 minutes (currently YES)

M. Private mode (currently YES)

N. Printer redirection (currently Remote)

O. Maximum call length (currently unlimited)

P. Create consistent .HO file

Q. Abandon changes

S. Save changes and leave installation program

Selection:

Refer to section 4 for the meaning of each option. The

password is not so critical here, since a user must be able to

log into the mini-computer before getting through to the node

PC. The mini-computer line will not provide any carrier

information, so this must be ignored. "Baud" rates must NOT be

synchronized, since a NULL character transmitted at 2400 bps

would be received as a BREAK by the mini-computer at 9600

bps!  To avoid the node PC getting logged out, HOHOST must

send a NULL character to "wake up" the mini-computer every few

minutes. This will be ignored by HO. Printer output is

redirected back to the users. The LAN PC is run with screen

blank and keyboard dead for added privacy and security.

Next the LAN manager selects option P from the main menu, and

enters the name of the configuration file he wishes to use for

the node PC:

HANDS ON Host Program .HO File Name Selection

Edit existing string:

lan2

HANDS ON  Remote Access   HO.DOC  manual      Page 46

Finally the LAN manager exits HOHOSTIN by selecting option S

to save the changes. He can now make up a disk with all the

installed HANDS ON files on it to give to the remote users.

Due to the way HOFIFO buffers input the delay between sending

keys needs to be 165 ms to avoid losing characters!  (There

could be another reason, and we would appreciate an update on

this) Therefore, the line containing:

56    Time to wait between "typing" keys (ms)

must be changed to:

165   Time to wait between "typing" keys (ms)

Also, since we are using a mini-computer link, the line

containing:

"DIRECT"    Type of link (MODEM, DIRECT, or OTHER)

must be changed to:

"OTHER"     Type of link (MODEM, DIRECT, or OTHER)

Finally, HO is capable of automating the mini-computer login

process. If this is desired, then something like the following

lines must be added at the end of the file:

"DISCONNECTED"      "\D250\r"      "\r"    5000

"Enter RESOURCE ? " "\D100G9000\r" "\r"    5000

"CONNECTED TO"    ""               ""      5000

"login: "     "\D100doug\r"  "\B\r"         500

"Password:"   "\D100password\r"    "\r"   10000

"Selection: " "\D100d\r"     ""           40000

"/users/doug>""\D100HOFIFO lan2\r" ""     40000

"and run HO"  "\D1000"       ""           40000

The exact strings will depend on the mini-computer. This

example (apart from the password) works for the author.

This completes the installation stage.

When the remote user wishes to connect into the LAN, he will

run HO:

HO LAN2 which will automatically log into the mini-computer

and run HOFIFO to connect to the node PC (in this case lan2).

HANDS ON will then exchange passwords and give the remote user

control of the node PC.

When the remote user is finished, he will exit HO using option

3, which will send a BREAK to the mini-computer. This will

terminate the HOFIFO program which the remote user was

running. The HOFIFO program that the node PC was running will

detect that this has happened, and send a HANDS ON reboot

HANDS ON  Remote Access   HO.DOC  manual      Page 47

command to the node PC to ensure that the next user cannot

pick up anything the previous user left running. The node PC's

HOFIFO program will then prepare for the next remote user. The

node PC will complete rebooting, and run HOHOST from

AUTOEXEC.BAT ready for the next remote login.

A5 Home/Office via Modem

Here a user wishes to dial in from home and take over the PC

on his desk at work. The office PC has a USR Courier

V32bis/V42bis modem connected to COM2, and the home PC has an

identical modem connected to COM1.

One problem is that since the office PC will be unattended,

all it takes is for the user to run a program which "hangs"

the computer, and that will be the end of the office PC until

the user goes back to work and presses the reset button!

(Note that HANDS ON does support rebooting the host PC, but

this can only work if interrupts are not disabled and if the

relevant code has not been overwritten. As a rule, HANDS ON

can get out of any situation that <Ctrl-Alt-Del> could get out

of, but if the hardware reset button was needed HO would be

helpless.)

Since the user is putting in overtime working at night without

getting paid for it, it seems reasonable that the office

'phone should at least pay for the call. However, how can the

office computer know when to call the user at home?  One

possibility would be to set up a batch file to run HODIAL at a

predetermined time. However, it would be better if the user

could choose any convenient time. HANDS ON can achieve this by

monitoring the RING signal from the modem, and responding

appropriately.  The basic idea is that the user will dial the

office number, and then hang up as soon as the 'phone starts

ringing. HANDS ON will detect the 'phone ringing, and respond

by calling the user at home.

Since the user does not want to be bothered by nuisance calls

from the office computer, HANDS ON needs to be quite selective

about the ring patterns it will respond to. Too short a ring

may mean a telephone company routine line test. Too long a

ring will mean a person trying to ring the office.

The 'phone number of the office PC is 123 4568.

The 'phone number of the home PC is 765 4321.

First the user must run HOHOSTIN, and make the changes

HANDS ON  Remote Access   HO.DOC  manual      Page 48

necessary to achieve something like the following display:

Hands On  Host 5.00 Installation   (C) 1994  P. C. Softsmith

Select serial port (currently COM2)

Change password (currently "fhkjeuyibn45938eh8fdfff631wjds874dffd")

Change modem initialization string (currently:

"at &B1 &H1 S0=0\r")

Change modem dialing format string (currently:

"\r%s\r")

Select maximum "baud" rate (currently 38400)

Use modem CARRIER DETECT signal (currently YES)

Flow control (currently CTS)

Generate INT 9 on each key (currently NO)

Respond to modem RING signal (currently YES)--|- 1 too short  220 ms

Reboot on loss of carrier (currently YES)     |- 2 too long  5000 ms

Synchronize "baud" rate (currently NO)        |- 3 delay    10000 ms

Send wakeup every 2 minutes (currently NO)    |- 4 reset    10000 ms

Private mode (currently YES)                  |- 5 send to modem

Printer redirection (currently Remote)       |- 6 response

Maximum call length (currently unlimited)         "ATDT 765 4321\r"

Create consistent .HO file

Abandon changes

Save changes and leave installation program

Selection:

Refer to section 4 for the meaning of each option.

Since anyone could try to get into the office PC the password

is important - notice that it is both long and quite random.

The modem initialisation string changes a USR V32bis modem

from its factory default settings to the following:

&B1 Fixed DTE/DCE rate. This means that the modem will always

talk to the PC at 38400 bps, regardless of what kind of

telephone connection is made.

&H1 Hardware flow control when transmitting data to the modem

(no flow control is needed in the other direction, since HANDS

ON can keep up with data arriving at 38400 bps).

S0=0 The office phone should never answer calls.

Since the modem can transmit 1700 bytes per second, and V42bis

compression may improve this to around 3400 bytes per second

for things like screen updates, the "baud" rate should be at

least 34000 bps, and 38400 is the lowest speed over 34000 that

HANDS ON  Remote Access   HO.DOC  manual      Page 49

the modems can accommodate.

It is essential that HOHOST takes notice of the CD signal,

since the host computer will be automatically rebooted if the

carrier is lost (see below).

Flow control is essential, since the PC will be sending data

to the modem at 38400 bps and the modem can only guarantee to

get rid of it at about 17000 bps!  Hardware flow control (CTS)

is selected to match the modem configuration.

The inter-character gap should be set to a fairly long time;

the modems support error correction, so HOHOST should be

patient and give the modems time to sort out problems

introduced by line noise. 5-10 seconds is a reasonable time:

HANDS ON Host Program Flow Control Selection

0. Completed, no (further) change

1. No flow control

=> 2. Hardware (RTS/CTS) flow control

3. Software (XON/XOFF) flow control

8. Both hardware and software flow control

8. Set inter-character gap time (currently 5000 ms)

Selection:

The host computer is to monitor the 'phone ringing and respond

accordingly (again, see below).

"Baud" rate synchronization is not appropriate here, since the

DTE/DCE speed is locked.

The host is set in PRIVATE mode so that no-one walking through

the office can read what is on the screen or type commands on

the keyboard (very useful if the user will be logging into a

LAN as SUPERVISOR!).

Printer output is redirected to the Remote PC, so that any

printout comes out at home rather than in the office.

Next the user selects option P from the main menu, and enters

the name of the configuration file he wishes to use for the

office:

HANDS ON  Remote Access   HO.DOC  manual      Page 50

HANDS ON Host Program .HO File Name Selection

Edit existing string:

office

Finally the user exits HOHOSTIN by selecting option S to save

the changes. He can now make up a disk with all the installed

HANDS ON files on it to take home.

Before leaving the office, the user must ensure that

AUTOEXEC.BAT has the directory containing all the HANDS ON

files on the path, and that HOHOST is run whenever the system

boots. However, when the user is at the office he will not

want HOHOST loaded, so AUTOEXEC.BAT should contain something

like:

prompt $p$g

path c:\HO

HOASK 5 "Do you want to load HOHOST? (y/n): \a\a" "yn"

if errorlevel 1 goto no

HOHOST

:no

This will ask the user at the host PC whether or not to load

HOHOST (the \a makes HOASK beep), and if the user does not

type anything within 5 seconds HOHOST will be loaded anyway.

If the user is at the office, then typing 'n' will prevent

HOHOST from loading.

At home the user needs to edit the file named OFFICE.HO to

install the office PC's 'phone number. This means changing the

line:

""          Connect command for modem

to:

"at d 123 4567\r" Connect command for modem

so that when the user runs "HO OFFICE" the number will be

dialled automatically.

Next, since the home modem is on COM1 the line:

02F8/IRQ3   Communications port (COMn or base/interrupt)

must become:

"COM1"  Communications port (COMn or base/interrupt)

Also, since the modems support error correction, and since

line noise can introduce a significant gap in the transmitted

data at any time, the timeouts should be made fairly generous

by changing the lines:

HANDS ON  Remote Access   HO.DOC  manual      Page 51

3000  Time to wait for first char before timeout (ms)

500   Time to wait for subsequent chars in block (ms)

to:

10000 Time to wait for first char before timeout (ms)

10000 Time to wait for subsequent chars in block (ms)

Finally, HO should respond to the modem ringing by answering

the 'phone.  This means adding an extra line at the end of the

file containing:

"RING"  "\D250ata\r"    ""   40000

(Note the \D250 to wait a little bit for the modem to finish

sending the carriage return and line feed after RING before

sending the answer command. With some modems this may not be

necessary;  with others, a different delay may work better. If

the answer command is ignored, then try a different delay.)

This completes the installation stage.

Before leaving the office, the user must remember to reboot

the PC (thus loading HOHOST) and leave it turned on!  However,

it would be a good idea to switch the monitor off.  Note that

you can leave the HOMENU screen saver running with HOHOST in

memory and readily dial in.

When the user is ready to dial into the office PC from home,

he will enter the command:  HO OFFICE This will cause the

modem to dial the office PC. The user must then listen to the

modem and wait until he hears the 'phone ring once. As soon as

he hears this, he must press any key (say the space bar) to

terminate the call. HOHOST will start a timer going when the

'phone rings, and then wait 10 seconds, keeping track of the

total time the 'phone has spent ringing. If, during the 10

seconds, the 'phone was ringing for more than 0.2 seconds but

less than 5 seconds, then HOHOST will send the dial command

(ATDT 765 4321) to the modem, causing the office modem to ring

home. When the user's 'phone rings, the modem will send the

message:

RING to the home PC, and HO will trigger on this to respond

to the modem with:

ATA which will answer the 'phone. HANDS ON will then

exchange passwords and give the user control of the office

PC.

When the user is finished, he will exit HO using option 3,

which will cause the modem to hang up and terminate the 'phone

call. The office PC will then automatically reboot, ready for

HANDS ON  Remote Access   HO.DOC  manual      Page 52

the user to call in again.

APPENDIX B - Sample File Transfer Session

In this example, we will assume that a good link has been

established as described in scenario A2. The user (now Fred)

has logged into the LAN from home, and wishes to transfer all

the .DBF xbase datafiles from the LAN directories:

F:\USERDATA\FRED\TEST

F:\USERDATA\FRED\UTILS

F:\USERDATA\FRED\PROBLEM

to the corresponding directories on his home PC:

C:\PROGRAMS\TEST

C:\PROGRAMS\UTILS

C:\PROGRAMS\PROBLEM

The current working directory on the LAN PC is

F:\USERDATA\FRED. The current working directory on the home PC

is C:\HO.

First, the user needs to make C:\PROGRAMS the current working

directory on his home PC. To do this, he must hold down Ctrl,

Alt, LeftShift and RightShift all at the same time!  This will

bring up a screen similar to:

HANDS ON   (C) 1994  P. C. Softsmith

Key         Effect

===         ======

<F2>      - simulate carrier - not used with MODEM link

<Ctrl-F2> - simulate NO carrier - not used with MODEM link

<F6>      - send break, restart session

<F8>      - perform warm boot on host

<F9>      - shell to DOS

<F10>     - exit HANDS ON

<Alt-X>   - exit HANDS ON

<Home>    - refresh display from HOST

<PgUp>    - upload file using OMI protocol

<PgDn>    - download file using OMI protocol

<ESC>     - do nothing

Press key:

Then he should press <F9> to shell to DOS, and enter the

command:

HANDS ON  Remote Access   HO.DOC  manual      Page 53

CD \PROGRAMS to make C:\PROGRAMS the current working

directory. Then he can enter the command:

EXIT to return to HO.

Next he needs to start the download. To do this he must again

hold down Ctrl, Alt, LeftShift and RightShift, and then press

<PgDn>. Another box will appear:

Name(s) of file(s) to be downloaded (use DOS wildcards):

WARNING: Files with the same names on this PC will be overwritten!

and he must enter the relative paths and wildcards:

Name(s) of file(s) to be downloaded (use DOS wildcards):

WARNING: Files with the same names on this PC will be overwritten!

TEST\*.DBF UTILS\*.DBF PROBLEM\*.DBF

If there were some problem (for example, if OMIPUT.EXE were not

on the path of the LAN PC), then the following could be displayed:

HANDS ON Error:

no ready marker from host

Type <ESC> to continue:

However, normally the download will then commence, leaving

the following display:

File Transfer in Progress...

File:       test\payroll.c

Error:

Throughput: 233.6 cps

You may:

1. Abort the file transfer

2. Shell to DOS

Selection:

HANDS ON  Remote Access   HO.DOC  manual      Page 54

The number of bytes remaining to be transfered is displayed in

the top right corner of the screen. The most recent error will

be reported in the appropriate line of the box, and the

throughput of the transfer (in characters per second) may help

to relieve the boredom while waiting for the transfer to

complete!  The box also indicates which file is currently

being transfered.

At this stage, the user can either wait for the transfer to

finish, or else select option 2 from the menu and shell to

DOS. In this case, he can use his home PC for other jobs while

the transfer takes place in the background. HO will continue

to display the number of bytes remaining in the top right

corner until the transfer is completed, at which time the

number in the top right corner will become the string

"Completed" (or, possibly, "Doubtful" if one or more files

could not be created on the home PC). However, the user must

be careful not to do things like delete the file that is being

downloaded!

While shelled out to DOS in this way, the user may exit back

into HO at any time. If the file transfer is still in

progress, then the file transfer window will still be there;

otherwise, the exit will leave HO displaying the normal Remote

of the host PC's display.

APPENDIX C - Modem Cables

A full 25-wire straight-through male-to-female RS232 cable is

recommended for HANDS ON (since there is less room for error

in the cable construction/selection).  Contact us if you need

details for possible subset cables (some information has been

chopped out of the manual because most people went cross eyed

reading it).

APPENDIX D - DESQview Compatibility

HO is "DESQview-Aware" and runs nicely in a small DESQview

window.  HOHOST needs to put keys into the BIOS keyboard

buffer, and this means that what is typed at the remote end

cannot be distinguished by DESQview from what is typed at the

real keyboard at the host end. The net effect is that there is

no point in trying to run HOHOST in a DESQview window, and so

HOHOST makes no attempt to be "DESQview-Aware". Also, DESQview

is activated by "tapping" the ALT key, but HOHOST has no way

HANDS ON  Remote Access   HO.DOC  manual      Page 55

to make this appear to happen, hence DESQview cannot be

activated remotely.  HOHOST and DESQview really don't mix!  We

have not tested Novell DOS.

For anyone wishing to set up DESQview for HANDS ON, these

settings work:

1  Change a Program

Change a Program

Program Name............: HO

Keys to Use on Open Menu: HO  Memory Size (in K):  70

Program...: C:\HO\HO

Parameters: REMOTE

Directory.: \HO

Options:

Writes text directly to screen.....: [N]

Displays graphics information......: [N]

Virtualize text/graphics (Y,N,T)...: [N]

Uses serial ports (Y,N,1,2)........: [1]

Requires floppy diskette...........: [N]

Press F1 for advanced options   Press <-Y when you are DONE

and advanced options:

1  Change a Program

Change a Program Advanced Options

System Memory (in K):  0    Maximum Program Mem Size (in K)..:  70

Script Buffer Size:  100    Maximum Expanded Mem Size (in K):    0

Text Pages: 1  Graphics Pages: 0  Initial Mode:  Interrs: 00 to 2F

Window Position:

Maximum Height: 25     Start Height: 25     Start Row...:  1

Maximum Width.: 80     Start Width.: 80     Start Column:  1

Shared Program

Pathname..:

HANDS ON  Remote Access   HO.DOC  manual      Page 56

Data......:

Close on exit (Y,N,blank).: [Y]  Uses its own colors...........: [Y]

Allow Close Window command: [N]  Runs in background (Y,N,blank): [Y]

Uses math coprocessor.....: [N]  Keyboard conflict (0-4).......: [0]

Share CPU when foreground.: [Y]  Share EGA when foreground/....: [Y]

Can be swapped out (Y,N, ): [N]  Protection level (0-3)........: [0]

Press F1 for standard options        Press <-Y when you are DONE

When running under DESQview, the "shell to DOS" option during

a file transfer is disabled, since DESQview does not permit

interrupt handlers to call DOS and BIOS functions (necessary

for reading and writing files).  However, this is not a

serious limitation under DESQview, since DESQview will allow

you to run multiple simultaneous programs.

The normal "shell to DOS" option may be used under DESQview,

but in this case more than 70K of memory will need to be

allocated - with 4DOS, it has been reported that at least 150K

is required!

APPENDIX E - Acknowledgments

This product has been developed from TeleReplica with the

co-operation of the author, and a proportion of each

registration is being forwarded to the primary author Douglas

Thomson.

The following individuals in particular have had a significant

impact on the development of this product:

Mr Douglas Thomson, Monash University College, Gippsland, Australia

Mr Lindsay Smith, Monash University College, Gippsland, Australia

Prof. Timo Salmi, University of Vaasa, Finland

APPENDIX F - ShareWare Vendor Notice (if any)

If you are a shareware vendor, please feel free to add your

your standard notice below.