550RESET.DOC  formatted Wed  10-19-1994  15:11:27

This program clears the 16550A's FIFO to prevent system hangs

when "Super I/O" cards with SMC665 or SMC666 chips are used

with communications programs that enable the 16550A UART. Just

run this program before you run your favorite communications

program and you should *not* experience further system hangs.

NOTE:  If you have an external device connect it and turn it

on BEFORE you run this program.

The specific problem is that if you output a 0x7 to the FIFO

control register of these chips while there is data in the

FIFO the chip will go into a state where the Receive Ready

flag (bit 0 in the LSR) remains high NO MATTER HOW MANY TIMES

YOU READ THE RECEIVER BUFFER REGISTER.  If your communication

program is unfortunate enough to enable interrupts while the

UART is in this condition your system will lockup, and require

a hardware reset or power cycle to restore operation.

Needless to say this is inconvenient!

I discovered this problem when I upgraded my "SUPER I/O card"

to a "SIDE jr L" vlb card.  While the features are great I

immediately ran into problem with EVERY program I have that

uses serial I/O.  I found that I could only execute Windows

programs that used serial I/O once, the whole system locked up

the second time I ran the program.  This occurred with several

different Windows programs.  Searching the Microsoft Knowledge

base yielded the answer... an updated SERIAL.386 driver

(search for WG1001 for the new driver).  Unfortunately the new

Windows driver did nothing for DOS programs I use regularly

such as Qmodem and Telix, that is why I wrote this program.

The Microsoft writeup provided enough information on the

problem for this program to be written.

Comments are welcome:

Skip Hansen WB6YMH.

internet: wb6ymh@amsat.org

BBS: (310) 541-2503

Packet: WB6YMH@WB6YMH.#SOCAL.CA.USA

If you find this program of value I am pleased, send no cash!