From: Dave Hershberger.

I have noticed and found a fix for a problem which occurs when

you use PROCOMM on COM3 and/or COM4. COM3 and COM4, when used

with PROCOMM 2.4.2, share the same interrups as COM1 and COM2.

That is, COM1 and COM3 share interrupt IRQ4 while COM2 and COM4

share IRQ3. This is fine as long as all programs which use the

COM ports tell the UART to "shut up" and get off the interrupt

bus before the programs terminate. Otherwise, bus contention may

occur. Two UARTs can both be tri-stated "on" trying to drive the

same interrupt. I have had the problem using PROCOMM on COM3 and

a Logitech mouse on COM1 with Generic Cadd. After using COM3 with

PROCOMM, for instance, then sometimes I could not run Generic

Cadd using the mouse on COM1. It was as though COM1 was not there

anymore. Similarly, sometimes after using Generic Cadd using the

mouse on COM1, COM3 did not seem to be there using PROCOMM.

CNTRL-ALT-DEL is not sufficient to clear the problem. The only

way to clear the bus contention was to turn the power off! After

conferring with a friend (Jim Michener, who gets credit for

writing the "RS232OFF" program), a solution was found. The

RS232OFF program simply sends a "shut up" command to all four

UARTs. The simple ten-line "c" source code as well as the

compiled .EXE files are included. If you have programs which

alternately use COM1/COM3 or COM2/COM3, just run RS232OFF between

them.