Personal Communications Library

For the C Language

(PCL4C)

REFERENCE MANUAL

Version 5.0

November 20, 1995

This software is provided as-is.

There are no warranties, expressed or implied.

Copyright (C) 1995

All rights reserved

MarshallSoft Computing, Inc.

Post Office Box 4543

Huntsville AL 35815

Voice : 205-881-4630

FAX : 205|880|0925

BBS : 205-880-9748

email : msc@traveller.com

Anon FTP : ftp.traveller.com  /pub/users/msc

web : www.traveller.com/~msc/

_______

____|__     |                (R)

--+       |    +-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

--+--+   |   +---------------------

|___|___|    MEMBER

PCL4C Reference Manual                                              Page 1

C O N T E N T S

Chapter                                     Page

Table of Contents.............................2

Introduction..................................3

SioBaud....................................4

SioBrkKey..................................4

SioBrkSig..................................5

SioCTS.....................................5

SioDCD.....................................6

SioDelay...................................6

SioDone....................................7

SioDSR.....................................7

SioDTR.....................................8

SioError...................................8

SioFIFO....................................9

SioFlow....................................9

SioGetc...................................10

SioGetDiv.................................10

SioGets...................................11

SioInfo...................................11

SioIRQ....................................12

SioLine...................................12

SioLoopBack...............................13

SioModem..................................13

SioParms..................................14

SioPorts..................................14

SioPutc...................................15

SioPuts...................................15

SioRead...................................16

SioReset..................................16

SioRI.....................................17

SioRTS....................................17

SioRxBuf..................................18

SioRxClear................................18

SioRxQue..................................19

SioTimer..................................19

SioTxBuf..................................20

SioTxClear................................20

SioTxFlush................................21

SioTxQue..................................21

SioUART...................................22

SioUnGetc.................................22

Function Summary.............................23

Error Code Summary...........................24

Code Examples................................25

PCL4C Reference Manual                                              Page 2

Introduction

This manual lists all of the  PCL4C  functions  in  alphabetical  order.  Every

library function will return a value as follows:

1.   Negative  values  for error conditions. See last page of this manual for a

list of error values and their meanings.

2.  Non-negative values when returning data (eg: SioLine).

3.  Zero otherwise.

When debugging an application, be sure to test all return values. Use  SioError

to print the associated text for errors.

Example Code Segment

+---------------------------------------------------------------+

| int Code;                 /* MUST be 'int', not 'char' */     |

|                                                               |

| Code = SioFunction( );    /* any PCL4C function */            |

| if(Code<0)                                                    |

|   {/* error returned */                                       |

|    SioError(Code);        /* SioError prints error text */    |

|    SioDone(Port);         /* always call SioDone last */      |

|    exit(1);                                                   |

|   }                                                           |

| /* no errors */                                               |

| ...your application code...                                   |

|                                                               |

+---------------------------------------------------------------+

For  more  examples,  examine each of the example programs provided (MINIMAL.C,

SIMPLE.C, LOGIN.C, DOOR.C,  SPAWN.C,  SELFTEST.C,  etc.)  in  the  distribution

archive.

You may also wish to examine the TERM.C example program in the protocol library

(PPL4C).

Refer to the User's Manual (PCL4C.USR) for additional information.

PCL4C Reference Manual                                              Page 3

+-------------+---------------------------------------------------------------+

|   SioBaud   |  Sets the baud rate of the selected port.                     |

+-------------+---------------------------------------------------------------+

Syntax    int SioBaud(int Port,int BaudCode)

/* Port:      Port selected (COM1 - COM16) */

/* BaudCode:  Baud code */

Remarks  The SioBaud function sets the baud rate without resetting  the  port.

It is used to change the baud rate after calling SioReset.

Code   Rate    Name             Code   Rate   Name

0     300    Baud300            5    9600   Baud9600

1     600    Baud600            6   19200   Baud19200

2    1200    Baud1200           7   38400   Baud38400

3    2400    Baud2400           8   57600   Baud57600

4    4800    Baud4800           9  115200   Baud115200

Returns   -4 : No such port. Expect 0 to MaxPort.

-11 : Bad baud rate code. See above code values.

See Also  SioReset

+-------------+------------+-------------------+------------------------------+

|  SioBrkKey  |  Return non|zero if the Control|Break key was pressed.        |

+-------------+------------+-------------------+------------------------------+

Syntax  int SioBrkKey(void)

Remarks  The  SioBrkKey function  returns  a  TRUE  value  (non-zero)  if  the

Control-BREAK key was pressed, else it returns a zero.  Use SioBrkKey

as a safety exit from a polling loop.

Returns  -1 : Control-BREAK was pressed.

0 : Control-BREAK was NOT pressed.

See Also  SioBrkSig

PCL4C Reference Manual                                              Page 4

+-------------+------------------------------------------------------------+

|  SioBrkSig  |  Asserts, cancels, or detects BREAK signal.                |

+-------------+------------------------------------------------------------+

Syntax  int SioBrkSig(int Port,char Cmd)

/* Port:  Port selected (COM1 thru COM20) */

/*  Cmd:  ASSERT ('A'), CANCEL ('C'), or DETECT ('D')*/

Remarks  The SioBrkSig function controls the BREAK  bit  in  the  line  status

register. The legal commands are:

ASSERT_BREAK ('A') : to assert BREAK

CANCEL_BREAK ('C') : to cancel BREAK

DETECT_BREAK ('D') : to detect BREAK

ASSERT_BREAK,  CANCEL_BREAK, and DETECT_BREAK are defined in PCL4C.H.

SioBreak  is often used by one side to signal the other than an error

condition has occurred.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-6 : Illegal command. Expected 'A', 'C', or 'D'.

>0 : BREAK signal detected (DETECT command only)

See Also  SioBrkKey

+-------------+---------------------------------------------------------------+

|   SioCTS    |  Reads the Clear to Send (CTS) modem status bit.              |

+-------------+---------------------------------------------------------------+

Syntax  int SioCTS(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioCTS function is used to read the Clear  to  Send  (CTS)  modem

status bit.

The  CTS  line  is  used by some error correcting modems to implement

hardware flow control.  CTS is dropped by the  modem  to  signal  the

computer  not  to  send  data and is raised to signal the computer to

continue.

Refer to the User's Manual for a discussion of flow control.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : CTS is clear.

>0 : CTS is set.

See Also  SioFlow, SioDSR, SioRI, SioDCD, and SioModem.

PCL4C Reference Manual                                              Page 5

+-------------+---------------------------------------------------------------+

|   SioDCD    |  Reads the Data Carrier Detect (DCD) modem staus bit.         |

+-------------+---------------------------------------------------------------+

Syntax  int SioDCD(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioDCD function is used to read the  Data  Carrier  Detect  (DCD)

modem status bit. Also see SioModem.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : DCD is clear.

>0 : DCD is set.

See Also  SioDSR, SioCTS, SioRI, and SioModem.

+-------------+---------------------------------------------------------------+

|   SioDelay  |  Delays one or more timer tics.                               |

+-------------+---------------------------------------------------------------+

Syntax  int SioDelay(int Tics)

/* Tics: # timer tics to delay */

Remarks  The SioDelay function is used to delay one or more timer tics,  where

each  timer  tic  is  approximately 55 milliseconds (18.2 tics to the

second). Be carefull calling SioDelay since it will not return  until

the specified delay has occurred.

Returns  zero.

See Also  SioTimer

PCL4C Reference Manual                                              Page 6

+-------------+---------------------------------------------------------------+

|   SioDone   |  Terminates further serial processing.                        |

+-------------+---------------------------------------------------------------+

Syntax  int SioDone(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The  SioDone function terminates further serial  processing.  SioDone

MUST be called before exiting your application so that interrupts can

be  restored  to  their original state.  Failure to do this can crash

the operating system.  If you forget to call SioDone before  exiting,

be  sure  to  re-boot  your  computer.   You can call SioDone even if

SioReset has not been called, so it is good practice to  always  call

SioDone before exiting your application.

Also note that SioDone de-references the transmit and receive buffers

set  up  by  SioRxBuf  and  SioTxBuf.   However, it does NOT free the

transmit and receive buffers.   It  is  good  practice  to  free  the

transmit  and  receive  buffers  (after calling SioDone) if they were

allocated by malloc (or fmalloc).

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also  SioReset.

+-------------+---------------------------------------------------------------+

|    SioDSR   |  Reads the Data Set Ready (DSR) modem status bit.             |

+-------------+---------------------------------------------------------------+

Syntax  int SioDSR(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The  SioDSR function is used to read the Data Set Ready  (DSR)  modem

status bit. The DSR line is typically used by the serial device (such

as a modem) to indicate that it is connected.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : DSR is clear.

>0 : DSR is set

See Also  SioCTS, SioRI, SioDCD, and SioModem

PCL4C Reference Manual                                              Page 7

+-------------+---------------------------------------------------------------+

|    SioDTR   |  Set, clear, or read Data Terminal Ready (DTR) status bit.    |

+-------------+---------------------------------------------------------------+

Syntax  int SioDTR(int Port,char Cmd)

/* Port: Port selected (COM1 thru COM20) */

/* Cmd:  DTR command (SET, CLEAR, or READ) */

Remarks  The SioDTR function controls the Data Terminal Ready (DTR) bit in the

modem  control  register.   The  DTR  line  is typically used by your

program to tell the serial device that you are connected. DTR  should

always  be  set when communicating with a modem. Commands (defined in

PCL4C.H) are:

SET_LINE ('S') : to set DTR (ON)

CLEAR_LINE ('C') : to clear DTR (OFF)

READ_LINE ('R') : to read DTR

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-5 : Not one of 'S', 'C', or 'R'.

0 : DTR is OFF (READ_LINE Command).

>0 : DTR is ON (READ_LINE Command).

See Also  SioRTS.

+-------------+---------------------------------------------------------------+

|   SioError  |  Displays error in text.                                      |

+-------------+---------------------------------------------------------------+

Syntax  int SioError(int Code)

/* Code: PCL4C error code */

Remarks  The SioError function displays the error in text corresponding to the

error  code  returned  from a PCL4C function. During development of a

communications application, it is a good idea to always  test  return

codes, and print out their descriptions with SioError.

Returns  zero.

PCL4C Reference Manual                                              Page 8

+-------------+---------------------------------------------------------------+

|   SioFIFO   |  Sets the FIFO trigger level (16550 UART only).               |

+-------------+---------------------------------------------------------------+

Syntax  int SioFIFO(int Port,int LevelCode)

/* Port:      Port selected (COM1 thru COM20) */

/* LevelCode: FIFO level code (see below) */

Remarks  The  SioFIFO  function  is  used to enable both transmit  and receive

FIFOs for 16550 UARTS, and to set the trigger level at which  receive

interrupts are generated.

For  example, if the FIFO level is set to 8, then the 16550 UART will

not generate an interrupt until 8  bytes  have  been  received.  This

reduces   the  number  of  interrupts  generated  and  allows  faster

processing with slower machines or when running simultaneous ports.

In order to test if your port is a 16550 UART, call  SioFIFO  with  a

LevelCode  of other than FIFO_OFF. SioFIFO can be called for the 8250

and 16450 UART without ill effect.

Code  PCL4C.H Name  Trigger Level

-1      FIFO_OFF     Disable FIFO

0      LEVEL_1      1 byte

1      LEVEL_4      4 bytes

2      LEVEL_8      8 bytes

3      LEVEL_14     14 bytes

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

>0 : FIFO level set.

0 : FIFO level not set (not #16550).

+------------+----------------------------------------------------------------+

|   SioFlow  |  Sets hardware (RTS/CTS) flow control.                         |

+------------+----------------------------------------------------------------+

Syntax  int SioFlow(int Port,int Tics)

/* Port: Port selected (COM1 thru COM20) */

/* Tics: # tics before timeout */

Remarks  The SioFlow function is used  to  enable  or  disable  hardware  flow

control.  Hardware flow control uses RTS and CTS to control data flow

between the modem and the computer.  Refer to the User's Manual for a

discussion  of  flow  control.  To enable hardware flow control, call

SioFlow with Flag >= 0.

"Tics" is the number of timer tics (18.2 per second) before a call to

SioPutc or SioPuts will time out because CTS was not set.

If you enable hardware flow control, do not modify the RTS  line  (by

calling SioRTS).

Flow  control  is  disabled  after  resetting  a port.  To explicitly

disable hardware flow control, call SioFlow with Tics=-1.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

=0 : Flow control disabled.

>0 : Flow control enabled.

See Also  SioPutc and SioPuts

PCL4C Reference Manual                                              Page 9

+------------+----------------------------------------------------------------+

|   SioGetc  |  Reads the next character from the serial line.                |

+------------+----------------------------------------------------------------+

Syntax  int SioGetc(int Port,int Tics)

/* Port: Port selected (COM1 thru COM20) */

/* Tics: Timeout */

Remarks  The SioGetc function reads a byte from the selected serial port.  The

function  will wait for the number of system tics given by the 'Tics'

argument before returning 'timed out'.  There are 18.2  tics  to  the

second.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-1 : If timed out.

>0 : Character read.

See Also  SioUnGetc and SioPutc.

+---------------+-------------------------------------------------------------+

|   SioGetDiv   |  Reads the baud rate divisor.                               |

+---------------+-------------------------------------------------------------+

Syntax  int SioGetDiv(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioGetDiv function reads the baud rate divisor. The baud rate can

then be determined from the divisor by using the following table:

Baud   Divisor      Baud  Divisor      Baud  Divisor

300    0180        4800   0018       38400   0003

1200    0060        9600   000C       57600   0002

2400    0030       19200   0006      115200   0001

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>0 : Baud rate divisor.

See Also  SioParms.

PCL4C Reference Manual                                              Page 10

+------------+----------------------------------------------------------------+

|   SioGets  |  Receive a buffer from the serial line.                        |

+------------+----------------------------------------------------------------+

Syntax  int SioGets(int Port,char *Buffer,int Length)

/* Port: Port selected (COM1 thru COM20) */

/* Buffer: Character buffer */

/* Length: Length of Buffer (# of requested bytes) */

Remarks  The SioGets function reads bytes from the selected serial port.  This

function will read either the number of requested bytes or read fewer

if  no character is immediately available. SioGets cannot return more

characters than are in the receive buffer at the time it  is  called.

The number of characters actually read is returned.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-1 : If timed out.

>0 : Number of characters read into Buffer.

See Also  SioUnGetc and SioGetc.

+-------------+---------------------------------------------------------------+

|   SioInfo   |  Returns PCL4C library information.                           |

+-------------+---------------------------------------------------------------+

Syntax  int SioInfo(char Cmd)

/* Cmd: Command (see below) */

Remarks  The SioInfo function returns a word value depending on the  character

argument   in   the  table  below.  This  function  is  usefull   for

understanding  what  the  hardware  is  doing.  Note that the command

argument is case sensitive.

Info Codes

'V' : Version [as hex byte XY means version X.Y]

'I' : TRUE if library compiled with TX interrupts enabled.

'M' : Memory model (0=small,1=compact,2=medium,3=large)

'P' : TRUE if compiled for protected mode.

'T' : Total (over all ports) transmitter interrupts.

'R' : Total (over all ports) receiver interrupts.

'd' : Total (over all ports) times RTS dropped.

'r' : Total (over all ports) times RTS raised.

'c' : Total (over all ports) times CTS drop detected.

Returns   -1 : Function argument not recognized.

>=0 : Value as per above table.

PCL4C Reference Manual                                              Page 11

+-------------+---------------------------------------------------------------+

|    SioIRQ   |  Assigns an IRQ line to a port.                               |

+-------------+---------------------------------------------------------------+

Syntax  int SioIRQ(int Port,intIRQcode)

/* Port:    Port selected (COM1 thru COM20) */

/* IRQcode: IRQ number [IRQ2..IRQ15] */

Remarks  The SioIRQ function assigns an IRQ line to a port.  That  is,  SioIRQ

maps  an  IRQ to a port.  SioIRQ (like SioUART) must be called before

calling SioReset. Unless you have a non-standard  (COM1  &  COM3  use

IRQ4  while COM2 & COM4 use IRQ3) serial port configuration (or don't

want to run more than 2 ports concurrently), you  will  not  need  to

call  SioIRQ.   Be  EXTREMELY careful with SioIRQ as it can lock your

machine up if given incorrect information.

In particular, remember that your port  hardware  must  generate  the

interrupt  that  you have specified.  You should refer to your serial

board hardware manual for specfic instructions  in  configuring  your

hardware.  Be sure to call SioPorts first to setup DigiBoard ports if

you  have  them.   Refer  to  the  PCL4C  Users Manual for additional

information.

Returns  -4 : No such port. Expect 0 to MaxPort.

15 : Port already enabled.  SioReset has already been called.

-17 : No such IRQ.

-18 : ISR limit (maximum of 4 PC IRQs) exceeded.

0 : Otherwise

See Also  SioUART and SioPorts.

+-------------+---------------------------------------------------------------+

|   SioLine   |  Reads the line status register.                              |

+-------------+---------------------------------------------------------------+

Syntax  int SioLine(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioLine function reads the line status register.  The  individual

bit masks are as follows:

0x40  = Transmitter empty.

0x20  = Transmitter buffer empty.

0x10  = Break detected.

0x08  = Framming error.

0x04  = Parity error.

0x02  = Overrun error.

0x01  = Data ready.

The above are documented in the file PCL4C.H.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>0 : Line status (rightmost byte of word).

See Also  SioModem.

PCL4C Reference Manual                                              Page 12

+-------------+---------------------------------------------------------------+

| SioLoopBack |  Does a UART loopback test.                                   |

+-------------+---------------------------------------------------------------+

Syntax  int SioLoopBack(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  SioLoopBack makes use of the built in loopback test capability of the

INS8250  family  UART.   Normally  SioLoopBack  will never need to be

called unless you suspect that your UART is bad. Many UARTs  must  be

reset after running a loopback test.

Returns    0 : Loopback test is successfull.

-2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

-12 : Loopback test fails.

+-------------+---------------------------------------------------------------+

|   SioModem  |  Reads the modem status register.                             |

+-------------+---------------------------------------------------------------+

Syntax  int SioModem(int Port, int Mask)

/* Port: Port selected (COM1 thru COM20) */

/* Mask: Modem function mask */

Remarks  The SioModem function reads the modem register.  The bit  definitions

for the function mask are as follows:

Bit Name  Function               Bit  Name      Function

7  DCD   Data Carrier Detect     3   DeltaDCD  DCD has changed

6  RI    Ring Indicator          2   DeltaRI   RI has changed

5  DSR   Data Set Ready          1   DeltaDSR  DSR has changed

4  CTS   Clear To Send           0   DeltaCTS  CTS has changed

Bits 4 through 7 represent the absolute  state  of  their  respective

RS-232  inputs.   Bits 0 through 3 represent a change in the state of

their respective RS-232 inputs since last read. Once UART register  3

is  read, the Delta bits are cleared.  Thus, it is best not to depend

on the Delta bits.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>0 : Modem status (rightmost byte of word).

See Also SioCTS, SioDCD, SioDSR and SioRI.

PCL4C Reference Manual                                              Page 13

+-------------+---------------------------------------------------------------+

|  SioParms   |  Sets parity, stop bits, and word length.                     |

+-------------+---------------------------------------------------------------+

Syntax   int  SioParms(int Port,int Parity,int StopBits, int DataBits)

/* Port:     Port selected (COM1 - COM16) */

/* Parity:   Parity code [0,1,2] */

/* StopBits: Stop bits code [0,1] */

/* DataBits: Word length code [0,1,2,3] */

Remarks  The SioParms function sets the parity, stop bits, and word length. If

the  default  parity (none), stop bits (1), or word length (8) is not

acceptable, then they can be changed by  calling  SioParms.  SioParms

can  be  called  either  before  or  after calling SioReset. See file

PCL4C.H. (8 = default)

Value   Description    PCL4C.H Name

ParityCode:    *0      no parity      NoParity

1      odd parity     OddParity

3      even parity    EvenParity

StopBitsCode:  *0      1 stop bit     OneStopBit

1      2 stop bits    TwoStopBits

WordLengthCode: 0      5 data bits    WordLength5

1      6 data bits    WordLength6

2      7 data bits    WordLength7

*3      8 data bits    WordLength8

Returns   -4 : No such port. Expect 0 to MaxPort.

-7 : Bad parity code selected. Expecting  0 to 2.

|8 : Bad stop bits code. Expecting  0 or 1.

-9 : Bad word length code. Expecting  0 to 3.

See Also  SioReset.

+-------------+---------------------------------------------------------------+

|  SioPorts   |  Sets number of PC & Digiboard / BOCA Board ports.            |

+-------------+---------------------------------------------------------------+

Syntax  int  SioPorts(int NbrPorts,int FirstPort,int StatusReg,int Code)

/* NbrPorts:  Total number of ports */

/* FirstPort: First DigiBoard port */

/* StatusReg: DigiBoard Status Register */

/* Code:      PC_PORTS, DIGIBOARD, or BOCABOARD */

Remarks  The  SioPorts  function  must  be  called  before  ANY  other  serial

functions.   The purpose of the SioPorts function is to set the total

number of ports, the first DigiBoard (or BOCA  board)  port  and  the

DigiBoard (or BOCA board) status register address.

Once SioPorts is called, all COM ports starting with "FirstPort" will

be  treated as DigiBoard (or BOCA board) ports.  The default setup is

4  standard  PC  ports   and   no   DigiBoard   or   BOCA   ports   [

SioPorts(4,4,0,PC_PORTS) ].

Refer to  ther  PCL4C  users  manual  for  more  information  on  the

DigiBoard   and   BOCA   board.   Many  other  multiport  boards  are

functionally equivalent to either the DigiBoard or the BOCA board.

Returns  -4 : No such port. Expect 0 to 9.

0 : No error (sets MaxPort to NumberPorts-1).

See Also  SioUART, SioIRQ, and Code Examples.

PCL4C Reference Manual                                              Page 14

+-------------+---------------------------------------------------------------+

|   SioPutc   |  Transmit a character over a serial line.                     |

+-------------+---------------------------------------------------------------+

Syntax  int SioPutc(int Port,char Ch)

/* Port: Port selected (COM1 thru COM20) */

/* Ch:   Character to send */

Remarks  The SioPutc function transmits one character over the selected serial

line.

If  flow control has been enabled, then SioPutc may return a -1 (time

out) if the number of tics specified  in  the  SioFlow  function  was

exceeded  waiting  for  the  modem  to raise CTS. Refer to the User's

Manual for a discussion of flow control.

If  transmitter  interrupts  are enabled, SioPutc returns a -1 if the

transmit buffer is already full.  Otherewise the byte  is  placed  in

the  transmit  buffer,  awaiting  transmission by the PCL4C interrupt

service routine. If transmitter interrupts are not enabled, the  byte

is loaded directly into the UART transmit buffer.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-1 : Timed out waiting for CTS (flow control enabled), or

transmit buffer already full.

See Also  SioPuts, SioFlow, and SioTxFlush.

+-------------+---------------------------------------------------------------+

|   SioPuts   |  Transmit a buffer over a serial line.                        |

+-------------+---------------------------------------------------------------+

Syntax  int SioPuts(int Port,char *Buffer,int Length)

/* Port: Port selected (COM1 thru COM20) */

/* Buffer: Character buffer */

/* Length: Length of Buffer */

Remarks  The  SioPuts  function  transmits  each  buffer  character  over  the

selected  serial line.  The number of characters actually transmitted

is returned. This function can take a long time for large buffers  if

transmitter interrupts are not enabled!

If  flow  control  has  been enabled, then SioPuts may transmit fewer

characters than requested if the number  of  tics  specified  in  the

SioFlow  function  was  exceeded  waiting for the modem to raise CTS.

Refer to the User's Manual for a discussion of flow control.

If transmitter interrupts are enabled, SioPuts  will  transmit  fewer

than  the number of characters requested if the transmit buffer would

overflow. The actual number of characters transmitted is returned.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>=0: Number of characters actually transmitted.

See Also  SioPutc, SioFlow, and SioTxFlush.

PCL4C Reference Manual                                              Page 15

+-------------+---------------------------------------------------------------+

|   SioRead   |  Reads any UART register.                                     |

+-------------+---------------------------------------------------------------+

Syntax  int SioRead(int Port,int Reg)

/* Port: Port selected (COM1 thru COM20) */

/* Reg:  UART register (0 to 7) */

Remarks  The  SioReset function directly reads the contents of any  of  the  7

UART  registers.   This function is useful when debugging application

programs and as a method for verifying UART contents.  Refer  to  the

PCL4C Users Manual for a discussion of the 7 UART registers.

The line status register (register 5) can also be read  with  SioLine

while  the  modem  status register (register 6) can also be read with

SioModem.  Refer to the PCL4C User's Manual for a discussion  of  the

UART registers.

Returns   -3 : No buffer available. Call SioRxBuf first.

-4 : No such port. Expect 0 to MaxPort.

See Also  SioLine and SioModem.

+-------------+---------------------------------------------------------------+

|  SioReset   |  Initialize a serial port for processing.                     |

+-------------+---------------------------------------------------------------+

Syntax  int SioReset(int Port,int BaudCode)

/* Port:     Port selected (COM1 thru COM20) */

/* BaudCode: Baud code or -1 */

Remarks  The SioReset function initializes the selected serial port.  SioReset

should be called after calling SioParm and SioRxBuf but before making

any other calls to PCL4C.  SioReset uses the parity, stop  bits,  and

word length value previously set if SioParm was called, otherwise the

default values (see SioParm) are used.

Recall  that  COM1  and  COM3  share  the  same  interrupt vector and

therefore cannot operate simultaneously. Similiarly,  COM2  and  COM4

cannot operate simultaneously. Any other combination of two ports can

be used.

By  specifing  NORESET (-1) for the baud rate code, the port will NOT

be  reset.   This  is  used  to  "take  over"  a  port  from  a  host

communications   program  that  allows  a  "DOS  gateway".   External

protocols can be implemented this way.  See SioBaud for a list of the

baud rate codes, or see "PCL4C.H".

Returns   -3 : No buffer available. Call SioRxBuf first.

-4 : No such port. Expect 0 to MaxPort.

-11 : Bad baud rate code selected. Expecting  0 to 9.

|13 : UART undefined.  SioUART(Port,0) was called previously.

|14 : Bad or missing UART.  You may not have hardware present.

|15 : Port already enabled.  SioReset has already been called.

-16 : Interrupt already in use.

See Also  SioBaud, SioParms, SioRxBuf, SioTxBuf, SioDone, and SioUART.

PCL4C Reference Manual                                              Page 16

+-------------+---------------------------------------------------------------+

|    SioRI    |  Reads the Ring Indicator (RI) modem status bit.              |

+-------------+---------------------------------------------------------------+

Syntax  int SioRI(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioRI function is used to read  the  Ring  Indicator  (RI)  modem

status bit. Also see SioModem.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : RI is clear.

>0 : RI is set (RING has occurred).

See Also  SioDSR, SioCTS, SioDCD, and SioModem.

+-------------+---------------------------------------------------------------+

|    SioRTS   |  Sets, clears, or reads the Request to Send (RTS) line.       |

+-------------+---------------------------------------------------------------+

Syntax  int SioRTS(Port, char Cmd)

/* Port: Port selected (COM1 thru COM20) */

/* Cmd:  RTS command (SET, CLEAR, or READ) */

Remarks  The  SioRTS  function  controls  the Request to Send  (RTS bit in the

modem control register.

The  RTS  line  is  used by most error correcting modems to implement

hardware flow control.  RTS is dropped by the computer to signal  the

modem  not  to  send  data,  and  is  raised  to  signal the modem to

continue.  RTS should be set when communicating with a  modem  unless

Flow Control is being used.

Refer to the User's Manual for a discussion of flow control. Commands

(defined in PCL4C.H) are:

SET_LINE ('S') : set RTS (ON)

CLEAR_LINE ('C') : clear RTS (OFF)

READ_LINE ('R') : read RTS

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-5 : Command is not one of 'S', 'C', or 'R'.

0 : RTS is OFF (READ_LINE Command).

>0 : RTS is ON  (READ_LINE Command).

See Also  SioFlow and SioDTR.

PCL4C Reference Manual                                              Page 17

+------------+----------------------------------------------------------------+

|  SioRxBuf  |  Sets up receive buffers.                                      |

+------------+----------------------------------------------------------------+

Syntax  int SioRxBuf(int Port,int Selector,SizeCode)

/* Port:     Port selected (COM1 thru COM20) */

/* Selector: Receive buffer segment [selector for PM] */

/* SizeCode: Buffer size code */

Remarks  The SioRxBuf function passes the address  segment  and  size  of  the

receive buffer to PCL4C.  Recall that PCL4C requires a receive buffer

for each port in simultaneous operation since the receive function is

interrupt  driven.   It must be called before any incoming characters

can be received. SioRxBuf must be  called  before  SioReset.   Buffer

size codes are listed in "PCL4C.H".

Size Code       Buffer Size    PCL4C.H Name

4            128 bytes        Size128

5            256 bytes        Size256

6            512 bytes        Size512

7           1024 bytes        Size1024 or Size1K

8           2048 bytes        Size2048 or Size2K

9           4096 bytes        Size4096 or Size4K

10           8192 bytes        Size8192 or Size8K

11          16384 bytes        Size16384 or Size16K

12          32768 bytes        Size32768 or Size32K

Returns  -4 : No such port. Expect 0 to MaxPort.

-10 : Bad buffer size code. Expecting 0 to 11.

See Also  SioReset, SioTxBuf and Code Examples.

+------------+----------------------------------------------------------------+

| SioRxClear |  Cleares (clears) the receive buffer.                          |

+------------+----------------------------------------------------------------+

Syntax  int SioRxClear(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The  SioRxClear  function  will  delete any characters in the receive

buffer for the specified port.  After execution, the  receive  buffer

will  be  empty.   Call  SioRxBuf  after resetting a port in order to

delete any spurious characters.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also  SioRxQue.

PCL4C Reference Manual                                              Page 18

+-------------+---------------------------------------------------------------+

|  SioRxQue   |  Returns the number of bytes in the receive queue.            |

+-------------+---------------------------------------------------------------+

Syntax  int SioRxQue(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioRxQue function will return the number  of  characters  in  the

receive  queue  at  the time of the call. It can be used to implement

XON/XOFF flow control.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also   SioRxClear.

+------------+----------------------------------------------------------------+

|  SioTimer  |  Returns the number of system clock tics since midnight.       |

+------------+----------------------------------------------------------------+

Syntax  long SioTimer(void)

Remarks  The SioTimer function will return the number  of  system  clock  tics

since midnight, at 18.2065 tics per second.  This function is usefull

for timing various functions.

See Also  SioDelay

PCL4C Reference Manual                                              Page 19

+------------+----------------------------------------------------------------+

|  SioTxBuf  |  Sets up transmitter buffer.                                   |

+------------+----------------------------------------------------------------+

Syntax  int SioTxBuf(int Port,int Selector,int SizeCode)

/* Port:     Port selected (COM1 thru COM20) */

/* Selector: Receive buffer segment [selector in PM] */

/* SizeCode: Buffer size code */

Remarks  The SioTxBuf function passes the address  segment  and  size  of  the

transmit  buffer  to  PCL4C, provided that you will link with a PCL4C

library with transmitter  interrupts  enabled.   SioTxBuf()  must  be

called before SioReset.

Size Code       Buffer Size    PCL4C.H Name

4            128 bytes        Size128

5            256 bytes        Size256

6            512 bytes        Size512

7           1024 bytes        Size1024 or Size1K

8           2048 bytes        Size2048 or Size2K

9           4096 bytes        Size4096 or Size4K

10           8192 bytes        Size8192 or Size8K

11          16384 bytes        Size16384 or Size16K

12          32768 bytes        Size32768 or Size32K

Returns   -4 : No such port. Expect 0 to MaxPort.

-10 : Bad buffer size code. Expecting 0 to 11.

See Also  SioRxBuf, SioReset and Code Examples.

+------------+----------------------------------------------------------------+

| SioTxClear |  Cleares (clears) the transmitter buffer.                      |

+------------+----------------------------------------------------------------+

Syntax  int SioTxClear(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioTxClear function will delete any characters  in  the  transmit

buffer  for  the  specified  port, provided that you will link with a

PCL4C library with transmitter interrupts enabled.  After  execution,

the transmit buffer will be empty.

Once this function is called, any character in  the  transmit  buffer

(put  there  by  SioPutc) will be lost and therefore not transmitted.

This function is not  used  unless  the  transmitter  interrupts  are

enabled   Refer   to  the  PCL4C  Users  Manual  for  information  on

transmitter (TX) interrupts.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also  SioTxFlush and SioTxQue.

PCL4C Reference Manual                                              Page 20

+------------+----------------------------------------------------------------+

| SioTxFlush |  Flushes the transmitter buffer.                               |

+------------+----------------------------------------------------------------+

Syntax  int SioTxFlush(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioTxFlush function will re-enable transmitter interrupts if they

had been disabled by CTS dropping. This has the effect of forcing all

data out of the transmitter queue unless stopped again by  CTS  going

down.

Once the transmitter interrupt has been disabled (by CTS going down),

then the only way to restart it is by calling one of the  serial  I/O

functions  (SioPutc,  SioPuts,  SioGetc,  or  SioGets),  SioTxQue, or

SioTxFlush.

The primary purpose of SioTxFlush is to provide an explicit means  of

restarting  transmitter  interrupts.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also  SioTxQue.

+------------+----------------------------------------------------------------+

|  SioTxQue  |  Returns the number of bytes in the transmit queue.            |

+------------+----------------------------------------------------------------+

Syntax  int SioTxQue(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioTxQue function will return the number  of  characters  in  the

transmit  queue  at the time of the call, provided that you will link

with a PCL4C library with transmitter interrupts enabled.

This function is not  used  unless  the  transmitter  interrupts  are

enabled.   Refer  to  the  PCL4C  Users  Manual  for  information  on

transmitter (TX) interrupts.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also  SioTxFlush.

PCL4C Reference Manual                                              Page 21

+------------+----------------------------------------------------------------+

|  SioUART   |  Sets the UART base address.                                   |

+------------+----------------------------------------------------------------+

Syntax  int SioUART(int Port,int Address)

/* Port:    Port selected (COM1 thru COM20) */                           int Port;     /* COM1 to COM4 */

/* Address: UART address */

Remarks  The SioUART function sets the UART base  address  for  the  specified

port.   SioUART  must  be  called  before  SioReset  in order to have

effect.  Be extremely sure that you know what you  are  doing!   Note

that  PCL4C  uses  the  standard  PC/XT/AT  port addresses, interrupt

request  lines,  and  interrupt  service  vectors.   Therefore,  this

function is only needed for non-standard ports.

Returns  >0 : The previous base address for this port.

-4 : No such port.  Expect 0 to MaxPort.

-15 : Port already enabled.  SioReset has already been called.

See Also  SioPorts, SioIRQ, and SioReset.

+------------+-----+----------------------------------------------------------+

| SioUnGetc  |  "Un|gets" the last character read with SioGetc().             |

+------------+-----+----------------------------------------------------------+

Syntax  int SioUnGetc(int Port,char Ch)

/* Port: Port selected (COM1 thru COM20) */

/* Ch:   Character to unget */

Remarks  The SioUnGetc function returns ("pushes") the character back into the

serial input buffer.  The character pushed will be the next character

returned  by  SioGetc.   Only  one character can be pushed back. This

function works just like the "ungetc" function in the C language.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also  SioReset.

PCL4C Reference Manual                                              Page 22

Function Sumary

+-------------+----------+----------+----------+----------+

| Function    |   Arg1   |   Arg2   |   Arg3   |   Arg4   |

+-------------+----------+----------+----------+----------+

| SioBaud     | Port     | BaudCode |          |          |

| SioBrkKey   |          |          |          |          |

| SioBrkSig   | Port     | Cmd      |          |          |

| SioCTS      | Port     |          |          |          |

| SioDCD      | Port     |          |          |          |

| SioDelay    | Tics     |          |          |          |

| SioDone     | Port     |          |          |          |

| SioDSR      | Port     |          |          |          |

| SioDTR      | Port     | Cmd      |          |          |

| SioError    | Code     |          |          |          |

| SioFIFO     | Port     | LevelCode|          |          |

| SioFlow     | Port     | Tics     |          |          |

| SioGetc     | Port     | Tics     |          |          |

| SioGetDiv   | Port     |          |          |          |

| SioGets     | Port     | Buffer   | Length   |          |

| SioInfo     | Cmd      |          |          |          |

| SioIRQ      | Port     | IRQcode  |          |          |

| SioLine     | Port     |          |          |          |

| SioLoopBack | Port     |          |          |          |

| SioModem    | Port     | Mask     |          |          |

| SioParms    | Port     | Parity   | StopBits | DataBits |

| SioPorts    | NbrPorts | FirstPort| StatusReg| Code     |

| SioPutc     | Port     | Ch       |          |          |

| SioPuts     | Port     | Buffer   | Length   |          |

| SioRead     | Port     | Reg      |          |          |

| SioReset    | Port     | BaudCode |          |          |

| SioRI       | Port     |          |          |          |

| SioRTS      | Port     | Cmd      |          |          |

| SioRxBuf    | Port     | Segment  | SizeCode |          |

| SioRxClear  | Port     |          |          |          |

| SioRxQue    | Port     |          |          |          |

| SioTimer    |          |          |          |          |

| SioTxBuf    | Port     | Segment  | SizeCode |          |

| SioTxClear  | Port     |          |          |          |

| SioTxFlush  | Port     |          |          |          |

| SioTxQue    | Port     |          |          |          |

| SioUART     | Port     | Address  |          |          |

| SioUnGetc   | Port     | Ch       |          |          |

+-------------+----------+----------+----------+----------+

PCL4C Reference Manual                                              Page 23

Error Code Summary

+------+--------------------------------------------------------+

| Code |  Description                                           |

+------+--------------------------------------------------------+

|   0  |  No error.                                             |

|  -1  |  Timeout waiting for I/O.                              |

|  |2  |  Port not enabled. Call SioReset first.                |

|  |3  |  No buffer available. Call SioRxBuf first.             |

|  |4  |  No such port. Expect 0 to MaxPort. (COM1 = 0)         |

|  |5  |  Expected 'S', 'C', or 'R' as 2nd argument.            |

|  |6  |  Expected 'A', 'C', or 'D' as 2nd argument.            |

|  |7  |  Bad parity code specified. Expected 0 to 7.           |

|  |8  |  Bad stop bits code specified. Expected 0 or 1.        |

|  -9  |  Bad wordlength code specified. Expect 0 to MaxPort.   |

| -10  |  Bad buffer size code specified. Expected 0 to 11.     |

| |11  |  Bad baud rate code. Expected 0 to 9.                  |

| |12  |  Loopback test fails.                                  |

| |13  |  UART undefined.                                       |

| |14  |  Missing or bad UART. (no UART hardware ?)             |

| |15  |  Port already enabled.                                 |

| |16  |  ISR (interrupt) already in use.                       |

| |17  |  No such IRQ. (Should be 2 to 7)                       |

| |18  |  ISR limit (maximum of 4 PC IRQs) exceeded.            |

| |19  |  Shareware screen cannot be displayed.                 |

+-+----+--------------------------------------------------------+

PCL4C Reference Manual                                              Page 24

Code Examples

SioRxBuf / SioTxBuf Example

int BufSize = 1024;

int SizeCode = Size1024;

char far *Ptr;

int BufSeg;

...

/* allocate receive buffer + 16 bytes */

Ptr = (char far *) _fmalloc(BufSize+16);

BufSeg = FP_SEG(Ptr) + ((FP_OFF(Ptr)+15)>>4);

/* pass receive buffer segment to PCL4C */

SioRxBuf(Port,BufSeg,SizeCode);

/* allocate transmit buffer + 16 bytes */

Ptr = (char far *) _fmalloc(BufSize+16);

BufSeg = FP_SEG(Ptr) + ((FP_OFF(Ptr)+15)>>4);

/* pass transmit buffer segment to PCL4C */

SioTxBuf(Port,BufSeg,SizeCode)

DigiBoard Example

/* use 0x140 for odd IRQs & 0x141 for even IRQs */

SioPorts(8,COM1,0x140,DIGIBOARD);

for(i=COM1;i<=COM8;i++)

{/* set DigiBoard UART addresses */

SioUART(i,0x100+8*i);

/* set DigiBoard for IRQ5 */

SioIRQ(i,IRQ5);

}

BOCA Board Example

/* use base port + 7 */

SioPorts(16,COM1,0x100+7,BOCABOARD);

for(i=COM1;i<=COM16;i++)

{/* set BOCA Board UART addresses */

SioUART(i,0x100+8*i);

/* set DigiBoard for IRQ5 */

SioIRQ(i,IRQ5);

}

PCL4C Reference Manual                                              Page 25