SwapIRQ

User Guide

Computer Telecommunication Systems, Inc.

3847 Foxwood Road, Suite 1000

Duluth, Georgia 30136-6100

Orders: 1-800-380-2666

Voice: 1-770-263-8623

FAX: 1-770-263-0124

Email: 76662.2315@compuserve.com

(C) Copyright 1993-1995 Computer Telecommunication Systems, Inc.

All Rights Reserved

No part of this User Guide may be reproduced, transmitted, transcribed,

stored in a retrieval system, or translated into any language (natural

or computer), in any form or by any means, without the prior written

permission of Computer Telecommunication Systems, Inc.

Information in this manual is subject to change without notice and does

not represent a commitment on the part of Computer Telecommunication

Systems, Inc

Computer Telecommunication Systems, Inc. may make improvements and/or

changes in the manual and/or in SwapIRQ at any time

Trademarks and Copyrights

-------------------------

IBM(R) and PS/2(R) are registered trademarks of International Business

Machines Corporation.

Hayes(R) is registered trademark of Hayes Microcomputer Products, Inc.

The CTS Serial Port Utilities(TM), IRQInfo and SwapIRQ(TM) are

trademarks of Computer Telecommunication Systems, Inc.

TABLE OF CONTENTS

Introducing SwapIRQ. . . . . . . . . . . . . . . . . . . . . 1

Installing SwapIRQ . . . . . . . . . . . . . . . . . . . . . 2

Before Using SwapIRQ . . . . . . . . . . . . . . . . . . . . 2

Using SwapIRQ. . . . . . . . . . . . . . . . . . . . . . . . 3

Help Screen. . . . . . . . . . . . . . . . . . . . . . 3

Loading SwapIRQ. . . . . . . . . . . . . . . . . . . . 3

Format 1: SwapIRQ /ih /is . . . . . . . . . . . . 3

Format 2: SwapIRQ /ih /is /n /M . . . . . . . . . 4

Format 3: SwapIRQ /ih /is /Aaaa /M. . . . . . . . 4

Stopping SwapIRQ . . . . . . . . . . . . . . . . . . . 5

Running SwapIRQ From a Batch File. . . . . . . . . . . 5

Using SwapIRQ with Windows . . . . . . . . . . . . . . . . . 5

Examples . . . . . . . . . . . . . . . . . . . . . . . . . . 6

Example 1: Your modem is on Com3 using IRQ 5, but

software only uses Com3 with IRQ 4. . . . . 6

Example 2: Your serial ports support high IRQs, but

your software doesn't . . . . . . . . . . . 6

Example 3: Your modem is on Com3 using IRQ 5, and

your mouse is on Com1 using IRQ 4. Your

software uses Com3 on IRQ 4, which

conflicts with your serial mouse (also

using IRQ 4). . . . . . . . . . . . . . . . 7

Appendix A: Conventional IRQ and Address

Assignments for Serial Ports . . . . . . . . . . . . . 8

Appendix B: Using SwapIRQ with Games . . . . . . . . . . . . 9

Appendix C: Compatibility list . . . . . . . . . . . . . .  10

Introducing SwapIRQ(TM)

-----------------------

SwapIRQ is a powerful but tiny DOS utility which allows your

existing communications software to use any interrupt (IRQ)

supported by your serial port or modem. You can also use it in a

"DOS session" under Windows to provide access to an IRQ not

supported by a DOS application. SwapIRQ works under DOS versions 2.0

and higher, and uses less than 1K (actually, less than 800 bytes) of

memory.

Serial cards and board modems in your computer system are installed

at a particular address and use an IRQ (an Interrupt ReQuest line)

to signal the controlling software that they require attention. Some

cards only allow selection of a Com port: Com1, Com2, etc., and use

the industry standard addresses and IRQs as described in Appendix A.

Other serial ports let you independently select a port address and

IRQ which do not follow any particular standards.

Communication software requires two pieces of information to work

with a serial card or a board modem. First, software must know the

address used by the board so that commands can be issued. Secondly,

the software must know which IRQ is used by the board to request

attention. Finally, you must be able to configure the software to

use the address and IRQ used by the serial card. Otherwise, the

board or modem will not work with the software.

Communication software normally lets you select the port to use -

Com1, Com2, and so on. A few applications are limited to using the

two "standard" serial ports: Com1 at address 3F8H (hexadecimal) with

IRQ 4, and Com2 at address 2F8H with IRQ 3. There are some serial

ports that allow Com1 and Com2 to use other IRQs, and SwapIRQ will

allow these applications to work with the ports on other IRQs.

Other applications allow you to use more serial ports - Com3 and

Com4 are common. However, many of these applications may require

that the IRQ used by Com3 and Com4 are the same as the IRQs used by

Com1 and Com2. SwapIRQ enables these applications to use Com3 and

Com4 (or any serial port) on any available IRQ supported by the

serial port.

SwapIRQ does this by "redirecting the interrupt". For example, let's

say that your software is set up to use Com3 on IRQ 4, but your

serial port is set up as Com3 using IRQ 5. When your board generates

the interrupt request (IRQ 5), your software won't see it because it

isn't watching the correct IRQ (it's looking for IRQ 4). The result

is a "hung application", or worse, a system lockup.

But if you install SwapIRQ before using the application, then

SwapIRQ "watches for" IRQ 5 for your application. When your serial

board activates IRQ 5, SwapIRQ detects it and signals the IRQ your

software uses (IRQ 4, in this case) that the interrupt occurred.

This allows the software to operate normally. When you are finished

with the application, you unload SwapIRQ to put your system back in

its normal state.

-1-

Installing SwapIRQ

------------------

SwapIRQ is normally placed in a subdirectory on your hard disk. The

following instructions assume you will put SwapIRQ in a directory

named "SWAPIRQ":

1.  Create an "SWAPIRQ" subdirectory on your hard disk

using the DOS Make Directory (MD) command:

Example: C>MD SWAPIRQ <Enter>

NOTE: <Enter> means to press the Enter (Return) key.

2.  Change the default directory to the directory you

created using the DOS Change Directory (CD)

command:

Example: C>CD SWAPIRQ <Enter>

3.  Copy the distribution files to the SWAPIRQ directory. If

the distribution files are on a floppy diskette in drive A,

you would enter:

Example: C:\SWAPIRQ>COPY A:\*.* <Enter>

If your directory contains a README.TXT file, then check this file

for additional information not included in this User Guide.

SwapIRQ is ready to run. To make SwapIRQ available to all of your

applications, you can add the SwapIRQ directory to the PATH command

in your AUTOEXEC.BAT file. An alternative is to use a batch file for

each of your applications requiring SwapIRQ as described on Page 8.

Before Using SwapIRQ

--------------------

Before you use SwapIRQ with your serial ports, you should verify

that your system has no serious serial port or interrupt conflicts.

If you have been using your current system for some time without

problems, then you can probably assume that your system is

configured properly. If you have changed your system setup recently,

or if you have occasional difficulties, then a little additional

testing is worth the effort. If you have the CTS Serial Port

Utilities, then run PortInfo, paying careful attention to the error

and warning messages regarding port and interrupt conflicts. If

PortInfo detects serial port conflicts, then these should be

resolved before using SwapIRQ.

If you do not have the CTS Serial Port Utilities, then we suggest

you contact us to obtain a copy, particularly if you observe any

problems with SwapIRQ.

After you have verified that your system is in good working order,

then you need to determine which interrupt your hardware is

generating, and which interrupts your software can use. You will use

this information when starting SwapIRQ.

-2-

Using SwapIRQ

-------------

SwapIRQ is a TSR application that you load before running your

application which requires the special interrupt handling, and

unload when you are finished. SwapIRQ is controlled through command

line switches, and is aware of CTSCOMn environment variables. There

are several possible command lines formats:

Help Screen

You can display the help screen from the DOS prompt by entering

either:

C:\SWAPIRQ>SwapIRQ <Enter>

-OR-

C:\SWAPIRQ>SwapIRQ /? <Enter>

Loading SwapIRQ

There are three formats for loading SwapIRQ. The first is fine

to use under most circumstances. However, if you experience

problems with SwapIRQ when using the first format, then try the

second. The second format will make sure that another serial

port will not generate interrupts that conflict with SwapIRQ.

You can even completely disable a serial mouse that uses the

same IRQ required by your software for something else!

Format 1: SwapIRQ /ih /is

This command installs the SwapIRQ program, and swaps hardware

IRQ 'h' to software IRQ 's'. That is, the serial port is using

IRQ 'h' to signal the software, which is looking for IRQ 's'

where:'h' is the IRQ generated by the hardware

's' is the IRQ expected by the software

NOTE: They must be in the sequence shown.

The valid 'h' and 's' values are decimal 2-7 for an IBM PC/XT

computer, and 3-7, 9-12, 14 and 15 for an IBM AT or compatible

machine.

NOTE: if your board has a setting for IRQ 2, it is actually

using IRQ 9 in an IBM AT or compatible computer. You should

use IRQ 9 as the hardware IRQ. You can use IRQ 2 as the

software IRQ.

-3-

For example, using the command line:

SwapIRQ /i5 /i4

SwapIRQ will translate IRQ 5 generated by the serial port to IRQ

4 for the software.

NOTE: There is one important limitation when using SwapIRQ. The

"software interrupt" cannot be used by a hardware device at the same

time that SwapIRQ is in use. In the example above, the IRQ 4 line

must be dedicated to SwapIRQ. The Com1 serial port, which normally

uses IRQ 4, must be inactive while SwapIRQ is loaded. See command

Format 2 on how to disable interrupts from a serial port or serial

mouse while SwapIRQ is loaded.

Format 2: SwapIRQ /ih /is /n /M

where 'n' is a valid serial port number of 1-8. The '/n'

switch instructs SwapIRQ to disable interrupts from serial

port 'n'. Use this switch if you are switching a hardware

interrupt over to an interrupt line used by another serial

port. This will prevent a stray interrupt from the serial port

from contesting with the swapped interrupt.

If you add a '/M' switch to the command line, this will

completely disable a serial mouse on port 'n'. Applications

that use a mouse may require this command format to run

correctly with SwapIRQ.

NOTE: If you use the CTS Serial Port Utilities and the CTSCOMn

environment variables, then SwapIRQ will use these port

assignments.

Format 3: SwapIRQ /ih /is /Aaaa /M

where 'aaa' is a valid serial port address, in hexadecimal.

You can use this option to disable interrupts from a serial

port at address 'aaa'. Use this command if you are swapping a

hardware IRQ over to an IRQ line used by a non-standard serial

port, and you do not have a CTS Serial Port Utility CTSCOMn

environment variable defined for the port.

The '/M' switch behaves just as described above.

-4-

Stopping SwapIRQ

Stop SwapIRQ from running when you have finished with the

application that needs it. This is done with the command:

C:\SWAPIRQ>SwapIRQ /D <Enter>

This command removes SwapIRQ from memory and restores all

interrupt control back to a normal status. If another

application has been loaded since SwapIRQ was installed, then

SwapIRQ will only be disabled and not removed from memory. In

this case, you will lose about 1K of system memory until your

next system startup (reboot).

NOTE: it is recommended that you load SwapIRQ just before running

your application and then unload it immediately afterwards. You can

put the SwapIRQ commands into a batch file to automate the process.

Running SwapIRQ From a Batch File

You can set up batch files to automatically load and unload

SwapIRQ by doing the following (this assumes that SwapIRQ is

located in a directory SWAPIRQ on the "C" drive):

Create a batch file in the directory of the program that

requires SwapIRQ that starts SwapIRQ, then runs the

communication application you need, and then unloads SwapIRQ

when you have finished.

FILE: SAMPLE.BAT

c:\SWAPIRQ\SwapIRQ /i5 /i4  (to swap IRQ 5 to IRQ 4)

CommProg       (put the program startup string here)

c:\SWAPIRQ\SwapIRQ /d       (unloads SwapIRQ)

Using SwapIRQ with Windows

--------------------------

You can use SwapIRQ in Windows just like you do in DOS. Before using

SwapIRQ with Windows, it is important to have Windows properly

configured for the serial ports you have in your system. You can do

this either through your Windows control panel, or by editing the

WIN.INI file with a text editor. See your Microsoft Windows

documentation for additional information.

You use SwapIRQ under Windows in a DOS box to provide access for

DOS-based communication applications to IRQs not supported directly

by the application. Load SwapIRQ as a described for DOS in a Windows

DOS box before running your DOS communications program. Then run the

application just as you would under DOS.

-5-

Examples

--------

Example 1: Your modem is on Com3 using IRQ 5, but software only uses

Com3 with IRQ 4

You have a modem on Com3, IRQ 5. Most of your software supports this

modem. However, one application allows use of Com3, but won't let

you specify the IRQ used by the port.

Solution: Use SwapIRQ to "redirect" IRQ 5 generated by the modem to

IRQ 4 (the conventional IRQ for Com3) when using this application.

To be on the safe side, you also disable Com1 from generating

interrupts on IRQ 4 while SwapIRQ is loaded. You can do this with

the command line "SWAPIRQ /i5 /i4 /1" where:

SwapIRQ /i5 /i4 /1

^   ^  ^

|   |  |

|   |  +---- Disable Com1 IRQs (Com1 uses IRQ 4)

|   |

|   +------- The software uses IRQ 4

|

+----------- The modem uses IRQ 5

Com1 is now disabled until you unload SwapIRQ or until another

application re-enables it. Run your application that needed the

interrupts swapped. When you are finished with the application, you

unload SwapIRQ to restore your system to a "normal" status.

Example 2: Your serial ports support high IRQs, but your software

doesn't:

You have a 16-bit serial board that supports high IRQs (10-12, 14

and 15), but some of your software doesn't allow you to access these

IRQs.

Solution: Use SwapIRQ to redirect the high IRQs to the conventional

IRQs supported by most communication software. For example, if you

need to use Com4, IRQ 11 with an application that expects Com4 to

use IRQ 3, then you load SwapIRQ before running the application with

the command:

C:\SWAPIRQ>SwapIRQ /i11 /i3 <Enter>

- OR -

C:\SWAPIRQ>SwapIRQ /i11 /i3 /2 <Enter>

(to prevent Com2 from generating interrupts on IRQ 3)

NOTE: the command line "switches" like "/i", "/a", and "/d" can be

either uppercase or lowercase letters.

-6-

Example 3: Your modem is on Com3 using IRQ 5, and your mouse is on

Com1 using IRQ 4. Your software uses Com3 on IRQ 4, which conflicts

with your serial mouse (also using IRQ 4).

Solution: You use SwapIRQ to get access to the modem on IRQ 4 and to

prevent the mouse from interfering.

NOTE: you will not be able to use the mouse in this application,

since SwapIRQ shuts off the mouse!

To do this, you load SwapIRQ with the command:

SwapIRQ /i5 /i4 /1 /M

^   ^  ^  ^

|   |  |  |

|   |  |  +------- Disables the mouse

|   |  |

|   |  +---------- Disables IRQs from Com1

|   |              (which uses IRQ 4)

|   |

|   +------------- The software uses IRQ 4

|

+----------------- The modem uses IRQ 5

Then run your application. When you are finished with your

application, stop SwapIRQ. Your mouse will be available to the next

application you use after stopping SwapIRQ.

-7-

Appendix A: Conventional IRQ and Address Assignments for Serial Ports

I. ISA (IBM AT and Compatible) Computers

Serial Port    IRQ        Address (first address, in hexadecimal)

Com1         4         3F8   (this is a STANDARD!)

Com2         3         2F8   (this is a STANDARD!)

Com3         4         3E8   (a convention)

Com4         3         2E8   (a convention)

NOTE: there are no conventions for Com5 or higher.

II. Microchannel Architecture (IBM PS/2) Computers

Serial Port    IRQ        Address (first address, in hexadecimal)

Com1         4         3F8

Com2         3         2F8

Com3         3         3220

Com4         3         3228

Com5         3         4220

Com6         3         4228

Com7         3         5220

Com8         3         5228

NOTE: These are standards as defined by IBM.

-8-

Appendix B: Using SwapIRQ with Games

Many DOS games are set up to only work with one IRQ - usually IRQ 5

or IRQ 7. If your sound card is not set up to work on that IRQ, you

miss all of the sound. While SwapIRQ was developed specifically for

communications, you can use it with many DOS games to allow the

sound card IRQ to get through to the game software.

Operation with games is similar to that for communication software.

Simply replace the word "modem" with "sound card" in all earlier

sections of the User Guide.

Example 4: Your sound card uses IRQ 5, but the game software

requires the sound card on IRQ 7:

Solution: Use SwapIRQ to "redirect" IRQ 5 generated by the sound

card to IRQ 7.

SwapIRQ /i5 /i7

^   ^

|   |

|   +------- The game software uses IRQ 7

|

+----------- The sound card uses IRQ 5

-9-

Appendix C: Compatibility List

SwapIRQ has been used extensively for over 2 years with a wide range

of communications software. At the time of this initial shareware

release, the only known incompatibilities are with the America

Online for DOS and Prodigy for DOS programs.

I would like to compile a list of games that do and do not operate;

please let us know how SwapIRQ works for you by contacting us via

any means listed on the title page of this User Guide.

Association Of Shareware Professionals Member

=============================================

_______

____|__     |                (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

This program is produced by a member of the Association of

Shareware Professionals (ASP). ASP wants to make sure that the

shareware principle works for you. If you are unable to resolve a

shareware-related problem with an ASP member by contacting the

member directly, ASP may be able to help. The ASP Ombudsman can

help you resolve a dispute or problem with an ASP member, but

does not provide technical support for members' products.  Please

write to the ASP Ombudsman at 545 Grover Road, Muskegon, MI

49442-9427 USA, FAX 616-788-2765 or send a CompuServe message via

CompuServe Mail to ASP Ombudsman 70007,3536.

-10-

Name: ______________________________        Telephone: ____________________

Company: ________________________________   E-mail ID: ____________________

Address: __________________________________________________________________

__________________________________________________________________

QTY   Description                               Price           Subtotal

===   ===========                               =====           ========

___   SwapIRQ with on-disk User Guide             $20           ________

___   SwapIRQ with Printed User Guide             $24           ________

Other Products (special prices valid only with SwapIRQ order):

(see the descriptions and in PRODUCTS.DOC)

___   IRQInfo ....... Normally $28.95 - only ....  24           ________

___   IRQInfo PRO ... Normally $41.95 - only ....  35           ________

___   PortFix ....... Normally $14.95 - only ....  10           ________

___   The CTS Serial Port Utilities

(see the description and the order form in PRODUCTS.DOC)

___   The CTS Modem Utilities

(available soon - call or write for details)

========

Sub-Total     $________

Sales Tax:                  Georgia residents add 5%     $________

Shipping & Handling:                                     $    4.00

=========

TOTAL (US dollars, please)   $________

PAYMENT METHOD:  Check __    Money Order __    EuroCheq __     Discover __

Visa __    MasterCard __    Am. Express __    Purchase Order ($50 min) __

========================================================================

CREDIT CARD INFORMATION

Name as it appears on the card: _________________________________

Card No. ________  ________  ________  ________  Expires  ___/___

Cardholder signature: ___________________________________________

Please send, FAX, email or call:

+-------------------------------------------------------------------+

|              Computer Telecommunication Systems, Inc.             |

|                   3847 Foxwood Road, Suite 1000                   |

|                   Duluth, Georgia  30136-6100 USA                 |

+------------------------------+------------------------------------+

|           Telephone          |          Electronic Mail           |

|           ---------          |          ---------------           |

|       Orders: 1-800-380-2666 |       CompuServe: 76662,2315       |

|        Voice: 1-770-263-8623 | Internet:76662.2315@compuserve.com |

|          FAX: 1-770-263-0124 |                                    |

+------------------------------+------------------------------------+

NOTE: Federal TaxID: 58-1880693. Prices are subject to change without

notice.