Tiny Stuff and Tiny View

by  David Mumper

ST Picture Compression

Any ST user who has pictures knows that, no matter how good (or

bad) they look, they always take roughly 32128 bytes of disk space.

This can be quite discouraging when you realize that this means that

you can only fit 10 pictures on a single sided disk.  From NeoChrome

0.5, the memory is used as follows...

4 bytes telling the resolution (0=low, 1=medium, 2=high)

32 bytes for the colors

12 bytes set aside for a filename (always "        .   ")

2 bytes telling which colors are set for rotating

(first is always $80)

2 bytes telling the speed and direction of rotation

2 bytes telling how long to rotate

74 reserved bytes (for expansion)

32000 bytes of screen memory

-----

32128 total

A more efficient way of saving the pictures would be to do a

compression on the screen memory so that, if there were repetative

sections, it wouldn't be as long.  An example of how to do this could

be to look at the Micro-Illustrator (Koala pad) compaction for the

8-bit computers.  A straight memory save of a Graphics 8 (or 15) screen

would take up 7680 bytes.  Saving a picture with the Micro-Illustrator

format can result in a picture as small as 32 bytes (if the screen was

blank).  This type of compression could be applied to the ST as well

resulting in very large memory savings.  TINYSTUFF is an implementation

of just such a compression algorhythm that produces an average

compression of about 64%.  Comparing it to the Micro-Illustrator

format, it's smallest save would be 42 bytes.

USING THE COMPACTOR

To use the compactor program, place it on a freshly formatted disk

and add some pictures.  For best results, place half as many

uncompacted pictures on the disk as you could actually fit (5 on single

sided and 10 on double sided).  Next, double click on the compacter

program.  It will then load and begin executing.  The program will

begin by asking you the formats for the pictures to convert.  As an

additional feature, you can convert Degas format pictures to Neo (or

vice versa) as well as compacting or uncompacting them.  After you have

chosen the source and destination formats for the pictures, the program

will begin loading all the pictures that match the source format from

the same disk and pathname as the program.  The program will load and

display each picture that has the correct extender for that file type

(i.e. '.NEO' for NeoChrome, '.TNY' for Tiny or '.PI?' for Degas).  As

the program displays the pics, it will convert them and save them back

to the same disk / pathname as the program.  Each of the pictures will

have the same name as they start with but the extender will be changed

to that of the destination format.

Having your picture collection in a compacted format is handy to

save space but is a nuisance if you can't look at them that way.

Therefore, TINYVIEW is a displayer program for pictures in Tiny format.

To look at the pictures, just place as many of them on a disk with the

displayer program as will fit then double click the display program.

It will search the directory for the first filename with a '.TNY'

extender and then start displaying them from there.  It will load and

display all the pictures on the disk, cycling back to the beginning

when the end of directory is reached.  Pressing the space bar will

cause the program to pause and wait for another key to be pressed

before going on to the next picture.  To exit the program, press any

key other than the space bar while not paused.  The picture that is

being loaded will be displayed and then the program will return to the

desktop.  Any pictures that have color rotation information, will have

that saved with the picture when it's compacted and displayed by this

program.

The file format of the compacted files is as follows...

1 byte that tells which resolution and if there is rotation infor-

mation (if there is rotation information it is stored as follows

1 byte for the start and end color to rotate (start is high 4 bits

end is low 4 bits)

1 byte for direction and speed of rotation (negative for left

rotation positive for right, actual number is the number of

vertical blank interrupts to wait between each rotation)

2 bytes for the number of revolutions to make)

2 bytes that tell how many control bytes there are

2 bytes that tell how many data bytes there are

32 bytes that tell the computer the colors

3-10667 control bytes

2-32000 data bytes

-----

42-32044 minimum-maximum size

The screen is scanned column by column, plane by plane (assuming

low res format) as the program does the compaction.

If there is rotation information, the resolution has 3 added to it

to signify this.

Control bytes are interpreted as follows...

X < 0 -- 1-127 unique words are taken from the data section

X = 0 -- two more words are taken from the control section that is a

value 128-32767 which is the number of times to repeat the

next data word

X = 1 -- two more bytes are taken from the control section that

is a value 128-32767 of unique words to take from data section

X > 1 -- 1 byte of repetative information is taken from data

section and repeated 2-127 times.

Due to this compression algorythm, I've been able to place as many

as 95 pictures on a double sided disk.  (That's an 850% improvement!!

and not common) Average disks will hold roughly 20-30 pictures.  So far

I've seen many very nicely done pictures on bulletin boards and

visiting friends, so hopefully, by allowing the average user the

ability to keep more pictures on less disks, we'll all be able to see

many more artists showing their abilities on the ST.

####�#####�#��#��8x#@#�####�##�#�##f#R#Q######�###L#�