Ultra Fast RAM Disk for ATARI 520/1040 ST

Version 1.0

April, 1986

by Charles Smeton

****************************************************

*                                                  *

*      This program  can be  freely  distributed   *

*   (excluding  media  costs), it is and  should   *

*   remain  in the  PUBLIC DOMAIN. Any questions   *

*   should  be forwarded  to the author  through   *

*   Compuserve  at:  [73047,2565]. The author is   *

*   is not  held  responsilbe  for any  problems   *

*   arising  from  the  use  of  this  program.    *

*                                                  *

****************************************************

This program sets aside a portion of the ST's memory to

act as an Ultra-Fast Disk Drive.  Unlike other RAM Disk

programs that are installed as desk accessories, this

RAM Disk can be installed automatically by placing it

in AUTO folder on boot disk.  This save a desk accessory

slot for more useful purposes. The RAM disk can also be

installed by renaming it to a ".TTP" extension and clicking

it from the desktop.

To install the RAM Disk for automatic setup upon booting:

1)  Install the RAM Disk icon with drive letter you want

the RAM Disk to be.  This is accomplished from the

desktop by clicking Floppy Disk A or B's icon once

(causing it to turn black).  Then select "Install Disk

Drive" from the "Options" menu in the Desk Top.

Change the "Drive Identifier" to the drive you want the

RAM Disk to be and the "Icon Label" to "RAM DISK".  Then

select the "Install" box. Drag the RAM DISK icon to

where you want it to be on the desk top.  Insert your

boot disk in drive A and then select "Save Desktop"

from the "Options" menu. This creates the "DESKTOP.INF"

file on the boot disk.

2)  Create a new folder on the boot disk called "AUTO",

then copy the RAMDISK.PRG file into this folder.

3)  Using a standard text editor create a file called

"RAMDISK.INF" in the main directory of the boot

disk. This file need only contain the following:

a) the drive letter for the RAM Disk

b) the size of the RAM Disk in Kilobytes.

An example would be:

C200

for drive C, 200K bytes.

The drive letter can be any letter upper or lower

case from "C" through "P". The first letter found in

the file in this range is interpreted as the drive

letter.  After the drive letter is determined, then

the first numeric character located starts the drive

size number, this number is determined complete when

any non numeric character is found or the end of the

file is detected.  If the no number is found, then

a default RAM Disk size of 100K is assumed.  Likewise,

if no drive letter is found, then "D" is assumed as

the drive and 100K bytes as the size. If the file

"RAMDISK.INF" is not found on the boot disk, then

the RAM Disk program will also assume the above

defaults.

Note: If a size less than 50K is given, then 50K

is used as the drive size. If there is not enough

memory available, then the RAM Disk is not

installed by the program.

The RAM Disk contents will survive a resolution change

on a  color monitor.  Since the DESKTOP.INF file will be

re-loaded after a resolution change, make sure it has the

RAM Disk Drive icon installed. It is a good idea to have

the boot disk in drive A during this time, otherwise you

will need to reinstall RAM disk icon manually.

The RAM Disk data transfer has been optimized for speed.

This should out perform many commercial RAM Disks. Try the

RAM Disk with a slide show to see how fast it is.

The RAM Disk can be initialized manually by double

clicking the program file name or icon from the desktop.

If you rename the program to have a ".TTP" extension,

then the RAM Disk Drive parameters can be entered in the

dialog box just as they would be from the RAMDISK.INF file.

If no parameters are given, then the RAMDISK.INF file is

checked for the valid parameters as described above. If

the command line is entered incorrectly, then the default

parameters are used.

The RAM Disk can be initialized from a Command line

interpreter environment, but  it is advisable to use

one of the other methods. The command line can be typed

as follows:  RAMDISK D200.

The problem using this method is that the memory used

by the command program is below the RAM Disk and cannot

be released to GEMDOS. The system may lockup or the

RAM Disk contents may be lost if the user tries to exit

back to the desktop. If the user plans to stay in the

command environment, then ther is no problem.

RAM Disk Size specification:

Any size RAM Disk from 50K bytes to 60000 bytes below

the total amount of free memory may be specified. As

stated before attempting to use less than 50K bytes,

gives you a 50K byte minimum RAM Disk. The 60000 byte

saftey margin is to insure that there will be enough

memory for the desktop to operate as well as the minimum

to run a small program. The RAM Disk will not be installed

if you specify an amount above this limit.

Note: On a normal 520ST with TOS in ROM, there is enough

room for a 351K RAM Disk. This will not leave much room

for programs to run, but for copying files/disks it

gives the same memory as a single sided disk. On a 1040ST

or expanded 520ST, RAM Disks up to 700K bytes should be

no problem.

Note: You cannot copy a Floppy Disk to/from a RAM Disk by

dragging the disk icon on top of the other icon. This

is true no matter what size RAM Disk is used. To get around

having to draw a box around all the files in a window and

dragging them to the other window or disk icon (this may

take several iterations), just drag the disk icon into the

open (empty) window of the destination. This will copy

each file from the source to the destination disk.

The RAM Disk size specified is the total number of data

clusters available for storage in the RAM Disk. Where one

cluster = two sectors = 1K bytes.  The memory needed for the

boot sector (1 sector), the two copies of the File Allocation

Table (FAT) (2 * 5 sectors) and the directory (7 sectors) is

allocated in addition to the specified size . For some reason,

GEMDOS reserves 2K bytes of the specified RAM Disk size for its

own use. The total memory displayed by a "SHOW INFO" of the RAM

Disk's icon be 2048 bytes less than the RAM Disk size * 1024 bytes.

If the RAM Disk is successfully installed, a message indicating

the Drive and size allocated is displayed. If the RAM Disk can

not be installed, then and Error message is printed and two

console beeps are generated to grab your attention. These message

appear for only a second or two, since the desktop will wipe

them out, but there is plenty of time to view them when the

RAM Disk is installed from the AUTO folder.

###################################################################By##0#(_ _/