2.0 Example of circuit description

==============================

1  IS       0 TO 1  1 AMPS

0 DEG

2  L        1 TO 2  2 E-4 H

3  R        2 TO 3  .33 OHM

4  C        3 TO 0  2.2 E-4 F

5  R        2 TO 0  20 OHM

Note in the above example that circuit elements are described by their connec-

ting nodes, their values, and a branch or element number. It is this branch

number which is specified when requesting branch voltages, currents, or pow-

ers for output. You would also specify this number when deleting an element.

Due to the way the program orders itself, as you add independent current sour-

ces they are added to the top of the list. As you add any other elements they

are added to the bottom of the list.

CAP is devided into three sections a) Input of circuit, b) Selection of output

quantities and frequency sweep, and c) Plotting.

These sections are completely independent in that you can change any parameter

of any section and retain all other parameters.