Instruction Manual: MC_TALK4.ACC

================================

Table of Contents

-------------------

1. General

2. Booting

3. The Control Elements

a. Net Map

b. System Time

c. Printer Selection

d. Connection (?)

e. Telephone

f. Mailbox

g. "Peeking"

4. MC_TALK.INF

5. Procedures in case of program malfunctions

6. Processes under GEM

7. Epilogue

1. General:

-----------

MCTalk is a utility written for MIDI_COM.ACC. The majority of

programmable functions of MidiCom can be used with help from MCTalk.

Requirements for proper functioning of MidiCom and therefore the

useage of MCTalk: all computers to be used in the network must be

connected to each other in a "Ring" via the MIDI ports; all net-worked

computers are booted with MIDI_COM.ACC.

The physical connection of the computers to be networked is

accomplished via normal cables (similar to those used with the

majority of stereo systems) in that one connects the net-worked

computers in a "Ring." That means the MIDI Out port of the first

computer is connected to the MIDI In port of the next computer and so

foth until the MIDI Out port of the last computer in the "Ring" is

finally connected to the MIDI In port of the first computer. According

to various public sources it should be possible to use cable lengths

up to 200 meters (656.2 ft.).

You can lookup more information in your Atari ST's "User's Manual"

regarding the cable connections (Appendix D: Output port

specifications).

MidiCom can connect up to seven computers.

2. Booting

----------

As already said, MIDI_COM.ACC must be booted on all computers

connected in the "Ring." MidiCom does not use menu entries and

controls all communication in the MIDI-Ring.

MCTalk is booted as a normal ACC. The file MC_TALK.INF can be

optionally located on the boot disk. The meaning of this file will be

explained later.

The menu entry MCTalk can be now selected after bootup. MCTalk will

then attempt to "register" itself with MidiCom. MCTalk will be

successful if MidiCom is active -- that is, when the MIDI-Ring is

complete. If MCTalk's first attempt to register itself with MidiCom

fails, it can be repeated. Otherwise a window with MCTalk's operation

elements appears on the screen. Closing this window will not lead to a

disconnection. MCTalk remains active.

3. The Control Elements

-----------------------

When the MCTalk window is open and active, all control elements can be

accessed via the mouse (double-click) as well as the keyboard.

For keyboard access, use the corresponding first letter with the ALT

key. As an example, ALT+N brings up on the screen the "Network Map."

3.a. Net Map (ALT+N)

--------------------

This function gives one an overview of all computers currently

part of the "Ring" outside of the computer one is working at. It

indicates which program is currently running under MidiCom (the

desktop means that there is no program currently running under

MidiCom), if a printer is connected to a particular computer and,

if MCTalk is active on a particular computer, the name of the

user as well as an arbitrary description of the connected

computer.

The computer numbers on the left side indicate the active

computers in the Ring. For example, numbers (1) and (2) would be

indicated if there were three computers connected. The order of

the computers is based on the cabling of the computers in the

Ring.

3.b. System Time <ALT+S>

------------------------

This option set the date and time of the computer. In addition

one can choose if the set date and time will be applied to all

computers in the Ring or locally; that means only on the computer

of the user.

3.c. Printer Selection <ALT+D>

------------------------------

This option can limit MidiCom's search for a free printer in the

net to just one or more. If >EGAL< is chosen all other settings

will be ineffective and MidiCom will then search for the first

free and print-ready printer. Otherwise MidiCom reaches a

selection only from the printers indicated by the user.

(Translation Note: The word "egal" means roughly "it doesn't

matter.")

The following functions are of significance if MCTalk is installed and

registered on at least one other computer in the Ring.

3.d. Registration <ALT+A>

-------------------------

Using the registration function, one can change, as well as

enter, the names under which one wants to appears to the other

MCTalk users. The name can contain up to eight charachters and

should not contain empty spaces or special characters. The

reasoning for this is found under point 3.f.

3.e. Telephone <ALT+T>

----------------------

This function allows a direct exchange of information with other

MCTalk users in the Ring. If this function is selected, the

program will go into telephone mode.

(In the event that no previous registration has yet occured, the

registration function will be called up.)

A number of telephone symbols will appear in the upper part of

the screen which allows one to control communications. A solid

telephone is displayed for all computers in the Ring that have

MCTalk registered. All other computers appear greyed out. The

small line of text beneath each Symbol indicates under which name

each user is registered.

The screen is divided into two halves. The upper half shows the

last 9 received lines while the lower half shows the last

messages sent. The lowest line serves as input of messages.

Each press of the RETURN key sends out a line of text to all

selected recepients. The same occurs when the maximum line length

of 67 characters is exceeded.

The only editing keys available are the Backspace, Delete and

Esc. The cursor keys are not available.

I. Calling/Selecting the Recepients

By double-clicking on one of the active telephone symbols one can

establish connection to the respective user. The telephone symbol

will then be displayed inverted. Double-clicking on an inverted

telephone symbol disconnects one from the respective user

represented by the inverted telephone symbol.

The call can also be accomplished via the function keys 1 to 6.

One can send out one or more lines. It is possible to

simultaneously send out a message to all currently logged-on

users. All computers whose telephone symbol is inverted will

receive the message regardless of whether or not they are in

telephone mode. One can also select any combination of recepients

to receive a particular message.

As soon as the "called" individual switches into telephone mode,

he will see the received text displayed (inverted) under the

appropriate telephone symbol. The sent message will always be

received by the intended individual(s) and will be seen by the

recepient as soon as he enters telephone mode. In this way can

one leave a message although the inteded recepient is at the

moment not ready to receive a message.

Telephone mode is exited by clicking on the close button of the

window. Other users in the ring will be automatically informed of

a user leaving telephone mode.

II. Receiving Messages

When a computer receives a new call, MCTalk responds with a short

"ring" symbol. There are various responses from MCTalk to an

incoming call, dependent on MCTalk's current operating status.

II.1. The MCTalk window is not active or does not even appear on

the screen.

Two small bell symbols will appear in the upper right

corner of the screen to indicate a call has come in. The

symbols will be erased when the MCTalk window is activated.

II.2. The MCTalk Window is active.

The telephone symbols will be displayed inverted. The

inversion remains as long as the caller remains in

telephone mode. If the caller "hangs up," the telephone

symbol of the recepient will be displayed normally.

II.3. MCTalk is already in telephone mode.

The text beneath the symbol of the called computer will be

displayed inverted. In addition, the caller will be

automatically notified that the called user/computer is

ready to receive a message.

Every received message is tagged with the name of the sender so

that the recepient can determine from whom the message

originated.

3.f. Mailbox <ALT+M>

-----------------------

The Mail System enables one to copy files from one computer to

another. This is subject to several limitations. In particular is

the .INF file which dictates where in-coming mail should be

written; the sender doesn't have any influence in this matter.

The sender also has no say in the naming of the Mail-Files.

In-coming mail is saved under the name of the sender (plus a

consecutive number) in the path determined by the .INF file. If

no path has been designated then receiving of mail will be

denied.

To send mail select the function "Mailbox." (The function

"Anmeldung" will be called up in the event MCTalk is currently

inactive. The program consists of the entry of, at the minimum,

one character.) The selection dialog appears allowing one to

select the file to be sent. Another dialog box appears allowing

the selection of the receiver to whom the mail should be sent. A

file can be sent to all users in one pass. While the mail is

being transferred, this dialog box remains on the screen. Upon

completion of the transfer process one can determine if the

transfer was successful or not via a message in the dialog box.

If, instead of the the computer name, an OK appears then all mail

was successfully transferred. A failure of the transfer process

is indicated by an ERROR message. A mail/file transfer failure

can be due to several causes. In general there are two major

possibilities. First, there is no mail path defined for the

receiver or, second, the storage media/mechanism of the receiver

has been changed so that the specific directory defined by the

mail path is no longer accessible.

3.g. "Peek" Function <ALT+K>

----------------------------

The "Peek" function allows you to import the screen display of

another computer to your own screen and, if desired, save it to

disk in STAD format. First, you determine, via the Net Map, the

computer whose screen you want to import. You can only select

computers which have MCTalk active.

Before the selected computer's screen is displayed on your own, a

message is displayed indicating the peek function can be left by

pressing the right mouse button. If you press the right mouse

button to exit Peek, MCTalk queries if you want the imported

screen saved.

Possible error messages:

a) Not enough memory (nicht genug Speicher):

Your computer must have a minimum of 32Kb free RAM to save the

imported screen.

b) Unable to read (kann nicht gelesen werden):

The "target computer" needs 64Kb free RAM for the pack

routine. If this amount of free RAM is not available or the

screen cannot be packed for whatever reason, you will see this

message.

4. MC_TALK.INF

--------------

This file is a simple ASCII file which you can edit with a text

editor. It contains two lines. The first line determines the name

MCTalk uses in identifying itself to other users. The line can

contain up to 8 characters maximum. Avoid using the "spacebar"

character and other special characters because the name used in

sending mail is also used as the file name. (See 3.d. and 3.f.).

The line can be empty as well.

The second line determines the path where in-coming mail will be

saved. If this line is empty or doesn't contain an acceptable

pathname, then no mail will be received. Once MCTalk is active

there is no possibility thereafter to enter a mail path or to

change a mail path. The pathname can be any desired name up to 68

characters in length. The pathname must have "\" at the end.

5. What to do in case of errors

-------------------------------

Unfortunately there is no program that is completely free of bugs.

Although I hate to admit it, that also applies to this program.

However, with several simultaneously running programs (for every

computer in the Ring, that means two more) it is difficult to

anticipate all possible problems. Since time-dependent errors are

often the case (that means something occurs in the same hundreths

of a second as another and triggers an error) they are very

difficult to localize. And when "foreign" programs come into play,

one can only hope that everything goes well.

Should a computer in the Ring suddenly crash all computers

accessing the Ring at the time of the crash will be "blocked." That's

because for every sent message MidiCom expects a reception

confirmation before any new incoming messages will be accepted. On

the other hand, MCTalk waits for every message that is to be sent

until these messages are received. A break in the Ring will cause

other MCTalk users in the Ring to remain in a wait state until the

crashed computer is rebooted. Thereafter everything should

generally function at once as before the crash occurred. It is

therefore possible at any point in time to reboot one or more

computers in the Ring.

6. Processes under GEM

----------------------

Because MidiCom operates in conjunction with an accessory, the Ring

has certain limitations. As is probably well-known, the operating

system initiates a process switchover only through the calling of an

AES routine. That means, for one, that on all computers in the Ring

only GEM progams may be executed otherwise MidiCom will not receive

a share of processor time and the Ring is broken.

All disk operations are problematic due to the same reason. While a

process initiates a disk operation, no switchovers can occur. This

is true for MidiCom as well as other programs. Normally MidiCom is

layed out in such a way that it poses as small a hinderance as

possible to other currently running programs. If a copy request is

initiated for example, reading or writing) the computer on which

the copy request occurs experiences a relatively strong hinderance.

This occurs because during the time MidiCom uses for reading or

writing, no other program on the affected computer can receive

processor time. Unfortunately disk operations are time consuming.

Therefore MidiCom was layed out in such a way that a copy request

would be completed as quickly as possible. That means a computer

from which a file is read is temporarily "claimed" by MidiCom. You

should therefore watchout for large amounts of data to read from a

computer in the Ring that is currently being used.

Disk operations on a computer in the Ring lead to impedement of all

other users of MCTalk because, in this case, for the duration of

the disk operation MidiCom doesn't receive any processor time and

messages cannot be passed on. For example, there can be a delay of

several minutes if a diskette is copied on a computer in the Ring.

Data, however, will not be lost.

7. Epilogue

-----------

For those who would like to develop programs that work with MidiCom

(whether it be user applications or games) you can order directly

from me a complete description of MidiCom. Programming with MidiCom

is not dependent on any one particular progamming language. A

complete library for ST-Pascal+ already exists and needs only to be

linked.

Translator's Note: Since I don't have the program myself, my

translation might be a little vague in some areas. If you would

like to make any corrections feel free to do so. But I request only

one thing. If you intend to circulate your updated version of this

doc via on-line services or giving copies to friends, please

indicate your name, address (snail-mail or e-mail), and the date of

your modification. I've probably already made some "mistakes" in

translation due to the above indicated reason. However, I don't

want to be blamed for someone else's goofs! Thanks!

Translated on 08 August 1994 by Lowe Llaguno (GEnie address

L.LLAGUNO) an American ExPat in Germany!