DigiUbik                                          (c) 1991 GHungerford

DigiUbik is a digital sound file editing tool. Digitized

sound samples may be merged, appended, packed or played.

Loading Buffers:

To load a buffer, click on the number of the buffer you want to load.

If the buffer is already loaded, you'll be given a choice of clearing

it out and continuing or of cancelling the request.

If you continue (or the buffer wasn't loaded), you will be shown a

file selector box. Select the file you want or cancel the request.

If you cancel the request, the buffer will remain empty. If you get

an 'Insufficient Ram' message or a file won't load, try clearing out

any loaded buffers this way to gain space.

If the file has the extension '.PAK', then it is assumed to be a packed

file and will be loaded accordingly. Packing is a way of storing digi-

tal sound files in one half the space.

Frequency:

Click directly on the Frequency to change it.

0   =  5   Khz

1   =  7.5 Khz

2   = 10   Khz

3   = 15   Khz

4   = 20   Khz

5   = 31   Khz

6   = 40   Khz (requires cartridge sound output)

7   = 50   Khz (            ditto              )

UpDownButton:

(The arrow in the box to the right of the Target Buffer display.)

Clicking on this button will toggle the arrow up and down.

Initially the arrow is pointing up, indicating that the numbered

buttons (like the Frequency button above) will increase when

clicked on. Likewise, if the UpDownButton is set to a down arrow,

the numbers will decrease when clicked on.

Target Buffer:

The 'current' buffer to be played, saved or packed - the destination

buffer of merged and appended buffers. You will be notified if you

are about to overwrite a currently loaded buffer. Click directly on

the Target Buffer number to change it.

Sources:

The source buffers that are to be merged or appended. Both source

buffers may be the same (for appending a buffer to itself).

Merge Percent:

These two percentages relate to the source buffers. A 50/50 setting

will merge the two sources equally. If the volume of one source

is too low or one source needs to be emphasized, increase the Merge

Percent for that source buffer.

Play:

If the current Target Buffer is loaded then that buffer will be played.

Append:

Joins the current source buffers and places the result in the current

Target Buffer.

Pack:

Packs the current Target Buffer to a disk file. The sound quality

is lessened, but the file size is one half of the original. Packed

files MUST have the '.PAK' extension to load back properly. Packing

is accomplished by trashing the lower nybble of each byte in the

sample.

Save:

Saves the current Target Buffer to disk.

Merge:

Mixes the sounds from the source buffers and places the result

in the current Target Buffer.

NOTES:

Since sound files are just a bunch of bytes, this program

can also be used to quickly append ANY kind of file to ANY

kind of file.

Packing ONLY makes sense for sound files and will trash any

other type of data.

Although only proper sound data files normally make sense,

any file may be listened to. Usually this equates to a lot

of noise, but some programs (games) have sounds hidden in them...

SHAREWARE:

This program may be freely distributed as long as this doc

file (DIGIUBIK.DOC) is included. If you use this program and

wish to encourage more programs of this nature, please send

a $5.00 donation to :

G. Hungerford

1718 Ocala Ave. #117

San Jose, Ca. 95122

Developed using TDI Modula-2/ST