MDF.EXE  --  MIDI DATA FILER ver 2.3, Apr 28 1991 by Kai Risku

===============================================================

This is a very simple and easy-to-use program that I use to

store sysex-dumps from my synthesizer in files on my PC.

The program requires a MPU-compatible MIDI-interface at the

default address (0330h), no IRQ is used.

The program was made with Borland's Turbo C 2.0.

The program MDF.EXE takes one filename with optional

path as a command-line argument and if the filename

has no extension, .SYX will be added.

How to save MIDI-data to a file:

================================

You just start MDF with a non-existent filename as argument.

MDF will then create a file with that name (and add SYX to

the name if there isn't one already). When MDF reports

'Ready to receive' you start a sysex-dump manually from your

synthesizer. After the dump is done, MDF will report how

many bytes that have been received, save those in the file

and exit automatically.

How to send the data back to the synthesizer:

=============================================

Oh, you have already guessed that one, but I just want to say

it anyway. Run MDF with the name of the file you want to send

to your synthesizer as the argument. If the filename has the

extension .SYX, you don't have to write it out, as the program

will add it automatically. MDF will send the contents of

the file and inform you of how many bytes that has been sent.

Configuring MDF using the MDF.INI file:

=======================================

If you have a fast computer, you can encounter problems because

the data is sent out too fast for your synthesizer. To cure this

MDF has options to let you slow down the output. These options

are found in the file name MDF.INI and can be edited to your

needs with an ordinary ASCII editor. When you start MDF, the

program will look for the INI file in the same place as MDF.EXE

resides. A default INI file is created if it cannot be found.

The options in the file are of the syntax Option=value and every

option must be on a line of its own. At this moment there are

two options available and these both control the output speed when

sending sysex-data. The options are named Delay1 and Delay2

respectively. Delay1 is controlling the delay between each byte

being sent, and Delay2 specifies the delay between the sysex-messages.

Current limitations:

====================

- The maximum size of a single dump is limited to 64 kbytes.

If you have a need for more than that, then contact me

and I'll happily make an improved version.

- There can be some problems when receiving a dump if the

synthesizer dumps data too fast. You should therefore keep

an eye on the size of the received data as this will be

too small if the buffer has flooded.

- You cannot use handshake-dumps with this program, because

practically every synth has its own format for this.

You can only use transfers without handshaking and therefore

the dumps must always be manually started from the synth.

- If you for some reason have changed the address of the

MIDI-interface to anything else than 0330h, then you can't

use this program at all. Sorry.

Revisions:

==========

Changes from version 2.2:

Added an INI-file for user-configurable delays while

sending. Modified text output and removed some

unnecessary texts.

Changes from version 2.1:

More code cleanup. Incoming data during sysex-send

to synthesizer will be sent back out when possible.

Error-handling improved.

Changes from version 2.0:

The default extension MDF for files created was changed

to SYX for compatibility with other software.

Active sensing has earlier caused some trouble, but

this has now been fixed as well as a small bug causing

a time-out if you don't start the dump fast enough

after you started the program.

Changes from version 1.0:

Document added (this piece of text). Improved routines

with faster code. Corrected some minor bugs.

Credits and more:

=================

This program is completely in the public domain. You can distribute

it to all your friends as long as you don't modify this program

in any way without permission from the author. If you really think

this is a useful program and you have a Roland D-5/10/20/110

then you could always mail me some great sounds (I have a D-110).

If you have any questions, ideas, suggestions for improvements

or if you are interested in the source then feel free to

contact me...

Snailmail:      Kai Risku         Email (internet):  Kai.Risku@hut.fi

Kronvik                         or   krisku@vipunen.hut.fi

65410 Sundom

Finland