Combat Arms

2869 Grove Way

Castro Valley, California 94546-6709

Telephone (415) 538-6544

BBS Phone: (415) 537-1777

COLT DETECTIVE SPECIAL

Description of Operation

1.   Pulling the trigger in double action mode

a.   The rearward pull of the trigger causes the trigger to pivot

about a stud raising the rear bevel of the trigger. The hand is

attached to the trigger's rear area and raises with the trigger.

b.   The rising hand presses against the forward underside of the

rebound lever and causes it to begin to rise also.

c.   The right middle side of the rebound lever (tang) raises the rear

of the bolt and the front of the bolt begins to lower as its

pivots around its bolt screw attached to the right side of the

frame.

d.   The top middle portion of the rebound lever presses against the

lower spring leaf. This starts compressing the spring.

e.   The back of the trigger pushes upwards on the strut (Colt's name

for the sear), which causes the hammer to pivot rearward.

f.   The rearward motion of the hammer pulls down the upper spring

leaf. The upper spring leaf is attached to the hammer's stirrup.

g.   The safety lever (Colt's name for the hammer block) is pulled

downward by the articulating connecting link to the rear of the

trigger. This causes the safety lever to clear the firing pin

hole in the frame in front of the hammer.

h.   When the hammer has traveled about half way back, the rear of the

bolt slips off the tang of the rebound lever, raising the front

end of the bolt and locking the cylinder.

2.   The trigger bevel clears the sear.

a.   The upper spring leaf pulls upwards on the hammer's stirrup,

pivoting the hammer forward.

b.   The firing pin attached to the front of the hammer clears the

firing pin hole in the frame and strikes the primer, discharging

the cartridge.