M1 Garand Compensator

Staff Evaluation

IF you own an M1 Garand you are aware of the strong recoil  these

rifles can generate.  Western Ordnance, a firm which  specializes

in  high  quality after market accessories for  guns  and  custom

rebuilt firearms, has developed a new compensator for these rites

which  installs quickly with only a wide blade flat  head  screw#

driver and can significantly tame the strong kick and muzzle rise

generated by firing these big guns. Similar compensators are also

available for the AR-15/M16, Galils (both calibers), AR-180,  Dae

Woo and AK-47 rites as well as custom made compensators for  such

rifles as the Mini-14. We tried out one of the MI Garand  compen#

sators  sent to us by Western Ordnance on our test  Federal  Ord#

nance Tanker Garand in 7.62 NATO caliber.

The  compensator  is a 2 1/2-inch-long cylinder made  of  solid

steel bar stock and parkerized. It has six small holes on top and

two  more,  behind the first six on top, which  face  toward  the

muzzle. It's designed to aid in rapid target reacquisition  with#

out altering the rifle.

All  that's needed to install the compensator is a  wide  blade

flat head screwdriver. You might also want a bore brush to  clean

the  threads of the gas cylinder and around the muzzle which  the

compensator fits onto.

To  install you unscrew the gas cylinder lock screw and  remove

gas cylinder lock and clean of the threading if needed. Then  the

compensator is screwed onto the barrel and the gas cylinder  lock

screw is tightened down to hold it in place. That's all there  is

to it.

The  smaller,  lighter  Tanker Garand  model  has  considerable

recoil and muzzle climb in either 7.62 NATO (.308 Win.) or  30.06

calibers.  After firing it without the compensator, we  then  in#

stalled  the  compensator and noticed a pronounced  reduction  in

felt recoil and reduced muzzle flip. While there is still a  fair

amount  of  both kick and muzzle climb, it's  definitely  not  as

strong as without the compensator.

We  will  shortly be trying out the compensator on  a  recondi#

tioned  Fed Ord G.I. Garand in 30.06 caliber and will report  the

result.  But we're satisfied this great little device works  well

from our test with the Tanker model.

The  Western Ordnance MI Garand Compensator is $59.95  postpaid

(dealer discounts offered). The other compensators range in price

from  $25  to  $35 for custom made types. For  ordering  or  more

information contact Western Ordnance, Dept. ASG, 325 South  West#

wood, FL, Mesa, AZ 85210; (602) 964-1799.

(This article was optically scanned from :

American Survival Guide / January 1992

Subscription Information

American Survival Guide

Subscription Dept.

2145 W. La Palma Ave

Anaheim, CA 92801-1785