Combat Arms

2869 Grove Way

Castro Valley, California 94546-6709

Telephone (415) 538-6544

����������������������������������������ͻ

�                                        �

�       CLEANING THE RUGER MINI-14       �

�                                        �

����������������������������������������ͼ

1.   Gather together the proper sized cleaning rod with a .223/5.56mm brass

or bronze bristle bore brush and a slotted tip into which a cleaning

patch can be inserted. Also needed are clean cloth patches and clean

soft cloths.

2.   Do not use stainless steel brushes on the weapon. Stainless steel

brushes will remove the finish on the Ruger Mini-14 rifle. A bronze or

brass "toothbrush" is especially helpful for scrubbing parts. Do not

use the bore brush for scrubbing anything except the barrel.

3.   Combat Arms and the United States military recommend that you use

Break-Free CLP(TM). Under no circumstances should you ever use

WD-40(TM) on any weapon! If you need a solvent, Combat Arms recommends

that you use Shooter's Choice(TM) followed by CLP. The Ruger Mini-14

should be cleaned at least every 400 rounds (if the firing period is a

long one) and after each firing period.

4.   Using a cleaning rod with a slotted tip, run a CLP wetted patch

through the full length of the barrel several times to remove loose

material. Then attach a bronze or brass bristle bore brush to the

cleaning rod and wet it in CLP. Run the bore brush back and forth

through the full length of the barrel at least a dozen times. Make

sure that the rod is pushed all the way through the barrel until the

entire bore brush clears the muzzle end. Do not reverse the direction

of the brush until it has completely exited from the muzzle end of the

barrel or the bore brush may bind up inside the barrel. If there is

metal fouling in the bore, remove it completely because metal fouling

(lead or copper) can destroy the accuracy of the weapon. Please note

that Hoppe's No. 9(TM) does not remove metal fouling as well as

Shooter's Choice, in the opinion of many benchrest shooters. Combat

Arms shares that opinion and recommends Shooter's Choice. If you want

to remove copper fouling the fastest way, Combat Arms recommends that

an uncontaminated bottle of Hoppe's No. 9 (meaning it has never been

used before) be mixed in a 50-50 solution with 28% Aqua Ammonia. This

is chemically known as AMMONIUM HYDROXIDE - 26�on the Baume' scale.

This is potent stuff and is concentrated ammonia. It is three times

more powerful than household ammonia. Do not use this inside a closed

room! When poured down the barrel from the breech end (not the muzzle

end!), the copper will be dissolved and fall out. It takes about 15

minutes.

5.   Using the toothbrush soaked in CLP, scrub the area around the chamber

until it is clean. Remove the bore brush from the cleaning rod and

attach the slotted tip again. Run several dry patches through the

barrel until they come out clean. If necessary, attach the bore brush

to the cleaning rod again and scrub the barrel with a CLP soaked bore

brush. Continue the process until the barrel is clean.

6.   Failing to do this type of cleaning consistently and properly will

cause fouling of the barrel with a natural loss of accuracy. Grease

accumulation in the chamber area can interfere with the proper feeding

of cartridges from the magazine.

7.   Using a loose clean patch or soft rag soaked with CLP, remove all

powder residue from all components of the mechanism. If grease or dirt

cannot be removed by merely wiping with the patch, then use your brass

toothbrush soaked with CLP. After using the brass toothbrush, wipe all

of the scrubbed areas with a clean CLP wetted patch. After cleaning

the bore, run a clean dry patch through the barrel and then follow it

with a patch that has a light coat of CLP on it. This will put a light

coat of CLP on the inside of the barrel and protect it from corrosion.

8.   Wipe all surfaces with a clean cloth that has a light coat of CLP on

it. If the lower receiver and its mechanism is especially dirty, flood

it with CLP, leave it sit soaking for 2 hours and then flush it out

with more CLP. Remove the grip panels before soaking or flushing the

rifle and make certain that all foreign matter is removed after

cleaning. Lightly lubricate the lower receiver with CLP.

9.   When cleaning the bolt face, breech areas, etc. with the toothbrush,

work carefully so as not to damage components.

10.  Do not clean the gas port on the Mini-14. It is self cleaning. Also

note that Chris Peters, in charge of Ruger's armorer training program,

recommends that the gas pipe (piston) and the piston hole in the slide

be thoroughly cleaned. Failure to thoroughly clean these areas may

result in an accumulation of powder residue and moisture, which can

result in a seizing of the gas piston to the slide.

11.  Reassemble the rifle as detailed later in this manual. If the Mini-14

rifle is to be stored for an extended period, the final wiping should

be done with a heavier coat of CLP.

12.  Clean the internal and external parts of the disassembled magazine

with CLP. The inside of the magazine can be cleaned with a soft rag or

patch soaked in CLP. Remove all loose material with several clean

patches, just like you did with the cleaning of the barrel. Leave a

very light coat of CLP on the inside of the magazine and on the

magazine spring.

�����������������������������������������������������������������������ͻ

�                    - MAGAZINE CLEANING CAUTION -                      �

�                                                                       �

�   Be careful not to scratch the interior and exterior of the          �

�   Mini-14 magazine, particularly along the front section where the    �

�   bullet tips ride. Be very careful not to bend the magazine lips.    �

�                                                                       �

�����������������������������������������������������������������������ͼ

13.  Only a very small amount of CLP is needed to provide adequate

lubrication of all moving parts in the Ruger Mini-14 and to prevent

rust. Accumulation of CLP can attract particles of dust and dirt which

can interfere with the safe and reliable function of the rifle. Do

not, therefore, apply excess CLP unless storing the rifle for an

extended period of time. If storing the weapon and using a heavy

coating of CLP, the weapon should be disassembled and re-cleaned and

lubricated with a light coating of CLP before firing.

������������������������������������������������������������������������ͻ

�                           - STORAGE WARNING -                          �

�                                                                        �

�    If the rifle has been stored, before firing it again, completely    �

�    disassemble it as detailed elsewhere in this manual, remove all     �

�    grease and lubricant, apply a light coating of CLP to all parts     �

�    as described in the section on cleaning and be sure to run a        �

�    dry patch through the barrel after cleaning with a bore brush       �

�    soaked in CLP. Do not keep the rifle stored in a leather case       �

�    or carrier. Leather attracts moisture, even though the carrier or   �

�    case may appear to be perfectly dry. Never store the Ruger Mini-14  �

�    rifle in such a manner as it may be dislodged. Always store the     �

�    rifle securely and unloaded.                                        �

�                                                                        �

������������������������������������������������������������������������ͼ

������������������������������������������������������������������������ͻ

�                       - LUBRICATION WARNING -                          �

�                                                                        �

�   Firing the rifle with oil, grease or any other material even         �

�   partially obstructing the barrel may result in damage to the         �

�   rifle and death or injury to the user and those persons nearby.      �

�   Never spray or apply any lubricants directly to the ammunition.      �

�   If the powder charge of a cartridge is affected by the lubricant,    �

�   it is possible that the charge will not ignite but the energy        �

�   from the primer in the cartridge case may be sufficient to push      �

�   the bullet into the barrel where it may be lodged. Firing a          �

�   subsequent bullet into the obstructed barrel may damage the rifle    �

�   and cause death or injury to the user and persons nearby. Use all    �

�   lubricants properly and according to the lubricant manufacturer's    �

�   suggestions and recommendations.                                     �

�                                                                        �

������������������������������������������������������������������������ͼ

- Acknowledgment of Trademarks -

Combat Arms wishes to acknowledge the trademarks of the following

products along with their respective companies.

Mini-14                       Sturm, Ruger and Co. Inc.

Break-Free, CLP               San-Bar Corp.

WD-40                         WD-40 Company

Shooter's Choice              Venco Industries Inc.

Gun Scrubber                  Birchwood Laboratories, Inc.

Hoppe's No. 9                 Penguin Industries Inc.