%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%%                                %%

%%        ================        %%

%%        RIFLE MICROPHONE        %%

%%        ================        %%

%%                                %%

%%       Another file from:       %%

%%                                %%

%%    ------------------------    %%

%%    Omnipotent, Incorporated    %%

%%    ------------------------    %%

%%                                %%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

This is a most unusual project;  we are going to mount a sensitive

microphone behind a series of acoustic wave guides and  hope  for  the

best.  Actually, our wave guides are aluminum tubes cut to resonate at

frequencies  near the human voice range.  This arrangement also serves

to make the unit highly directional as sound other than that  arriving

at  the  mouth  of  the  tubes  arrives  at the mic in an out-of-phase

condition and cancels itself out.

With this easily constructed device it is possible  (under  ideal

conditions)  to  pick  up conversations several HUNDRED feet away,  in

fact, reports of recording conversations up to 30 yards away THROUGH A

CLOSED WINDOW have been made...

The device is quite easy to construct:  First  get  together  the

following materials:

[1]  56' of 3/8" outside diameter

aluminum tubing (hardware stores)

[2]  1 sensitive amplifier with

operating paraphenalia

[3]  1 sensitive microphone element -

use either a good condenser or a

crystal, as their outputs tend to

be a bit higher than their dynamic

counterparts.

[4]  1 funnel

[5]  Assorted small hardware

Cut  the tubing into 37 pieces ranging from 1" through 36" in one

inch divisions.  Now bundle them together symmetrically with  one  end

flush (as shown).  As you assemble them, blue them together in several

places  to  assure they remain in the state you have put them in (were

it only so easy with people...).

OOO

OOOOO

OOOOOOO

OOOOOOOOO

OOOOOOOOO

OOOOOOOOO

OOOOOOO

OOOOO

OOO

Flush ends of tubes

Take  the  funnel  and  fit  it  around the flush end of the tube

bundle.  Being astute enough to read this,  I am sure you will  notice

the  tubes  do  not exactly fit into the funnel.  At this realization,

take a small hammer and beatt the funnel to nearly  as  possible.  Now

fill the remaining spaces with a good rubber based caulking compound.

The  microphone element is placed in the neck of the funnel (with

a rubber grommet if it is small enough to  fit  up  into  the  neck  -

otherwise  set  it  into the other end of the neck before caulking the

tubing in place).  Glue the mic into place and  caulk  any  air  space

around it.

Run  the  microphone  wires  out the neck of the funnel,  glue in

place, and also caulk to give it a closed,  fairly air invincible seal

around the entire unti.  If you really feel up to it, fiberglass resin

can be used instead of caulking compound.

M    ###\

I    ### \

C    ###  \

###   \_________#  wire

E    ###             #______

L => ###    funnel   #

E    ###    _________#

M    ###   /         ^

E    ###  /       grommet

N    ### /

T    ###/

Run  the output wires to your amplifier and place the entire unit

on some sort of support,  a camera tripod being ideal.  (Need I remind

you  to  check matching impedances?).  The unit should be aimed at the

subject and then panned about  for  maximum  volume  (sound  does  not

travel in a straight line, but may vary with wind, etc).

If  wind  noise overrides your target noise you can place a piece

of  cheesecloth  across  the  end  of  the  unit.  If  a  particularly

difficult noise interferes with your pick-up you can often plug up the

tube(s) carrying this noise with a cork.

Good hunting,  avoid paranoid people who may feel your rifle mic

more than resembles the real thing...

Coming soon from Omnipotent:

=-=-=-=-=-=-=-=-=-=-=-=-=-=

THE BLITZ BOX!